# 3GPP TSG-RAN WG4 Meeting # 102-e						R4-2205531

# Electronic Meeting, 21st  Feb –3rd March, 2022

Source:		Ericsson

Title:			Simulation results updates for phase tolerance for PUSCH and PUCCH repetition

Agenda item:		10.18.1

Document for:		Approval

# Introduction

In WF[1], there are some new updates related to the phase and power consistency tolerance simulation:

## Issue 1-1: Model of phase variation

Candidate options:

•	Phase offset Option 1: for each individual slot k (k=1…n) within the bundle, an independent offset is generated and applied with respect to the slot 0.

•	Phase offset Option 2: for each individual slot k (k=1…n) within the bundle, an independent offset is generated and applied with respect to the slot k-1. (i.e., the offset is allowed to accumulate)

WF recommendation:

•	Discuss the phase tolerance in Issue 1-2 directly.

## Issue 1-2: Phase continuity tolerance

WF recommendation:

- Down select between the following two options:

  - Option 1: Adopt [-30, 30] degrees if Phase offset Option 1 in Issue 1-1 is agreed.

  - Option 2: Adopt [-15, 15] degrees if Phase offset Option 2 in Issue 1-1 is agreed.

In this paper, we provide the updated simulation results with the more repetition number.

# Discussion

## Scenario parameters

Table 1: Basic setup of LLS for joint channel estimation on PUSCH, for FDDat 700 MHz

| System | - Carrier frequency 700MHz - 15 kHz SCS - FDD - 52PRBs BWP size |
| --- | --- |
| UE speed | - 3 km/h |
| Payload / tx scheme | - MCS 4 unless otherwise stated, 4 PRBs, 14 symbols - 2 DMRS symbols per UL slot unless otherwise explicitly stated - 32 (actual) repetitions, no re-transmissions - No frequency hopping |
| Channel | - TDL-C (NLoS), 30 ns delay spread, medium correlation |
| Impairments | - 0.10 ppm CFO (70 Hz) - Phase offsets between slots as/if indicated in respective section - No time offset errors between slots |
| Antennas | - 1T2R |
| Receiver | - Practical (delay spread, CFO, etc not known to receiver), except CFO known where explicitly indicated in Section. |

## Phase error and power error tolerance for FR1

The simulation results are updated to investigate new phase offset modelling on the JCE performance.

Figure 1 shows the performance comparison between phase offset model of option 1 and option 2 with the same phase offset range with 32 repetition.  It can be seen that phase model option 2 is performing worse than phase model option 1 for [-40, 40].

Phase model of option 2 has worse JCE performance compared with phase offset modeling of option 1 for the phase offset range.

- :

Figure 1: BLER performance for JCE and non-JCE with added phase offset modelling at FR1 (option 1 and option 2 with different ranges)

# Conclusions-

In this contribution, the updated link level simulation assumption is discussed for phase discontinuity tolerance study on JCE with below proposal:

- Phase model of option 2 has worse JCE performance compared with phase offset modeling of option 1 for the phase offset range.

# References

- R4-2202418,	WF on phase continuity and power consistency for PUCCH and PUSCH transmissions, Huawei