3GPP TSG-WG4 Meeting #102-e	 R4-2205586

# Electronic Meeting, Feb 21- Mar 03, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | <Rev#> | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for TS 38.101-1: On time mask for SL intra-band con-current operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduce time mask for switching between Uu and SL for intra-band con-current operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Time mask for both same carrier and different carrier case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for switching between Uu and SL are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3E.3.4, 6.3E.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change>

#### 6.3E.3.4	Transmit ON/OFF time mask for V2X inter-band con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.3.3 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.3 shall apply for the sidelink in licensed band or Band n47.

#### 6.3E.3.4	Transmit ON/OFF time mask for V2X intra-band con-current operation

For the intra-band con-current NR V2X operation, the switching time mask requirements specified in this clause are applied for same carrier and different carriers respectively.

Requriements in Figure 6.3E.3.5-1 and 6.3E.3.5-2 are for same carrier case.

Figure 6.3E.3.5-1: Time mask for same carrier where switching occurs in NR SL slot

Figure 6.3E.3.5-2: Time mask for same carrier where switching occurs in NR UL slot

Requriements in Figure 6.3E.3.5-3 and 6.3E.3.5-4 are for different carrier case.

Figure 6.3E.3.5-3: Time mask for different carriers where switching occurs in NR SL slot

Figure 6.3E.3.5-4: Time mask for different carriers where switching occurs in NR UL slot

## <End of Change>