3GPP TSG-RAN WG4 Meeting # 102-e 	                                              R4-2205654

Electronic Meeting, February 21st – March 3rd, 2022

Title: 	Selection of UE duplexer and REFSENS for band n256 in TS 38.101-5

Source: 	Hughes/EchoStar, Skyworks

Agenda item:	10.13.4

Document for:	Discussion and Approval

# Introduction

This contribution provides the rationale for the selection of UE duplexer bandwidth, and corresponding reference sensitivity for band n256 in document TS 38.101-5.

# Reference

[1] R4-2200331

[2] R4-2200074

[3] R4-2200332

[4] R4-2203113, “Email discussion summary for [101-bis-e][308] NTN_Solutions_Part3”

[5] TS 38.101-1 V17.4.0 2021-12-01

# Rationale

- Feedback from front-end (FE) vendors indicate that re-using the 90 MHz band n65 duplexer for band n256 would lead to simplification and ease of implementation of the NTN UE receiver [3], [4].

- As Skyworks (a duplexer manufacturer) noted at the end of round 1 [4], “for UEs to support efficiently, in terms of size and cost, multiple bands and multiple band combinations, it is essential to reuse the [mature] RF front end HW as much as possible. As the n65 duplexer is mature there is little performance optimisation to gain by using a narrower filter. In any case a UE implementing an n65 duplexer shall be able to support band n256”.

- As MediaTek reported at the end of round 1 [4], they received feedback from evaluation of a new 30MHz duplexer by additional FE vendor, indicating that rejection performance related to band n34 would be broadly the same for the 30MHz duplexer and 90MHz duplexer (only a few dB difference), and the improvement of insertion loss will not be 0.5 dB.

- From the flexibility and service continuity between NTN service (in band n256) and CGC service (in band n65) it may be preferred for both services to use the same duplexer.

- A selection of the 90MHz duplexer for band n256 would be compatible with the REFSENS specifications of band n65 [1], [2], [4]. See document TS 38.101-1, Table 7.3.2-1a [5].

# Conclusion

Based on the above rationale in section3, please consider the following proposals:

Proposal 1: Reuse the 90MHz band n65 duplexer for UE operation in band n256

Proposal 2: Define REFSENS for band n256 based on Table 7.3.2-1a, band n65 in [5] (provided below as reference):

Table 7.3.2-1a: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n65 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |