3GPP TSG-RAN WG4 Meeting #102-e	R4-2205697

Electronic Meeting, 21 February – 03 March 2022

Source:	Ericsson, BT plc

Title:	TP to TR 37.717-11-21: Addition of DC configurations for DC_20_n3-n67

Agenda item:	9.19.2

Document for:	Approval

# Background

This text proposal for TR 37.717-11-21 to add DC_20A_n3A-n67A configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_20_n3-n67

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n3-n67 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n3 | 1805 MHz | – | 1880 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n67 | N/A |  |  | 738 MHz |  | 758 MHz | SDL |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n3A-n67A | DC_20A_n3A | 20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n67 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

Based on co-existence studies there is IMD4 impact from UL 20_n3 into DL n67.

### 6.x.4	∆TIB and ∆RIB values

For DC_20_n3-n67, the TIB,c and RIB,c values are derived from DC_3-20_n28 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n3-n67 | 20 | 0.5 |
|  | n3 | 0.3 |
|  | n67 | 0.5 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n3-n67 | 20 | 0.1 |
|  | n3 | 0 |
|  | n67 | 0.1 |

### 6.x.5	MSD

As can be seen in the co-existence studies there is a need to define MSD. MSD value from R4-2200794.

Table 6.x.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_20_n3-n67 | 20 | 837 | 5 | 25 | 796 | N/A | FDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | FDD | N/A |
|  | n67 | N/A | 5 | 25 | 746 | 9.4 | FDD | IMD4 |

##### ---End of changes---

# Reference

[1]		RP-212876, Revised WID on DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) in Rel-17, LG Electronics