3GPP TSG-RAN WG4 Meeting #102-e	R4-2205764

# Electronic Meeting, 21st Feb – 3rd Mar, 2022

Title: 	Discussion on satellite NTN demod PUSCH

Source: 	Huawei, HiSilicon

Agenda item:	10.13.6.2.1

Document for:	Discussion

Background

During RAN#101bis-e meeting, WF [1] on NTN satellite demodulation was approved. In this contribution, we share our views about NTN satellite PUSCH demodulation requirements.

Discussion

PUSCH

PUSCH requirements

- Define the following requirements and further discuss on Tx/Rx configurations - Transform precoding disabled - Transform precoding enabled - UL timing adjustment - repetition Type A - Further discuss whether to define the following requirements - Option 1: 2-step RA type requirement - Option 2: Mapping Type B with non-slot transmission requirements

Higher latency and lower SNR point are expected for NTN deployment comparing to the legacy TN network. Considering typical NTN channel and deployment, we propose to not consider URLLC scenario, so we propose to not consider mapping Type B with non-slot transmission requirements.

In addition, we think 2-step RA type is benefit for NTN scenario since there is very small frequency offset and delay offset after UE pre-compensation. So we propose to define requirements for 2-step RA type requirement.

For NTN PUSCH, do not define requirements for mapping Type B with non-slot transmission requirements.

For NTN PUSCH, define requirements for 2-step RA type requirement.

SCS/CBW set for PUSCH requirements

- Consider both 15kHz and 30kHz SCS - FFS on CBW and test metric - Option 1: 15kHz SCS: 5/10/20MHz, 30kHz SCS: 10/20MHz - Option 2: FFS based on the link budget

As per RF agreements, two new NTN satellite bands n256/n255 is introduced with {5, 10, 15, 20}MHz bandwidth configuration for 15kHz SCS and {10, 15, 20}MHz bandwidth configuration for 30kHz/60kHz SCS. For the normal BS performance requirements, only 5MHz, 10MHz and 20MHz bandwidth configuration case is defined for 15kHz SCS while 10MHz and 20MHz bandwidth configuration case is defined for 30kHz SCS. For NTN satellite PUSCH performance requirements definition, we propose to use same typical bandwidth and SCS configuration as normal TN BS performance requirements.

For NTN satellite PUSCH performance requirements, select 5MHz, 10MHz and 20MHz bandwidth for 15kHz SCS while 10MHz and 20MHz bandwidth for 30kHz SCS.

Modulation order for PUSCH requirements

- Introduce QPSK for PUSCH requirements - FFS whether to introduce 16QAM and 64QAM for PUSCH requirements - Companies are encouraged to further check if it is feasible for requirement

Here we show the LLS evaluation results for QPSK and 16QAM as Figure 2.1.3-1 and Table 2.1.3-1 and link budget evaluation based on TR 38.863 as Table 2.1.3-2.

Figure 2.1.3-1 Evaluation results for NTN PUSCH

Table 2.1.3-1 Evaluation results for NTN PUSCH

|  |  |  | NTN-TDLA100-200 | NTN-TDLB300-200 | NTN-TDLC100-200 | NTN-TDLD300-200 |
| --- | --- | --- | --- | --- | --- | --- |
| 2Rx | Rank1 | MCS2 | -4.76 | -4.50 | -4.11 | -4.09 |
|  |  | MCS16 | 8.83 | 9.04 | 9.13 | 9.23 |
|  | Rank2 | MCS2 | -0.82 | -0.59 | -0.47 | -0.43 |
|  |  | MCS16 | 19.25 | 20.03 | 22.03 | 21.51 |
| 4Rx | Rank1 | MCS2 | -7.81 | -7.60 | -7.53 | -7.48 |
|  |  | MCS16 | 5.04 | 5.23 | 4.98 | 5.14 |
|  | Rank2 | MCS2 | -4.21 | -4.12 | -4.00 | -4.07 |
|  |  | MCS16 | 10.20 | 10.27 | 10.42 | 10.25 |
| 8Rx | Rank1 | MCS2 | -10.38 | -10.30 | -10.25 | -10.26 |
|  |  | MCS16 | 2.37 | 2.40 | 2.32 | 2.34 |
|  | Rank2 | MCS2 | -6.95 | -6.96 | -6.86 | -6.91 |
|  |  | MCS16 | 5.86 | 5.96 | 5.91 | 6.05 |

Table 2.1.3-2 Link budget for NTN PUSCH

|  | LEO600-rural | LEO600-urban | LEO1200-rural | LEO1200-urban | GEO-rural | GEO-urban | LEO600-rural | LEO600-urban | LEO1200-rural | LEO1200-urban | GEO-rural | GEO-urban |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE Max Tx power (dBm) | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| Coupling loss 50%-tile | 125.73 | 125.54 | 131.49 | 131.39 | 139.94 | 140 | 125.73 | 125.54 | 131.49 | 131.39 | 139.94 | 140 |
| Noise figure for NTN satellite (dB) | 4.3 | 4.3 | 4.3 | 4.3 | 7.4 | 7.4 | 4.3 | 4.3 | 4.3 | 4.3 | 7.4 | 7.4 |
| SCS (kHz) | 15 | 15 | 15 | 15 | 15 | 15 | 30 | 30 | 30 | 30 | 30 | 30 |
| Thermal Noise | -103.17 | -103.17 | -103.17 | -103.17 | -100.07 | -100.07 | -103.72 | -103.72 | -103.72 | -103.72 | -100.62 | -100.62 |
| SNR (UL 5MHz) | 0.44 | 0.63 | -5.32 | -5.22 | -16.87 | -16.93 | 0.99 | 1.18 | -4.77 | -4.67 | -16.32 | -16.38 |

We can see that even with 5MHz bandwidth is configured, it is still not feasible for MCS 16 for both rank1 and rank2 with 8Rx that is the highest number of receiving antenna configuration during the test. Therefore, we propose to only consider QPSK for NTN PUSCH requirements for NTN PUSCH requirements definition.

Only consider QPSK for NTN PUSCH requirements for NTN PUSCH requirements definition.

Proposals

In this contribution, we discuss on satellite PUSCH demodulation requirements for NTN. Our observations and proposals are:

- For NTN PUSCH, do not define requirements for mapping Type B with non-slot transmission requirements.

For NTN PUSCH, define requirements for 2-step RA type requirement.

For NTN satellite PUSCH performance requirements, select 5MHz, 10MHz and 20MHz bandwidth for 15kHz SCS while 10MHz and 20MHz bandwidth for 30kHz SCS.

Only consider QPSK for NTN PUSCH requirements for NTN PUSCH requirements definition.

Reference

R4-2203043, WF on NTN SAN demodulation requirements, RAN4#101bis-e, Huawei, HiSilicon