3GPP TSG-RAN WG4 Meeting # 102-e	R4-2xxxxxxx

# Electronic Meeting, Feb.21st  –Mar.3rd , 2022

Title: 	Simulation results for inter-cell MMSE-IRC CQI requirements

Source: 	Huawei, HiSilicon

Agenda item:	10.12.2.1.2

Document for:	Discussion

Background

As per [1], RAN 4 agreed to provide the simulation results for inter-cell MMSE-IRC CQI requirements. In this paper, we provide the corresponding simulation results

Simulation results

Simulation results are captured in Figure 2-1 and Table 2-1. In the simulation, three schemes are simulated:

- Scheme 1: Transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with INR=10.04dB (DIP=-0.41dB); Demodulation is based on MMSE-IRC processing and CSI reporting is based on MMSE-IRC processing.

- Scheme 2: Transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with INR=10.04dB (DIP=-0.41dB). Demodulation is based on MMSE-IRC processing and CSI reporting is based on MMSE-MRC processing.

- Scheme 3: Transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source.

Figure 2-1: Simulation results for MMSE-IRC CQI test

Table 2-1: Summary of simulation results for MMSE-IRC CQI test

| 2RX | SINR (dB) | TP ratio(TP with scheme 1/TP with scheme3) | TP ratio(TP with scheme 2/TP with scheme3) | BLER of scheme 1 |
| --- | --- | --- | --- | --- |
|  | -6 | 2.72 | 1.54 | 0.02 |
|  | -5 | 2.74 | 1.45 | 0.03 |
|  | -4 | 3.02 | 1.50 | 0.04 |
|  | -3 | 3.04 | 1.49 | 0.03 |
|  | -2 | 3.00 | 1.534 | 0.03 |
|  | -1 | 2.88 | 1.56 | 0.06 |
|  | 0 | 2.95 | 1.56 | 0.07 |
|  | 1 | 2.94 | 1.54 | 0.07 |
|  | 2 | 2.79 | 1.47 | 0.06 |
|  | 2 | 2.52 | 1.37 | 0.07 |
|  | 4 | 2.46 | 1.35 | 0.07 |
|  | 5 | 2.40 | 1.37 | 0.09 |
|  | 6 | 2.48 | 1.41 | 0.08 |
|  | 7 | 2.29 | 1.39 | 0.10 |
|  | 8 | 2.26 | 1.39 | 0.09 |
|  | 9 | 2.09 | 1.30 | 0.07 |
|  | 10 | 2.02 | 1.28 | 0.08 |
| 4RX | SINR (dB) | TP ratio(TP with scheme 1/TP with scheme3) | TP ratio(TP with scheme 2/TP with scheme3) | BLER of scheme 1 |
|  | -6 | 3.43 | 1.22 | 0 |
|  | -5 | 3.38 | 1.22 | 0.004 |
|  | -4 | 3.16 | 1.202 | 0.02 |
|  | -3 | 3.05 | 1.20 | 0.02 |
|  | -2 | 2.63 | 1.23 | 0.05 |
|  | -1 | 2.69 | 1.22 | 0.03 |
|  | 0 | 2.68 | 1.18 | 0.02 |
|  | 1 | 2.36 | 1.12 | 0.06 |
|  | 2 | 2.10 | 1.08 | 0.1 |
|  | 2 | 2.27 | 1.12 | 0.06 |
|  | 4 | 2.30 | 1.18 | 0.06 |
|  | 5 | 2.12 | 1.16 | 0.07 |
|  | 6 | 2.14 | 1.16 | 0.05 |
|  | 7 | 2.03 | 1.11 | 0.04 |
|  | 8 | 1.97 | 1.08 | 0.04 |
|  | 9 | 1.96 | 1.11 | 0.02 |
|  | 10 | 1.86 | 1.10 | 0.02 |

Conclusion

In this paper, we provide the simulation results for CQI requirements for inter cell MMSE-IRC receiver.

Reference

- R4-2203009, WF on CSI requirements for inter-cell interference MMSE-IRC. Ericsson