3GPP TSG-RAN4 Meeting #102-e	R4-2205815

# Electronic Meeting, 21st February – 3rd March, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on requirements for UE Rx-Tx measurement for propagation delay compensation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE Rx-Tx measurement based on TRS or PRS within te serving cell is introduced for propagation delay compensation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define measurement requirements for UE Rx-Tx measurement for propagation delay compensation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance for UE Rx-Tx measurement for propagation delay compensation cannot be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 9.12	Measurement for Propagation Delay Compensation

### 9.12.1	Introduction

The requirements in this clause are applicable for UE capable of RTT based propagation delay compensation based on TRS and SRS or based on PRS and SRS,  indicated by [TBD].

### 9.12.2	Requirements Applicability

The requirements in clause 9.12 apply for [periodic/semi-persistent/aperiodic] and triggered UE Rx-Tx time difference measurements, provided:

-	If UE Rx-Tx time difference measurement is based on PRS, the related side conditions given in clause [TBD] are met for a corresponding band.

-	If UE Rx-Tx time difference measurement is based on TRS, the related side conditions given in clause [TBD] are met for a corresponding band.

-	SRS is configured on at least one of the PCell.

### 9.12.3	Measurement Capability

If UE Rx-Tx time difference measurement is based on PRS, the capability is as indicated by the UE in NR-Multi-RTT-ProvideCapabilities, according to TS 37.355 [34].

If UE Rx-Tx time difference measurement is based on TRA, the capability is as indicated by the UE in [TBD]?

### 9.12.3	Measurement Reporting Requirements

#### 9.12.3.1 Measurement Reporting Requirements for PRS

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The UE Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause [TBD].

The UE Rx-Tx time difference measurement accuracy for all measured DL PRS resources shall be fulfilled according to the accuracy requirements specified in clause [TBD].

#### 9.12.3.2 Measurement Reporting Requirements for TRS

Measurement report delay requirement is [TBD]

The UE Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause [TBD].

The UE Rx-Tx time difference measurement accuracy for all measured DL TRS resources shall be fulfilled according to the accuracy requirements specified in clause [TBD].

### 9.12.4	Measurement period requirements

#### 9.12.4.1 PRS Measurement Period

The UE shall be able to measure UE Rx-Tx time difference on PCell after receiving [TBD, command from network that triggers the UE Rx-Tx measurement] within TUERx-Tx_PRS, where

TUERx-Tx_PRS = TBD

UE is only required to perform UE Rx-Tx time difference on PRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on PRS, the requirements apply provided that the SCS of the PRS is same as that of the active BWP on PCell.

#### 9.12.4.2 TRS Measurement Period

The UE shall be able to measure UE Rx-Tx time difference on PCell after receving [TBD, command from network that triggers the UE Rx-Tx measurement] within TUERx-Tx_TRS, where

TUERx-Tx_TRS = TBD

UE is only required to perform UE Rx-Tx time difference on TRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on TRS, the requirements apply provided that the SCS of the TRS is same as that of the active BWP on PCell.

<End of Change 1>