3GPP TSG-RAN WG4 Meeting#102-e                	R4-2205898

Electronic Meeting, 21 February – 3 March 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On remaining RRM measurement open issues for FR2 HST

Agenda item:	10.9.3.2

Document for:	Discussion

Introduction

During the RAN4#101bis-e meeting, a way forward on FR2 HST RRM requirements [1] was approved, outlining some agreements and remaining open issues. In this contribution, we address the open issues and provide our views.

Discussion

NR SA intra-frequency cell identification time

From [1], the open issues for intra-frequency cell identification time without measurement gaps are

Way forward: - Discuss the following options to define the scaling factors: - Option 1: Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 6 for Set 1 and 18 for Set 2 - Option 2: Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 10 for Set 1 and 24 for Set 2 - Companies are encouraged to check, whether 600ms lower bound is makes sense when 80ms< DRX cycle≤ 320ms

From TS 38.113, the scaling factors for UE supporting power classes 3 and 4 are Mpss/sss_synch_w/o_gaps = Mmeas_period_w/o_gaps = 24.

It can be observed in Option 2 that the scaling factor for Set 2 equals to the non-HST scenario for UE supporting power classes 3 and 4. However, the number of UE RX sweeping beams has been reduced for HST scenarios in relation to the agreement in [2], which is

GtW Agreements: - Define only two sets of enhanced RRM requirements in terms of number of RX beams (i.e. RX beam sweeping scaling factor) per UE - Set 1: 2 RX beams - Set 2: 6 RX beams - Introduce network signalling to configure UE to follow either Set 1 or Set 2 RRM requirements - Note: the applicability of Set 1/2 requirements to the FR2 HST scenarios will be captured in the TR

Based on the above agreement for the number of UE RX beams for Set 2, the enhanced PSS/SSS detection and measurement time should be faster than the non-HST case for UE supporting power classes 3 and 4, which corresponds to no enhancements.

Observation 1: In Option 2, the scaling factor (Mpss/sss_synch_w/o_gaps = Mmeas_period_w/o_gaps = 24) for Set 2 is equal to non-HST scenarios for UE supporting power classes 3 and 4.

Observation 2: For Set 2 in Option 2, the PSS/SSS detection and measurement time is not faster than the non-HST case for UE supporting power classes 3 and 4, which corresponds to no enhancements and not suitable for HST.

Based on the above observations, the enhancements for intra-frequency measurement requirements for connected mode are shown below, which is also specified in a draft CR.

In connected mode, the scaling factor for PSS/SSS detection and measurements should be Option 1: Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 6 for Set 1 and 18 for Set 2.

The intra-frequency PSS/SSS detection and measurement enhancements for FR2 HST are specified in a draft CR [4].

SA Cell reselection (in idle mode)

From [1], the agreement and open issue for cell reselection in idle mode are

Agreement: Defined enhanced requirements for Cell reselection in IDLE/INACTIVE mode for DRX 320 ms in HST FR2 deployments: DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; 2.56 x N1 x M2 (8 x N1 x M2); 0.32 x N1 x M3 (1 x N1 x M3); 0.96 x N1 x M4 (3 x M4). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: [When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraRAT-MeasurementEnhancement-r16].. N1 refers to the number of Rx beams and equals 2 for Set 1, and 6 for Set 2. Way forward: The companies are encouraged to check the Notes and to reformulate Note 2.

Note 2 is not applicable and should be removed.

Proposal 2: For the enhanced requirements for Cell reselection in IDLE/INACTIVE mode, Note 2 in the table is not needed.

Conclusion

The document has discussed possible enhancements on RRM measurement requirements for FR2 HST, which include the following observations and proposals:

Observation 1: In Option 2, the scaling factor (Mpss/sss_synch_w/o_gaps = Mmeas_period_w/o_gaps = 24) for Set 2 is equal to non-HST scenarios for UE supporting power classes 3 and 4.

Observation 2: For Set 2 in Option 2, the PSS/SSS detection and measurement time is not faster than the non-HST case for UE supporting power classes 3 and 4, which corresponds to no enhancements and not suitable for HST.

- In connected mode, the scaling factor for PSS/SSS detection and measurements should be Option 1: Scaling factors (Mpss/sss_synch_w/o_gaps and Mmeas_period_w/o_gaps) equal 6 for Set 1 and 18 for Set 2.

- For the enhanced requirements for Cell reselection in IDLE/INACTIVE mode, Note 2 in the table is not needed.

References

[1]    R4-2202594, WF on FR2 HST RRM (part 1), Nokia, Nokia Shanghai Bell

[2]   R4-2120292, WF on FR2 HST RRM (part 1), Nokia, Nokia Shanghai Bell

[3]   R4-2120416, WF on FR2 HST RRM requirements (part 2), Samsung

[4]   R4-2204629, CR to TS 38.133: intra-frequency measurements without gaps for for FR2 NR HST, Nokia, Nokia Shanghai Bell