3GPP TSG-RAN WG4 # 102-e	R4-2205903

Electronic Meeting, February 21 – March 3, 2022

Agenda item:	10.12.2.2

Source:	MediaTek Inc.

Title:	Discussion on the MMSE-IRC receiver for intra-cell interference

Document for:	Discussion

- Introduction

In RAN4#101-bis-e, a way forward for MMSE-IRC receiver for intra-cell inter-user interference was agreed [1]. In this contribution, we provide our views for the remaining issues and present simulation results based on the agreed simulation assumptions.

- Discussion

In the last meeting, RAN4 discussed UE feature and capability signaling for MMSE-IRC receiver for scenarios with intra-cell interference and the candidate options are show as below.

UE feature list MMSE-IRC requirements for scenarios with intra-cell inter-user interference

- Option 1: No need to introduce new UE feature, requirements release independent from Rel-15 - Option 2: Mandatory with or without UE capability signaling - Option 3: Optional with or without UE capability and applicable from Rel-17 - Option 4: Optional without UE capability and applicable from Rel-15

To address the concern that there is no intra-cell inter-user interference considered in Rel-15/16 and extra testing cost for Rel-15/16 UEs. We prefer “Optional without UE capability signalling for Rel-15/16 UE”. Besides, the MU-MIMO capability is essential to increase the throughput of network and it is up to gNB to pair the co-scheduled UE. We think it is not necessary to define UE capability signaling. Also, as we consider the intra-cell inter-user interference scenario in Rel-17, it should be mandatory from Rel-17.

Proposal 1: For UE feature list MMSE-IRC requirements for scenarios with intra-cell inter-user interference, we propose “Optional without UE capability signalling for Rel-15/16 UE and mandatory from Rel-17”.

- Simulation Results

Based on the simulation assumptions in Annex provided by [1], we present our simulation results as below.

Table 1: Summary of simulation results for FDD

| Parameters |  |  |  |  |  | SNR for 70% of max T-put, [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| #CDM | #Tx | #Rx | MCS | Propagation channel | PMI | MMSE-IRC |
| 1 | 2 | 2 | 13 | TDLC300-100 | Rand | 15.6 |
|  | 2 | 4 | 13 | TDLC300-100 | Rand | 9.3 |
| 2 | 4 | 4 | 13 | TDLA30-10 | Orthog | 16.9 |

- Conclusion

In this contribution, we provide simulation results and our views on the remaining issues for the requirements of MU-MIMO. The proposals are summarized as below:

Proposal 1: For UE feature list MMSE-IRC requirements for scenarios with intra-cell inter-user interference, we propose “Optional without UE capability signalling for Rel-15/16 UE and mandatory from Rel-17”.

- Reference

- R4-2203011, “Way Forward on MMSE-IRC for intra-cell inter-user interference”, Huawei, HiSilicon