3GPP TSG-RAN WG4 Meeting #102-e 	R4-2205930

Electronic Meeting, Feb 21st - March 3rd 2022

Source:	T-Mobile USA

Title:	TP for TR38.841: PC2 and PC1.5 n77 for CA_n66A-n77A

Agenda item:	9.30.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC1.5 n77 for DL CA_n66A-n77A.

There is no PC3 cross band isolation of n77 into n66, but it was proposed to start with n77 PC2 MSD into n41 and derate to estimate MSD for PC2 n77 into n66. 1.5 dB of cross band isolation MSD is proposed for n77 PC1.5 into n66, and 3 dB of MSD for PC1.5 n77 into n66.

- Reference

[1]

- Text Proposal

<Start of text proposal>

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n5A-n77A requirements are applied for PC2 CA_n5A-n77A.

## 5.7 	CA_n66-n77

### 5.7.1	Configurations

Table 5.7.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n66A-n77A CA_n66A-n77(2A) | n778,9, CA_n66A-n77A8 | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.7.2	Maximum output power

| Uplink CA configuration | Power class 2 cases for CA_n66A-n77A | CA power class | Carrier n66 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n66A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 5.7.3	REFSENS requirements

According to the PC3 CA_n66A-n77A study, the 2nd and 5th order IMD products generated from dual uplinks of band 66 and n77 may fall into band 66 Rx frequency range. Thus additional MSD should be considered to mitigate the impact of the interference for the PC2 CA_n66A-n77A combination.

#### 5.7.3.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 CA_n66A-n77A are defined in table 5.7.3.1-1.

Table 5.7.3.1-1: MSD test points for PCell due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD for PC2 | Duplex mode | Source of IMD |
| Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n66A-n77A CA_n66A-n77(2A) | n66 | 1730 | 5 | 25 | 2130 | 34.33 | FDD | IMD2 |
|  | n77 | 3860 | 10 | 50 | 3860 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | 11.27 | FDD | IMD5 |
|  | n77 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |

#### 5.7.3.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B CA_n66A-n77A are same as the Case A defined in table 5.7.3.1-1.

Although no cross-band isolation MSD was identified for PC3 n77 into n66, it was suggested that for PC2 n77 cross band isolation into n41 be considered and de-rated.

PC2 n77 into n41: MSD=6.5dB => UL noise ~5.5dB above REFSENS

PC2 n77 into n66 => UL noise at 5.5-9=-4.5dB => ~1.5dB MSD

Table 5.7.3.1-2: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 n77 single band uplink into n66

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n66 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |  |  |  |  |  |  |

5.13.3.6	Single uplink n77 Power class 1.5

There was intermodulation MSD identified for PC2 for CA_n66A-n77A. However, PC1.5 is only for single band uplink so it will not impact intermodulation MSD.

Although no cross-band isolation MSD was identified for PC3 n77 into n66, it was suggested that for PC1.5 n77 cross band isolation into n41 be considered and de-rated.

PC1.5 n77 into n41 => UL noise 8.5dB above RESENS => ~9dB MSD

PC1.5 n77 into n66 => UL noise = REFSENS => ~3dB MSD

Therefore, the following is proposed

Table 5.13.3.6-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 n77 single band uplink into n66

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n66 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |  |  |  |  |  |  |

### 5.7.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n66A-n77A requirements are applied for PC2 CA_n66A-n77A.