3GPP TSG RAN WG4 Meeting #102-e	R4-2205988

e-Meeting, February 21 – March 3, 2022

Agenda Item:	10.16.2

Source:	Huawei, HiSilicon

Title:	Discussion on the channel raster and sync raster in FR2-2

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN#92-e, WI for extending current NR operation to 71 GHz was revised as following [1]:

- In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - Limited sync raster entry numbers - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz. - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS. - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - 960 kHz numerology for the SSB is not supported by the UE for initial access in Rel-17.

The following related agreements were made in RAN4 [2][3]:

RAN4#101-e: Band definition Unlicensed band n263 covering 57 to 71 GHz has been agreed and introduced into specs - n263: 57000 MHz – 71000 MHz (both UL and DL), with TDD duplex mode Channelization Option 1C (no IEEE 802.11ad/ay alignment and floating channelization) and Option 1D (hybrid between IEEE and no IEEE alignment with fixed channelization) will be evaluated in the next RAN4 meeting based on: - Number of sync raster entries and cell searching complexity - Support of CA in this frequency range - Whether flexibility is needed for minimum channel bandwidth, and how much it benefits system performance - Co-existence with IEEE channels (find out whether there is similar activity in IEEE) - Whether to consider and how to ensure future proof for adding new frequency bands or channel bandwidth - Other aspects are not precluded If no agreement is reached in the next RAN4 meeting, different channelizations will be considered for licensed and unlicensed bands RAN4#101b-e: Agreement: Consider the different channelization for licensed band(s) and unlicensed band(s) - Fixed sync raster for unlicensed bands - Fixed scheme should not be constrained by IEEE channel raster - Send LS to RAN1 to make sure that RAN1 accommodates the solution already now for both fixed and floating sync raster. - For the contiguous carrier aggregation, the channel spacing of adjacent channels should be multiple of the larger SCS, i.e., 960KHz, used by two channels/CCs - Floating sync raster for licensed bands - Stick to the agreement last meeting for 3 x 17.28MHz as the minimum granularity. - Refer to gap between adjacent GSCN values is not smaller than 3 x 17.28MHz - FFS: Unlicensed bands tries to use the sub-set of sync raster for licensed bands

This contribution provides our views and proposals about channel raster and sync raster for unlicensed band n263 and the possible licensed band in 66-71 GHz spectrum. This is the revision of [4].

# Channel raster for licensed/unlicensed band

There is 14GHz contiguous spectrum in unlicensed band n263. The channelization design should be flexible enough to support NR operation with different channel bandwidths and numerologies. The current ARFCN for 24.25 GHz~100 GHz with 60 kHz step size can be considered as the starting point for the raster design.

For licensed band of 66.0~71.0 GHz, floating channel raster similar as FR2-1 is preferred. The proposed channel raster for the licensed band is as follows and also summarized in Table 1:

- For 120 kHz SCS, the minimum channel bandwidth is 100MHz. For NREF=2713332, the corresponding frequency is 66049.98 MHz which could be used as the as the lowest center frequency for a 100 MHz channel bandwidth. For NREF=2794998, the corresponding frequency is 70949.94 MHz which could be used as the highest center frequency for a 100MHz channel bandwidth.

- For 480/960 kHz SCS, the minimum channel bandwidth is 400MHz. For NREF=2715832, the corresponding frequency is 66199.98 MHz which could be used as the as the lowest center frequency for a 400 MHz channel bandwidth. For NREF=2792504, the corresponding frequency is 70800.3 MHz which could be used as the highest center frequency for a 400 MHz channel bandwidth.

Table 1: Applicable NR-ARFCN for band 66~71 GHz

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| nxxx | 120 | 2713332– <2> – 2794998 |
|  | 480 | 2715832 – <8> – 2792496 |
|  | 960 | 2715832 – <16> – 2792504 |

Proposal 1: For licensed band, support floating channelization with 120/480/960 kHz step gap for 120/480/960kHz SCS respectively. The corresponding ARFCN are given in Table 1.

For unlicensed band n263, considering the minimum channel bandwidth is 100MHz, it is beneficial to divide the whole 14GHz spectrum into 140 non-overlapped 100MHz channels in order to facilitate intra/inter RAT interference mitigation. The 100MHz channel(s) that overlap with the 2.16GHz channel boundaries of 802.11ad/ay can be skipped if the alignment with 802.11ad/ay channelization is expected although we do not see a strong motivation. Raster locations close to 100MHz steps could be chosen from the ARFCN set for n263 because 100MHz is not divisible by the 60 kHz raster in the current ARFCN. We suggest to choose 99.84 MHz = 1664*60 kHz as the raster step for 100MHz channel bandwidth. For 200 MHz channel bandwidth, 2*99.84 MHz = 3328*60 kHz is used as the channel raster gap. For channel band width equal or larger than 400 MHz, 4*99.84 = 6656*60 kHz is used as the channel raster gap. The channel raster design for unlicensed band will follow the principle that the channel raster of unlicensed band in the overlapping frequency range with licensed band (66-71GHz) is a subset of the channel raster of licensed band, while the lowest frequency of the channel bandwidth is as close as to 57 GHz.

Our proposed channelization for n263 is as follows and summarized in Figure 1 and Table 2:

- For NREF=2563334, the corresponding frequency is 57050.46 MHz which could be used as the as the lowest center frequency for a 100MHz channel bandwidth. For NREF=2794908, the corresponding frequency is 70944.48 MHz which could be used as the highest center frequency for a 100MHz channel bandwidth.

- For NREF=2564166, the corresponding frequency is 57000.02 MHz which could be used as the as the lowest center frequency for a 200MHz channel bandwidth. For NREF=2793798, the corresponding frequency is 70877.94 MHz which could be used as the highest center frequency for a 200MHz channel bandwidth.

- For NREF=2565832, the corresponding frequency is 57199.98 MHz which could be used as the as the lowest center frequency for a 400MHz channel bandwidth. For NREF=2792136, the corresponding frequency is 70778.22 MHz which could be used as the highest center frequency for a 400MHz channel bandwidth.

- For NREF=2569164, the corresponding frequency is 57399.9 MHz which could be used as the as the lowest center frequency for an 800MHz channel bandwidth. For NREF=2788812, the corresponding frequency is 70578.78 MHz which could be used as the highest center frequency for an 800MHz channel bandwidth.

- For NREF=2575832, the corresponding frequency is 57799.98 MHz which could be used as the as the lowest center frequency for a 1600MHz channel bandwidth. For NREF=2782168, the corresponding frequency is 70180.14 MHz which could be used as the highest center frequency for a 1600MHz channel bandwidth.

- For NREF=2579164, the corresponding frequency is 57999.92 MHz which could be used as the as the lowest center frequency for a 2000MHz channel bandwidth. For NREF=2778884, the corresponding frequency is 69980.7 MHz which could be used as the highest center frequency for a 2000MHz channel bandwidth. Table 2: Applicable NR-ARFCN for band 57~71 GHz

| Channel Bandwidth | Allowed NREF (First – <Step size> – Last) |
| --- | --- |
| 100 MHz | 2563334 – <1664> – 2794908 |
| 200 MHz | 2564166 – <3328> – 2793798 |
| 400 MHz | 2565832 – <6656> – 2792136 |
| 800 MHz | 2569164 – <6656> – 2788812 |
| 1600 MHz | 2575832 – <6656> – 2782168 |
| 2000 MHz | 2579164 – <6656> – 2778844 |

Figure 1. High level description of channel raster for unlicensed band

Proposal 2: For the unlicensed band n263, support fixed channelization. For 100MHz or 200MHz channel bandwidth, the gap between two adjacent channels is about 100MHz or 200 MHz respectively. For channel bandwidth not smaller than 400MHz, the gap between two channel rasters is about 400MHz. the corresponding ARFCN are given in Table 2.

# Sync raster for licensed/unlicensed band

The global synch raster introduced in 38.101-2 can be used as the starting point to design SS raster entries for FR2-2.  For licensed band of 66~71 GHz, to reduce the SSB search complexity, we propose to support 3*17.28 MHz SS raster steps for 120 kHz SCS sync raster. For 480 kHz SCS sync raster step, considering the SSB bandwidth of 480 kHz is 4 times than that of 120 kHz, 4*3*17.28 MHz will be adopted. For GSCN = 24675, the corresponding frequency is 66050.4 MHz which could be used as the as the center frequency of the lowest 120 kHz SSB. For GSCN = 24960, the corresponding frequency is 70940.88 MHz which could be used as the center frequency of the highest 120 kHz SSB. For GSCN = 24684, the corresponding frequency is 66205.92 MHz which could be used as the as the center frequency of the lowest 480 kHz SSB. For GSCN = 24948, the corresponding frequency is 70767.84 MHz which could be used as the center frequency of the highest 480 kHz SSB. The sync raster for licensed band is summarized in Table 3.

Table 3: Applicable SS raster entries for 66~71 GHz

| NR Operating Band | SS Block SCS | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- |
| Nxxx | 120 kHz | 24675 - <3> - 24960 |
|  | 480 kHz | 24684 - <12> - 24948 |

For unlicensed band of n263 spanning 14 GHz, reusing the 17.28 MHz SS raster step size in FR2-1 bands results in a prohibitive initial search complexity. A simple approach to reduce such complexity is to down-select some of the GSCN values for SS raster entries. For 120 (480) kHz SCS, SSB occupies 28.8 (115.2) MHz and the minimum channel bandwidth is 100 (400) MHz. We propose to support 2 candidate SSB positions per unit of 100 (400) MHz channel bandwidth for 120 (480) kHz SCS. An example is plotted in figure 2. Detail GSCN for 120kHz and 480kHz SSB are provided in Table 4 and Table 5 in the appendix.

Figure 2. High level description on selection of GSCN locations unlicensed band for 100/400MHz channels.

Based on Table 3, Table 4 and Table 5, the total number of unique sync raster for both licensed and unlicensed band with 120 kHz SCS SSB and 480 kHz SCS SSB is 345 + 89 = 434 which is under the limit of 665 given by the WID.

Proposal 3: For 66~71 GHz licensed band, support candidate SS raster in Table 3 for 120 kHz and 480 kHz SCS. For n263 unlicensed band, support candidate SS raster in Table 4 and Table 5 for 120 kHz and 480 kHz SCS, respectively.

# Conclusions

This contribution provides our view about the design of channel raster and sync raster for band n263 and unlicensed band, following proposals are made:

Proposal 1: For licensed band, support floating channelization with 120/480/960 kHz step gap for 120/480/960kHz SCS respectively. The corresponding ARFCN are given in Table 1.

Proposal 2: For the unlicensed band n263, support fixed channelization. For 100MHz or 200MHz channel bandwidth, the gap between two adjacent channels is about 100MHz or 200 MHz respectively. For channel bandwidth not smaller than 400MHz, the gap between two channel rasters is about 400MHz. the corresponding ARFCN are given in Table 2.

Proposal 3: For 66~71 GHz licensed band, support candidate SS raster in Table 3 for 120 kHz and 480 kHz SCS. For n263 unlicensed band, support candidate SS raster in Table 4 and Table 5 for 120 kHz and 480 kHz SCS, respectively.

# References

[1]  RP-211584	Revised WID: Extending current NR operation to 71GHz

[2]  R4-2119928 Email discussion summary for [101-e][128] NR_ext_to_71GHz_Part_1

[3]  R4-2202327 Email discussion summary for [101-bis-e][127] NR_ext_to_71GHz_Part_1

[4]  R1-2200652 Discussion on the channel raster and sync raster in FR2-2	Huawei, HiSilicon

# Appendix.

Table 4   Sync Raster for 120 kHz SSB in 57GHz-71GHz (SSREF = 24250.08 MHz + Sync Raster *17.28 MHz)

| Bandwidth | Sync Raster | Bandwidth | Sync Raster | Bandwidth | Sync Raster | Bandwidth | Sync Raster |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 57.0-57.1 | 1897,1900 | 60.5-60.6 | 2099,2102 | 64.0-64.1 | 2302,2305 | 67.5-67.6 | 2504,2507 |
| 57.1-57.2 | 1903,1905 | 60.6-60.7 | 2105,2108 | 64.1-64.2 | 2308,2310 | 67.6-67.7 | 2510,2513 |
| 57.2-57.3 | 1908,1911 | 60.7-60.8 | 2111,2114 | 64.2-64.3 | 2313,2316 | 67.7-67.8 | 2516,2519 |
| 57.3-57.4 | 1914,1917 | 60.8-60.9 | 2117,2119 | 64.3-64.4 | 2319,2322 | 67.8-67.9 | 2522,2525 |
| 57.4-57.5 | 1920,1923 | 60.9-61.0 | 2122,2125 | 64.4-64.5 | 2325,2328 | 67.9-68 | 2527,2530 |
| 57.5-57.6 | 1926,1928 | 61.0-61.1 | 2128,2131 | 64.5-64.6 | 2331,2334 | 68.0-68.1 | 2533,2536 |
| 57.6-57.7 | 1931,1934 | 61.1-61.2 | 2134,2137 | 64.6-64.7 | 2337,2339 | 68.1-68.2 | 2539,2542 |
| 57.7-57.8 | 1937,1940 | 61.2-61.3 | 2140,2143 | 64.7-64.8 | 2342,2345 | 68.2-68.3 | 2545,2548 |
| 57.8-57.9 | 1943,1946 | 61.3-61.4 | 2146,2148 | 64.8-64.9 | 2348,2351 | 68.3-68.4 | 2551,2553 |
| 57.9-58.0 | 1949,1952 | 61.4-61.5 | 2151,2154 | 64.9-65.0 | 2354,2357 | 68.4-68.5 | 2556,2559 |
| 58.0-58.1 | 1955,1957 | 61.5-61.6 | 2157,2160 | 65.0-65.1 | 2360,2363 | 68.5-68.6 | 2562,2565 |
| 58.1-58.2 | 1960,1963 | 61.6-61.7 | 2163,2166 | 65.1-65.2 | 2365,2368 | 68.6-68.7 | 2568,2571 |
| 58.2-58.3 | 1966,1969 | 61.7-61.8 | 2169,2172 | 65.2-65.3 | 2371,2374 | 68.7-68.8 | 2574,2577 |
| 58.3-58.4 | 1972,1975 | 61.8-61.9 | 2175,2177 | 65.3-65.4 | 2377,2380 | 68.8-68.9 | 2580,2582 |
| 58.4-58.5 | 1978,1981 | 61.9-62.0 | 2180,2183 | 65.4-65.5 | 2383,2386 | 68.9-69.0 | 2585,2588 |
| 58.5-58.6 | 1984,1987 | 62.0-62.1 | 2186,2189 | 65.5-65.6 | 2389,2391 | 69.0-69.1 | 2591,2594 |
| 58.6-58.7 | 1989,1992 | 62.1-62.2 | 2192,2195 | 65.6-65.7 | 2394,2397, | 69.1-69.2 | 2597,2600 |
| 58.7-58.8 | 1995,1998 | 62.2-62.3 | 2198,2200 | 65.7-65.8 | 2400,2403 | 69.2-69.3 | 2603,2606 |
| 58.8-58.9 | 2001,2004 | 62.3-62.4 | 2203,2206 | 65.8-65.9 | 2406,2409 | 69.3-69.4 | 2608,2611 |
| 58.9-59.0 | 2007,2010 | 62.4-62.5 | 2209,2212 | 65.9-66.0 | 2412,2415 | 69.4-69.5 | 2614,2617 |
| 59.0-59.1 | 2012,2015 | 62.5-62.6 | 2215,2218 | 66.0-66.1 | 2418,2420 | 69.5-69.6 | 2620,2623 |
| 59.1-59.2 | 2018,2021 | 62.6-62.7 | 2221,2224 | 66.1-66.2 | 2423, 2426 | 69.6-69.7 | 2626,2629 |
| 59.2-59.3 | 2024,2027 | 62.7-62.8 | 2227,2229 | 66.2-66.3 | 2429,2432 | 69.7-69.8 | 2632,2635 |
| 59.3-59.4 | 2030,2033 | 62.8-62.9 | 2232,2235 | 66.3-66.4 | 2435,2438 | 69.8-69.9 | 2637,2640 |
| 59.4-59.5 | 2036,2038 | 62.9-63.0 | 2238,2241 | 66.4-66.5 | 2441,2444 | 69.9-70.0 | 2643,2646 |
| 59.5-59.6 | 2041,2044 | 63.0-63.1 | 2244,2247 | 66.5-66.6 | 2446,2449 | 70.0-70.1 | 2649,2652 |
| 59.6-59.7 | 2047,2050 | 63.1-63.2 | 2250,2253 | 66.6-66.7 | 2452,2455 | 70.1-70.2 | 2655,2658 |
| 59.7-59.8 | 2053,2056 | 63.2-63.3 | 2256,2258 | 66.7-66.8 | 2458,2461 | 70.2-70.3 | 2661,2663 |
| 59.8-59.9 | 2059,2062 | 63.3-63.4 | 2261,2264 | 66.8-66.9 | 2464,2467 | 70.3-70.4 | 2666,2669 |
| 59.9-60.0 | 2065,2067 | 63.4-63.5 | 2267,2270 | 66.9-67.0 | 2470,2472 | 70.4-70.5 | 2672,2675 |
| 60.0-60.1 | 2070,2073 | 63.5-63.6 | 2273,2276 | 67.0-67.1 | 2475,2478 | 70.5-70.6 | 2678,2681 |
| 60.1-60.2 | 2076,2079 | 63.6-63.7 | 2279,2282 | 67.1-67.2 | 2481,2484 | 70.6-70.7 | 2684,2687 |
| 60.2-60.3 | 2082,2085 | 63.7-63.8 | 2284,2287 | 67.2-67.3 | 2487,2490 | 70.7-70.8 | 2690,2692 |
| 60.3-60.4 | 2088,2091 | 63.8-63.9 | 2290,2293 | 67.3-67.4 | 2493,2496 | 70.8-70.9 | 2695,2698 |
| 60.4-60.5 | 2093,2096 | 63.9-64.0 | 2296,2299 | 67.4-67.5 | 2499,2501 | 70.9-71.0 | 2701,2704 |

Table 5   Sync Raster for 480 kHz SSB in 57GHz-71 GHz(SSREF = 24250.08 MHz + Sync Raster *17.28 MHz)

| Bandwidth | Sync Raster | Bandwidth | Sync Raster |
| --- | --- | --- | --- |
| 57.0-57.4 | 1900,1914 | 64.2-64.6 | 2316,2331 |
| 57.4-57.8 | 1923,1937 | 64.6-65.0 | 2339,2354 |
| 57.8-58.2 | 1946,1960 | 65.0-65.4 | 2363,2377 |
| 58.2-58.6 | 1969,1983 | 65.4-65.8 | 2386,2400 |
| 58.6-59.0 | 1992,2007 | 65.8-66.2 | 2409,2423 |
| 59.0-59.4 | 2015,2030 | 66.2-66.6 | 2432,2446 |
| 59.4-59.8 | 2039,2053 | 66.6-67.0 | 2455,2470 |
| 59.8-60.2 | 2062,2076 | 67.0-67.4 | 2478,2493 |
| 60.2-60.6 | 2085,2099 | 67.4-67.8 | 2502,2516 |
| 60.6-61.0 | 2108,2122 | 67.8-68.2 | 2525,2539 |
| 61.0-61.4 | 2131,2145 | 68.2-68.6 | 2548,2562 |
| 61.4-61.8 | 2154,2169 | 68.6-69.0 | 2571,2585 |
| 61.8-62.2 | 2177,2192 | 69.0-69.4 | 2594,2608 |
| 62.2-62.6 | 2201,2215 | 69.4-69.8 | 2617,2632 |
| 62.6-63.0 | 2224,2238 | 69.8-70.2 | 2640,2655 |
| 63.0-63.4 | 2247,2261 | 70.2-70.6 | 2664,2678 |
| 63.4-63.8 | 2270,2284 | 70.6-71.0 | 2687,2701 |
| 63.8-64.2 | 2293,2308 |  |  |