3GPP TSG-RAN WG4 Meeting #102-e	R4-2206059

e-Meeting, Feb.-Mar. 2022

Title:	On FR2 power control for inter-band NR-DC

Source:	Qualcomm Incorporated

Agenda item:	13.2.1

Document for:	Approval

# 1.	Introduction

We discuss our findings towards enabling FR2+FR2 inter-band ULCA as it applies to one case of the enquiry from RAN1 [1] regarding NR-DC power control.

# 2. 	Discussion

RAN1 enquires about the feasibility of independent power control for two cases [1].

Prior progress and discussion on this topic are captured well in [2]. The latest WF [3] also identifies independent power control for ULCA as the precedent for RAN4’s stand on NR-DC power control for FR2.

In this contribution, we focus on case 1).

The ‘total power concept’ has been identified as a possible reason for not establishing independent per-band configured power. Any impact from ‘total power concept’ is being discussed in terms of a relaxation to the peak EIRP requirement and is intended to be absorbed into the parameter delta(TIB_peak), per agreement in WF [7], section 2.4.b. This parameter applies to the single-band min. peak EIRP requirement, and therefore is defined per-band. Once specified as a numerical value in the standard, no further cross-band dependence is necessary.

Observation: ‘Total power concept’ does not motivate a configured power requirement that is linked between the UL bands.

In addition, delta(TIB_peak) like delta (MB_peak), is easily accommodated into a per-band configured power, as demonstrated in the draft CR [6], confirming that an independent per-band treatment is feasible for ‘total power concept’.

There are other known mechanisms that indirectly link the transmit powers over the UL FR2 bands, however. These are listed below:

- Emissions: FR2+FR2 inter-band ULCA generates far out of band IM3 products that must also comply with emissions requirements. It is agreed to use the MPR framework for ULCA [4], where CA_MPR = max { MPRPA-PA, MPRwaveform&modulation&BW&etc }. Moreover, it is agreed that CA_MPR is defined per-band, per-band_combination rather than per band combination so once specified as a numerical value, no further cross-band dependence is necessary.

- MPE: A UE may use P-MPR to self-regulate its UL when proximity to biological tissue is detected. P-MPR is unregulated for single CC UL. Extending this to FR2 ULCA, it is left up to UE implementation how much P-MPR is taken in each band when conditions demand. There is neither standards need nor regulatory need to institute cross-band dependence on P-MPR.

In summary, both, MPR and P-MPR are independent per-band quantities, without further necessity for cross-band linkage. Both, MPR and P-MPR are already present as explicit terms in the single CC configured power requirement, so they would naturally have a home in an independent per-band configured power requirement modelled after the single CC requirement. This confirms that an independent per-band treatment is feasible for MPR and P-MPR.

We therefore conclude in [5] that for FR2+FR2 inter-band ULCA, independent per-band power control is feasible.

Proposal 1: RAN4 to confirm that independent power control per cell group is feasible when ‘uplink CCs of MCG and uplink CCs of SCG are in different frequency bands in FR2’.

# 3. 	References

- R4-2107605, Further Reply LS on power control for NR-DC, RAN4#99-e; RAN1

- R4-2115081, WF on FR2 power control for NR-DC, OPPO, RAN4#100-e

- R4-2118291, ‘Discussion and Reply LS on Further Reply LS on power control for NR-DC’, Vivo, RAN4#101-e

- R4-2202343, ‘WF on FR2+FR2 inter-band UL CA’, Qualcomm, RAN4#101-Bis-e, Jan. 2022

- R4-2206054, ‘delta(TIB) for FR2+FR2 ULCA’, Qualcomm, RAN4#102-e, Feb.-Mar. 2022

- R4-2206057, ‘draft CR to introduce FR2+R2 inter-band ULCA’, Qualcomm, RAN4#102, Feb 2022

- R4-2202343, ‘WF on FR2+FR2 inter-band UL CA’, Qualcomm, RAN4#101-Bis-e, Jan. 2022