3GPP TSG-RAN WG4 Meeting # 102-e	R4-2206098

# Electronic Meeting, February 21 – March 3, 2022

Agenda Item:	8

Source: 		Qualcomm Incorporated

Title: 			Rel-17 Feature List

Document for:	Discussion

# 1.	Introduction

In this contribution, we propose the following Rel-17 feature lists according to the proposals [1,2]:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X1-Y | Network controlled small gap (NCSG) | Support of per-FR NCSG (independentGapConfig) | X1(NCSG) | yes | no | UE cannot signal per-FR NCSG capability | per-UE | No | No |  |  | Optional with capability signalling |
| X1-Y+1 | Network controlled small gap (NCSG) | Supported NCSG patterns | X1(NCSG) | yes | no | Network does not know whether some NCSG patterns can be configured to UE | per-UE | No | No |  |  | Optional with capability signalling NCSG patterns #0, #1, [x1, y1, …] are conditional mandatory if UE support X-1 |
| X1-Y+2 | Network controlled small gap (NCSG) | Supported NR-only NCSG patterns | X1(NCSG) | yes | no | Network does not know whether some NR-only NCSG patterns can be configured to UE | per-UE | No | No |  |  | Optional with capability signalling NCSG patterns #0, #1, [x2, y2, …] are conditional mandatory if UE support X-1 |
| X2 – Y1 | FR2 HST | Support of one time large UL timing adjustment | (Applicable to PC 6 UEs only) | Yes | no | Network does not know whether UE support one time large UL timing adjustment | Per-UE | No | No |  |  | Optional with capability signaling |
| X3-Y2 | Pre-configured gap | Support of per-FR pre-configured gap (independentGapConfig) | X3(pre-configured gap) | yes | no | UE cannot signal per-FR pre-configured gap capability | per-UE | No | No |  |  | Optional with capability signalling |

# 2. Conclusion

Proposal 1: The following capabilities should be introduced:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X1-Y | Network controlled small gap (NCSG) | Support of per-FR NCSG (independentGapConfig) | X1(NCSG) | yes | no | UE cannot signal per-FR NCSG capability | per-UE | No | No |  |  | Optional with capability signalling |
| X-1Y+1 | Network controlled small gap (NCSG) | Supported NCSG patterns | X1(NCSG) | yes | no | Network does not know whether some NCSG patterns can be configured to UE | per-UE | No | No |  |  | Optional with capability signalling NCSG patterns #0, #1, [x1, y1, …] are conditional mandatory if UE support X-1 |
| X1-Y+2 | Network controlled small gap (NCSG) | Supported NR-only NCSG patterns | X1(NCSG) | yes | no | Network does not know whether some NR-only NCSG patterns can be configured to UE | per-UE | No | No |  |  | Optional with capability signalling NCSG patterns #0, #1, [x2, y2, …] are conditional mandatory if UE support X-1 |
| X2 – Y1 | FR2 HST | Support of one time large UL timing adjustment | (Applicable to PC 6 UEs only) | Yes | no | Network does not know whether UE support one time large UL timing adjustment | Per-UE | No | No |  |  | Optional with capability signaling |
| X3-Y2 | Pre-configured gap | Support of per-FR pre-configured gap (independentGapConfig) | X3(pre-configured gap) | yes | no | UE cannot signal per-FR pre-configured gap capability | per-UE | No | No |  |  | Optional with capability signalling |

# 3. Reference

[1] R4-2203715

[2] R4-2203711