3GPP TSG-RAN WG4 Meeting RAN4#102-e     				R4-2206099 Online

February 21st – March 3rd

Agenda Item:	4.1.1.1

Source:	Lenovo

Title:	MIMO EVM Measurement for FR1

Document for:  Approval

- Introduction

It has been proposed in [1,2] that the pseudo-inverse be used to define and measure EVM for two-layer uplink MIMO in FR1.  This proposal is based on the contents of TR 38.884 [3] in which the pseudo-inverse is used to define and measure the EVM for two-layer uplink MIMO in FR2.

In [4,5], some issues were identified with the use of the pseudo-inverse and these issues are also addressed briefly in this contribution.  The main issues are the following:

- The expression used for the pseudo-inverse of a square matrix is not the pseudo-inverse and is undefined if the effective channel does not have full rank;

- The pseudo-inverse is not needed because if the effective channel does not have full rank, the layers cannot be separated and EVM is failed.  If the channel has full rank, the channel inverse should be used.

Based on the above, it is proposed that the effective channel inverse be used to define and measure EVM for two-layer uplink MIMO in FR1.

- EVM for Two-Layer Transmission

In Section 5.2.3.1.1.2 of TR 38.884-120 on “Method 1 MIMO Equalization,” there is the following text:

“The ZF equalizer coefficients are calculated as pseudo inverse of effective channel matrix, in general:

GZF=H+=(HHH)-1HH ,"

where the effective channel matrix is given by

H=HP= h0,0h0,1h1,0h1,1

and P is the precoding matrix.  This text indicates that the pseudo-inverse of the 2x2 matrix H is given by

H+=(HHH)-1HH,

However, this is not correct.  If this were in fact the pseudo-inverse, then it would be defined for the case that the matrix H is rank 1.  Consider the following example of a rank 1 matrix:

H= 1-1-11 .

In this case,

(HHH)-1HH=2-2-22-11-1-11

Since the determinant of

2-2-22-1

is 0, its inverse does not exist, and thus

(HHH)-1HH=2-2-22-11-1-11

does not exist and is not the pseudo-inverse.

From [4], the actual pseudo-inverse H+ of a square matrix is given by

H+=V Σ+UH

where the singular value decomposition of the matrix is given by

H=U Σ VH,

and Σ+ is derived from Σ by taking the inverse of the non-zero diagonal elements and leaving the zero elements in place. For this example, the singular value decomposition of H is given by

H=U Σ VH=1212-1212200012-121212

and

H+=V Σ+UH=1212-12121200012-121212=121-1-11 .

Observation 1:  The expression (HHH)-1HH is not the pseudo-inverse of a square matrix.

If the effective channel matrix H does not have full rank, then no matrix H+ exists such that

H+H=I

where I is the identity matrix. As a result, it will not be possible to separate the MIMO layers and EVM will be failed.

Observation 2:	Since the MIMO layers cannot be separated if the matrix H does not have full rank, there is no need for the pseudo-inverse.

If the effective channel matrix  H does have full rank, then the zero-forcing receiver is given by H-1, since

H-1 H=I .

If H has full rank, it can be noted that

(HHH)-1HH=H-1(HH)-1HH=H-1

so that the two expressions are equivalent.  However, the expression (HHH)-1HH requires two more matrix multiplications than the simple inverse H-1, and these two additional matrix multiplications are of no benefit and may result in a loss of accuracy.

Observation 3:	The zero-forcing receiver should be defined using the simple H-1 rather than using the expression (HHH)-1HH which is not the pseudo-inverse and requires two additional matrix multiplications.

Based on the above observations, we have the following proposal:

Proposal 1:  	For two-layer uplink MIMO in FR1, define the zero-forcing receiver as the inverse of the effective channel matrix so that

GZF=H-1

- Summary

In Section 5.2.3.1.1.2 of TR 38.884-120 on “Method 1 MIMO Equalization,” there is the following text:

“The ZF equalizer coefficients are calculated as pseudo inverse of effective channel matrix, in general:

GZF=H+=(HHH)-1HH ."

# In this contribution, the following observations have been made:

Observation 1:  The expression (HHH)-1HH is not the pseudo-inverse of a square matrix.

Observation 2:	Since the MIMO layers cannot be separated if the matrix H does not have full rank, there is no need for the pseudo-inverse.

Observation 3:	The zero-forcing receiver should be defined using the simple H-1 rather than using the expression (HHH)-1HH which is not the pseudo-inverse and requires two additional matrix multiplications.

Based on these observations, we have the following proposal:

Proposal 1:  	For two-layer uplink MIMO in FR1, define the zero-forcing receiver as the inverse of the effective channel matrix so that

GZF=H-1.

# References

- R4-2107113, ‘Discussion on FR1 UL MIMO EVM measurements’, Rohde and Schwartz, 3GPP TSG-RAN WG4 Meeting #101-e.

- R4-2118058, “Discussion on FR1 UL MIMO EVM measurement method,” Rohde and Schwartz, 3GPP TSG-RAN WG4 #101-e.

- R4-2203128, TR 38.884-120 Study on enhanced test methods for FR2 NR UEs, 3GPP TSG-RAN WG4 #101-bis-e.

- R4-2201990, “Issues with MIMO EVM Measurement Using the Pseudo-Inverse,” Lenovo, Motorola Mobility, 3GPP TSG-RAN WG4 Meeting #101-bis-e.

- R4-2119555, “Issues with FR2 MIMO EVM Measurement Using the Pseudo-Inverse,” Lenovo, Motorola Mobility, 3GPP TSG-RAN WG4 Meeting #101-e.