3GPP TSG-RAN WG4 Meeting # 102-e	R4-2206119

# Electronic Meeting, 21 Feb – 03 Mar, 2022

Source: 	Huawei

Title: 	Discussion on the remaining BS RF requirements for FR2-2: Tx requirements

Agenda Item:	10.16.4.1

Document for:	Discussion

# Introduction

During previous RAN4#101bis-e meeting, the following was agreed on the remaining TX requirements [1]:

|  | WF form last meeting (R4-2203016) |
| --- | --- |
| TAE | Contiguous intra-band CA (baseline assumption): 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS. RAN4 will make final decision on February RAN4#102-e meeting. |
|  | Non-contiguous intra-band CA: [260 ns]. (Note: Whether the requirement need to be specified in Rel-17 specification pending on further confirmation whether NC CA is within Rel-17 scope). |
|  | MIMO: Option 1 baseline assumption; RAN4 will make final decision on February RAN4#102-e meeting. - Option 1: 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS. - Option 2: 32.5 ns for 480 kHz, 16.25 ns for 960kHz SCS. |
| EVM | Re-use 50% of the normal CP EVM window length for FR-2-2 for 120 kHz SCS. FFS for 480kHz and 960kHz SCSs; RAN4 will make conclusion on February RAN4#102-e meeting. |
| OBUE | ΔfOBUE = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6-71GHz. - FFS for the upper bound of FDL,high – FDL,low. |

In this contribution, we provide feedback on the remaining BS RF Tx requirements for FR2-2.

# Discussion

Below we briefly summarize our views in Proposals 1-4.

|  | WF form last meeting (R4-2203016) | Huawei proposals |
| --- | --- | --- |
| TAE | Contiguous intra-band CA (baseline assumption): 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS. RAN4 will make final decision on February RAN4#102-e meeting. | Proposal 1: Confirm the TAE requirement for contiguous intra-band CA proposal from WF |
|  | Non-contiguous intra-band CA: [260 ns]. (Note: Whether the requirement need to be specified in Rel-17 specification pending on further confirmation whether NC CA is within Rel-17 scope). | Proposal 2: ok, but seems not needed in Rel-17 due to CA scope discussion. |
|  | MIMO: Option 1 baseline assumption; RAN4 will make final decision on February RAN4#102-e meeting. - Option 1: 65 ns for 480 kHz SCS, 32.5 ns for 960 kHz SCS. - Option 2: 32.5 ns for 480 kHz, 16.25 ns for 960kHz SCS. | Proposal 3: Option 1 |
| EVM | Re-use 50% of the normal CP EVM window length for FR-2-2 for 120 kHz SCS. FFS for 480kHz and 960kHz SCSs; RAN4 will make conclusion on February RAN4#102-e meeting. | FFS |
| OBUE | ΔfOBUE = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6-71GHz. - FFS for the upper bound of FDL,high – FDL,low. | Proposal 4: For the upper bound: suggest to limit it to the n263 bandwidth, and leave if for future if RAN4 would need any wider bands (if possible at all). |

# Conclusions

Consider the above Proposals 1 – 4 for the final decisions on the remaining BS RF Tx requirements for FR2-2.

# References

[1]		R4-2203016 		WF on BS RF Tx requirements, RAN4#101bis-e