3GPP TSG-RAN WG4 Meeting #102-e                                                    	R4-2206267

Online, 21st February – 3rd March, 2022

Source:	Vodafone

Title:	TP for TR 37.717-51-11: DC_7-20-28-32-38_n1

Agenda item:			9.18.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-51-11 to include DC_7-20-28-32-38_n1.

The TIB,c and RIB values provided are derived from the relevant subsets already present in 36101 and 38101-3.

# 2. Reference

# 3. Text Proposal

<Start of Text Proposal>

### 5.1.X	DC_7-20-28-32-38_n1

#### 5.1.X.1	Configuration for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (six bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-28A-32A-38A_n1A | DC_20A_n1A DC_28A_n1A |

#### 5.1.X.2	∆TIB and ∆RIB values

<Editor’s note: rows in ΔTIB,c andΔRIB tables shall be deleted if ΔTIB,c andΔRIB,c values are zero. >

Table 5.1.X.2-1: ΔTIB,c due to EN-DC (six bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-28-32-38_n1 | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.7 |

Table 5.1.X.2-2: ΔRIB,c due to EN-DC (six bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-20-28-32-38_n1 | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 38 | 0.2 |

<End of Text Proposal>