3GPP TSG-RAN WG4 Meeting #102-e	R4-2206275

Electronic Meeting, 21 February – 03 March 2022

Source:	Ericsson, Telstra

Title:	TP for TR 37.827 Addition of DC_3_n5-n78

Agenda item:			9.31.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.827 to add DC_3A_n5A-n78A, DC_3C_n5A-n78A.

# 2. Text Proposal

---Start of changes---

## 5.x	DC_3_n5-n78

### 5.x.1	Transmitter Characteristics

#### 5.x.1.1	Maximum Output Power

Table 5.x.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_3A_n78A | 266 | +2/-3 |
| DC_3C_n78A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 5.x.1.2	Configurations for EN-DC

Table 5.x.1.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A_n5A-n78A DC_3C_n5A-n78A | DC_3A_n78A DC_3C_n78A |

#### 5.x.1.2	Co-existence study

Co-existence studies shows that no IMD products will affect own Rx frequencies of band n5:

### 5.x.2	Receiver Characteristics

#### 5.x.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

Based on the co-existence studies there is no need to define MSD.

---End of changes---