3GPP TSG-RAN WG4 Meeting #102-e	R4-2206372

# Electronic meeting, February 21-March 3, 2022

Source:		Ericsson

Title:			WF on on system parameters for the 6 GHz licensed band

Agenda item:		9.3.2

Document for:		Approval

# System parameters

## Channel bandwidths and SCS

Agreement:

Following channel bandwidths and SCS shall be supported for the new 6GHz licensed band:

| NR band | SCS (kHz) | channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| [n104] | 15 |  |  |  | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  |  |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |

## Spectrum utilization

Agreement:

Reuse the existing FR1 spectral utilization for 6425-7125MHz.

## Channel raster and sync raster

Way Forward:

Companies are encouraged to further investigate the following 3 options, comparing pros and cons:

- Option 1:

  - Channel raster:

    - Legacy approach based on 15/30 kHz SCS:

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n104] | 15 | 828334 – <1> –875000 | 828334 – <1> –875000 |
|  | 30 | 828334 – <2> –875000 | 828334 – <2> –875000 |

  - Sync raster

    - SS block: 30kHz SCS– Case C pattern

    - Step size value from [1 to 7] (companies are also encouraged to indicate their preferred value, higher value than 7 is not precluded if justified).

- Option 2:

  - Channel raster:

    - 5 MHz resolution:

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n104] | 15 | 828667 – <333, 334> – 874667 | 828667 – <333, 334> – 874667 |

The ARFCN available for this band as follows

ARFCN0 = 828667

ARFCNn+1 = ARFCNn + 333 if n mod 3 ≠ 0

ARFCNn+1 = ARFCNn + 334 if n mod 3 = 0

ARFCN138 = 8874667

where n = {0, 1, 2, … 138}.

  - Sync raster

    - SS block: 30kHz SCS– Case C pattern

    - based on ~ 5MHz channel raster and the raster entries are FFS

- Option 3:

  - Channel raster:

    - 5 MHz resolution:

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n104] | 15 | 828667 – <333, 334> – 874667 | 828667 – <333, 334> – 874667 |

The ARFCN available for this band as follows

ARFCN0 = 828667

ARFCNn+1 = ARFCNn + 333 if n mod 3 ≠ 0

ARFCNn+1 = ARFCNn + 334 if n mod 3 = 0

ARFCN138 = 8874667

where n = {0, 1, 2, … 138}.

  - Sync raster

    - SS block: 30kHz SCS– Case C pattern

    - Step size value from [1 to 7] (companies are also encouraged to indicate their preferred value, higher value than 7 is not precluded if justified).

# References

- R4-2203919, System parameters for 6GHz licensed band, CATT

- R4-2204565, Discussion on system parameters for 6GHz licensed spectrum, CMCC

- R4-2205120, Discussion the remaining issues on system parameters for 6G license band, Xiaomi

- R4-2205145, System parameters for 6GHz NR licensed band, Huawei, HiSilicon, China Unicom

- R4-2205453, Discussion on system parameters for 6425-7125MHz, ZTE Corporation

- R4-2206102, Channel raster and sync raster for the 6 GHz licensed band, Qualcomm Incorporated

- R4-2206127, 6GHz licensed band system parameters, MediaTek (Chengdu) Inc.