3GPP TSG-RAN WG4 Meeting # 102-e 														            R4-2206409

Electronic Meeting, 1st February – 3rd March, 2022

Agenda item:			9.6 Issues arising from basket WIs but not subject to block approval

Source:	Dominique Brunel (Skyworks Solutions Inc.)

Title:	Round 2 Email discussion summary for [102-e][109] NR_Baskets_Part_1

Document for:	Information

# Introduction

Email discussion for contributions submitted under agenda item 9.6 “Issues arising from basket WIs but not subject to block approval” for UE RF and NR-U intra-band contiguous ULCA, and additional documents submitted to NR band related band combination baskets that require discussion.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Discussion and potential approval of CRs. Proposals for way forward.

- 2nd round: Finalization of CRs and way forwards.

Topics:

- LB-LB cases

- 3rd band MSD

- CA_n46-48-n96 and related fall backs

- Other  BC: fall-backs

- Triple beat and NC ULCA IMD issues

- NR-U contiguous UL CA

- Low MSD for CA and DC

- Documents moved from block approval (if applicable)

# Topic #1: LB-LB cases

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2204213 CA_n18-n28 and DC_18_n28 LB_LB_MSD | Qualcomm Incorporated | CA_n18-n28 MSD UL band; DL band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB). n18; n28; [31.1]; [28.4]. UL config UL band; DL band; SCS of UL band (kHz); 5 MHz; 10 MHz; 15 MHz. n18; n28; 15; 25; 25. DC_18_n28 MSD UL band; DL band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB); 25 MHz (dB); 30 MHz (dB). n18; n28; [31.1]; [28.4]; [26.6]; [23.9]; [12.0]. UL config UL band; DL band; SCS of UL band (kHz); 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz. n18; n28; 15; 25; 25; 25; 25; 25. |
| R4-2206140 Corrections for CA_n18-n28, DC_18_n18 MSD | Skyworks Solutions Inc. | Proposal 1: For DC_18_n28, remove the n28 frequency range restriction to 703-733 MHz for the UL and 758-788 MHz in Table 5.5B.4.1-1. Proposal 2: For CA_n18-n28, adopt MSD specifications captured in Table 7.3A.6-1 and Table 7.3A.6-2. Table 7.3A.6-1: Reference sensitivity exceptions (MSD) UL band; DL band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB). n18; n28; 31.4; 28.9. Table 7.3A.6.2: Uplink configuration UL band; DL band; SCS of UL band (kHz); 5 MHz; 10 MHz; 15 MHz. n18; n28; 15; 25; 25. Proposal 3: For DC_18_n28, adopt MSD specifications captured in Table 7.3B.2.3.4-1 and Table 7.3B.2.3.4-2: Table 7.3B.2.3.4-1: Reference sensitivity exceptions (MSD) UL band; DL band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB); 25 MHz (dB); 30 MHz (dB). 18; n28; 31.4; 28.9; 27.2; 24.6; 12.7. Table 7.3B.2.3.4-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1 UL band; DL band; SCS of UL band (kHz); 5 MHz (LCRB); 10 MHz (LCRB); 15 MHz (LCRB); 20 MHz (LCRB); 25 MHz (LCRB); 30 MHz (LCRB). 18; n28; 15; 25; 25; 25; 25; 25. |
| R4-2204680 Draft Correction CR to R17 TS38.101-1 on MSD for CA_n18-n28 | Samsung, KDDI, Skyworks Solutions Inc., Qualcomm | Moderator: CR to capture outcome of discussion of above two papers, comments on CR can be done directly in CR comment section, I suggest to merge input from MediaTek and add as co-author |
| R4-2204681 Draft Correction CR to R17 TS38.101-3 on MSD for DC_18_n28 | Samsung, KDDI, Skyworks Solutions Inc., Qualcomm | Moderator: CR to capture outcome of discussion of above two papers, comments on CR can be done directly in CR comment section |
| R4-2204480 Discussion on CA_n18_n28 | MediaTek Inc. | proposes to add note 2 to restrict applicable spectrum for n28: NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. The restriction also apply for any band combinations when CA_n20-n28 is a subset of a higher order band combination. |
| R4-2204482 draft CR to 38101-1-h40 improve note for CA_n18-n28 | MediaTek Inc. | Moderator: CR to capture proposed Note in R4-2204480, comments on CR can be done directly in CR comment section |
| R4-2206141 Corrections for CA_n5-n28 MSD | Skyworks Solutions Inc. | Proposal 1: For CA_n5-n28, remove the n28 frequency range restriction to 703-733 MHz for the UL and 758-788 MHz for the DL in Table 5.2A.2.1-1. Proposal 2: For CA_n5-n28, adopt MSD specifications captured in Table 7.3A.6-1 and Table 7.3A.6-2. Table 7.3A.6-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1 UL band; DL band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB); 25 MHz (dB); 30 MHz (dB). n5; n28; 17.5; 15.8; 14.0; 11.7; 2.9. Table 7.3A.6.2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1 UL band; DL band; SCS of UL band (kHz); 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz. n5; n28; 15; 20; 20; 20; 20; 20. |
| R4-2206134 CR to R17 TS38.101-1 on MSD for CA_n5-n28 | Skyworks Solutions Inc. | Moderator: CR to capture outcome of discussion of above two papers, comments on CR can be done directly in CR comment section |

## Open issues summary

### Sub-topic 1-1: CA and DC of band n18/18 and band n28

Issue 1-1a: spectrum restriction for n28

Proposals

- Skyworks: For DC_18_n28, remove the n28 frequency range restriction to 703-733 MHz for the UL and 758-788 MHz in Table 5.5B.4.1-1.

- MediaTek: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL.

Recommended WF

- Needs to be discussed but based on available spectrum in Japan, not based on operator’s holding

Issue 1-1b: CA_n18-n28 MSD

Proposals

- Both contributions are close for Qualcomm and Skyworks

|  | UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) |
| --- | --- | --- | --- | --- | --- |
| Qualcomm | n18 | n28 | [31.1] | [28.4] |  |
| Skyworks | n18 | n28 | 31.4 | 28.9 |  |
| Average |  |  | 31.3 | 28.7 |  |

Recommended WF

- Use averaged value between Qualcomm and Skyworks

Issue 1-1c: DC_18_n28 MSD

Proposals

- Both contributions are close for Qualcomm and Skyworks

|  | UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm | 18 | n28 | [31.1] | [28.4] | [26.6] | [23.9] |  | [12.0] |
| Skyworks | 18 | n28 | 31.4 | 28.9 | 27.2 | 24.6 |  | 12.7 |
| Average |  |  | 31.3 | 28.7 | 26.9 | 24.3 |  | 12.4 |

Recommended WF

- Use averaged value between Qualcomm and Skyworks

### Sub-topic 1-2: CA_n5-n28

Issue 1-2a: spectrum restriction for n28

Proposals

- For CA_n5-n28, remove the n28 frequency range restriction to 703-733 MHz for the UL and 758-788 MHz for the DL in Table 5.2A.2.1-1.

Recommended WF

- Since there is no difference in MSD for n28 full range this seems agreeable.

Issue 1-2b: CA_n5-n28 MSD

Proposals

- Skyworks input

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | n28 | 17.5 | 15.8 | 14.0 | 11.7 |  | 2.9 |

Recommended WF

- Unless there is disagreement by experts, MSD values are adopted

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1

| Company | Comments |
| --- | --- |
| KDDI | Issue 1-1a spectrum restriction for n28 The operating frequency of n28 in Japan is the same as TS38.101-1, UL: 703-748MHz DL: 758-803MHz. Issue 1-1b CA_n18-n28 MSD We agree with recommended WF. Issue 1-1c DC_18_n28 MSD We agree with recommended WF. |
| MediaTek | Issue 1-1a spectrum restriction for n28 Our understanding is The Japan operators own n28 spectrum higher than 788MHz does not have n18 spectrum. We think the n28A only restriction is still valid. Issue 1-1b CA_n18-n28 MSD Not sure if n28B filter rejection to n18 uplink can reach minimum 30~35dB. Need more time to check. We would like to hear more voice on filter rejection performance from UE/filter companies. Issue 1-1c DC_18_n28 MSD Same with issue 1-1b |
| Qualcomm | Issue 1-1a spectrum restriction for n28 We had the same view as MediaTek but considering the RAN plenary agreement about available spectrum Vs operator holding, the worst-case MSD may need to be analyzed assuming operation in the upper block. |
| Skyworks | Issue 1-1a spectrum restriction for n28 CA_n18-n28 is a KDDI specific combination in which KDDI the band n28 spectrum holding can be considered as either an n28A range or an n28B range. Even though n28 in Japan is the same as TS 38.101-1 range definition, our understanding is that the n28B range has been allocated to operators. We justify the n28B MSD analysis based on RAN WF RP-210639 that "Regional spectrum restriction is allowed with a clarification note, but not operator specific spectrum allocation". Issue 1-1b CA_n18-n28 MSD We agree with the moderator recommended WF. Issue 1-1c DC_18_n28 MSD We agree with the moderator recommended WF. To Mediatek: adopting MSD specifications for the n28B range does not preclude a UE from using the n28A range. |
| Huawei | Issue 1-1a spectrum restriction for n28 Should we consider two MSD sets respectively based n28A and n28B filter? At least the better performance can be recorded as an agreement with a specific condition if we specify only one MSD set for this band combination. |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2a spectrum restriction for n28 Issue 1-2b CA_n5-n28 MSD |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204680 Draft Correction CR to R17 TS38.101-1 on MSD for CA_n18-n28 | Skyworks: this CR can be revised to capture the proposed average MSD values. |
|  | Company B |
| R4-2204681 Draft Correction CR to R17 TS38.101-3 on MSD for DC_18_n28 | Skyworks: this CR can be revised to capture the proposed average MSD values. |
|  | Company B |
| R4-2206134 CR to R17 TS38.101-1 on MSD for CA_n5-n28 | Company A |
|  | Company B |
| R4-2204482 draft CR to 38101-1-h40 improve note for CA_n18-n28 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1.1 | Tentative agreements: Good agreement on worst case MSD based on band 28B assumption for Japan and based on regional spectrum approach as agreed in RAN. Candidate options: Use averaged MSD values from Qualcomm and Skyworks for MSD Recommendations for 2nd round: Discuss if lower MSD based on 28A should be captured on top. All agreements to be captured in revisions of Samsung CRs |
| Sub-topic #1.2 | Tentative agreements: No comments on discussion paper Candidate options: CR is agreeable and discussion paper is noted Recommendations for 2nd round: No further discussion in round 2 |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2204680 Draft Correction CR to R17 TS38.101-1 on MSD for CA_n18-n28 | To be revised to capture averaged MSD form Qualcomm and Skyworks and capture low MSD test points if agreed Moderator: MediaTek may be added as Co-author |
| R4-2204681 Draft Correction CR to R17 TS38.101-3 on MSD for DC_18_n28 | To be revised to capture averaged MSD form Qualcomm and Skyworks and capture low MSD test points if agreed Moderator: MediaTek may be added as Co-author |
| R4-2206134 CR to R17 TS38.101-1 on MSD for CA_n5-n28 | No comment: agreeable Moderator: author to cross check CR correctness in case revision is needed |
| R4-2204482 draft CR to 38101-1-h40 improve note for CA_n18-n28 | To be merged with revisions of R4-2204680 and R4-2204681 and capture low MSD test points if agreed |

## Discussion on 2nd round (if applicable)

Moderator: the addition of the lower MSD based on 28A spectrum is further discused in round 2

### Sub-topic 1-1: additional MSD test point for 28A

Issue 1-1: spectrum restriction for n28

Proposals

- Addition of lower MSD based on 28A spectrum on top of agreed 28B MSD

Recommended WF

- To be discussed

## Companies views’ collection

### Open issues

Sub topic 1-1

| Company | Comments |
| --- | --- |
| XXXX | Issue 1-1 spectrum restriction for n28 |
| Samsung | Thanks for all experts’ comments, we also in favour of two test points for n28A and n28B individually, please find our revision4 of R4-2204680/1 in Round2 folder. Thanks. https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_102-e/Inbox/Drafts/%5B102-e%5D%5B109%5D%20NR_Baskets_Part_1/Round%202 |
| MediaTek | After further offline discussion, we suggest to remove the 2nd MSD due to cross band isolation which is relevant to IMD5. Not sure if this is ok for everyone? |
| Qualcomm | The offline explanations are clear and note 7 is clear enough to protect legacy UE to meet requirement. |
| Skyworks | we agree with the latest revision with a single test point. A WF assigned to MediaTek is also available if companies feel that a second test point can be studied for next meeting |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2206378 Revision of R4-2204680 Draft Correction CR to R17 TS38.101-1 on MSD for CA_n18-n28 | Samsung: Please find the revision4 here https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_102-e/Inbox/Drafts/%5B102-e%5D%5B109%5D%20NR_Baskets_Part_1/Round%202 |
|  | Skyworks: we still have an issue with the spectrum note related to the lower MSD value. If we are fine to provide more information to operators depending on where the n28 spectrum sits and have two tets points, the allocated sectrum in Japan corresponds to 28B and thus a UE implementing 28B or 28F filter for Japan must be enabled to pass both test points. If further evaluation is needed we keep the value in bracket and seek for resolution in next meeting. |
|  | Skyworks: we agree with the latest revision with a single test point. |
| R4-2206379 Revision of R4-2204681 Draft Correction CR to R17 TS38.101-3 on MSD for DC_18_n28 | Samsung: Please find the revision4 here https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_102-e/Inbox/Drafts/%5B102-e%5D%5B109%5D%20NR_Baskets_Part_1/Round%202 |
|  | Skyworks: we still have an issue with the spectrum note related to the lower MSD value. If we are fine to provide more information to operators depending on where the n28 spectrum sits and have two tets points, the allocated sectrum in Japan corresponds to 28B and thus a UE implementing 28B or 28F filter for Japan must be enabled to pass both test points. If further evaluation is needed we keep the value in bracket and seek for resolution in next meeting. |
|  | Skyworks: we agree with the latest revision with a single test point. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2206378 Draft Correction CR to R17 TS38.101-1 on MSD for CA_n18-n28 | Agreeable, all experts have approved the latest revision shared |
| R4-2206379 Draft Correction CR to R17 TS38.101-3 on MSD for DC_18_n28 | Agreeable, all experts have approved the latest revision shared |
| R4-206584 WF on CA_n18- n28 and DC_18-n28 low MSD second test point | Agreeable, all experts have approved the latest revision shared |

# Topic #2: 3rd band MSD

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203623 Discussion on UE RF requirements for DC_8-28_n3 | VODAFONE Group Plc | This contribution is a draft text proposal for TR 37.717-21-11 to include DC_8-28_n3. Since this is a combination with two low bands and has an IMD2 issue, discussion is required to determine the appropriate UE RF requirements. |
| R4-2203624 Discussion on UE RF requirements for DC_8-28_n78 | VODAFONE Group Plc | This contribution is a draft text proposal for TR 37.717-21-11 to include DC_8-28_n78. Since this is a combination with two low bands and has both IMD4 and IMD5 issues, discussion is required to determine the appropriate UE RF requirements. |
| R4-2203625 Discussion on UE RF requirements for DC_8-32_n78 | VODAFONE Group Plc | This contribution is a draft text proposal for TR 37.717-21-11 to include DC_8-32_n78. Since this combination has an IMD3 hit in B32 as highlighted in section 5.x.4, input is needed on a suitable MSD test point. |
| R4-2203626 Discussion on UE RF requirements for DC_20-28_n78 | VODAFONE Group Plc | This contribution is a draft text proposal for TR 37.717-21-11 to include DC_20-28_n78. Since this is a combination with two low bands and has both IMD4 and IMD5 issues, discussion is required to determine the appropriate UE RF requirements. |
| R4-2203627 Discussion on UE RF requirements for DC_20-38_n8 | VODAFONE Group Plc | This contribution is a draft text proposal for TR 37.717-21-11 to include DC_20-38_n8. Since this is a combination with two low bands and has IMD3 issues, discussion is required to determine the appropriate UE RF requirements. |

## Open issues summary

### Sub-topic 2-1: DC_8-28_n3

Issue 2-1: test point and MSD

Proposals

- Test point required for IMD2 hit in band 28 for DC_8_n3 UL.

Recommended WF

- Experts to provide MSD test point and if possible MSD level

### Sub-topic 2-1: DC_8-28_n78

Issue 2-1: test point and MSD

Proposals

- Test points required for IMD5 with DC_28_n78 UL and IMD4 with DC_8_n78 UL.

Recommended WF

- Experts to provide MSD test point and if possible MSD level

### Sub-topic 2-1: DC_8-32_n78

Issue 2-1: test point and MSD

Proposals

- Test point required for band 32 IMD3 hit from DC_8_n78 UL.

Recommended WF

- Experts to provide MSD test point and if possible MSD level

### Sub-topic 2-1: DC_20-28_n78

Issue 2-1: test point and MSD

Proposals

- Test points required for band 28 IMD4 with DC_20_n78 UL and band 20 IMD5 with DC_28_n78 UL.

Recommended WF

- Experts to provide MSD test point and if possible MSD level

### Sub-topic 2-1: DC_20-38_n8

Issue 2-1: test point and MSD

Proposals

- MSD test points required for band 38 IMD3 hit due to DC_20_n8 UL and band 20 IMD3 hit due to DC_38_n8 UL.

Recommended WF

- Experts to provide MSD test point and if possible MSD level

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1 DC_8-28_n3

| Company | Comments |
| --- | --- |
| Qualcomm | Can use same MSD test point as DC_8A_n3A-n28A 8; 912.5; 5; 25; 957.5; N/A; N/A. 3; 1712.5; 5; 25; 1807.5; N/A; N/A. n28; 745; 5; 25; 800; 30.4; IMD2. |
| Skyworks | Question for clarification: is DC_28A_n3A the only requested uplink configuration for this EN-DC combination? If so, co-existence studies for UL DC_8_n3 are not needed, and test point for IMD2 hit in band 28 is not needed. |
| Apple | Agree with Skyworks that UL DC_8A_n3A is not specified in the TP. Need clarification as whether UL DC_8A_n3A is needed. For Band 8 2nd harmonic hit to n3A DL, the requirement should have been specified in fallback combination DC_8A_n3A. There is no need to replicate the MSD requirement table in the TP. |
| Qualcomm | The proponent of the TP needs to confirm the UL configurations of the band combination otherwise why are we discussing the DC_8_n3 in the subtopic. |

Sub topic 2-2 DC_8-28_n78

| Company | Comments |
| --- | --- |
| Qualcomm | Can use same IMD4 test point as DC_8A_n28A-n78A 8; 910; 5; 25; 955; N/A; N/A. 28; 710; 5; 25; 765; 11.6; IMD4. n78; 3495; 10; 50; 3495; N/A; N/A. Can use same IMD5 test point as DC_28A_n8A-n78A 28; 713; 5; 25; 768; N/A; N/A. 8; 890; 5; 25; 935; 4.3; IMD5. n78; 3787; 10; 50; 3787; N/A; N/A. |
| Skyworks | We were about to make same proposal. Agree with Qualcomm. |

Sub topic 2-3 DC_8-32_n78

| Company | Comments |
| --- | --- |
| Qualcomm | Can use same IMD3 test point as DC_8A-11A_n78A 8; 910; 5; 25; 955; N/A; N/A. n78; 3311; 10; 50; 3311; N/A; N/A. 32; N/A; 5; 25; 1491; 18.8; IMD3. |
| Skyworks | same comment as topic 2-2. |

Sub topic 2-4 DC_20-28_n78

| Company | Comments |
| --- | --- |
| Qualcomm | Can use same IMD4 test point as DC_20A_n28A-n78A 20; 837; 5; 25; 796; N/A; N/A. n78; 3310; 10; 50; 3310; N/A; N/A. 28; 744; 5; 25; 799; 9.4; IMD4. Can use similar MSD IMD5 as DC_28A_n8A-n78A 28; 713; 5; 25; 768; N/A; N/A. 20; 847; 5; 25; 806; 4.3; IMD5. n78; 3658; 10; 50; 3658; N/A; N/A. |
| Skyworks | same comment as topic 2-2. |

Sub topic 2-5 DC_20-38_n8

| Company | Comments |
| --- | --- |
| Qualcomm | Can use similar MSD IMD3 test points as DC_7A-20A_n8A 38; 2575; 5; 25; 2575; N/A; N/A. n8; 885; 5; 25; 930; N/A; N/A. 20; 846; 5; 25; 805; 17.4; IMD3. 38; 2580; 5; 25; 2580; 21.1; IMD3. n8; 900; 5; 25; 945; N/A; N/A. 20; 840; 5; 25; 799; N/A; N/A. 38; 2624; 5; 25; 2624; 18.8; IMD3. n8; 910; 5; 25; 955; N/A; N/A. 20; 857; 5; 25; 816; N/A; N/A. |
| Skyworks | A TP has already been proposed at last meeting and a CR has been endorsed. Please refer to R4-2202038 for TP and R4-2202278 for CR. |
| Apple | Is there a reason to have two IMD3 test points for Band 38 where the bottom one is outside of Band 38 frequency range? |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2.1 | Tentative agreements: If it is confirmed that DC_8A_n3A UL configuration is not needed Candidate options: Proponent captures agreement in TP W/Wo DC_8A_n3A related IMD2 MSD and accounts for Apples comment on H2 in fallback (check if captured) Recommendations for 2nd round: Proponent to confirm requested UL configuration and provide TP for revie |
| Sub-topic #2.2 | Tentative agreements: proposed IMD4 and IMD5 test points and MSD to be added in TP Candidate options: Recommendations for 2nd round: Proponent to provide TP for review |
| Sub-topic #2.3 | Tentative agreements: proposed IMD3 points and MSD to be added in TP Candidate options: Recommendations for 2nd round: Proponent to provide TP for review |
| Sub-topic #2.4 | Tentative agreements: proposed IMD4 and IMD5 test points and MSD to be added in TP Candidate options: Recommendations for 2nd round: Proponent to provide TP for review |
| Sub-topic #2.5 | Tentative agreements: BC already captured in in R4-2202038 for TP and R4-2202278 for CR. Candidate options: Recommendations for 2nd round: No further discussion |

## Discussion on 2nd round (if applicable)

Moderator: TPs from proponent capturing proposed MSDs are commented directly when available. We also recomend to proponent to consider adding Qualcomm as co-author for the TP as they provided the test points.

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2206389 TP for TR 37.717-21-11 for DC_8-28_n3 | Skyworks: This TP is agreeable. |
|  | Company B |
| R4-2206390 TP for TR 37.717-21-11 for DC_8-28_n78 | Skyworks: This TP is agreeable. |
|  | Company B |
| R4-2206391 TP for TR 37.717-21-11 for DC_8-32_n78 | Skyworks: This TP is agreeable. |
|  | Company B |
| R4-2206392 TP for TR 37.717-21-11 for DC_20-28_n78 | Skyworks: This TP is agreeable. |
|  | Company B |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2206389 TP for TR 37.717-21-11 for DC_8-28_n3 | Agreeable TPs uses proposed MSD from Qualcomm and agreed by experts |
| R4-2206390 TP for TR 37.717-21-11 for DC_8-28_n78 | Agreeable TPs uses proposed MSD from Qualcomm and agreed by experts |
| R4-2206391 TP for TR 37.717-21-11 for DC_8-32_n78 | Agreeable TPs uses proposed MSD from Qualcomm and agreed by experts |
| R4-2206392 TP for TR 37.717-21-11 for DC_20-28_n78 | Agreeable TPs uses proposed MSD from Qualcomm and agreed by experts |

# Topic #3: CA_n46-48-n96 and related fall backs

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2204214 CA_n46-n48-n96_Async_MSD | Qualcomm Incorporated | Proposal 1: Define exclusion region for n48 2nd harmonic as shown in Tables 2.1-2 using the existing specification framework Table 2.1-2: NR-U reference sensitivity measurement exclusion region in MHz. NR Band / Harmonic order / Channel BW in UL. Band; Harmonic order; 5MHz; 10MHz; 15MHz; 20 MHz; 40MHz. n48; 2; +/- 10; +/- 20; +/- 30; +/- 40; +/- 80. Proposal 2: Use harmonic mixing MSD for CA_n48-n96 fallback combination of CA_n46-n48-n96 as shown in Tables 2.2-1 and 2.2-2. Table 2.2-1: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1 NR Band / Channel bandwidth of the affected DL band (MHz) / MSD (dB). UL band; DL band; 5; 10; 15; 20; 25; 30; 40; 50; 60; 70; 80; 90; 100. n96; n482; 5.8; 3.7; 2.7; 2.2; 1.6; 1.2; 1.0; 0.8; 0.7; 0.6; 0.6; 0.5. Table 2.2-2: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1 Operating band / SCS / Channel bandwidth / Duplex-mode. Operating Band; SCS; 5; 10; 15; 20; 30; 40; 50; 60; 70; 80; 90; 100; Duplex. n96; 15; 25; 50; 75; 100; 100; 100; 100; 100; 100; 100; 100; 100; TDD. Proposal 3: Use cross band MSD for fallback combinations of CA_n46-n48-n96 as shown in Tables 2.3-1 and 2.3-2 and revisit the existing crossband noise between the fallback combination of CA_n46-n48. Table 2.3-1: MSD due to cross band isolation Operating Band / Channel bandwidth of the affected DL band. CA Configuration; UL band; DL band; 5; 10; 15; 20; 25; 30; 40; 50; 60; 80; 90; 100. CA_n46A-n48A; n46; n48; 13.3; 13.3; 11.8; 10.7; -; 9.4; 8.5; 7.9; 7.3; 7.0; 6.4; 6.2. n48; n46; -; -; -; 15.7; -; -; 15.7; -; 15.7; 15.7; -; -. CA_n48A-n96A; n96; n48; 4.3; 4.1; 4.0; 3.9; 3.9; 3.9; 3.9; 3.8; 3.8; 3.8; 3.8. n48; n96; 5.8; 5.6; 5.5; 5.5. Table 2.3-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation Operating Band / SCS / Channel bandwidth of the affected DL band. UL; DL; SCS; 5; 10; 15; 20; 25; 30; 40; 50; 60; 80; 90; 100. n46; n48; 30; 216; 216; 216; 216; 216; 216; 216; 216; 216; 216. n48; n46; 15; 216; 216; 216; 216. n96; n48; 30; 216; 216; 216; 216; 216; 216; 216; 216; 216; 216; 216. n48; n96; 15; 216; 216; 216; 216. |
| R4-2204090 On simultaneous Tx/Rx for constituents of CA_n46-n48-n96 | Skyworks Solutions Inc. | Proposal for CA_n46-n96: co-banding of n46 and n96 should be the baseline assumption and results of only supporting non-simultaneous Tx/Rx. As such, the TP introducing CA_n46-n96 should not include any MSD and should have the first table with following notes: - NOTE 9: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. (from 38.101-1 table 5.2A.2.1-1) - NOTE XX: The minimum requirements for inter-band CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable CA configuration is a subset of a higher order CA configuration. (derived from 38.101-3 Table 5.5B.4.1-1 Note 11 for co-banding) - NOTE YY: The combination is not used alone as fall back mode of other band combinations in which UL in Band n48 is not used. (derived from 38.101-3 Table 5.5B.4.1-1 Note 9 for UL configuration). Proposal for CA_n46-n48: - Simultaneous Tx/Rx is supported - Cross band MSD for n48 UL in n46 and cross band MSD of n46 UL in n48 are specified. Proposal for CA_n48-n96: - Simultaneous Tx/Rx is supported - Harmonic MSD of 2xn48 UL in n96 is specified - Harmonic mixing of 2xn48DL = n96 UL is specified - Cross band MSD for n48 UL in n96 and cross band MSD of n96 UL in n48 are specified if necessary. |
| R4-2203538 TP to TR 38.717.02-01 for CA_n46-n96 | Charter Communications, Inc | Moderator: TR to capture outcome of discussion of above two papers, comments on CR can be done directly in CR/TP comment section |
| R4-2205669 TP to TR 38.717.02-01 for CA_n48-n96 and DC_n48-n96 | Charter Communications, Inc | Moderator: TR to capture outcome of discussion of above two papers, comments on CR can be done directly in CR/TP comment section |
| R4-2203539 TP to TR TR38.717-03-01 for CA_n46-n48-n96 | Charter Communications, Inc | Moderator: TR to capture outcome of discussion of above two papers, comments on CR can be done directly in CR/TP comment section |
| R4-2203540 TP to TR 38.717.03-02 for CA_n46-n48--n96 | Charter Communications, Inc | Moderator: TR to capture outcome of discussion of above two papers, comments on CR can be done directly in CR/TP comment section |

## Open issues summary

### Sub-topic 3-1: CA_n46-n96

Issue 3-1: Non-simultaneous Tx/RX, Co-banding

Proposals

- BC does not exist as stand-alone fallback

- Non simultaneous Tx/Rx is assumed, no MSD

- Co-banding assumptions.

Recommended WF

- Agree non simultaneous Tx/Rx and co-banding assumptions and discuss Notes and TP directly

### Sub-topic 3-2: CA_n46-n48

Issue 3-2: Simultaneous Tx/Rx, cross band MSD

Proposals

- Simultaneous Tx/Rx is supported

- Cross band MSD for n48 UL in n46 and cross band MSD of n46 UL in n48 are specified.

- Cross band MSD

| Operating Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | UL band | DL band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |
| CA_n46A-n48A | n46 | n48 | 13.3 | 13.3 | 11.8 | 10.7 | - | 9.4 | 8.5 | 7.9 | 7.3 | 7.0 | 6.4 | 6.2 |
|  | n48 | n46 | - | - | - | 15.7 | - | - | 15.7 | - | 15.7 | 15.7 | - | - |

Recommended WF

- Agree Simultaneous Tx/Rx assumptions and cross band MSD

### Sub-topic 3-3: CA_n48-n96

Issue 3-3: Simultaneous Tx/Rx, Harmonic and Harmonic mixing, cross band MSD

Proposals

- Simultaneous Tx/Rx is supported

- Harmonic MSD of 2xn48 UL in n96 is specified

| NR Band / Harmonic order / Channel BW in UL |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | Harmonic order | 5MHz | 10MHz | 15MHz | 20 MHz | 40MHz |
| n48 | 2 | +/- 10 | +/- 20 | +/- 30 | +/- 40 | +/- 80 |

- Harmonic mixing of 2xn48DL = n96 UL is specified

| NR Band / Channel bandwidth of the affected DL band (MHz) / MSD (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| n96 | n482 | 5.8 | 3.7 | 2.7 | 2.2 |  | 1.6 | 1.2 | 1.0 | 0.8 | 0.7 | 0.6 | 0.6 | 0.5 |

- Cross band MSD for n48 UL in n96 and cross band MSD of n96 UL in n48 are specified if necessary.

| Operating Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | UL band | DL band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |
| CA_n48A-n96A | n96 | n48 | 4.3 | 4.1 | 4.0 | 3.9 |  | 3.9 | 3.9 | 3.9 | 3.8 | 3.8 | 3.8 | 3.8 |
|  | n48 | n96 |  |  |  | 5.8 |  |  | 5.6 |  | 5.5 | 5.5 |  |  |

Recommended WF

- Agree Simultaneous Tx/Rx assumptions H2 exception and harmonic + cross band MSD

## Companies views’ collection for 1st round

### Open issues

Sub topic 3-1 CA_n46-n96

| Company | Comments |
| --- | --- |
| Charter Communications Inc. | Agree with the recommended WF; non simultaneous Tx/Rx and co-banding assumptions and discuss Notes and TP directly |
| MediaTek | Agree with recommended WF |
| ZTE | Agree with recommended WF, Co-banding assumptions is ok due to no gap between the two bands. |
| Apple | Agree with recommended WF |

Sub topic 3-2 CA_n46-n48

| Company | Comments |
| --- | --- |
| Charter Communications Inc. | Agree with recommended WF; Simultaneous Tx/Rx assumptions and cross band MSD |
| MediaTek | Agree with recommended WF |
| Apple | Agree with recommended WF |

Sub topic 3-3 CA_n48-n96

| Company | Comments |
| --- | --- |
| Charter Communications Inc. | Agree with recommended WF; Simultaneous Tx/Rx assumptions H2 exception and harmonic + cross band MSD |
| MediaTek | Considering share path design on n46 and n96, we propose same values for MSD due to cross band isolation as CA_n46-n48: Operating Band / Channel bandwidth of the affected DL band. CA Configuration; UL band; DL band; 5; 10; 15; 20; 25; 30; 40; 50; 60; 80; 90; 100. CA_n48A-n96A; n96; n48; 13.3; 13.3; 11.8; 10.7; -; 9.4; 8.5; 7.9; 7.3; 7.0; 6.4; 6.2. n48; n96; -; -; -; 15.7; -; -; 15.7; -; 15.7; 15.7; -; -. We are fine with other MSD mechanism proposals |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2203538 TP to TR 38.717.02-01 for CA_n46-n96 | Charter Communications Inc. A revision of R4-2203538 has been uploaded for review supporting the agreement in 3-1 |
|  | Skyworks: based on draft revision review: sections 7XXX should not be part of this TP but in revision of 5669 |
|  | ZTE: For NOTE X, would it better to modify a bit: NOTE X: Simultaneous Rx/Tx capability does not apply for UEs supporting CA_n46-n96. Same restrictions are applied when applicable NR CA configuration is part of a to related higher order configurations In addition, similar with Skyworks, no need to include section 7.3G.5.X in this TP |
|  | Charter Communications Inc. A second revision has been uploaded with the changes identified by Skyworks and ZTE |
| R4-2205669 TP to TR 38.717.02-01 for CA_n48-n96 and DC_n48-n96 | Charter Communications Inc. A revision of R4-2205669 has been uploaded for review supporting the agreement in 3-2 and 3.3 |
|  | Skyworks: based on draft revision review: harmonic 2 exception of n48 in n96 missing. Harmonic mixing n48 H2 = n96 missing . there is still text saying non-simultaneous Tx/Rx for CA_n48-n96. There are CA_n46-n48 related cross band in CA_n48-n96 section. There is need for a specific CA_n46-n48 section to update cross band based on simultaneous Tx/Rx |
|  | MediaTek: Considering share path design on n46 and n96, we propose same values for MSD due to cross band isolation as CA_n46-n48: Operating Band / Channel bandwidth of the affected DL band. CA Configuration; UL band; DL band; 5; 10; 15; 20; 25; 30; 40; 50; 60; 80; 90; 100. CA_n48A-n96A; n96; n48; 13.3; 13.3; 11.8; 10.7; -; 9.4; 8.5; 7.9; 7.3; 7.0; 6.4; 6.2. n48; n96; -; -; -; 15.7; -; -; 15.7; -; 15.7; 15.7; -; -. |
|  | ZTE: This sentence is not correct: There are no harmonic or IMD overlaps as a result of non-simultaneous Tx/Rx. In terms of the issue 3-3, Simultaneous Tx/Rx is supported. A note should be added in table 6.X.1.1-1, like you did in 2203538. For the cross band isolation MSD tables in section 7.3G.5.X, it should be remove to section 6.X.1.5. |
|  | Charter Communications Inc. A second revision has been uploaded with the changes identified by Skyworks and ZTE |
| R4-2203539 TP to TR TR38.717-03-01 for CA_n46-n48-n96 | Charter Communications Inc. A revision of R4-2203539 has been uploaded for review supporting Qualcomm’s proposal 3 of R4-2204214 |
|  | Skyworks: based on draft revision review: there is no need for harmonic coex in 3band case. same for cross band: they should be in the 2 band fallback |
|  | ZTE: Similar with skyworks, no need to include cross band isolation for 1UL/3DL, it was already included in the 2DL fallback. |
|  | Charter Communications Inc. A second revision has been uploaded with the changes identified by Skyworks and ZTE |
| R4-2203540 TP to TR 38.717.03-02 for CA_n46-n48--n96 | Charter Communications Inc. A revision of R4-2203540 has been uploaded for review supporting Qualcomm’s proposal 3 of R4-2204214 |
|  | Skyworks: based on draft revision review: there is an added “-“ in the band combination in titles. Contiguous ULCA is not finalized for NR-U so UL configurations with n96B shall be removed. Text says “Based on above table, there is no harmonic interference.” In an IMD analysis section. No need for cross band: they should be in the 2 band fallback |
|  | ZTE: Please remove section 7.3G.5.X. In 2UL/3DL TR, only IMD MSD should be checked. |
|  | Charter Communications Inc. A second revision has been uploaded with the changes identified by Skyworks and ZTE |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #3.1 | Tentative agreements:Agreement to use non-simultaneous Tx/Rx and co-banding assumptions and BC no existing stand alone for CA_n46-n96 Candidate options: Recommendations for 2nd round:Revision of TP to capture agreements and associated notes and final review in round 2 |
| Sub-topic #3.2 | Tentative agreements:Agreement on simultaneous Tx/Rx and associated cross band MSD for CA_n46-n48 Candidate options: Recommendations for 2nd round: Revision of TP to capture agreements and final review in round 2 |
| Sub-topic #3.3 | Tentative agreements:Agreement on simultaneous Tx/Rx, n96 2xn48 harmonic exclusion, harmonic mixing and cross band MSD using MediaTek numbers for CA_n48-n96 Candidate options: Recommendations for 2nd round: Revision of TP to capture agreements and final review in round 2 |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2203538 TP to TR 38.717.02-01 for CA_n46-n96 | to be revised based on draft rev2 and further checks in Rd2 Moderator: consider adding companies that helped as co-author Charter Communications Inc Revision 2 of R4-2203538 was uploaded with all of the comments provided and is ready for final review. We also agree with moderator’s suggestion to consider adding companies that helped as co-authors. |
| R4-2205669 TP to TR 38.717.02-01 for CA_n48-n96 and DC_n48-n96 | to be revised based on draft rev2 but still needs to move 1UL MSDs to the 1UL chapters and remove bands that are not part of the combinations. further checks in Rd2 Moderator: consider adding companies that helped as co-author Charter Communications Inc Revision 3 of R4-2205669 was uploaded with all of the comments provided and is ready for final review. We also agree with moderator’s suggestion to consider adding companies that helped as co-authors. |
| R4-2203539 TP to TR TR38.717-03-01 for CA_n46-n48-n96 | to be revised based on draft rev2 but still needs to remove text on synch. further checks in Rd2 Moderator: consider adding companies that helped as co-author Charter Communications Inc Revision 3 of R4-2203539 was uploaded with all of the comments provided and is ready for final review. We also agree with moderator’s suggestion to consider adding companies that helped as co-authors. |
| R4-2203540 TP to TR 38.717.03-02 for CA_n46-n48--n96 | to be revised based on draft rev2 but still needs to remove text on synch. further checks in Rd2 Moderator: consider adding companies that helped as co-author Charter Communications Inc Revision 3 of R4-2203540 was uploaded with all of the comments provided and is ready for final review. We also agree with moderator’s suggestion to consider adding companies that helped as co-authors. |

## Discussion on 2nd round (if applicable)

Moderator: TPs from proponent capturing proposed MSDs are commented directly when available.

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2206380 Revision of R4-2203538 TP to TR 38.717.02-01 for CA_n46-n96 | Charter Communications Inc. Revision 2 of R4-2203538 was uploaded with all of the comments provided and is ready for final review. |
|  | Charter Communications Inc: Draft revision of R4-2206380 has been uploaded for final review |
|  | Skyworks: the sections 6.X.1.3 UE Co-existence studies says that there is no harmonic or harmonic mixing issue. In fact there are it is just that it is not an issue due to non-simultaneous Tx/Rx, the comment below tables should say: there are no harmonic/harmonic mixing issues due to non-simultaneous Tx/Rx |
|  | Charter Communications Inc: Thanks Skyworks for your feedback. Revision 2 of R4-2206380 has been uploaded with correction |
| R4-2206381 Revision of R4-2205669 TP to TR 38.717.02-01 for CA_n48-n96 and DC_n48-n96 | Charter Communications Inc. Revision 3 of R4-2203559 was uploaded with all of the comments provided and is ready for final review. |
|  | MediaTek: The revision 3 of R4-2203559 not yet address our comment for MSD due to cross band isolation for CA_n48-n96. We propose use same values as CA_n46-n48 |
|  | Charter Communications Inc. Charter Communications Inc: Draft revision of R4-2206381 has been uploaded for final review and includes Mediatek’s change |
|  | MediaTek2; Thanks for the Draft revision of R4-2206381. We are fine with it |
|  | Skyworks: the sections 6.X.1.3 UE Co-existence studies says that there is no harmonic or harmonic mixing issue where there are and they have MSD. this comments below the tables should be removed |
|  | Charter Communications Inc: Thanks Skyworks for your feedback. Revision 2 of R4-2206381 has been uploaded with correction |
| R4-2206382 Revision of R4-2203539 TP to TR TR38.717-03-01 for CA_n46-n48-n96 | Charter Communications Inc. Revision 3 of R4-2203539 was uploaded with all of the comments provided and is ready for final review. |
|  | Charter Communications Inc: Draft revision of R4-2206382 has been uploaded for final review |
|  | Skyworks: OK with rev 3 |
| R4-2206383 Revision of R4-2203540 TP to TR 38.717.03-02 for CA_n46-n48-n96 | Charter Communications Inc. Revision 3 of R4-2203540 was uploaded with all of the comments provided and is ready for final review. |
|  | Charter Communications Inc: Draft revision of R4-2206383 has been uploaded for final review |
|  | Skyworks: OK with rev 3 |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2206380 Revision of R4-2203538 TP to TR 38.717.02-01 for CA_n46-n96 | Agreeable TP addresses corrections based on expert’s review |
| R4-2206381 Revision of R4-2205669 TP to TR 38.717.02-01 for CA_n48-n96 and DC_n48-n96 | Agreeable TP addresses corrections based on expert’s review |
| R4-2206382 Revision of R4-2203539 TP to TR TR38.717-03-01 for CA_n46-n48-n96 | Agreeable TP addresses corrections based on expert’s review |
| R4-2206383 Revision of R4-2203540 TP to TR 38.717.03-02 for CA_n46-n48-n96 | Agreeable TP addresses corrections based on expert’s review |

# Topic #4: Other  BC: fall-backs

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203709 Issue of many missing fallbacks in 38.101 specifications | Apple | Proposal 1: RAN4 should add the missing fallbacks that don’t need extra specification work with a CR. Proposal 2: RAN4 should generate a list of missing fallbacks that cannot be added with a CR. These fallbacks need to be added to the band combination request sheet for the usual processing with the Proposal 3: After completing the steps in the proposals above, the higher order parent combinations of these still missing fallbacks will be removed from the 38.101-x specifications. Moderator: have a look to the attached tables of missing fallbacks |
| R4-2204216 DC_2_n25_Fallback_MSD | Qualcomm Incorporated | Proposal 1: If treating DC_2_n25 with UL only in n25 like intra-band DC_25_n25 with n25 UL only, then use ΔRIBNC as shown in Table 2.1-2. Use test points same as LTE CA_25A-25A and not include 3rd order effect. DC configuration; Aggregated bandwidth; Wgap / (MHz); UL NR allocation (LCRB); ΔRIBNC (dB); Duplex mode. NR; E-UTRA. DC_25A_n25A; 5MHz; 5MHz; 30.0 < Wgap ≤ 55.0; 101; 5.0; FDD. 0.0 < Wgap ≤ 30.0; 251; 0. 5MHz; 10MHz; 25.0 < Wgap ≤ 50.0; 101; 4.5. 0.0 < Wgap ≤ 25.0; 251; 0. 5MHz; 15MHz; 20.0 < Wgap ≤ 45.0; 101; 4.3. 0.0 < Wgap ≤ 20.0; 251; 0. 5MHz; 20MHz; 15.0 < Wgap ≤ 40.0; 101; 4.1. 0.0 < Wgap ≤ 15.0; 251; 0. 10MHz; 5MHz; 15.0 < Wgap ≤ 50.0; 10 (RBstart = 33); 5.5. 0.0 < Wgap ≤ 15.0; 321; 0. 10MHz; 10MHz; 10.0 < Wgap ≤ 45.0; 10 (RBstart = 33); 5.0. 0.0 < Wgap ≤ 10.0; 321; 0. 10MHz; 15MHz; 5.0 < Wgap ≤ 40.0; 10 (RBstart = 33); 4.5. 0.0 < Wgap ≤ 5.0; 321; 0. 10MHz; 20MHz; 0.0 < Wgap ≤ 35.0; 10 (RBstart = 33); 4.2. 15MHz; 5MHz; 10.0 < Wgap ≤ 45.0; 10 (RBstart = 44); 7.6. 0.0 < Wgap ≤ 10.0; 321; 0. 15MHz; 10MHz; 5.0 < Wgap ≤ 40.0; 10 (RBstart = 44); 6.7. 0.0 < Wgap ≤ 5.0; 321; 0. 15MHz; 15MHz; 0.0 < Wgap ≤ 35.0; 10 (RBstart = 44); 5.6. 15MHz; 20MHz; 0.0 < Wgap ≤ 30.0; 10 (RBstart = 44); 4.8. 20MHz; 5MHz; 0.0 < Wgap ≤ 40.0; 12 (RBstart = 62); 8.0. 20MHz; 10MHz; 0.0 < Wgap ≤ 35.0; 12 (RBstart = 62); 6.7. 20MHz; 15MHz; 0.0 < Wgap ≤ 30.0; 12 (RBstart = 62); 6.1. 20MHz; 20MHz; 0.0 < Wgap ≤ 25.0; 12 (RBstart = 62); 5.7. Proposal 2: If treating DC_2_n25 like inter-band ENDC combination with only n25 as uplink, then use MSD as shown in Table 2.2-1 and Tab le 2.2-2 to include 3rd order effect. Requirements are only required to be met with DL frames aligned within 3usec and PSD difference between carriers is 6dB. Table 2.2-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1 UL band; DL band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n25; 2; 33; 33; 33; 33. Table 2.2-1: UL configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1 UL band; DL band; SCS of UL band (kHz); 5 MHz (LCRB); 10 MHz (LCRB); 15 MHz (LCRB); 20 MHz (LCRB). n25; 2; 15; 40; 40; 40; 40. |
| R4-2205701 TP for TR 37.717-11-11 to include DC_2_n25 | Ericsson, Bell Mobility | Moderator: TP to capture outcome of discussion on DC_2_n25 MSD, comments on CR can be done directly in CR comment section |
| R4-2205702 TP for TR 37.717-21-11 to include DC_2-7_n25 | Ericsson, Bell Mobility | Moderator: only pending DC_2_n25, review TP directly |
| R4-2205703 TP for TR 37.717-31-11 to include DC_2-7-66_n25 | Ericsson, Bell Mobility | Moderator: only pending DC_2_n25, review TP directly |
| R4-2205704 TP for TR 37.717-31-11 to include DC_2-7-13_n25 | Ericsson, Bell Mobility | Moderator: only pending DC_2_n25, review TP directly |

## Open issues summary

### Sub-topic 4-1: Missing fallbacks

Issue 4-1:

Proposals

- RAN4 should add the missing fallbacks that don’t need extra specification work with a CR.

- RAN4 should generate a list of missing fallbacks that cannot be added with a CR. These fallbacks need to be added to the band combination request sheet for the usual processing with the

- After completing the steps in the proposals above, the higher order parent combinations of these still missing fallbacks will be removed from the 38.101-x specifications..

Recommended WF

- Discuss proposals and check the lists, ideally fallbacks without additional spec should be added with CR in this meeting

- Moderator: may be proponent can propose the two lists corresponding to the first two bullets

### Sub-topic 4-2: DC_2_n25

Issue 4-2:

Proposals

- Proposal 1: If treating DC_2_n25 with UL only in n25 like intra-band DC_25_n25 with n25 UL only, then use ΔRIBNC as shown in Table 2.1-2. Use test points same as LTE CA_25A-25A and not include 3rd order effect.Cross band MSD for n48 UL in n46 and cross band MSD of n46 UL in n48 are specified.

- Proposal 2: If treating DC_2_n25 like inter-band ENDC combination with only n25 as uplink, then use MSD as shown in Table 2.2-1 and Tab le 2.2-2 to include 3rd order effect. Requirements are only required to be met with DL frames aligned within 3usec and PSD difference between carriers is 6dB.

Recommended WF

- Discuss approaches and MSD

## Companies views’ collection for 1st round

### Open issues

Sub topic 4-1 Missing fallbacks

| Company | Comments |
| --- | --- |
| Nokia | Agree with proposals. hopefully proposal 3 is not needed and no combinations needs to be removed. CR for combinations that can be added without technical work would be good to get approved in this meeting, |
| SoftBank | Thanks Apple for the big efforts. We have the same view of Nokia. |
| Apple | We planned to have the CR in the next meeting based the March specifications. If Nokia generates the CR in this meeting, we are happy to agree the CR in this meeting. |
| AT&T | We agree with the proposal. Although, some of the missing fallbacks were completed in the January meeting. It would be good to revise the spreadsheet accordingly in the second round or treat it during post meeting email approval to collect the outcome of January and February meetings. In addition, it would be good to identify the list of missing fallbacks that cannot be added with a CR due to the need for additional specification work during the second round so that companies can add the band combinations to the combination request sheet for the revised WIDs for consideration at the March Plenary. We are OK with the Apple proposal to take the CR at the next meeting to give time for companies to assess and to ensure that combinations do not need to be removed. |

Sub topic 4-2 DC_2_n25

| Company | Comments |
| --- | --- |
| Skyworks | Thank you Qualcomm for providing detailed MSD analysis. We have a slight preference for the inter-band cross band isolation MSD because it is more generic than the intra-band MSD proposal. |
| Apple | Thanks to Qualcomm for providing the two framework options for consideration. We also have the preference to define the requirement based on inter-band MSD due to cross-band isolation. |
| Qualcomm | The MSD numbers will be in square brackets to allow other companies to contribute. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2205701 TP for TR 37.717-11-11 to include DC_2_n25 | Company A |
|  | Company B |
| R4-2205702 TP for TR 37.717-21-11 to include DC_2-7_n25 | Company A |
|  | Company B |
| R4-2205703 TP for TR 37.717-31-11 to include DC_2-7-66_n25 | Company A |
|  | Company B |
| R4-2205704 TP for TR 37.717-31-11 to include DC_2-7-13_n25 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #4.1 | Tentative agreements: Agreement that missing fallbacks are needed and add by CR if feasible and list remaining missing fallbacks eventually removing higher order BC that do not have Candidate options: Generate a WF that contains the list of missing fall backs that can be added with CR and the remaining ones in view of bringing CRs in May meeting. This will also allow to check if some missing fallbacks have not been provided during meeting 101b or 102. Recommendations for 2nd round: WF to Apple to provide the lists for the different specs based on their checks |
| Sub-topic #4.2 | Tentative agreements:Agreement that DC_2_n25 MSD can be based on cross band isolation framework Candidate options: Recommendations for 2nd round:Revision of TP to capture Qualcomm’s suggested cross band MSD |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2205701 TP for TR 37.717-11-11 to include DC_2_n25 | To be revised to capture cross band MSD based on Qualcomm’s input Moderator: consider adding Qualcomm as co-author for the TP |
| R4-2205702 TP for TR 37.717-21-11 to include DC_2-7_n25 | Return to, pending approval of the revision of R4-2205701 |
| R4-2205703 TP for TR 37.717-31-11 to include DC_2-7-66_n25 | Return to, pending approval of the revision of R4-2205701 |
| R4-2205704 TP for TR 37.717-31-11 to include DC_2-7-13_n25 | Return to, pending approval of the revision of R4-2205701 |

## Discussion on 2nd round (if applicable)

Moderator: Revised TP on DC_2_n25 is reviewed directly when available

### Sub-topic 4-1: Missing fallbacks

Issue 4-1:

Proposals

- Build list of missing fallbacks that can be introduced with a CR and the list of remaining missing fallbacks

Recommended WF

- Discuss and capture in WF on missing fall back list for 36.101 and 38.101-1 and -3

## Companies views’ collection

### Open issues

Sub topic 4-1

| Company | Comments |
| --- | --- |
| XXXX | Issue 4-1 Missing fallbacks |
| Nokia | Apple proposal for them to bring CR in next meeting is ok for us. |
| AT&T | We are OK with the draft Apple WF. |
| Skyworks | We support the way forward |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2206384 Revision of R4-2205701 TP for TR 37.717-11-11 to include DC_2_n25 | Ericsson: Please find updated TP based on 1st round comments: revision of R4-2205701 TP for TR 37 717-11-11 to include DC_2_n25 |
|  | Qualcomm: We are fine with the revision. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2206393 WF on missing fall back list for 36.101 and 38.101-1 and -3 | Agreeable WF provides guidelines on how RAN4 will resolve missing fallbacks for R17 |
| R4-2206384 TP for TR 37.717-11-11 to include DC_2_n25 | Agreeable TP addresses corrections |

# Topic #5: Triple beat and NC ULCA IMD issues

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2204217 Triple_Beat_MSD_update | Qualcomm Incorporated | Proposal 1: Based on observation 1, ignore 2nd order triple beat for 3_n41C, 25_n41C, and 8_n79C. Proposal 2: Use the worst-case power back off to calculate the triple beat MSD which implies a high-quality filter. The power back-off level per ULCC carrier will be 10*log10(3) = 4.78dB backoff per carrier without violating power class. Proposal 3: Use updated triple beat MSD per Table 2.5-1: Table 2.5-1: Triple beat MSD NR or E-UTRA Band / Channel bandwidth / NRB / MSD. EN-DC Configuration; EUTRA or NR band; UL Fc (MHz); UL/DL BW (MHz); UL LCRB; DL Fc (MHz); MSD (dB); Duplex mode; Triple beat order. DC_3A-n41C; 3; 1782.5; 5; 25; 1877.5; [13.4]; FDD; 1. n41C; 2555 2635; [80] [80]; [1 (RBstart=88)] [1 (RBstart=128)]; 2555 2635; N/A; TDD; N/A. DC_25A-n41C; 25; 1912.5; 5; 25; 1992.5; [18.0]; FDD; 1. n41C; 2545 2595; 90 100; 1 (RBstart=150) 1 (RBstart=122); 2545 2595; N/A; TDD; N/A. DC_8A-n79C; 8; 912.5; 5; 25; 957.5; [0]; FDD; 1. n79C; 4545 4645; 100 100; 1 (RBstart=212) 1 (RBstart=60); 4545 4645; N/A; TDD; N/A. |
| R4-2204483 draft CR to 38101-1-h40 missing MSD for CA_n5-n77(2A) | MediaTek Inc. | Table 7.3A.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA Band / Channel bandwidth / NRB / Duplex mode; Source of IMD. NR CA band combination; NR band; UL Fc (MHz); UL/DL BW (MHz); UL CLRB; DL Fc (MHz); MSD (dB); Duplex mode. CA_n5-n77; 5; N/A; 5; 25; 880; 27; FDD; IMD2. n77; 3310 4190; 10; 50; 3310 4190; N/A; TDD; N/A. |

## Open issues summary

### Sub-topic 5-1: Triple beat order

Issue 5-1:

Proposals

- Proposal 1: Based on observation 1, ignore 2nd order triple beat for 3_n41C, 25_n41C, and 8_n79C.

Recommended WF

- Discuss proposal

- Moderator: should we only look into 1st order triple beat only? (and remove order from table?)

### Sub-topic 5-2: back off for Triple beat

Issue 5-2:

Proposals

- Proposal 2: Use the worst-case power back off to calculate the triple beat MSD which implies a high-quality filter. The power back-off level per ULCC carrier will be 10*log10(3) = 4.78dB backoff per carrier without violating power class.

Recommended WF

- Discuss backoff: equal power per CC or equal power per band?

### Sub-topic 5-3: Triple beat MSD

Issue 5-3:

Proposals

- Proposal 3: Use updated triple beat MSD per Table 2.5-1:

Table 2.5-1: Triple beat MSD

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Triple beat order |
| DC_3A-n41C | 3 | 1782.5 | 5 | 25 | 1877.5 | [13.4] | FDD | 1 |
|  | n41C | 2555 2635 | [80] [80] | [1 (RBstart=88)] [1 (RBstart=128)] | 2555 2635 | N/A | TDD | N/A |
| DC_25A-n41C | 25 | 1912.5 | 5 | 25 | 1992.5 | [18.0] | FDD | 1 |
|  | n41C | 2545 2595 | 90 100 | 1 (RBstart=150) 1 (RBstart=122) | 2545 2595 | N/A | TDD | N/A |
| DC_8A-n79C | 8 | 912.5 | 5 | 25 | 957.5 | [0] | FDD | 1 |
|  | n79C | 4545 4645 | 100 100 | 1 (RBstart=212) 1 (RBstart=60) | 4545 4645 | N/A | TDD | N/A |

Recommended WF

- Discuss MSD and which cases are still valid to go in specification

### Sub-topic 5-4: IMD2 in LB from n77(2A)

Issue 5-4:

Moderator: new case for IMD or n77(2A) falling in LB

Proposals

- New MSD proposed

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n77 | 5 | N/A | 5 | 25 | 880 | 27 | FDD | IMD2 |
|  | n77 | 3310 4190 | 10 | 50 | 3310 4190 | N/A | TDD | N/A |

Recommended WF

- Discuss this new MSD type for IMD related to intra-band ULCA and see if it needs to be added to the specification framework

## Companies views’ collection for 1st round

### Open issues

Sub topic 5-1 Triple beat order

| Company | Comments |
| --- | --- |
| Skyworks | Thank you for providing the updated MSD analysis. We are in favor of focusing only on 1st order TB product for REL-17. For capture into TS 38.101-3, new table is not needed. The triple beat MSD test points can be specified in the current Table 7.3B.2.3.5.2-1. The IMD order could be captured as IMD3, other solutions are not precluded (like TB1 for 1st order TB product?). |
| Qualcomm | Current framework is fine |

Sub topic 5-2 back off for Triple beat

| Company | Comments |
| --- | --- |
| Skyworks | For EN-DC, up power control commands are sent to each CG, with LTE taking priority, with equal power per band. Then within the intra-band UL CA, we assume equal PSD per CC. For the example of DC_3_n41 measurements, we assumed 20dBm for LTE B3, 20dBm for NR n41, and equal PSD for each CC within band n41. |
| Qualcomm | For dynamic power sharing, the power need not be set equally per band. So, equal power per CC is possible. |

Sub topic 5-3 Triple beat MSD

| Company | Comments |
| --- | --- |
| Skyworks | We are fine to capture the MSDs for DC_3A-n41C and DC_8A-n79C in brackets []. To the best of our knowledge, DC_25A-n41C is not requested and so MSD test point is not needed. |
| Huawei | The UL frequency test point for DC_25A_n41C is not correct considering 90+100MHz. |
| Qualcomm | Can TMO confirm online that is combination is not required for release 17? If so, we can correct the testpoint. If not, we can remove. |

Sub topic 5-4 IMD2 in LB from n77(2A)

| Company | Comments |
| --- | --- |
| Skyworks | We want to thank MediaTek for bringing up this new issue of NC UL CA IMD2 falling on LB. We think it is useful to discuss this issue within the specification framework and guidelines for MSD due to IMD of intra-band UL CA: - With the current band definition, only n77, n46 and n96 have sufficient BW to generate an IMD2 in LB but n46 and n96 do not support NC ULCA yet. - Still there may be a question on which n77 BW may be applicable given the fact that currently NC UL CA maximum BW separation class is 600MHz (to cover Japan largest need). Given this, at least in release 17 we do not think such issue exists. |
| MediaTek | Thanks for valuable comments and further discussion. Agree that in Rel-17 such MSD issue does not exist. To make specs clear, the MSD due to IMD2 is listed with all values in N/A and update note 7. A revision of R4-2204483 is uploaded in the draft box. |
| Skyworks | Thank you for the revision, we are fine with the proposed changes. We would like to integrate this study into the guidelines we intended to provide at this meeting. We apologize for not having time to publish our views in due time and will contribute at next meeting. |
| Apple | Thanks to MediaTek for raising this issue which has the resemblance of n77 out of band blocking IMD2 issue when pairing with a low frequency band. We are not sure if we would endorse this draft CR in this meeting. If so, we notice that the draft CR does not seem to use the latest version of specifications as the CA_n5_n77 IMD test configuration does not look to be the same. On the other hand, we would prefer to use n77(2A) in the test configuration such that it is more clear why 2 UL frequencies are specified. |
| AT&T | Thanks MediaTek for the revision. We don’t think that the CA_n5A_n77(2A) should be shown in the first column as 38.101-1 does not list any combinations with higher order intra-band aspects to keep the tables generic. This aspect should be addressed based on our flag which indicated to place a superscript for Note 12 to n77 (as done for n2-n77 and n66-n77) since it is a test point for an intra-band non-contiguous uplink configuration. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204483 draft CR to 38101-1-h40 missing MSD for CA_n5-n77(2A) | MediaTek: A revision of R4-2204483 is uploaded in the draft box. MSD due to IMD2 is listed with all values in N/A and update note 7 |
|  | AT&T: Please see our comments on the revision in sub-topic 5-4. The revision is not in-line with the handling of similar combinations in the table. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #5.1 | Tentative agreements: agreed that first order TB is sufficient I.E it correspond to a 3 tone IMD3 Candidate options: Recommendations for 2nd round: Capture agreement in WF on triple beat |
| Sub-topic #5.2 | Tentative agreements: Still discussion whether the power split is based on equal power per band or equal power per CC. agreement that total power is at max. Candidate options: Further discuss in round 2 taking into account how the test may be performed Recommendations for 2nd round: Capture agreement in WF on triple beat |
| Sub-topic #5.3 | Tentative agreements:Triple beat MSD for DC_24A-n41C not needed as no request has been found in current basket. Test points and values are agreeable but need to agree on power split for future. Candidate options: Recommendations for 2nd round: Capture agreement in WF on triple beat and generate CR for 38.101-3 for the validated MSDs |
| Sub-topic #5.4 | Tentative agreements: Agreement that IMD2 MSD is N/A due to current regional spectrum limitations in n77 and the fact that maximum bandwidth separation class for NC ULCA is 600MHz Candidate options: Recommendations for 2nd round:CR is revised accordingly but needs to address the test point so that CA_n5-n77(2A) does not appear in first column (probably have all the test point value with N/A for MSD value only and check if note 7 is appropriate) |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2204483 draft CR to 38101-1-h40 missing MSD for CA_n5-n77(2A) | To be revised, revision shared in Rd1 as starting point. Note 7 needs further check and UL config needs clarification wo CA_n5-n77(2A) in first column |

## Discussion on 2nd round (if applicable)

### Sub-topic 5-1: back off for Triple beat

Issue 5-1:

Proposals

- Clarify if power split is based on equal power per band or equal power per CC

Recommended WF

- Discuss and capture in WF on capturing triple beat MSD in 38.101-1 and 38.101-3. All WF input to be discussed in 5-1

## Companies views’ collection

### Open issues

Sub topic 5-1

| Company | Comments |
| --- | --- |
| XXXX | Issue 5-1 back off for Triple beat and WF |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2206385 Revision of draft CR to 38101-1-h40 missing MSD for CA_n5-n77(2A) | AT&T: Thanks, MediaTek for the latest revision. We are OK with revision4 of the draft CR. Please add AT&T as co-sourcing company on the final version. |
|  | Skyworks: we support the last revision with the Note reffering to the bandwidth separation class constraint rather than operators spectrum |
| R4-2206395 CR to 38.101-3 to add triple beat MSD | Company A |
|  | Company B |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2206394 WF on capturing triple beat MSD in 38.101-1 and 38.101-3 | Agreeable WF provides guidelines for analysing and capturing triple beat cases |
| R4-2206395 CR to 38.101-3 to add triple beat MSD | Agreeable captures MSD for missing cases according to way forward |
| R4-2206385 Revision of draft CR to 38101-1-h40 missing MSD for CA_n5-n77(2A) | Agreeable Captures agreements for no IMD2 MSD |

# Topic #6: NR-U contiguous ULCA

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2206076 Proposals for NR-U Intraband Contiguous UL-CA requirements | Qualcomm Incorporated | Proposal 1: For full allocation, support tentatively extending NR-U single CC wideband MPR requirements to NR-U contiguous UL CA for the same aggregated total bandwidth (up to 80MHz aggregated BW), when the same waveform type and modulation order is used across the CCs. Proposal 2: If there are concerns over DFT-s-OFDM, consider extending NR-U single CC wideband requirements to contiguous UL-CA only for CP-OFDM at this stage. Proposal 3: If there are further concerns, discuss the introduction of a relaxation (e.g. 0.5 or 1 dB) for NR-U contiguous UL-CA MPR on top of existing NR-U single CC WB MPR requirements for full allocation. Proposal 4: For interlaced allocation, support tentatively extending NR-U single CC wideband MPR requirements to NR-U contiguous UL CA for the same aggregated total bandwidth (so up to 80MHz aggregated BW), when the same waveform type and modulation order is used across the CCs and the RB interlaces are aligned across different CCs with the minimum distance with respect to Single CC RB allocation; Proposal 5: If there are further concerns, discuss the introduction of a relaxation (e.g. 0.5 or 1 dB) for NR-U contiguous UL-CA MPR on top of existing NR-U single CC WB MPR requirements for interlaced allocation. |
| R4-2206138 MPR proposal for NR-U UL-CA | Skyworks Solutions Inc. | Proposal: Based on these observations, we propose to adopt: - 3.0dB MPR for all QPSK DFT-s-OFDM waveforms except for WB configuration [1111]+[1000] with full RB allocation for which [4.0]dB MPR is proposed; and - 4.0 MPR for all QPSK CP-OFDM waveforms except for WB configuration [1111]+[1000] with full RB allocation for which [5.5]dB MPR is proposed. |

## Open issues summary

### Sub-topic 6-1: MPR for NR-U ULCA

Issue 6-1:

Proposals

- Qualcomm proposes an approach were MPR is extended from single CC cases with potentially some 0.5/1dB relaxation

- Skyworks proposes MPR values based on measurements

  - 3.0dB MPR for all QPSK DFT-s-OFDM waveforms except for WB configuration [1111]+[1000] with full RB allocation for which [4.0]dB MPR is proposed; and

  - 4.0 MPR for all QPSK CP-OFDM waveforms except for WB configuration [1111]+[1000] with full RB allocation for which [5.5]dB MPR is proposed.

Recommended WF

- Compare proposals in terms of values w/wo relaxation to reach consensus on MPR table structure and possibly tentative values

## Companies views’ collection for 1st round

### Open issues

Sub topic 6-1 MPR for NR-U ULCA

| Company | Comments |
| --- | --- |
| Skyworks | Support moderator recommended WF. |
| Qualcomm | We support the WF. We have looked into the results submitted by Skyworks in their contribution for QPSK, and the comparison between results for UL CA and the corresponding wideband case is in accordance with our expectations. The proposed MPR values result in a more relaxed requirement than our own proposals but the scope proposed by Skyworks covers all the configuration included in the paper while ours only covered a subset. It seems reasonable for us to include Skywork’s proposal in the new MPR table, possibly using [] until other companies have further time to check. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #6.1 | Tentative agreements: Consensus that Skyworks input can be used to build MPR table in [] as more complete even if more relaxed than Qualcomm’s but based on measurements Candidate options: Recommendations for 2nd round: Generate R4-22xxxxx WF on NR-U contiguous ULCA MPR and CR to introduce in 38.101-1 |

## Discussion on 2nd round (if applicable)

### Sub-topic 6-1: NR-U UL CA MPR

Issue 6-1:

Proposals

- Build and MPR table proposal for NR-U ULCA

Recommended WF

- Discuss and capture in WF on NR-U contiguous ULCA MPR and CR to 38.101-1 to add NR-U contiguous UL CA MPR . All WF input to be discussed in topic 6-1 and CR commented directly

## Companies views’ collection

### Open issues

Sub topic 6-1

| Company | Comments |
| --- | --- |
| XXXX | Issue 6-1 MPR table proposal for NR-U ULCA |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2206397 CR to 38.101-1 to add NR-U contiguous UL CA MPR | Company A |
|  | Company B |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2206396 WF on NR-U contiguous ULCA MPR | Agreeable provides background and agreements for NRU ULCA MPR cases |
| R4-2206397 CR to 38.101-1 to add NR-U contiguous UL CA MPR | Agreeable captures MPR according to way forward |

# Topic #7: Low MSD for CA and DC

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2204088 On low MSD for CA and DC | Huawei, HiSilicon | Proposal 1: No need to have further discussion in Rel-17 for the low MSD for CA and DC |

## Open issues summary

### Sub-topic 7-1: Low MSD for CA and DC in R17

Issue 7-1:

Proposals

- No need to have further discussion in Rel-17 for the low MSD for CA and DC

Recommended WF

- Discuss proposal

- Moderator: our understanding is this was moving to R18 anyhow

## Companies views’ collection for 1st round

### Open issues

Sub topic 7-1 Low MSD for CA and DC in R17

| Company | Comments |
| --- | --- |
| Skyworks | In our view, low MSD is discussed again for Release 18 and given the status can’t see any further progress feasible in Release 17. Agree not to discuss further in R17 |
| MediaTek | Agree with the proposal. Since Rel-18 will discuss the topic, there’s no need to discuss it in R17 |
| AT&T | We think that we should follow the agreed way forward from RAN#94e which indicated that there would be no discussion in Q1 but any discussion in Q2 was to be further evaluated in March 2022 RAN#95e meeting as below. The following modified proposal 4 is agreed: - Stop discussion on “low MSD” in RAN4 meetings in Q1 2022. - Continue discuss the potential objectives for Rel-18 in February pre-RAN email discussion, and if the scope is stable further discuss whether the work can start from Q2 2022 in RAN March 2022. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #7.1 | Tentative agreements: Given the previous agreements the topic with be discussed in march RAN plenary Candidate options: Recommendations for 2nd round: No further discussion |

# Topic #8: Documents moved from block approval

Moderator Note: band combinations moved from #111/112 will be treated here or if they fall in one of the above topic it will be added to the topic.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2204736 TP for TR 37.717-11-11: Update MSD analysis of DC_(n)3AA | Huawei Technologies France | Only single switched UL mode is supported and UL-DL separation is not large (20 MHz), however no MSD is needed for the requested channel bandwidths. Flagged by Qualcomm and Skyworks |
| R4-2204806 TP for TR 37.717-21-11: Update DC_1A_(n)3AA | Huawei Technologies France | Moderator: Dependent on lower fallback in R4-2204736 |
| R4-2204754 TP for TR38.717-03-02: CA_n28A-n40A-n41A | ZTE Corporation | Moderator: flagged by Qualcomm on low IMD4 MSD |

## Open issues summary

### Sub-topic 8-1

Issue 8-1a: DC_(n)3AA

Proposals

- Only single switched UL mode is supported and UL-DL separation is not large (20 MHz), however no MSD is needed for the requested channel bandwidths

Recommended WF

- Proponent need to discuss with flagging companies to agree on MSD value

Issue 8-1b: CA_n28A-n40A-n41A

Proposals

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n40-n41 | n28 | 710 | 5 | 25 | 765 | 1.3 | FDD | IMD4 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |

Recommended WF

- Proponent need to discuss with flagging companies to agree on MSD value

## Companies views’ collection for 1st round

### Open issues

Sub topic 8-1

| Company | Comments |
| --- | --- |
| MediaTek | Issue 8-1a: If only single switched UL mode is supported, the “Uplink ENDC configurations” in table 1 shall be updated accordingly. In table 3, REFSENS is incorrect, REFSENS of LTE is -91dBm. MSD due to IMD5 when UL=n3,30MHz was not analyzed. |
| Moderator: Huawei? | @MediaTek: Thanks for the comment. Table one is an extract of Bandwidth combination requested in RP-212096. The Single switched uplink mode was decided based on Annex I 38.101-3. The REFSENS is updated to -91dBm, however still no MSD is required. Main; Div. LTE Rx BW; 20.000; 20.000. Receiver NF; 9.000; 9.000. TX Noise at RX band at the antenna; -89.000; -99.000. RX Thermal noise at the antenna; -92.447; -92.447. RX Total noise at the antenna; -87.380; -91.580. SNR; -3.620; 0.580. REFSENS; -91.000; -91.000. MRC RX total noise; -92.979. MSD; 2.479. SNR; 1.979. MSD at SNR=-1; 0.000. On the other hand, for the table 3 configuration Tx is indeed an NR TX with 30MHZ CBW and its noise is received at an LTE receiver with CBW=20MHz, as we mentioned “Table 3 contains the MSD analysis with a NR active Tx with a CBW of 30MHz and an inactive LTE Tx with a CBW of 20MHz.”. Also,Figure 1, corresponds to the case in Table 3. I will add a phrase mentioning that the table corresponds to the configuration in Figure 1 in the revised version. |
| MediaTek2 | In response to HW: Since the IMD5 of n3 uplink does not fall in the LTE DL receiving range, we can accept no MSD on LTE RX. As for single switched uplink, it needs to be indicated in Table 5.5B.2-1 of 38l101-3, not annex I, like below: Table 5.5B.2-1: Intra-band contiguous EN-DC configurations EN-DC configuration; Uplink EN-DC configuration (NOTE 1); Single UL allowed. DC_(n)3AA; DC_(n)3AA6; Yes6. DC_(n)5AA; DC_(n)5AA6; Yes6. |
| Qualcomm | MSD can be more significant with the 5th order distortion and RXBN at a lower duplex offset. We did RXBN measurements as well as CIM5 added to the intermodulation of CIM3 with the TX signal. All combinations were analyzed that can have potential MSD with a single NR uplink or a single EUTRA uplink. Other company views should be considered. We estimate 5.3dB MSD for the worst-case combination. MSD for all other possible combinations is listed in the table below. |
| Skyworks | Thank you for providing detailed MSD analysis. Our views is to study the MSD according to agreed WF R4-2107805 guidelines for intra-band contiguous EN-DC with single UL transmission MSD studies, i.e: - For each distinct Uplink band (aggressor) CBW, only specify the worst-case (highest) MSD test point. This corresponds to the collision of the lowest victim’s CBW which experiences the greatest overlap with the lowest IMD order leading to highest MSD ; - Test points for which the anchor point MSD is less than 0.5 dB do not need to be specified; - Only specify single UL REFSENS test points where NR is the Uplink Band and LTE is the Downlink affected band. For DC_(n)3AA, the TP presents analysis for NR UL = 30MHz CBW and DL LTE = 20MHz, let’s denote this case as N30L20. For DC_(n)3AA it is not clear which of these four configurations leads to worst case MSD: - N30L20: N30 IM5 is fully centered on L20 for small N30 Lcrb, example Lcrb=5, Rbstart=155. - N30L15: affected L15 may integrate the entire N30 IM5 PSD. This collision is similar to DC_(n)5AA with single UL Tx in n5 15MHz affecting DL LTE 10MHz for which the agreed MSD is 5.2dB. - N30L10: N30 IM5 partially overlaps the affected DL L10, - N30L05: N30 IM5 is adjacent to the affected DL L05. We suggest to capture MSD test points in round 2 via possible WF for FFS. Based on similarities with DC_(n)5AA, approximately [5.2]dB may be expected, MSD test point proposal could be refined at round 2. |
| Apple | We agree with Qualcomm and Skyworks that we need to take a closer look on the MSD analysis. Even with 50RB UL allocation, the sideband noise leaking into LTE carrier may not yet reach PA noise floor due to reduced duplex distance as compared to single band n3 with 30MHz CBW. |
| Qualcomm | Perhaps we can agree to test points in the current specification framework as a WF and get more thorough analysis next meeting. |

Sub topic 8-1b

| Company | Comments |
| --- | --- |
| Qualcomm | Issue 8-1b: Forward IMD4 products dominate in this band combination. Assume 60dB PA->LNA and PA->PA isolation and making sure IMD4 is measured at the PA output pin since PA is combined with front end filter as part of a module. The filter to reject IMD4 from the HB PA outputs is 35dB + 15dB from any N plexer =50dB. Our estimated MSD is 7.6dB |
| Skyworks | We need to come back at round 2 to verify this combination. |
| ZTE | Thanks Qualcomm providing the estimation. Much appreciate. Looking at the calculation, I have two soft questions: 1: Why the IMD4 at LNA for PRX and DRX are the same? In my understanding, there is 10dB(Ant. isolation) between PRX and DRX, so generally the IMD at LNA @DRX would be smaller than @PRX. 2: For the last two steps, by using -89.2dBm @PRX and @DRX, i could not got -90.9dBm MRC sensitivy by using the non-correlation MRC fomula: 2*(A*B)/(A+B), where A= B = 10*log10(10^(-89.2/10)), i got -93.2dBm@Ant, which is 5.3dB MSD. If considering IMD at LNA @DRX is smaller than @PRX, then smaller MSD would be got. |
| Qualcomm | To Skyworks, this calculation is a very quick IMD4 PA measurement in the lab and a preliminary MSD estimate. Other companies should present their views. Also, we can come with a more detailed assumption of architecture and parameters. To ZTE, the assumption was that B28 Primary RX can share same antenna as n40 and B28 diversity antenna can share same antenna as n41. The forward IMD4 dominates, and we can assume the equal IMD4 from each PA gets the same antennation to the respective LNAs. We can provide more detail in the 2nd round or part of the WF for the next meeting, but you can also have different PRX/DRX levels if the filtering path is different. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204736 TP for TR 37.717-11-11: Update MSD analysis of DC_(n)3AA | Company A |
|  | Company B |
| R4-2204806 TP for TR 37.717-21-11: Update DC_1A_(n)3AA | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #8.1a | Tentative agreements: Agreement that this intra-band ENDC case needs further discussion and especially IMD5 related MSD Candidate options: discuss MSD test point to be studied and capture in WF for evaluation for next meeting. If tentative value is found, it can be captured in revised TP Recommendations for 2nd round: Discuss test point in WF and/or revised TP if MSD value available |
| Sub-topic #8.1b | Tentative agreements:IMD4 based MSD needs further discussion Candidate options: discuss MSD test point to be studied and capture in WF for evaluation for next meeting. If tentative value is found, it can be captured in revised TP Recommendations for 2nd round: Discuss test point in WF and/or revised TP if MSD value available |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2204736 TP for TR 37.717-11-11: Update MSD analysis of DC_(n)3AA | To be revised, Need to capture Rd2 agreements and if MSD number available. WF also assigned in case not complete and evaluation of MSD is needed for next meeting |
| R4-2204806 TP for TR 37.717-21-11: Update DC_1A_(n)3AA | Return to, can be approved if revision of R4-2204736 is approved |
| R4-2204754 TP for TR38.717-03-02: CA_n28A-n40A-n41A | To be revised, Need to capture Round2 agreements on IMD4 MSD. WF also assigned in case not complete and evaluation of MSD is needed for next meeting |

## Discussion on 2nd round (if applicable)

### Sub-topic 8-1:

Issue 8-1: IMD5 MSD for DC_(n)3AA

Proposals

- Evaluate the need for IMD5 MSD

Recommended WF

- Discuss test point in WF and/or revised TP if MSD value available

Issue 8-1b: IMD4 MSD for CA_n28A-n40A-n41A

Proposals

- Evaluate IMD4 MSD value

Recommended WF

- Discuss test point in WF and/or revised TP if MSD value available

## Companies views’ collection

### Open issues

Sub topic 8-1a

| Company | Comments |
| --- | --- |
| XXXX | Issue 8-1a IMD5 MSD for DC_(n)3AA |
| Huawei (Mohammad) | @Skyworks Based on the categories that you mentioned(5RB155), here are the four configurations for N30L20, N30L15, N30L10, N30L5. Based on our calculations N30L15 N30L10: Greatest overlap with lowest IM (IM5) N30L5 None of them require MSD. On the other hand we need to establish a fixed way to set the RB_start and LCRB for test points. For example in WF R4-2105338 Slide 2, three options of RB allocations for the test points are proposed without a final decision on which to choose. We selected 50RB closest to the DL band in order to have the same configuration as the n3 REFSENS studies. We would also like to mention that your study (R4-1802205) showed that the no MSD was needed for n3 with band CBW=20,25,30MHz . @Qualcomm, Apple We are not against having couple of dBs on the MSD value to relax the conditions for UE manufacturers. However based on the discussions brought by different companies, it might be better to set the test points precisely (NxxLyy, RB start, LCRB) so companies have comparable results. |
| Skyworks | To Huawei: We can not agree to 0dB MSD. This needs to be further checked, we can come back at next meeting with our evaluation results. |
| Huawei (Mohammad) | @Skyworks: Any suggestion about the test point configurations (NxxLyy, RB start, LCRB)? |

Sub topic 8-1b

| Company | Comments |
| --- | --- |
| XXXX | Issue 8-1b IMD4 MSD for CA_n28A-n40A-n41A |
| Qualcomm | To ZTE regarding the MRC calculation: We do not dispute the calculation, but our assumption is that IMD4 interference on both antenna ports is correlated. Hence the MSD is higher. If you assume interference is uncorrelated as you do with thermal noise, then the MSD will be lower because you can take advantage of the diversity gain. This is not the worst-case scenario. |
| ZTE | To Qualcomm, yes, different RF architecture would cause different values. As discussed offline, the draft WF is available in the folder: https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_102-e/Inbox/Drafts/%5B102-e%5D%5B109%5D%20NR_Baskets_Part_1/Round%202/draft%20R4-2206399%20WF%20on%20MSD%20for%20CA_n28A-n40A-n41A.docx |
| Qualcomm | Added statement to study the feasibility of 4RX operation for this band combination in revision 3. It is acceptable WF after this comment. |
| Skyworks | Thank you for bringing this WF. We support the WF. We intend to check the RF-FE architecture assumptions on how to support 4Rx for this band combination and evaluation of MSD analysis at next meeting. |

### CRs/TPs comments collection

Major close to finalize Wis and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2206386 Revision of R4-2204736 TP for TR 37.717-11-11: Update MSD analysis of DC_(n)3AA | Company A |
|  | Company B |
| R4-2206250 Revision of R4-2204754 TP for TR38.717-03-02: CA_n28A-n40A-n41A |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2206399 WF on IMD4 MSD for CA_n28A-n40A-n41A | WF captures IMD4 test points for MSD evaluation and architecture discussions in next meeting |
| R4-2206250 TP for TR38.717-03-02: CA_n28A-n40A-n41A | Has captured test point from WF but can be finalyzed at next meeting based on inputs from companies on architecture |
| R4-2206398WF on MSD for DC_(n)3AA | withdrawn: no draft provided |
| R4-2206386 TP for TR 37.717-11-11: Update MSD analysis of DC_(n)3AA | postponed no agreement on IMD 5 test point and values – Moderator: companies are still invited to try to align work for next meeting offline |
| R4-2204806 TP for TR 37.717-21-11: Update DC_1A_(n)3AA | pending agreement on fallback DC_(n)3AA |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| R4-2206378 Revision of R4-2204680 Draft Correction CR to R17 TS38.101-1 on MSD for CA_n18-n28 | Samsung, KDDI, Skyworks Solutions Inc., Qualcomm, MediaTek | Needs to include averaged value for worst case MSD and may need to capture lower MSD test point based on Round2 agreements. |
| R4-2206379 Revision of R4-2204681 Draft Correction CR to R17 TS38.101-3 on MSD for DC_18_n28 | Samsung, KDDI, Skyworks Solutions Inc., Qualcomm, MediaTek |  |
| R4-2206389 TP for TR 37.717-21-11 for DC_8-28_n3 | VODAFONE Group Plc | Move to TP needs clarification if UL configuration DC_8_n3 is needed or not and thus the associated MSD |
| R4-2206390 TP for TR 37.717-21-11 for DC_8-28_n78 | VODAFONE Group Plc | Move to TP proposed IMD4 and IMD5 test points and MSD to be added |
| R4-2206391 TP for TR 37.717-21-11 for DC_8-32_n78 | VODAFONE Group Plc | Move to TP proposed IMD3 points and MSD to be added |
| R4-2206392 TP for TR 37.717-21-11 for DC_20-28_n78 | VODAFONE Group Plc | Move to TP proposed IMD4 and IMD5 test points and MSD to be added |
| R4-2206380 Revision of R4-2203538 TP to TR 38.717.02-01 for CA_n46-n96 | Charter Communications, Inc | revision based on draft rev2 and further checks in Rd2 |
| R4-2206381 Revision of R4-2205669 TP to TR 38.717.02-01 for CA_n48-n96 and DC_n48-n96 | Charter Communications, Inc | revision based on draft rev2 but still needs to move 1UL MSDs to the 1UL chapters and remove bands that are not part of the combinations. Further checks in Rd2 |
| R4-2206382 Revision of R4-2203539 TP to TR TR38.717-03-01 for CA_n46-n48-n96 | Charter Communications, Inc | revision based on draft rev2 but still needs to remove text on synch. Further checks in Rd2 |
| R4-2206383 Revision of R4-2203540 TP to TR 38.717.03-02 for CA_n46-n48-n96 | Charter Communications, Inc | revision based on draft rev2 but still needs to remove text on synch. Further checks in Rd2 |
| R4-2206393 WF on missing fall back list for 36.101 and 38.101-1 and -3 | Apple | WF to capture agreements on CRs to be generated and capturing the list of missing fall backs |
| R4-2206384 Revision of R4-2205701 TP for TR 37.717-11-11 to include DC_2_n25 | Ericsson, Bell Mobility | Capture cross band MSD based on Qualcomm’s input |
| R4-2206394 WF on capturing triple beat MSD in 38.101-1 and 38.101-3 | Skyworks, Qualcomm | WF to capture agreement on only first order TB and table template |
| R4-2206395 CR to 38.101-3 to add triple beat MSD | Qualcomm Incorporated | MSDs to be captured in CR for requested UL configurations only |
| R4-2206385 Revision of draft CR to 38101-1-h40 missing MSD for CA_n5-n77(2A) | MediaTek Inc. | Revision shared in Rd1 as starting point. Note 7 needs further check and UL config needs clarification wo CA_n5-n77(2A) in first column |
| R4-2206396 WF on NR-U contiguous ULCA MPR | Skyworks, Qualcomm | Capturing consensus between Qualcomm and Skyworks input |
| R4-2206397 CR to 38.101-1 to add NR-U contiguous UL CA MPR | Qualcomm, Skyworks | Capturing NRU UL CA MPR table in 38.101-1 |
| R4-2206398WF on MSD for DC_(n)3AA | Huawei Technologies France | Capturing test points to be specified together with experts, MSD values if any |
| R4-2206386 Revision of R4-2204736 TP for TR 37.717-11-11: Update MSD analysis of DC_(n)3AA | Huawei Technologies France | If MSD values are available this meeting they can be captured here, if not it will need to be postponed. |
| R4-2206399 WF on IMD4 MSD for CA_n28A-n40A-n41A | ZTE Corporation | Capturing test points to be specified together with experts, MSD values if any |
| R4-2206250 Revision of R4-2204754 TP for TR38.717-03-02: CA_n28A-n40A-n41A | ZTE Corporation | If MSD values are available this meeting they can be captured here, if not it will need to be postponed. |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2204213 | CA_n18-n28 and DC_18_n28 LB_LB_MSD | Qualcomm Incorporated | noted | MSD numbers can be averaged with Skyworks |
| R4-2206140 | Corrections for CA_n18-n28, DC_18_n18 MSD | Skyworks Solutions Inc. | noted | MSD numbers can be averaged with Qualcomm |
| R4-2204680 | Draft Correction CR to R17 TS38.101-1 on MSD for CA_n18-n28 | Samsung, KDDI, Skyworks Solutions Inc., Qualcomm | Revised, MediaTek added as co-author | Needs to include averaged value for worst case MSD and may need to capture lower MSD test point based on Round2 agreements. |
| R4-2204681 | Draft Correction CR to R17 TS38.101-3 on MSD for DC_18_n28 | Samsung, KDDI, Skyworks Solutions Inc., Qualcomm | Revised, MediaTek added as co-author |  |
| R4-2204480 | Discussion on CA_n18_n28 | MediaTek Inc. | Noted | lower MSD number may be added as second test point in CR revision |
| R4-2204482 | draft CR to 38101-1-h40 improve note for CA_n18-n28 | MediaTek Inc. | merged | merged with revision of 4680 and 4681 |
| R4-2206141 | Corrections for CA_n5-n28 MSD | Skyworks Solutions Inc. | noted | MSD numbers agreeable and already in CR |
| R4-2206134 | CR to R17 TS38.101-1 on MSD for CA_n5-n28 | Skyworks Solutions Inc. | Agreeable | No comment on MSD numbers |
| R4-2203623 | Discussion on UE RF requirements for DC_8-28_n3 | VODAFONE Group Plc | noted | Move to TP needs clarification if UL configuration DC_8_n3 is needed or not and thus the associated MSD |
| R4-2203624 | Discussion on UE RF requirements for DC_8-28_n78 | VODAFONE Group Plc | noted | Move to TP proposed IMD4 and IMD5 test points and MSD to be added |
| R4-2203625 | Discussion on UE RF requirements for DC_8-32_n78 | VODAFONE Group Plc | noted | Move to TP proposed IMD3 points and MSD to be added |
| R4-2203626 | Discussion on UE RF requirements for DC_20-28_n78 | VODAFONE Group Plc | noted | Move to TP proposed IMD4 and IMD5 test points and MSD to be added |
| R4-2203627 | Discussion on UE RF requirements for DC_20-38_n8 | VODAFONE Group Plc | noted | Already captured in R4-2202038 for TP and R4-2202278 for CR. |
| R4-2204214 | CA_n46-n48-n96_Async_MSD | Qualcomm Incorporated | Noted | proposed MSDs values to be captured in TPs |
| R4-2203538 | TP to TR 38.717.02-01 for CA_n46-n96 | Charter Communications, Inc | Revised | revision based on draft rev2 and further checks in Rd2 |
| R4-2205669 | TP to TR 38.717.02-01 for CA_n48-n96 and DC_n48-n96 | Charter Communications, Inc | Revised | revision based on draft rev2 but still needs to move 1UL MSDs to the 1UL chapters and remove bands that are not part of the combinations. Further checks in Rd2 |
| R4-2203539 | TP to TR TR38.717-03-01 for CA_n46-n48-n96 | Charter Communications, Inc | Revised | revision based on draft rev2 but still needs to remove text on synch. Further checks in Rd2 |
| R4-2203540 | TP to TR 38.717.03-02 for CA_n46-n48—n96 | Charter Communications, Inc | Revised | revision based on draft rev2 but still needs to remove text on synch. Further checks in Rd2 |
| R4-2204090 | On simultaneous Tx/Rx for constituents of CA_n46-n48-n96 | Skyworks Solutions Inc. | Noted | Recommendation on simultaneous Tx/Rx and MSD issues to be captured in TPs |
| R4-2203709 | Issue of many missing fallbacks in 38.101 specifications | Apple | Noted | Generate a WF for the list of missing fallbacks that can be added with CR and those that can’t. CR to be brought in May meeting based on latest specs. To chairman, is it acceptable to have the list as an embedded excel file? |
| R4-2204216 | DC_2_n25_Fallback_MSD | Qualcomm Incorporated | Noted | consensus to use the cross band isolation framework to capture the MSD |
| R4-2205701 | TP for TR 37.717-11-11 to include DC_2_n25 | Ericsson, Bell Mobility | Revised | Capture cross band MSD based on Qualcomm’s input |
| R4-2205702 | TP for TR 37.717-21-11 to include DC_2-7_n25 | Ericsson, Bell Mobility | Return to | pending approval of the revision of R4-2205701 |
| R4-2205703 | TP for TR 37.717-31-11 to include DC_2-7-66_n25 | Ericsson, Bell Mobility | Return to | pending approval of the revision of R4-2205701 |
| R4-2205704 | TP for TR 37.717-31-11 to include DC_2-7-13_n25 | Ericsson, Bell Mobility | Return to | pending approval of the revision of R4-2205701 |
| R4-2204217 | Triple_Beat_MSD_update | Qualcomm Incorporated | Noted | Proposed MSDs to be captured in CR for requested UL configurations only. WF to capture agreement on only first order TB and table template |
| R4-2204483 | draft CR to 38101-1-h40 missing MSD for CA_n5-n77(2A) | MediaTek Inc. | revised | Revision shared in Rd1 as starting point. Note 7 needs further check and UL config needs clarification wo CA_n5-n77(2A) in first column |
| R4-2206076 | Proposals for NR-U Intraband Contiguous UL-CA requirements | Qualcomm Incorporated | Noted | Consensus between Qualcomm and Skyworks to be captured in a WF and a CR can be discussed |
| R4-2206138 | MPR proposal for NR-U UL-CA | Skyworks Solutions Inc. | Noted |  |
| R4-2204088 | On low MSD for CA and DC | Huawei, HiSilicon | Noted | “Low MSD” work in RAN4 to be re-assessed in RAN plenary for R17 and R18 scope |
| R4-2204736 | TP for TR 37.717-11-11: Update MSD analysis of DC_(n)3AA | Huawei Technologies France | Revised | Capture test points based on Rd2 discussion and if MSD number available. WF assigned to capture agreements if not complete. |
| R4-2204806 | TP for TR 37.717-21-11: Update DC_1A_(n)3AA | Huawei Technologies France | Return to | depends on approval of revision of R4-2204736 or not |
| R4-2204754 | TP for TR38.717-03-02: CA_n28A-n40A-n41A | ZTE Corporation | Revised | Need to capture Rd2 agreements on IMD4 MSD. WF also assigned in case not complete |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2206378 | Revision of R4-2204680 Draft Correction CR to R17 TS38.101-1 on MSD for CA_n18-n28 | Samsung, KDDI, Skyworks Solutions Inc., Qualcomm, MediaTek | Agreeable | all experts have approved the latest revision shared |
| R4-2206379 | Revision of R4-2204681 Draft Correction CR to R17 TS38.101-3 on MSD for DC_18_n28 | Samsung, KDDI, Skyworks Solutions Inc., Qualcomm, MediaTek | Agreeable | all experts have approved the latest revision shared |
| R4-2206584 | WF on CA_n18- n28 and DC_18-n28 low MSD second test point | MediaTek, Skyworks, Qualcomm, Samsung, KDDI | Agreeable | All experts have approved the latest revision shared. Moderator to chairman, please add co-signing companies |
| R4-2206389 | TP for TR 37.717-21-11 for DC_8-28_n3 | VODAFONE Group Plc | Agreeable | TPs uses proposed MSD from Qualcomm and agreed by experts |
| R4-2206390 | TP for TR 37.717-21-11 for DC_8-28_n78 | VODAFONE Group Plc | Agreeable |  |
| R4-2206391 | TP for TR 37.717-21-11 for DC_8-32_n78 | VODAFONE Group Plc | Agreeable |  |
| R4-2206392 | TP for TR 37.717-21-11 for DC_20-28_n78 | VODAFONE Group Plc | Agreeable |  |
| R4-2206380 | Revision of R4-2203538 TP to TR 38.717.02-01 for CA_n46-n96 | Charter Communications, Inc | Agreeable | TP addresses corrections based on expert’s review |
| R4-2206381 | Revision of R4-2205669 TP to TR 38.717.02-01 for CA_n48-n96 and DC_n48-n96 | Charter Communications, Inc | Agreeable | TP addresses corrections based on expert’s review |
| R4-2206382 | Revision of R4-2203539 TP to TR TR38.717-03-01 for CA_n46-n48-n96 | Charter Communications, Inc | Agreeable | TP addresses corrections based on expert’s review |
| R4-2206383 | Revision of R4-2203540 TP to TR 38.717.03-02 for CA_n46-n48-n96 | Charter Communications, Inc | Agreeable | TP addresses corrections based on expert’s review |
| R4-2206393 | WF on missing fall back list for 36.101 and 38.101-1 and -3 | Apple | Agreeable | WF provides guidelines on how RAN4 will resolve missing fallbacks for R17 |
| R4-2206384 | Revision of R4-2205701 TP for TR 37.717-11-11 to include DC_2_n25 | Ericsson, Bell Mobility, Qualcomm | Agreeable | TP addresses corrections Moderator to chairman, please add co-signing company |
| R4-2205702 | TP for TR 37.717-21-11 to include DC_2-7_n25 | Ericsson, Bell Mobility | Agreeable | Was only pending on fallback approval |
| R4-2205703 | TP for TR 37.717-31-11 to include DC_2-7-66_n25 | Ericsson, Bell Mobility | Agreeable | Was only pending on fallback approval |
| R4-2205704 | TP for TR 37.717-31-11 to include DC_2-7-13_n25 | Ericsson, Bell Mobility | Agreeable | Was only pending on fallback approval |
| R4-2206394 | WF on capturing triple beat MSD in 38.101-1 and 38.101-3 | Skyworks, Qualcomm | Agreeable | WF provides guidelines for analysing and capturing triple beat cases |
| R4-2206395 | CR to 38.101-3 to add triple beat MSD | Qualcomm Incorporated | Agreeable | captures MSD for missing cases according to way forward |
| R4-2206385 | Revision of draft CR to 38101-1-h40 missing MSD for CA_n5-n77(2A) | MediaTek Inc., Skyworks, AT&T, Apple | Agreeable | Captures agreements for no IMD2 MSD Moderator to chairman, please add co-signing companies |
| R4-2206396 | WF on NR-U contiguous ULCA MPR | Skyworks, Qualcomm | Agreeable | provides background and agreements for NRU ULCA MPR cases |
| R4-2206397 | CR to 38.101-1 to add NR-U contiguous UL CA MPR | Qualcomm, Skyworks | Agreeable | captures MPR according to way forward |
| R4-2206398 | WF on MSD for DC_(n)3AA | Huawei Technologies France | withdrawn | No WF available |
| R4-2206386 | Revision of R4-2204736 TP for TR 37.717-11-11: Update MSD analysis of DC_(n)3AA | Huawei Technologies France | postponed | no agreement on IMD5 MSD test point or values. Companies are invited to bring analysis in next meeting for IMD5 issue |
| R4-2204806 | TP for TR 37.717-21-11: Update DC_1A_(n)3AA | Huawei Technologies France | postponed | pending agreement on fallback DC_(n)3AA |
| R4-2206399 | WF on IMD4 MSD for CA_n28A-n40A-n41A | ZTE Corporation, Qualcomm | Agreeable | WF captures IMD4 test points for MSD evaluation and architecture discussions in next meeting Moderator to chairman, please add co-signing company |
| R4-2206250 | Revision of R4-2204754 TP for TR38.717-03-02: CA_n28A-n40A-n41A | ZTE Corporation | Postponed | Has captured test point from WF but can be finalyzed at next meeting based on inputs from companies on architecture |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Skyworks Solutions Inc. (moderator) | Dominique Brunel | dominique.brunel@skyworksinc.com |
| Nokia | Petri Vasenkari | petri.j.vasenkari@nokia.com |
| MediaTek | Huanren | huanren.fu@mediatek.com |
| Skyworks Solution Inc. | Laurent Noel | laurent.noel@skyworksinc.com |
| Apple | James Wang | fucheng_wang@apple.com |
| Huawei | Peng (Henry) Zhang | zhangpeng169@huawei.com |
| AT&T | Ron Borsato | ronald.borsato@att.com |
| Huawei | Mohammad Abdi Abyaneh | Mohammad.abdi.abyaneh@huawei.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)