3GPP TSG-RAN WG4 Meeting # 102-e 											       R4-2206450

Electronic Meeting, 21st February– 3rd March, 2022

Source: 	Huawei, HiSilicon, Skyworks Solutions Inc.

Title: 	WF on improvements to MSD table

Agenda Item:	9.27.1

Document for:	Approval

Background

Based on the approved WF R4-2202287 [1], companies provided their views on the number of test points and new MSD table format in this meeting.

In this meeting, companies provides some contributions [2][3][4] to further discuss these remaining issues.

Open issues

Issue 2-1-1: How many test points should be restricted for a given MSD type and a given band combination?

- Proposals

  - Option 1: When considering MSD table improvement for the harmonic or cross-band isolation (>ACLR2), two MSD requirements for a given MSD type and a given band combination can be considered at least, one for the minimum victim downlink channel bandwidth and the other for the largest victim downlink channel bandwidth.

  - Option 2: More than one MSD test point can be introduced for a given band combination based on the reasonable judgement, but the maximum number of MSD test point can be five and it’s allowed to test only one configuration.

  - Option 3:

    - For MSD due to cross-band isolation, keep 1 test point per NR-CA combination, and optionally, on a case-by-case basis, evaluate the necessity to retain one additional test point to account for C-IM interference.

    - For MSD due to harmonic interference, keep 1 test point per NR-CA combination for direct harmonic hit, and 1 test point for near miss cases

  - Option 4: When considering MSD table improvement, for a given band combination:

    - two MSD test points for the case of direct-hit harmonic interference can be considered,

    - two MSD test points for cross-band isolation interference can be considered,

    - one MSD test point for the case of near-miss harmonic interference can be considered.

Note: For the case when 2 MSD test points can be considered at least, one test point is specified for the minimum victim downlink channel bandwidth.

Companies are encouraged to bring proposals on criteria

1)	 how to select the second MSD test point (is it based on maximum DL CBW? based on MSD level? other?); and,

2)	 how to capture the test point in the new table format in case new “lowest” or new “highest” CBW are introduced in the future.

3)	Other options are not precluded, including if 2 test points are needed for near miss harmonic MSD.

- Recommended WF

  - Option 4

Issue 2-1-2: The general table format for MSD due to cross band isolation is proposed as below.

WF: keep the previous agreement in R4-2202287.

Option 1:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1720 | 20 | 15 | 100 (RBstart=0) | 1515.5 | 5 | 2.6 | >ACLR2 |
| n34 | n3 | 2012.5 | 5 | 15 | 25 (RBstart=0) | 1877.5 | 5 | 3 | >ACLR2 |
| n46 | n78 | 5190 | 100 | 30 | 216 (RBstart=0) | 3795 | 10 | 10.4 | >ACLR2 |
| n46 | n78 | 5190 | 100 | 30 | 216 (RBstart=0) | 3750 | 100 | 5.1 | >ACLR2 |
| n77 | n41 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n77 | n41 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |
| NOTE X: When the victim DL bands are FDD bands, the UL RB allocation of victim FDD bands shouldn’t be configured |  |  |  |  |  |  |  |  |  |

WF on how to down-select the number of test points for MSD due to cross-band isolation to a maximum of 2 test points (option 1):

The question on how to configure the UL band remains open. Clear guidelines are needed to prevent introducing inconsistent MSD test points across band combinations.

Recommended WF:

- For FDD-FDD LB/LB and MB/MB combinations, the agreement from WF R4-2202275 can be used as baseline guidelines to down-select the number of MSD test points.

- For other types of cross-band isolation MSD test points, such as LB/MB or LB/HB (FDD-TDD or FDD-FDD), companies are invited to study how to extend/generalize WF R4-2202275 guidelines.

Issue 2-1-3: Further discuss on candidate options for the format of MSD table due to harmonics.

- Proposals

  - Option 1:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| nX | nY | TBD | 5 | 15 | 25 (RBstart=0) | TBD | 5 | 23.5 | UL2/DL1 |

  - Option 2:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| nX | nY | 5 | 15 | 25 (RBstart=0) | 10 | 23.5 | Note1 | UL2/DL1 |
| Note 1: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

- Recommended WF

  - Option 2

Issue 2-3-2: Is it agreeable that PC2 and PC3 MSD should be merged in the common table?

Five companies support option 1. One company support option 2.

- Recommended WF

  - Once the overlapping in different rapporteurs big CR for same table are resloved, then PC2 and PC3 MSD test points are merged in a common table. The notation “ULx/DLy” captured in the last column indicates if the MSD test point is a test point due to UL harmonic interference or due to Rx harmonic mixing,

- References

[1]	R4-2202287	WF on improvements to MSD table, Huawei, HiSilicon

[2] R4-2204053	Discussion on the number of test points for the MSD table improvement, CHTTL

[2] R4-2205280	Discussion on simplifying extended MSD table, Huawei, HiSilicon

[3] R4-2206142	MSD Tables Simplification Proposal for BCS4, Skyworks Solutions Inc.