3GPP TSG-RAN4 Meeting #102-e	R4-2206487

# Online, , 21st February – 3rd March 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for clarification on per band pair simultaneous RxTx capability for DC TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_Simult_RxTx-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In response to the introduction of the new capability signalling of per band pair simultaneous RxTx capability in RAN2#116-e, RAN4#101-bis-e discusses the clarification on mandatory applicaibility for band pairs included in higher order band combinations, and the following proposals were agreed in R4-2202295. Agreement: - Proposal 1: For inter-band EN-DC, NE-DC, NR CA, NR DC and SUL configurations, - If mandatory simultaneous RxTx capability apply for a band configuration, mandatory simultaneous RxTx capability also apply for the band pair of the configuration when the applicable configuration is a subset of a higher order band configuration. - Proposal 2: Clarification in Proposal 1 should apply from Rel-15 TS 38.101 series. - Proposal 3: FFS how to capture proposal 1 in TS 38.101 series. - Option 1: Add NOTEs in band configuration tables including higher order band configuration (more than 2 bands cases). - Option 2: Add description in general sections. This CR tries to reflect propsals 1 and 2 into TS 38.101-1. Regarding proposal 3, this CR is based on option 2 since it is redandant to put the same NOTEs into all band configuration tables. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following description is added into section 5.2B: If the mandatory simultaneous RxTx capability applies for a band configuration, the mandatory simultaneous RxTx capability also applies for the band pair of the configuration when the applicable configuration is a subset of a higher order band configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mandatory applicaibility for band pairs included in higher order band combinations is unclear considering the existence of per band pair simultaneous RxTx capability signalling of simultaneousRxTxInterBandCAPer-band-pair. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | As a related contribution, R4-2205439 is a draft CR to capture proposal 1 in the agreed WF(R4-2202295) into the case of CA and SUL in section 5.2A and 5.2C, respectibely, which does not include the case of DC because DC band combinations with mandatory simultaneous RxTx capability are specified from Rel-17 verision of TS 38.101-1. Therefore, this CR is a Cat F CR to capture the case of DC. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchaged sections are omitted>

5.2B	Operating bands for DC

The operating bands are specified in clause 5.5B for operation with NR dual connectivity configured, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a band combination, the mandatory simultaneous Rx/Tx capability also applies for the band combination when the applicable band combination is a subset of a higher order band combination.

<Unchaged sections are omitted>