3GPP TSG-RAN WG4 Meeting #102-e                                         R4-2206527

Electronic Meeting, 21 Feb – 3 Mar 2022

Source: 	Huawei, HiSilicon

Title: 	WF on MPR for intra-band V2X con-current operation

Agenda Item:	9.39

Document for:	Approval

# 1. GTW agreement

Agree with following high level principles

- Use the highest modulation order between sidelink and NR Uu

- RB size allocated for sidelink is 10 RB

- Taking intra-band UL CA requirement into account

- Have one requirement for con-current operation.

# 2. WF for PC3 MPR

Make changes for the PC3 MPR values from TR 38.785 as blow, and put the values in brackets for potential future revisit.

Table 1: MPR for contiguous RB allocation for power class 3 NR V2X con-current UE

| Higher Modulation order between Sidelink and Uplink |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocation | Outer RB allocation |
| CP-OFDM | QPSK | ≤ [1.0] [2.5] | ≤ [1.0] [4.5] |
|  | 16QAM | ≤ [1.0] [2.5] | ≤ [1.0] [4.5] |
|  | 64QAM | ≤ [1.0] [4.5] | ≤ [1.0] [5.0] |
|  | 256QAM | ≤ [2.0] [6.0] | ≤ [2.5] [6.0] |

Table 2: MPR for non-contiguous RB allocation for power class 3 NR V2X con-current UE

| Higher Modulation order between Sidelink and Uplink |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner RB allocation | Outer1 RB allocation | Outer2 RB allocation |
| CP-OFDM | QPSK | ≤ [1.0] [2.5] | ≤ [1.5] [4.0] | ≤ [3.0] [4.5] |
|  | 16QAM | ≤ [1.0] [2.5] | ≤ [1.5] [4.0] | ≤ [3.0] [4.5] |
|  | 64QAM | ≤ [1.0] [4.5] | ≤ [1.5] [4.5] | ≤ [3.0] [5.0] |
|  | 256QAM | ≤ [2.5] [6.0] | ≤ [2.5] [6.0] | ≤ [3.5] [6.0] |

# 3. WF for PC2 MPR

Make changes for the PC2 MPR values from TR 38.785 as below, and put the values in brackets for potential future revisit.

Table 3: MPR for contiguous RB allocation for power class 2 NR V2X con-current UE

| Higher Modulation order between Sidelink and Uplink |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocation | Outer RB allocation |
| CP-OFDM | QPSK | ≤ [1.5] [3.0] | ≤ [3.5] [5.5] |
|  | 16QAM | ≤ [2.0] [4.0] | ≤ [3.5] [5.5] |
|  | 64QAM | ≤ [3.0] [5.5] | ≤ [3.5] [6.0] |
|  | 256QAM | ≤ [5.0] [7.5] | ≤ [5.5] [7.5] |

Table 4: MPR for non-contiguous RB allocation for power class 2 NR V2X con-current UE

| Higher Modulation order between Sidelink and Uplink |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner RB allocation | Outer1 RB allocation | Outer2 RB allocation |
| CP-OFDM | QPSK | ≤ [2.0] [3.0] | ≤ [4.0] [5.5] | ≤ [6.0] [6.0] |
|  | 16QAM | ≤ [2.5] [4.5] | ≤ [4.0] [5.5] | ≤ [6.0] [6.5] |
|  | 64QAM | ≤ [3.5] [5.5] | ≤ [4.5] [6.5] | ≤ [6.0] [7.0] |
|  | 256QAM | ≤ [5.5] [8.0] | ≤ [5.5] [8.0] | ≤ [6.5] [8.0] |

# 4. Comments in 2nd round

| Company | Comments |
| --- | --- |

# References

[1] R4-2206331, Email discussion summary for [102-e][131] NRSL_enh_Part_2

[2] R4-2204153, TP on the RF requirements for the remaining open issues for SL enhancements, LG Electronics