3GPP TSG-RAN WG4 Meeting # 102-e		              		                R4-2206535

Electronic Meeting, February 21 – March 3, 2022

Agenda item:			10.16.1, 10.16.2, 10.16.7 and 10.16.9

Source:	Intel Corporation

Title:	WF on system parameters of FR2-2

Document for:	Approval

# Introduction

This Way Forward covers general aspects and system parameters of the NR extension to 71 GHz work item discussed in the email thread [133].

# Topics

## General

### UE feature list for NR ext. to 71 GHz

Candidate features:

- UE supported channel bandwidths for a single CC

- Improved ON/ON transient period capability

Agreement: TBD

### LS reply

#### Sensing beam characteristics

Further discussion is needed to confirm whether RAN4 needs to define LBT requirements. If so, what approach should we take (i.e., R4-2203938, other) and when can we reasonably conclude this. If needed, update LS reply (R4-2206533) accordingly.

Agreement: TBD

#### Minimum guard period between two SRS resources for antenna switching

Further discuss RAN1’s question to RAN4 and whether current LS reply (R4-2206534) is agreeable.

Agreement: TBD

## System parameters

### Band definition

Candidate option:

- Proposal: Specify 66 – 71 GHz band for licensed usage

Agreement: Endorse the draft CR to capture the solution of raster for 66-71 GHz band based on the agreement in RAN4#101-bis-e, and agree the CR in the future when the regulation is available in at least one country or region.

- 66-71GHz frequency range should be put in the [ ] in the draft CR.

- The agreement on the raster from RAN4#101-bis-e remains.

- The other RF requirements can also be captured if they are applicable to licensed band.

### Channelization

#### Unlicensed operation

Agreement: Use the fixed RF channel raster with the step size of 1680 (100.8 MHz) as baseline to define the channel raster for the unlicensed band, and accordingly provide the channel raster numbers

- To Check if the above solution can support CA with different bandwidth combinations

  - If there is issue identified, then the above agreement can be revisited

Agreement: Specify SSB SCS 960KHz in the GSCN and add a note to clarify that 960KHz SCS cannot be used for initial access.

#### Licensed operation

Agreement: GSCN for licensed band

- GSCN step sizes

  - 3 for 120 kHz

  - 12 for 480 kHz

  - 6 for 960 kHz

- Those values will be further checked after agreeing on the SU values.

Agreement: for RF channel raster for licensed band, SCS based channel raster is used for licensed bands, and the step size for channel raster is:

- 2 for 120kHz

- 8 for 480kHz

- 16 for 960kHz

### Mandatory channel bandwidths

Candidate options:

- Proposal 1: The optionality of CBW is agreed as follows:

  - 120 kHz: mandatory (100 MHz), optional (400 MHz)

  - 480 kHz: mandatory (400 MHz), optional (800 MHz, 1600 MHz)

  - 960 kHz: mandatory (400 MHz), optional (800 MHz, 1600 MHz, 2000 MHz)

- Proposal 2: As each SCS is optional to support, further optionality on maximum channel bandwidth support is not required.

Agreement: TBD

# References

- R4-2206433, “Email discussion summary for [102-e][133] NR_ext_to_71GHz_Part_1,” Moderator (Intel), RAN4 #102-e, March 2022