3GPP RAN WG4 Meeting #102-e		R4-2206559

Electronic meeting, February 21 – March 3, 2022

Agenda item:	11.2.1

Source:	Intel Corporation, Ericsson

Title:	TP to TR 38.844: Summary and Conclusions

Document for:	Discussion

# 1	Introduction

This contribution includes TP of proposed conclusions for section 7 of TR 38.844.

# Discussion

The specific changes include:

- Proposed conclusions added in 7.2

# 7	Conclusion

#### 7.1 Comparison Between Different Schemes

This section provides a comparison of the proposed schemes using different criteria.

The comparison between different schemes is summarized Table 7.2.1.

Table 7.2.1. Comparison of different schemes

| Comparison Criteria | Overlapping CA (two cells) – described in Section 6.2.1 | Combined UE CBW (One cell) – described in Section 6.2.2 | Overlapping UE CBW – described in Section 6.2.3 | Wider CBW (one cell) – described in Section 6.2.X |
| --- | --- | --- | --- | --- |
| Regulatory requirement | No issue | No issue | No issue | Potential issue on the BS side, gNB filters will be needed depending on BS implementation. |
| UE performance degradation relative to minimum requirements | Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. | Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. | No issue | UE ACS and in-band blocking degradation. Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. |
| gNB complexity | gNB has to support CA and schedule the data without collision in the two CC’s overlapping PRBs | gNB has to support splitting the signal into 2 RF carriers with a predefined phase relationship | gNB has to support the irregular channel BW (can also be implemented through RF combining of 2 channels) | No changes needed if the BS can meet regulatory requirements with the RF front end of the wider channel BW. Otherwise, gNB has to support the irregular channel BW |
| UE complexity | UE has to support intra-band NC CA. 1 less CC can be supported when irregular BW is used in combination from other bands, or more total CCs needed. | UE has to support RF architecture as in intra-band NC CA. Needs new capability to aggregate 2 RF channels in baseband. Complexity higher than CA because the baseband will need a new "combiner" module. 1 less CC can be supported when irregular BW is used in combination from other bands, or more total CCs needed. | No changes needed, supported by legacy UEs | No changes needed, supported by legacy UE |
| UE throughput | UEs supporting the feature can use the entire spectrum allocation, legacy UEs can use an already supported channel BW | UEs supporting the feature can use the entire spectrum allocation, legacy UEs can use an already supported channel BW | UE throughput based on existing channel BWs (5MHz for holdings <10MHz, 10MHz for holdings <15MHz, etc) | UE throughput based on how many RBs can be used |
| Spectral utilization | Channel edge guardband based on the aggregated channel BW (5MHz for <10MHz, 10MHz for <15MHz, etc), 2 SSBs are needed | Channel edge guardband based on the aggregated RF carrier BW (5MHz for <10MHz, 10MHz for <15MHz, etc), single SSB needed | Channel edge guardband based on the actual holding (can be same as Overlapping CA), 2 SSBs are needed | Depends on the usable number of RBs, single SSB needed |
| Cell Spectral utilization | Entire spectrum holding can be used even only with legacy UEs | Entire spectrum holding can be used even only with legacy UEs for some scenarios depending on whether a single SSB can be used to configure legacy channels at both edges of the spectrum holding. Otherwise, entire spectrum can be used only by new UEs, all legacy UEs have to use the same regular channel BW part of the spectrum holding. | Entire spectrum holding can be used even only with legacy UEs | Entire spectrum holding, but with wider guard bands than in the other methods, can be used even only with legacy UEs |
| Network capacity | Entire spectrum can be used by multiplexing different UEs(even legacy UEs) | Entire spectrum can only be used for new UEs, whether legacy UEs can be multiplexed to cover entire channel depends on the configuration and bandwidth | Entire spectrum can be used by multiplexing different UEs in the frequency domain | Entire spectrum can be used by any UE |
| Legacy UE support | Legacy UEs supported, can use one of the aggregated CCs | Legacy UEs can use part of the spectrum that contains the SSB | Legacy UEs supported | Legacy UEs supported |
| RAN1/2/4 Specification impact | RAN1/2 – new UE capabilities needed, RAN4 – new band combinations, changes to channel spacing definition, Overlapping CA reqs applicability, new demod requirements for UEs | RAN2 – impact on new capability RAN4 –core requirements equivalent to new channel BW for BS, new demod requirements for UEs | RAN4 – BS requirements for new channel BW | RAN4 – BS requirements for new channel BW, possibly restrictions of the suitable scenarios. New asymmetric bandwidth combinations for UE are needed. However these combinations would be “regular” BW combinations, so existing process. |

<<START OF CHANGES>>

#### 7.2 Study Item Conclusions

Four methods of addressing irregular channel BW have been presented.  Each method implements a means to add further RBs that utilize the irregular CBW up to the guard band edges.  The four methods each have trade-offs in terms of implementation complexity and spectral utilization from the UE and Network perspective.

The Overlapping UE CBW from the Network Perspective and the Larger Channel BW method offer benefits in improved spectral efficiency to utilize new irregular BWs without requiring new UE hardware.  The Larger Channel BW method allows improved UE spectral efficiency in addition to the NW spectral efficiency.  The Combined UE CBW and Overlapping CA methods can achieve a further improvement in spectral efficiency from both the UE and Network perspective, at the cost of greater specification development effort and significantly increased UE hardware complexity for the Combined UE CBW method compared to the other methods.

The Larger Channel BW method will have some ACI degradation at least when blockers exist on both sides of the operating of the spectrum block.  The degree of degradation is still a subject of further study.  The Combined UE CBW method requires a new UE architecture that modifies the RF and PHY sections of the UE.

The Overlapping UE CBW from the Network Perspective method is the only method that achieves full benefit with a legacy UE.  For the Larger Channel BW method, for the case of  legacy UEs the usable RBs are limited to the smaller CBW in UL and full irregularBW RBs in DL for UEs that support the needed asymmetric UL and DL CBW combination.  The Overlapping CA (two cells) and Combined UE CBW (one cell) methods will allow a legacy UE to connect but offer no additional RBs beyond the smaller CBW.

For spectral efficiency, all four of the methods offer significant benefit over the existing channel bandwidths defined in TS38.101-1 and TS 38.104.  Comparison data from the exemplary numbers from the method descriptions in section 6, and are summarized below in Table 7.2-2.  All four methods have a spectral efficiency from the network perspective that exceeds 90% for CBW greater than 7MHz.  For the Larger Channel BW, since the next wider guard band is used, this method is 1RB less efficient in some cases.  For Overlapping UE CBW from the Network Perspective, the spectral efficiency is improved from the network side, while the spectral efficiency from the UE side sees limited improvement in spectral efficiency.  Thus, all the methods allow the operator to utilize the DL full irregular spectrum in some way while some methods also improve UE spectral utilization.

Table 7.2-2: Spectrum utilization of different methods (15kHz SCS)

| Spectrum block [MHz] | Overlapping CA (two cells) – described in Section 6.2.1 [%] | Combined UE CBW (One cell) – described in Section 6.2.2 (without 100kHz raster alignment) [%] | Overlapping UE CBW from Network Perspective – described in Section 6.2.3 [%] | Wider CBW (one cell) – described in Section 6.1 [%] |
| --- | --- | --- | --- | --- |
| 6 | 90 | 90 | 90 | 87 |
| 7 | 90 | 92.6 | 90 | 90 |
| 11 | 93.3 | 94.9 | 93.3 | 91.6 |
| 12 | 93 | 94.5 | 93 | 93 |
| 13 | 92.8 | 95.5 | 92.8 | 92.8 |
| Note 1: The Overlapping UE CBW from Network Perspective method can achieve the same spectrum utilization as the Combined UE CBW method if the same implementation is used for the base station |  |  |  |  |

<<END OF CHANGES>>

# 4	References

RP-202103, "New SID: Study on Efficient utilization of licensed 	spectrum that is not aligned with existing NR channel bandwidths", T-Mobile USA, Ericsson