3GPP TSG-RAN WG4 Meeting # 102-e 	R4-2206560

# Electronic Meeting, February 21 – March 3, 2022

Source: 	Huawei, HiSilicon, [Nokia, Nokia Shanghai Bell]

Title: 	TP for overall method comparisons

Agenda Item:	11.2.3.4

Document for:	Approval

# 1 Introduction

A new SI has been approved on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths [1].

In last meeting the first TP on overall method comparisons was approved for TR 38.884. The contribution provides the update on the sub-clause 7.1 for the TR based on [2] and [3].

# 3 Conclusions

It is proposed to approve the TP for the TR.

# References

RP-202103, New SID: Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths, T-Mobile USA, Ericsson

R4-2205158, TP for overall method comparisons, Huawei, HiSilicon

R4-2205215, TP to TR 38.844 Overall method comparison, Nokia, Nokia Shanghai Bell

# Text Proposal for TR 38.844

==== Start of Changes ===

#### 7.1 Comparison Between Different UE Schemes

This section provides a comparison of the proposed schemes using different criteria.

The comparison between different schemes is summarized Table 7.1-1.

Table 7.1-1: Comparison of different schemes

| Comparison Criteria | Overlapping CA (two cells) – described in Section 6.2.1 | Combined UE CBW (One cell) – described in Section 6.2.2 | Overlapping UE CBW – described in Section 6.2.3 | Wider CBW (one cell) – described in Section 6.1 |
| --- | --- | --- | --- | --- |
| Regulatory requirement | No issue | No issue | No issue | . No issue |
| UE performance degradation relative to minimum requirements | Legacy UE: Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. New UE: requirements could be defined | Legacy UE: Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. New UE: requirements could be defined | No issue | New UE: UE ACS and in-band and narrow-band blocking degradation in some scenarios. Legacy UE: Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. FFS |
| gNB complexity | gNB has to support CA and schedule the data without collision in the two CC’s overlapping PRBs | gNB has to support splitting the signal into 2 RF carriers with a predefined phase relationship | gNB has to support the irregular channel BW (can also be implemented through RF combining of 2 channels) | gNB has to support the irregular channel BW(can also be implemented through RF combining of 2 channels) |
| UE complexity | UE has to support intra-band NC CA. 1 less CC can be supported when irregular BW is used in combination from other bands, or more total CCs needed. | UE has to support RF architecture as in intra-band NC CA. Needs new capability to aggregate 2 RF channels in baseband. Complexity higher than CA because the baseband will need a new "combiner" module. 1 less CC can be supported when irregular BW is used in combination from other bands, or more total CCs needed. | No changes needed, supported by legacy UEs | For asymmetric BW combinations, UE needs to support where UL CBW is SmallerCBW and DL CBW is WiderCBW. |
| UE throughput in DL | UEs supporting the feature can use the entire spectrum allocation, legacy UEs can use an already supported channel BW | UEs supporting the feature can use the entire spectrum allocation, legacy UEs can use an already supported channel BW | UE throughput based on existing channel BWs (5MHz for holdings <10MHz, 10MHz for holdings <15MHz, etc) | UEs supporting the feature can use the entire spectrum allocation. In some cases, 1RB less due to wider GB. |
| Spectral utilization | Channel edge guardband based on the aggregated channel BW (5MHz for <10MHz, 10MHz for <15MHz, etc), 2 SSBs are needed | Channel edge guardband based on the aggregated RF carrier BW (5MHz for <10MHz, 10MHz for <15MHz, etc), single SSB needed | Channel edge guardband based on the actual holding (can be same as Overlapping CA), 2 SSBs are needed (at least for an irregular bandwidth below 10 MHz) | Channel edge guardband based on the allotted spectrum block |
| Cell Spectral utilization | Entire spectrum holding can be used even only with legacy UEs | Entire spectrum holding can be used even only with legacy UEs for some scenarios depending on whether a single SSB can be used to configure legacy channels at both edges of the spectrum holding. Otherwise, entire spectrum can be used only by new UEs, all legacy UEs have to use the same regular channel BW part of the spectrum holding. | Entire spectrum holding can be used with legacy UEs | Entire licensed spectrumblock, but with wider guard bands than in the other methods, can be used even only with legacy UEs |
| Network capacity | Entire spectrum can be used by multiplexing different UEs(even legacy UEs) | Entire spectrum can only be used for new UEs, whether legacy UEs can be multiplexed to cover entire channel depends on the configuration and bandwidth | Entire spectrum can be used by multiplexing different UEs in the frequency domain | Entire spectrum can be used by any UE in DL direction. UL is smaller channel BW. |
| Legacy UE support | Legacy UEs supported, can use one of the aggregated CCs | Legacy UEs can use part of the spectrum that contains the SSB | Legacy UEs supported | Legacy UEs that support asymmetric BW can use full spectrum. Other UEs are limited to use at least a part of the spectrum (corresponding to the next smaller CBW) that contains the SSB. |
| RAN1/2/4 Specification impact | RAN1/2 – new UE capabilities needed, RAN4 – new band combinations, changes to channel spacing definition, Overlapping CA reqs applicability, new demod requirements for UEs | RAN2 – impact on new capability RAN4 –core requirements equivalent to new channel BW for BS, new demod requirements for UEs | RAN4 – BS requirements for new channel BW | RAN2 – impact by new UE capability New asymmetric bandwidth combinations for UE are needed. Handled on band by band basis and is as existing process. Possibly new ACS and blocking requirements for new UEs. |

==== End of Changes ===