3GPP TSG-RAN WG4 Meeting # 102-e 												R4-2206767

Electronic Meeting, February 21 – March 3, 2022

Agenda item:			10.16.8.1, 10.16.8.2

Source:	Moderator (Qualcomm)

Title:	Email discussion summary for [102-e][224] NR_ext_to_71GHz_RRM_1

Document for:	Information

# Introduction

In RAN #89e meeting a new WI on Extending current NR operation to 71GHz was approved. During the RAN4 #99 the initial scope of RRM work for NR_ext_to_71GHz WI was defined and captured in the way forward R4-2108354. Further discussion was split into two email threads. The discussion at RAN4 #100-e and RAN4 #101-e in the first email thread can be retraced through R4-2115405 with corresponding WF R4-2115351 and at R4-2120370 with corresponding WF R4-2120316. For the second email thread the discussion can be retraced through R4-2115406 with corresponding WF R4-2115352 for RAN4 #100-e and through R4-2120371 with corresponding WF R4-2120317 for RAN4 #101-e and through R4-2202734 with corresponding WF R4-2202659 for RAN4 #101-bis-e.

Current email discussion document focuses on the general requirements, timing requirements, scheduling restrictions and measurement procedures based on the documents submitted under AIs - 10.16.8.1, 10.16.8.2

Draft CRs are accepted in this meeting

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: The following list of open issues was identified, based on the contributions, for the 1st round

  - General

    - RX beam sweeping scaling factor

    - deriveSSB-IndexFromCell

    - Scheduling restrictions

    - SSB index identification time

    - PSharing_factor

  - Timing requirements

    - UE transmit timing error

    - Gradual timing adjustment

    - Timing advance

    - MRTD

    - MTTD

    - Intra-frequency measurements

- 2nd round: TBA

# Topic #1: General

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203889 | CATT | Proposal 1: If deriveSSB-IndexFromCell is not enabled, a UE could determine the LSBs of the SSB index using PBCH DMRS, which can only resolve up to 8 SSB indexes. Observation 1: In the synchronous network, the maximum frame boundary alignment will not exceed the range of 8 SSB indexes. Proposal 2: In a synchronous network, RAN4 is unnecessary to specify the frame boundary alignment tolerance when deriveSSB-IndexFromCell is disabled for 960 kHz SSB SCS. Proposal 3: Frame boundary alignment: - For 480kHz SCS: - 3 SSB symbols - For 960kHz SCS: when deriveSSB-IndexFromCell is enabled - 3 SSB symbols - For 960kHz SCS: when deriveSSB-IndexFromCell is disabled - not specify or add UE frame boundary assumption. Proposal 4: It is suggested that both synchronization error and propagation delay difference could be considered together, and beam switching time, TA adjustment and other factors could be considered together. Proposal 5: It is suggested to introduce a total of K (due to synchronization error, propagation delay difference) + L (due to beam switching time, TA adjustment accuracy, TA resolution error etc.) symbols scheduling restriction before and after SSB transmission. - For K, - K=3 for 480 kHz SCS - K=6 for 960 kHz SCS - For L, - L= 1 for 480/960kHz SCS Proposal 6: The total scheduling restriction symbols should be at least 7 symbols and 4 symbols for 960 kHz and 480 kHz respectively. |
| R4-2204188 | Mediatek | Proposal 1: For 960kHz SCS, when deriveSSB-IndexFromCell is enabled, relax deriveSSB-IndexFromCell tolerance to be 3 SSB symbols for SSB SCS (Option 2a). Proposal 2: For 480kHz SCS, when deriveSSB-IndexFromCell is enabled, reuse the existing deriveSSB-IndexFromCell tolerance as 2 SSB symbols (Option 1b). Proposal 3: On the unlicensed band, for 480 kHz SSB SCS, deriveSSB-IndexFromCell is always enabled by the network. Proposal 4: Regarding the scheduling restrictions during L3 measurements, for 480kHz SCS, support K=2 (Option 1), L=1 and M=0. It gives 3 data symbols restrictions before and after the SSBs. Proposal 5: Regarding the scheduling restrictions during L3 measurements, for 480kHz SCS, support K=3, L=1 and M=0. It gives 4 data symbols restrictions before and after the SSBs. Proposal 6: If deriveSSB-IndexFromCell is not enabled, additional time for PBCH detection for SSB index acquisition is allowed. |
| R4-2203531 | Nokia | Proposal 1: When deriveSSB-IndexFromCell is not enabled, use the following definition of TSSB_time_index_intra for FR2-2: DRX cycle; Without measurement gaps; With measurement gaps. No DRX; Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFintra; Max(200ms, ceil(MSSB_index_intra Kp) Max(MGRP, SMTC period)) CSSFintra. DRX cycle≤ 320ms; Max(200ms, ceil(1.5 MSSB_index_intra s Kp) Max(SMTC period, DRX cycle)) CSSFintra; Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(MGRP, SMTC period, DRX cycle)) CSSFintra. DRX cycle>320ms; ceil(MSSB_index_intra Kp) DRX cycle CSSFintra; ceil(MSSB_index_intra Kp) DRX cycle CSSFintra. MSSB_index_intra will depend on the outcome of the PBCH index detection discussion and RF decision on supported power classes for FR2-2.. Observation 1: Frame boundary alignment is a necessary assumption for the UE to be able to derive SSB index without demodulating PBCH. Observation 2: If deriveSSB-IndexFromCell is not enabled, the cell phase synchronization accuracy will also provide synchronization of a TDD network. Therefore, further frame boundary assumption in case of deriveSSB-IndexFromCell is not enabled could only pose more restriction on the deployment of FR2-2 networks. Proposal 2: No need to define frame boundary alignment when deriveSSB-IndexFromCell is not enabled. Observation 3: For 960 kHz, frame boundary alignment of 2 symbols would be below the 3 us cell phase synchronization requirements, therefore 2 symbols is not feasible for 960 kHz SCS. Proposal 3: Consider 3 SSB symbols frame boundary alignment for 960 kHz SCS. Observation 4: Larger tolerance for frame boundary alignment enables less restrictions on the deployment scenario, with larger possible inter-site distances. Observation 5: The difference in intra-cell scheduling availability is not significant when comparing 2 and 3 SSB symbols for tolerance for frame boundary alignment. Proposal 4: Use 3 SSB symbols for tolerance for frame boundary alignment with 480 kHz SCS. |
| R4-2204543 | Nokia | Observation 8: deriveSSB-IndexfromCell is optional for NR-U in FR1, and it is up to network configuration whether to enable it or not in FR2-2, for SCS equal to 960 kHz. Proposal 10: It is up to network configuration whether to enable deriveSSB-IndexfromCell or not for unlicensed operation in FR2-2. |
| R4-2204726 | Ericsson | Proposal 1: In RRM specifications, frame boundary alignment should not be specified. Proposal 2: DeriveSSB_IndexFromCell is not always enabled in unlicensed band in FR2-2. Proposal 3: For 480kHz SCS: - Option 1a: 3 SSB symbols For 960kHz SCS: when deriveSSB-IndexFromCell is enabled - Option 2a: 3 SSB symbols For 960kHz SCS: when deriveSSB-IndexFromCell is disabled - Option 3a: 6 SSB symbols Proposal 4: Instead of distinguishing M, adopt 3 SSB symbols tolerance for 480KHz and 960KHz SCS from the previous issue + 1 SSB symbol to represent Scheduling limitations. Scheduling limitations are 4 symbols for 480kHz SCS and 960kHz SCS ( deriveSSB-IndexFromCell is enabled) . Proposal 5: No need to define symbols level scheduling restrictions for 960KHz when deriveSSB-IndexFromCell is disabled with hypothetical deployment, i.e. coverage of 1km. Same to FR1, entire SMTC window duration shall be restricted for 960kHz SCS ( deriveSSB-IndexFromCell is disabled). Proposal 6: Psharing factor shall be 3 in any case upon 480KHz and 960KHz SCS. |
| R4-2200562 | LGE | Proposal 1: Consider tolerance for frame boundary alignment for the number of symbols for scheduling restriction instead of K and M value. And one symbol should be added depending on beam switching time. Proposal 2: When deriveSSB-IndexFromCell is not enabled, the scheduling restriction should be applied for all symbols within the SMTC window Proposal 3: Do not specify the tolerance of frame boundary alignment when deriveSSB-IndexFromCell is not enabled. Proposal 4: Define the requirement for SSB index detection for intra-frequency measurement when deriveSSB-IndexFromCell is not enabled. Proposal 5: Introduce 3 symbols for tolerance for frame boundary alignment for both 480 and 960kHz SCS. Proposal 6: Reuse the existing scaling factor for Rx beam sweeping for FR2-2 in CONNECTED mode. Proposal 7: Introduce a larger scaling factor for Rx beam sweeping ([8 x N])for FR2-2 in IDLE/INACTIVE mode. |
| R4-2204636 | Vivo | Observation 1: The Rx beam sweeping scaling factor may need to be increased for FR2-2 compared with FR2-1. Proposal 1: Cell synchronization accuracy, propagation delay difference and beam switching time need to be considered for scheduling restrictions during L3 measurement. Observation 2: The total time is 6.53us including the synchronization accuracy of 3us, the delay difference of 3.33us and beam switching time of 200ns for scheduling restrictions during L3 measurement. Proposal 2: The total scheduling restriction symbols during L3 measurement should be 6 symbols and 3 symbols for 960kHz and 480kHz respectively. Proposal 3: SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 1 sample compared with FR2-1 if using the channel model from RAN1, i.e., 6 samples is needed for SSB index acquisition delay for inter-frequency measurement for FR2-2. Proposal 4: The requirements of SSB index detection for FR2-2 for inter-frequency measurement can be defined as below: Table 2. Time period for time index detection for inter-frequency measurements Condition NOTE1,2; TSSB_time_index_inter. No DRX; Max(200ms, MSSB_index_inter Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(200ms, (1.5 MSSB_index_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; MSSB_index_inter DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group.. MSSB_index_inter = 6 *N, N is Rx beam sweeping factor for FR2-2. |
| R4-2204635 | Vivo | Observation 1: It is reasonable to only decode the PBCH DMRS which is similar to FR1 rather than PBCH in order to determine the SSB index when the deriveSSB-IndexFromCell is not enabled for 960kHz. Proposal 4: Specify the frame boundary alignment tolerance when the deriveSSB-IndexFromCell is not enabled for 960kHz. Proposal 5: The frame boundary alignment is 6 SSB symbols when the deriveSSB-IndexFromCell is not enabled for 960kHz. |
| R4- 2206115 | Qualcomm | Observation 9: For FR2, LSBs of the SSB index is indicated by the PBCH DMRS sequence. Observation 10: When deriveSSB-IndexFromCell is disabled for 960kHz SCS in FR2-2, the UE can use PBCH-DMRS detection to determine the neighbour cell SSB index by performing an ‘N’ point hypothesis testing, e.g., - N = 3 if the uncertainty is within +/-1 SSB index - N = 5 if the uncertainty is within +/-2 SSB indexes Observation 10: Defining the frame boundary alignment for 960kHz SCS when deriveSSB-IndexFromCell is disabled allows the UE to determine the number of PBCH-DMRS detection hypothesis to be performed. Observation 11: The SSB index identification requirements for FR2-2 intra-frequency measurements can be specified based on the number of PBCH-DMRS detection hypothesis Observation 12: Both network and the UE may benefit by defining the scheduling restrictions based on the affected symbols and not on the whole SMTC window. Proposal 3: RAN4 to also specify the frame boundary alignment tolerance even for the case when deriveSSB-IndexFromCell is disabled for 960kHz SCS. - FFS: Number of OFDM symbols could be double of that for 480kHz SCS Proposal 4: Define the neighbour cell SSB index identification requirements for intra-frequency measurements based on the PBCH-DMRS detection delay corresponding to the frame boundary alignment tolerance. Proposal 5: We prefer the following values for tolerance in frame boundary alignment: - For 480kHz SCS: - 3 SSB symbols - For 960kHz SCS: when deriveSSB-IndexFromCell is enabled - 3 SSB symbols - For 960kHz SCS: when deriveSSB-IndexFromCell is disabled - 6 SSB symbols |
| R4-2206005 | Intel | Proposal 9: RAN4 to specify the frame boundary alignment tolerance even for the case when deriveSSB-IndexFromCell is disabled Proposal 10: When deriveSSB-IndexFromCell is enabled frame boundary alignment tolerance is no worse than 3 symbols Proposal 11: When deriveSSB-IndexFromCell is disabled frame boundary alignment tolerance is no worse than 6 symbols Proposal 12: RAN4 to define requirements on Time period for time index detection as it is shown in Tables 2.4-2 and 2.4-3 Table 2.4-2. Time period for time index detection for intra-frequency measurements without measurement gaps TSSB_time_index_intra. No DRX; Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFinter. DRX cycle ≤ 320ms; Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; Ceil(MSSB_index_intra Kp) DRX cycle CSSFinter. Table 2.4-3. Time period for time index detection for intra-frequency measurements with measurement gaps TSSB_time_index_intra. No DRX; Max(200ms, ceil(MSSB_index_intra Kp) Max(MGRP, SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; Ceil(MSSB_index_intra Kp) DRX cycle CSSFinter. |
| R4-2204874 | Huawei | Proposal 1: Consider increasing the beam sweeping scaling factor in FR2-2. Observation 1: Symbol-level scheduling restriction are only used when deriveSSB-IndexFromCell is enabled. Observation 2: There is no need to define scheduling restrictions with data samples longer than tolerance for frame boundary alignment for deriveSSB-IndexFromCell. Proposal 2: Define the tolerance for frame boundary alignment when deriveSSB-IndexFromCell as 3 SSB symbols. Observation 3: The scheduling restriction is defined per unit of SSB symbols to be measure and there is no need to consider the scheduling restriction on multiple unit of SSB symbols to be measured jointly. Proposal 3: Define tolerance for frame boundary alignment and scheduling restrictions based on Table II. Proposal 4: Not to define frame boundary alignment tolerance when deriveSSB-IndexFromCell is disabled and define requirements for SSB index detection in a generic approach. |
| R4-2204189 | Mediatek | Scheduling restriction due to L3 measurements for FR2-2 |
| R4-2204190 | Mediatek | Scheduling restriction due to L1 measurements for FR2-2 |
| R4-2204728 | Ericsson | draftCR on cell reselection in Idle mode for FR2-2 CCA (Treated in thread 225) |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Rx beam sweeping scaling factor

Sub-topic description: Discussion on Rx beam sweeping scaling factor.

Open issues and candidate options before e-meeting:

Issue 1-1-1: Rx beam sweeping scaling factor

- Proposal 1 (Vivo, Huawei): The Rx beam sweeping scaling factor may need to be increased for FR2-2 compared with FR2-1.

- Proposal 2a (LGE): Reuse the existing scaling factor for Rx beam sweeping for FR2-2 in CONNECTED mode.

- Proposal 2b (LGE): Introduce a larger scaling factor for Rx beam sweeping ([8 x N])for FR2-2 in IDLE/INACTIVE mode.

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Support option 1. Considering more antenna elements, naturally the beam is narrower than FR2-1 |
| LGE | Basically we think that Rx beam sweeping scaling factor need to be increased for FR2-2 as proposal 1, but considering the timeline of Rel-17, we prefer to consider proposal 2a and 2b. |
| Ericsson | Even antenna element for FR2-2 is assumed in RF session, we need tradeoff between RF link budget and measurement delay. |
| CATT | Agree with proposal 1. |
| Nokia | Considering the timeline for Rel 17, we think Proposal 2a might be more feasible. For proposal 2b, what does 8xN mean? Is N the current FR2-1 beam sweeping factor? |
| Intel | More antenna elements comparing to FR2-1 are agreed in the RF room, which means that narrower beams can be formed. However, RRM operates with rough beams, so we don’t think that that RF agreement should be the decisive factor for FR2-2 Rx beam sweeping scaling factor. The measurement delays are already significant and increasing Rx beam sweeping scaling factor may cause mobility issues. Considering that and the timeline for Rel-17 we prefer to keep FR2-1 assumption for Rx beam sweeping scaling factor. |
| vivo | Proposal 1. The antenna elements double for FR2-2 compared with FR2-1 and the Rx beam for FR2-2 is narrower than FR2-1. To guarantee the coverage for FR2-2, the Rx beam sweeping scaling factor may need to be increased. Therefore, it is feasible to increase the Rx beam sweeping factor for FR2-2. |

### Sub-topic 1-2: deriveSSB-IndexFromCell

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-2-1: Frame boundary tolerance when deriveSSB-IndexFromCell is disabled

- Proposal 1 (Qualcomm, Intel, Vivo): Specify the frame boundary alignment tolerance for the case when deriveSSB-IndexFromCell is disabled for 960kHz SCS.

- Proposal 2 (CATT, Nokia, LGE, Ericsson, Huawei): Do not specify frame boundary alignment tolerance for the case when deriveSSB-IndexFromCell is disabled.

- Recommended WF

  - Consider agreeing to the proposal.

| Company | Comments |
| --- | --- |
| Qualcomm | Support proposal 1. As mentioned in our paper, UE can determine the neighbor cell SSB index based on the PBCH-DMRS detection and can reduce the number of hypothesis if the frame boundary tolerance is specified. The reason RAN4 has agreed that deriveSSB-IndexFromCell may not always be enabled is because the cell-phase synchronization error exceeds 3 SSB symbols and there may be an ambiguity of +/-1 SSB indexes. So, we don’t see a reason to not define the tolerance for 960kHz SCS considering the same cell-phase synchronization error and propagation delay as 480kHz SCS. If this is not specified, the UE cannot assume +/-1 SSB index ambiguity and may need to test more hypothesis leading to increased SSB index identification delay. |
| Huawei | We prefer proposal 2. We can understand the motivation of proposal 1 that UE can derive SSB index only by DMRS sequence with some of the bits of SSB index unknown. However, we think it is a very specific UE implementation, we think the requirements shall be defined in a more robust way where PBCH decoding is also allowed. |
| LGE | We don’t have strong view, but we slightly prefer option 2. |
| Ericsson | We don’t see necessity to add frame boundary alignment tolerance in spec. after ‘deriveSSB-IndexFromCell’. Boundary alignment tolerance relies on practical deployment, how UE utilizes residual resolution of SSBs without PBCH decoding is UE’s implementation. From high layer point of view, it is vague to define mapping ‘synchronized’/’asynchronized’ and ‘deriveSSB’/‘fragmental deriveSSB’/ ‘can not deriveSSB’ which cause more complexity, e.g.: - In FR2-1 and FR2-2 480KHz, ‘synchronized’ corresponds to ‘deriveSSB’ - In FR2-2 960KHz, ‘synchronized’ corresponds to ‘deriveSSB’/‘fragmental deriveSSB’. For FR2-1 and FR2-2 480KHz, even ‘asynchronized’ can use utilize ‘fragmental deriveSSB’, shall frame boundary alignment tolerance be defined also? Another issue on frame boundary alignment tolerance is its limit, limitation= 6 symbols(960KHz) is based on ‘synchronized’ with [1]km propagation delay, ‘fragmental deriveSSB’ can suffer more frame boundary alignment tolerance indeed. |
| CATT | Support Proposal 2. |
| Nokia | We still think Proposal 2 makes more sense. |
| Intel | Prefer Proposal 1. For the comment from Huawei: in our understanding Proposal 1 also assumes that the requirements shall be defined in a way where PBCH decoding is considered. Adding tolerance will only provide an option for some specific UE implementation to avoid PBCH decoding reducing delays. |
| vivo | Proposal 1. Prefer to specify the frame boundary alignment tolerance when deriveSSB-IndexFromCell is disabled for 960kHz SCS. As mentioned in our contribution, defining frame boundary alignment tolerance is helpful for UE to choose the correct SSB index from two candidate SSB indexes. In addition, as we mentioned in Issue 1-3-2, if the frame boundary alignment tolerance is specified when deriveSSB-IndexFromCell is disabled for 960kHz SCS, there is no need to define scheduling restriction on all the symbols within the SMTC window. It is feasible to define the scheduling restriction in terms of the number of symbols. |

Issue 1-2-2: Assumptions on deriveSSB-IndexFromCell during unlicensed operation

- Proposal 1 (Mediatek): For 480 kHz SSB SCS, deriveSSB-IndexFromCell is always enabled in the unlicensed band.

- Proposal 2 (Ericsson,Nokia): deriveSSB-IndexFromCell is not always enabled in unlicensed band in FR2-2.

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Nokia | We proposed Proposal 2, following Rel 16 NR-U |
| Intel | Support Proposal 2 to be aligned with NR-U agreements |

Issue 1-2-3: Frame boundary alignment tolerance - SSB

- Proposal (Moderator): Frame boundary alignment tolerance of SSB symbols:

  - For 480kHz SCS:

    - Option 1a: 3 SSB symbols (CATT, Nokia, Ericsson, LGE, Intel, Huawei)

    - Option 1b: 2 SSB symbols (Mediatek)

  - For 960kHz SCS: when deriveSSB-IndexFromCell is enabled

    - 3 SSB symbols (CATT, Mediatek, Nokia, Ericsson, LGE, Intel, Huawei)

  - For 960kHz SCS: when deriveSSB-IndexFromCell is disabled

    - 6 SSB symbols (Ericsson, Qualcomm, Intel, Vivo)

- Recommended WF: Consider agreeing to the following:

  - For 480kHz SCS:

    - 3 SSB symbols

  - For 960kHz SCS: when deriveSSB-IndexFromCell is enabled

    - 3 SSB symbols

  - For 960kHz SCS: when deriveSSB-IndexFromCell is disabled

    - FFS: 6 SSB symbols (Depends on Issue 1-2-1)

| Company | Comments |
| --- | --- |
| Qualcomm | Support the recommended WF. Also support 6 SSB symbols for 960kHz SCS when deriveSSB-IndexFromCell is disabled |
| Huawei | Support the recommended WF. |
| LGE | Support the recommended WF. |
| Ericsson | Ok with WF, but not support 6 symbols for 960kHz SCS when deriveSSB-IndexFromCell is disabled |
| CATT | Agree with Recommended WF and support not to define 6 SSB symbols for 960kHz SCS when deriveSSB-IndexFromCell is disabled. |
| Nokia | Agre with recommended WF. |
| MTK | Fine with the recommended WF. |
| Intel | Support the recommended WF |
| vivo | Support the Recommended WF. For 960kHz SCS when deriveSSB-IndexFromCell is disabled, we also support 6 SSB symbols. |

Issue 1-2-4: Frame boundary alignment tolerance – Data

- Proposal 1(Huawei): Define tolerance for frame boundary alignment of PDSCH as below:

| SSB SCS (KHz) | Data SCS (KHz) | deriveSSB-IndexFromCell tolerance of PDSCH symbols |
| --- | --- | --- |
| 120 | 120 | / |
| 120 | 480 | 3 480KHz symbol |
| 120 | 960 | 6 960KHz symbol |
| 480 | 120 | 1 120KHz symbol |
| 480 | 480 | 3 480KHz symbol |
| 480 | 960 | 6 960KHz symbol |
| 960 | 120 | 1 120KHz symbol |
| 960 | 480 | 2 480KHz symbol |
| 960 | 960 | 3 960KHz symbol |

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Qualcomm | Proposal 1 looks fine to us. |
| Huawei | As analyzed in our paper, the frame boundary alignment tolerance of data shall be updated accordingly ( 1 data symbol in current spec). The number of data symbols should be defined considering the frame boundary alignment tolerance when deriveSSB is enabled. |
| Ericsson | OK with proposal |
| CATT | Ok with proposal 1. |
| Nokia | We are fine with Proposal 1. |
| MTK | Fine with proposal 1, which is derived from tolerance of 3 SSB symbols for 480kHz/960kHz SSB in our understanding. |
| Intel | Ok with Proposal 1 |
| vivo | In general, we are Ok with Proposal 1. However, in our understanding, it only considers the case where deriveSSB-IndexFromCell is always enabled. If the frame boundary alignment tolerance needs to be specified when deriveSSB-IndexFromCell is disabled for 960kHz SCS based on Issue 1-2-1, the Table may need to be further revised. |

### Sub-topic 1-3: Scheduling restrictions

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-3-1: Scheduling restriction when deriveSSB-IndexFromCell is enabled

- Proposal 1: Introduce a total of K (due to synchronization error, propagation delay difference) + L (due to beam switching time, TA adjustment accuracy, TA resolution error etc.) symbols scheduling restriction before and after SSB transmission:

  - For 960kHz SCS: Total scheduling restriction (K+L) is

    - Option 1a: 7 (CATT)

    - Option 1b: 6 (Vivo)

    - Option 1c: 4 (Mediatek, LGE, Ericsson)

  - For 480kHz SCS: Total scheduling restriction (K+L) is

    - Option 2a: 4 (CATT,Ericsson)

    - Option 2b: 3(Mediatek, Vivo)

- Proposal 2 (Huawei): Define the scheduling restrictions based on Table below

| SSB SCS (KHz) | Data SCS (KHz) | Scheduling restriction including beam swtiching |
| --- | --- | --- |
| 120 | 120 | Existing requirements |
| 120 | 480 | (3+1) 480KHz symbols |
| 120 | 960 | (6+1) 960KHz symbols |
| 480 | 120 | 1 120KHz symbol |
| 480 | 480 | (3+1) 480KHz symbols |
| 480 | 960 | (6+1) 960KHz symbols |
| 960 | 120 | 1 120KHz symbol |
| 960 | 480 | (2+1) 480KHz symbols |
| 960 | 960 | (3+1) 960KHz symbols |

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Qualcomm | We are fine with proposal 2 |
| Huawei | As analyzed in our paper, it seems not feasible to define the number of scheduling restriction in a general approach. For example. When SCS is 480 KHz, if the SCS of SSB is also 480 KHz, then we agree that 4 symbols should be defined (3 for misalignments and 1 for beam switching). But if the SCS of SSB is 960 KHz, 2 symbol to accommodate the misalignment is enough (2 symbols of 480KHz cover 3 symbols of 960 KHz) and 1 symbol for beam switching. Another alternative approach is to define the requirements in a general approach by picking the minimum value in all SCS combinations. For instance, for 960 KHz SCS, 4 symbols is defined. Then the definition of derive SSB should be updated accordingly. |
| LGE | We think the scheduling restriction would be ‘deriveSSB-IndexFromCell tolerance + 1 (beam switching time)’. So, based on conclusion of deriveSSB-IndexFromCell tolerance, we can decide final value for scheduling restriction, and also consider RF conclusion of Rx beam switching time. |
| Ericsson | We understand Option 1c and Option 2a in Proposal 1 match Proposal 2. |
| CATT | We agree to consider different number of symbols for different combinations of SSB SCS and UL SCS. So we revise our opinion to support proposal 2. |
| Nokia | We agree with Option 1c and 2a. |
| MTK | Fine with proposal 2, which is derived from tolerance of 3 SSB symbols for 480kHz/960kHz SSB in our understanding. |
| Intel | Ok with Proposal 2. |
| vivo | We are fine with Proposal 2. However, as mentioned in our paper, we understand for some combinations (e.g., 480kHz SSB SCS and 480kHz Data SCS), the total time is 6.53us including the synchronization accuracy of 3us, the delay difference of 3.33us and beam switching time of 200ns. In that case, 3 symbols are enough to cover the total scheduling restriction. It may be no need to take one symbol for beam switching. |

Issue 1-3-2: Scheduling restriction when deriveSSB-IndexFromCell is disabled

- Proposal 1 (LGE, Ericsson): When deriveSSB-IndexFromCell is not enabled (for 960KHz SCS), the scheduling restriction should be applied for all symbols within the SMTC window:

- Proposal 2 (Qualcomm): If frame boundary alignment tolerance is defined when deriveSSB-IndexFromCell is not enabled for 960kHz SCS, no need to define scheduling restriction on all the symbols within the SMTC window

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Qualcomm | Support proposal 2. We can define the scheduling restriction in terms of the number of symbols and not on the whole SMTC window. |
| Huawei | We prefer proposal 1. We recognize the approach to derive the SSB index by combining the DMRS sequence index and frame boundary tolerance. However, we prefer a more robust way in real network. |
| LGE | Prefer proposal 1, but it depends on conclusion of issue 1-2-1. |
| Ericsson | Proposal 1, which remains in line with FR1. |
| CATT | Prefer proposal 1, and it depends on the conclusion of issue 1-2-2. |
| Nokia | Depends on Issue 1-2-2. Considering no frame boundary alignment in Issue 1-2-2, we agree on Proposal 1. |
| Intel | Ok with Proposal 1. |
| vivo | Agree with proposal 2. When deriveSSB-IndexFromCell is not enabled for 960kHz SCS, the frame boundary alignment is 6 SSB symbols. It is feasible to define the scheduling restriction in terms of the number of symbols. |

### Sub-topic 1-4: SSB index identification time

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-4-1: Intra-frequency SSB index identification time when deriveSSB-IndexFromCell is not enabled

- Proposal 1 (Nokia, Intel): When deriveSSB-IndexFromCell is not enabled, use the following definition of TSSB_time_index_intra for FR2-2:

| DRX cycle | Without measurement gaps | With measurement gaps |
| --- | --- | --- |
| No DRX | Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFintra | Max(200ms, ceil(MSSB_index_intra Kp) Max(MGRP, SMTC period)) CSSFintra |
| DRX cycle≤ 320ms | Max(200ms, ceil(1.5 MSSB_index_intra s Kp) Max(SMTC period, DRX cycle)) CSSFintra | Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(MGRP, SMTC period, DRX cycle)) CSSFintra |
| DRX cycle>320ms | ceil(MSSB_index_intra Kp) DRX cycle CSSFintra | ceil(MSSB_index_intra Kp) DRX cycle CSSFintra |
| MSSB_index_intra will depend on the outcome of the PBCH index detection discussion and RF decision on supported power classes for FR2-2. |  |  |

- Proposal 2a (CATT): If deriveSSB-IndexFromCell is not enabled, a UE could determine the LSBs of the SSB index using PBCH DMRS, which can only resolve up to 8 SSB indexes.

- Proposal 2b (Mediatek, LGE): If deriveSSB-IndexFromCell is not enabled, additional time for PBCH detection for SSB index acquisition is allowed.

- Proposal 2c (Qualcomm): If deriveSSB-IndexFromCell is not enabled, define the neighbour cell SSB index identification requirements for intra-frequency measurements based on the PBCH-DMRS detection delay corresponding to the frame boundary alignment tolerance.

- Proposal 2d (Intel, vivo): The MSSB_index_intra can be calculated as 3 x N, where N is the Rx beam sweeping scaling factor, which can be different depending on the UE type (Intel)

- Recommended WF

  - Consider agreeing to Proposal 1.

  - Further discuss how to determine MSSB_index_intra considering proposal 2a/2b/2c/2d.

| Company | Comments |
| --- | --- |
| Qualcomm | Fine with Proposal 1. Agree with proposal 2a and 2b and support proposal 2c in order to determine MSSB_index_intra |
| Huawei | Fine with recommended WF and 2b. |
| LGE | Fine with recommended WF. |
| Ericsson | Support Proposal 2b and 2d. We don’t see strong reason to adopt specific PBCH-DMRS detection in Proposal 2c. |
| CATT | Agree with proposal 1. And we are ok with proposal 2a and 2b. |
| Nokia | We agree with Proposal 1. |
| MTK | Fine with the recommended WF. |
| Intel | Support Proposal 1 and 2d. |
| vivo | Agree with Proposal 1. Support Proposal 2d. We understand that the requirements of SSB index detection for FR1 (e.g., 3 samples) may be reused in FR2-2. |

Issue 1-4-2: Inter-frequency SSB index identification time

- Proposal 1 (Vivo): SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 1 sample compared with FR2-1 if using the channel model from RAN1, i.e., 6 samples is needed for SSB index acquisition delay for inter-frequency measurement for FR2-2.

  - MSSB_index_inter = 6 *N in Table 9.3.4-4

  - N is Rx beam sweeping factor for FR2- 2

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Fine with proposal 1. |
| Nokia | We would like to clarify the proposal. We understand that this proposal is extending 1 sample for the power class 1. We think that PC 3 should be used as a baseline MSSB_index_inter=24=3*N, which would result in - MSSB_index_inter = 4 *N in Table 9.3.4-4 |
| vivo | Thanks Nokia to clarify the proposal. We would like to further clarify this issue. We reviewed the previous discussion in R15 on SSB index identification time. The agreement in RAN4 86 bis meeting was captured as below: Intra-frequency SBI Acquisition; Agreement: - FR1 max(120ms, 3 x SMTC period) - FR2 max(200ms, [5] x N2 x SMTC period) Editor’s note: The values of N2 are to be updated.. We understand the baseline assumption for FR2-1 is 5 samples. However, considering the long measurement delay, the final requirement is defined as 24 for FR2-1 for PC3. Therefore, from the perspective of measurement delay, we revise the proposal from ‘MSSB_index_inter = 6 *N, N is Rx beam sweeping factor for FR2- 2’ to ‘MSSB_index_inter_FR2_2 = MSSB_index_inter_FR2_1 +N (i.e., 24+N), N is Rx beam sweeping factor for FR2- 2’. |

### Sub-topic 1-5: Psharing_factor

Sub-topic description:

Open issues and candidate options before e-meeting

Issue 1-5-1: ‘Psharing factor’ in beam management

- Proposal 1 (Ericsson): Psharing factor in beam management, e.g., L1-RSRP, BFD and RLM shall be 3 in any case for 480KHz and 960KHz SCS

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| Huawei | By defining P as 3 for any case even for the case when RS for L1 is not overlapped with L3 measurement resource (including the symbols for scheduling restriction), we are wandering is it kind of relaxation compared with existing requirements? No strong views on this, if companies think tthis will not bring too much relaxation. |
| Ericsson | If following current rule, Proposal 1 is expected. |
| MTK | We understand Proposal 1 is following the current rule. @Huawei, our understanding is Psharing factor is applied for selected cases such as (e.g. in the RLM requirements) P=P_(sharing factor)/(1-T_SSB/MRGP), when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP) And for the case that “RS for L1 is not overlapped with L3”, the is no Psharing factor P=11-TSSBTSMTCperiod, when RLM-RS resource is not overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod). |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204189 Scheduling restriction due to L3 measurements for FR2-2 | Huawei: This depends on the issues under discussion. |
|  | Nokia: To be reviewed according to agreements on Issue 1-3-1 and 1-3-2 |
| R4-2204190 Scheduling restriction due to L1 measurements for FR2-2 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 Rx beam sweeping scaling factor | Issue 1-1-1: Rx beam sweeping scaling factor Companies’ views: Diverse views on whether to extend the scaling factor or not GTW agreements: - Rx beam sweeping scaling factor is FFS - Option 1: The Rx beam sweeping scaling factor is increased for FR2-2 compared with FR2-1 - Option 2: Reuse the existing FR2-1 scaling factor for Rx beam sweeping for FR2-2. Candidate options: Option 1: The Rx beam sweeping scaling factor is increased for FR2-2 compared with FR2-1 Option 2: Reuse the existing FR2-1 scaling factor for Rx beam sweeping for FR2-2. Recommendations for 2nd round: Session chair: Continue discussion in the 2nd round to identify candidate values for Option 1. For the next meeting companies are encouraged to bring analysis for all identified options. |
| Sub-topic #1-2 deriveSSB-IndexFromCell | Issue 1-2-1: Frame boundary tolerance when deriveSSB-IndexFromCell is disabled Companies’ views: GTW agreements: - Specify the frame boundary alignment tolerance for the case when deriveSSB-IndexFromCell is disabled for 960kHz SCS. - The tolerance is [6] SSB symbols - Requirements are defined under assumption that UE may read PBCH payload. Recommendations for 2nd round: No further discussion needed in the second round. |
|  | Issue 1-2-2: Assumptions on deriveSSB-IndexFromCell during unlicensed operation Companies’ views: Not many comments on this issue. Two companies don’t agree with the proposal to always enable deriveSSB-IndexFromCell during unlicensed operation. Tentative agreements: No agreement Candidate options: - Option 1: For 480 kHz SSB SCS, deriveSSB-IndexFromCell is always enabled in the unlicensed band. - Option 2: deriveSSB-IndexFromCell is not always enabled in unlicensed band in FR2-2. Recommendations for 2nd round: Can proponents of Option 1 compromise to Option 2? |
|  | Issue 1-2-3: Frame boundary alignment tolerance - SSB Companies’ views: Most companies agree with the recommended WF. The FFS item was agreed in the GTW. Tentative agreements: - Define frame boundary alignment tolerance of SSB symbols as below: - For 480kHz SCS – 3 SSB symbols - For 960kHz SCS: - When deriveSSB-IndexFromCell is enabled – 3 SSB symbols - When deriveSSB-IndexFromCell is disabled – [6] SSB symbols Candidate options: Recommendations for 2nd round: Consider agreeing to 6 SSB symbols tolerance for 960kHz SCS when deriveSSB-IndexFromCell is disabled |
|  | Issue 1-2-4: Frame boundary alignment tolerance – Data Companies’ views: Most companies agree with the recommended WF. One company highlighted the need to define the tolerance for 960kHz SCS when deriveSSB-IndexFromCell is disabled Tentative agreements: - Define frame boundary alignment tolerance of PDSCH when deriveSSB-IndexFromCell is enabled as below: SSB SCS (KHz); Data SCS (KHz); Frame boundary alignment tolerance of PDSCH symbols (deriveSSB-IndexFromCell enabled). 120; 120; /. 120; 480; 3 480KHz symbol. 120; 960; 6 960KHz symbol. 480; 120; 1 120KHz symbol. 480; 480; 3 480KHz symbol. 480; 960; 6 960KHz symbol. 960; 120; 1 120KHz symbol. 960; 480; 2 480KHz symbol. 960; 960; 3 960KHz symbol. Candidate options: Recommendations for 2nd round: Check if the following frame boundary alignment tolerance, when deriveSSB-IndexFromCell is disabled, can be agreed: SSB SCS (KHz); Data SCS (KHz); Frame boundary alignment tolerance of PDSCH symbols (deriveSSB-IndexFromCell disabled). 120; 120; /. 120; 480; /. 120; 960; 6 960KHz symbol. 480; 120; /. 480; 480; /. 480; 960; 6 960KHz symbol. 960; 120; 1 120KHz symbol. 960; 480; 3 480KHz symbol. 960; 960; 6 960KHz symbol. |
| Sub-topic #1-3 Scheduling restriction | Issue 1-3-1: Scheduling restriction when deriveSSB-IndexFromCell is enabled Companies’ views: Most companies seem to agree with the recommended WF. One company mentioned that for the case of (480,480), 3 symbols should be enough. Tentative agreements: Candidate options: Recommendations for 2nd round: Check if the following can be agreed: - Define the scheduling restrictions, when deriveSSB-IndexFromCell is enabled, based on Table below SSB SCS (KHz); Data SCS (KHz); Scheduling restriction including beam swtiching. 120; 120; Existing requirements. 120; 480; (3+1) 480KHz symbols. 120; 960; (6+1) 960KHz symbols. 480; 120; 1 120KHz symbol. 480; 480; (2+1) 480KHz symbols. 480; 960; (6+1) 960KHz symbols. 960; 120; 1 120KHz symbol. 960; 480; (2+1) 480KHz symbols. 960; 960; (3+1) 960KHz symbols. |
|  | Issue 1-3-2: Scheduling restriction when deriveSSB-IndexFromCell is disabled Companies’ views: Companies have diverse views on this. But based on the GTW agreement, Tentative agreements: No agreement Candidate options: - Option 1: When deriveSSB-IndexFromCell is not enabled (for 960KHz SCS), the scheduling restriction should be applied for all symbols within the SMTC window: - Option 2: When deriveSSB-IndexFromCell is not enabled for 960kHz SCS, no need to define scheduling restriction on all the symbols within the SMTC window Recommendations for 2nd round: Based on the GTW agreement on deriveSSB-IndexFromCell, consider agreeing to Option 2. |
| Sub-topic #1-4 SSB index identification time | Issue 1-4-1: Intra-frequency SSB index identification time when deriveSSB-IndexFromCell is not enabled Companies’ views: Most companies agree with proposal 1. Tentative agreements: - When deriveSSB-IndexFromCell is not enabled, use the following definition of TSSB_time_index_intra for FR2-2: DRX cycle; Without measurement gaps; With measurement gaps. No DRX; Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFintra; Max(200ms, ceil(MSSB_index_intra Kp) Max(MGRP, SMTC period)) CSSFintra. DRX cycle≤ 320ms; Max(200ms, ceil(1.5 MSSB_index_intra s Kp) Max(SMTC period, DRX cycle)) CSSFintra; Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(MGRP, SMTC period, DRX cycle)) CSSFintra. DRX cycle>320ms; ceil(MSSB_index_intra Kp) DRX cycle CSSFintra; ceil(MSSB_index_intra Kp) DRX cycle CSSFintra. MSSB_index_intra will depend on the outcome of the PBCH index detection discussion and RF decision on supported power classes for FR2-2.. Candidate options: Recommendations for 2nd round: Further discuss the values for MSSB_index_intra based on the GTW agreement that the requirements would be derived based on PBCH decoding and not PBCH-DMRS detection. |
|  | Issue 1-4-2: Inter-frequency SSB index identification time Companies’ views: One company commented seeking clarification on the proposal Tentative agreements: No agreement Candidate options: Recommendations for 2nd round: Continue discussion in the second round |
| Sub-topic #1-5 Psharing factor | Issue 1-5-1: ‘Psharing factor’ in beam management Companies’ views: Not many comments, needs further discussion Tentative agreements: No agreement Candidate options: Recommendations for 2nd round: Continue discussion in the second round |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2204189 Scheduling restriction due to L3 measurements for FR2-2 | To be revised |
| R4-2204190 Scheduling restriction due to L1 measurements for FR2-2 | To be revised |

## Discussion on 2nd round (if applicable)

# Topic #2: Timing requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203890 | CATT | Proposal 1: The possible SCS combinations can be analyzed according to the basic principles, i.e. Te such that the condition eRS + eDRIFT < Te < (TCP - TCH - TAC,Q)/2 holds. Proposal 2: Considering the difficulty for UE implementation, it is not suggested to specify UE transmit timing error requirements for the SCS combination of (120,960). Proposal 3: It is suggested to specify UE transmit timing error requirements for the SCS combination of (120,480) and (480,960). Observations 1: When the SSB periodicity is longer, it is more conducive to power saving on cells that are used for capacity boost, but it is more unfavorable to define UL timing requirements. Observations 2: When the SSB periodicity is shorter, it is more conducive to define UL timing requirements, but at the same time, it means that a large number of reference signal resources will be used. Observations 3: The timing drift difference between 20ms and 40ms SSB periodicity does not play a decisive role in the definition of UE transmit timing error requirements. Proposal 4: It is suggested to define a single set of requirements, that is, the SSB periodicity is 40ms. Proposal 5: It is suggested that MRTD for inter-band synchronous NR DC between FR1 and FR2-2 is 33us. Observations 4: The basic principles for MRTD requirements in FR2-2 contradict the definition of MRTD in legacy spec. Proposal 6: It is suggested to change the definition of MRTD for NR DC in legacy spec, so that it could be larger than 0.5 slot. Proposal 7: The MRTD definition for NR DC in 3GPP TS 38.133 can be changed as follows: “A UE shall be capable of handling a relative receive timing difference between slot timing boundary of a cell belonging to MCG and slot timing boundary from the same slot index of a cell belonging to the SCG to be aggregated for NR DC operation. A UE shall be capable of handling a relative receive timing difference among the closest slot timing boundaries of different carriers to be aggregated in NR carrier aggregation.” Proposal 8: Define MTTD requirements in FR2-2 based on the following rule: - MTTD = MRTD + (TA step size / 2+ TA adjustment accuracy + Te) in cc1 + ( TA step size / 2 + TA adjustment accuracy +Te) in cc2. |
| R4-2203806 | Apple | Proposal 1: Requirements for SSB SCS < UL SCS should be considered only if UE implementation challenges are adequately addressed. Proposal 2: It is proposed that - With T_SSB = 20ms, Te is set such that Te/Tcp = 40% for the cases (480kHz SSB, 480kHz UL signal) and (960kHz SSB, 960kHz UL signal). - RAN4 agrees that test cases for Te requirements for FR2-2 will be designed as having statistical nature, i.e., something like “The rate of UE meeting the Te requirement observed during repeated tests shall be at least 90%.” Proposal 3: The UE timing advance adjustment accuracy for 480/960kHz SCS is defined as shown in Table 2. |
| R4-2204635 | Vivo | Proposal 1: Define one set of Te requirement for FR2-2 based on SSB periodicity of 40ms. Proposal 2: For the Te requirements, update Table 7.1.2-1 for FR2-2 as below: Table 7.1.2-1: Te Timing Error Limit Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te. 1; 15; 15; 12*64*Tc. 30; 10*64*Tc. 60; 10*64*Tc. 30; 15; 8*64*Tc. 30; 8*64*Tc. 60; 7*64*Tc. 2-1; 120; 60; 3.5*64*Tc. 120; 3.5*64*Tc. 240; 60; 3*64*Tc. 120; 3*64*Tc. 2-2; 120; 120; 3.5*64*Tc. 480; 1.7*64*Tc. 480; 120; 3.5*64*Tc. 480; 1.7*64*Tc. 960; 120; 3.5*64*Tc. 480; 1.7*64*Tc. 960; 0.7*64*Tc. Note 1: Tc is the basic timing unit defined in TS 38.211 [6]. Proposal 3: RAN4 shall not define the Te requirements for the combination of (120kHz SSB SCS, 960kHz UL SCS) and (480kHz SSB SCS, 960kHz UL SCS). Observation 1: It is reasonable to only decode the PBCH DMRS which is similar to FR1 rather than PBCH in order to determine the SSB index when the deriveSSB-IndexFromCell is not enabled for 960kHz. Proposal 4: Specify the frame boundary alignment tolerance when the deriveSSB-IndexFromCell is not enabled for 960kHz. Proposal 5: The frame boundary alignment is 6 SSB symbols when the deriveSSB-IndexFromCell is not enabled for 960kHz. Proposal 6: The requirements of SSB index detection for FR2-2 for intra-frequency measurement can be defined as below: Table 1. Time period for time index detection for intra-frequency measurements TSSB_time_index_intra. No DRX; Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFinter. DRX cycle ≤ 320ms; Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; Ceil(MSSB_index_intra Kp) DRX cycle CSSFinter. MSSB_index_intra = 3 *N, N is Rx beam sweeping factor for FR2-2. Proposal 7: The existing MRTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2. Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2-1; 8 note1. Between FR1 and FR2-1; 25. Between FR1 and FR2-2; 25. Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA.. Proposal 8: The existing MRTD requirements for inter-band synchronous DC for FR1 and FR2-1 can be reused for inter-band synchronous DC for FR1 and FR2-2. Table 7.6.6-1: Maximum receive timing difference requirement for inter-band synchronous NR DC Frequency Range; Maximum receive timing difference (µs). Cell in MCG; Cell in SCG. FR1; FR1; 33. FR2-1; FR2-1; 8. FR1; FR2-1; 33. FR1; FR2-2; 33. Proposal 9: The existing MTTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2. Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of TAGs; Maximum uplink transmission timing difference (µs). FR1; 34.6. FR2-1; 8.5 Note1. Between FR1 and FR2-1; 26.1. Between FR1 and FR2-2; 26.1. Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA.. Proposal 10: The existing MTTD requirements for inter-band synchronous NR DC for FR1 and FR2-1 can be reused for inter-band synchronous NR DC for FR1 and FR2-2. Table 7.5.6-1: Maximum uplink transmission timing difference requirement for inter-band synchronous NR DC Frequency Range; Maximum uplink transmission timing difference (µs). Cell in MCG; Cell in SCG. FR1; FR1; 34.6. FR2-1; FR2-1; 8.5. FR1; FR2-1; 34.1. FR1; FR2-2; 34.1. |
| R4-2203532 | Nokia | Channel fading impact on upper bounds for Te Observation 1: When TDLA30 and TDLA20, are considered, 90% of the channel energy is concentrated in taps with delay smaller than 65 and 43 ns. Observation 2: For TDLA channels 81.5% of the energy of the taps is contained on taps occurring with a delay smaller than the channel delay spread. Proposal 1: Consider TCH as the delay of the tap with 90% accumulated energy of TDLA30 for 480 kHz and TDLA20 for 960 kHz SCS. Lower bound calculation for Te values Observation 3: From UE perspective, the boundaries for Te can be determined considering the DL timing detection accuracy eRS and the expected clock drift eDRIFT. Proposal 2: Choose Te such that the condition eRS+eDRIFT<Te<(TCP-TCH-TAC,Q)/2 holds, where eRS is the DL timing estimation accuracy, eDRIFT is the clock drift for the reference signal periodicity, TCP is the CP length, TCH is the delay of the tap with 90% accumulated energy of TDLA30 for 480 kHz and TDLA20 for 960 kHz SCS, TAC,Q is the timing advance command step. PUSCH performance impact on Te Observation 4: UL timing errors larger than 25% of the CP length can cause UL demodulation performance degradation of 4.5 dB for 480 kHz SCS and 14 dB for 960 kHz SCS with TDLA10 channel. Proposal 3: RAN4 to define UL timing accuracy requirements with Te < 22% of CP length SSB and UL signals combinations for Te requirements Observation 5: Some companies identified that it is technically challenging to meet feasible Te requirements for the combination of 960 kHz SCS in UL and 120 kHz SCS for the SSB. Proposal 4: RAN4 to define Te requirements for the following combinations of SSB and UL SCS: -SSB 120 kHz, UL 120 kHz -SSB 120 kHz, UL 480 kHz -SSB 480 kHz, UL 120 kHz -SSB 480 kHz, UL 480 kHz -SSB 480 kHz, UL 960 kHz -SSB 960 kHz, UL 120 kHz -SSB 960 kHz, UL 480 kHz -SSB 960 kHz, UL 960 kHz Te dimensioning guidelines Proposal 5: For 120 kHz SCS with 480 kHz and 960 kHz SCS used by SSB, consider improvement on DL detection timing due to wider SSB for the Te requirements. Proposal 6: Te requirements with multiple SSB availability periods are applicable to all the UEs. Observation 6: Increasing the SSB SCS spacing improves the UE margin for the Te accuracy. Observation 7: Reduction on the SSB availability interval XSSB improves the UE margin for the Te accuracy. Proposal 7: Increase of the SSB SCS should be translated in the calculation of the Te requirements. Proposal 8: Reduction on the availability of SSB should be translated in the calculation of the Te requirements. Proposal 9: Determine the Te requirements using the formula: Te=α144×κ×2-μ×Tc+10007.2×2μSSB -10007.2×2μSSB,min +0.1×XSSB-0.1×XSSB,max -where -κ=64; -μ and μSSB are the numerologies for UL signals and SSB; -XSSB is the SSB availability in ms; -μSSB,min is the minimum numerology used for the SSB for μ -XSSB,max is the maximum SSB availability specified for μ; - α controls the baseline Te value in relation to the CP length - Tc is the basic timing unit defined in TS 38.211 Statistical nature of Te RRM performance requirements Observation 8: RAN5 specify that at least 33 test runs are used for performance requirements with statistical nature, and the UE has to pass 90% of the test runs. Observation 9: Existing Te RRM performance requirements are not considered as of statistical nature. Observation 10: Consideration of Te requirements as of statistical nature is equivalent to relaxing the Te requirement. Proposal 10: Te requirements and classification of statistical test should be discussed together. Proposal 11: If RAN4 agrees that Te requirements are classified as statistical, smaller Te values should be specified. Te requirements proposals Proposal 12: Define Te requirements considering the Te calculation formula and the parameters below: 120 kHz SCS; 480 kHz SCS; 960 kHz SCS. Non statistical nature; Statistical nature; Non statistical nature; Statistical nature. α; 0.182; 0.22; 0.20; 0.18; 0.16. μSSB,min; 3; 3; 3; 5; 5. XSSB,max; 160; 80; 80; 80; 80. Proposal 13: RAN 4 to define Te requirements using the values on the table below: Non statistical nature; Statistical nature. SSB SCS; UL SCS; XSSB; Te*64*Tc; Te % CP; Te*64*Tc; Te % CP. 120; 120; 160; 3.50; 19.44; 3.50; 19.44. 240; 120; 160; 3.00; 16.67; 3.00; 16.67. 120; 480; 80; 0.99; 22.00; 0.90; 20.00. 40; 0.86; 19.11; 0.77; 17.11. 20; 0.80; 17.78; 0.71; 15.78. 480; 120; 160; 2.87; 15.94; 2.87; 15.94. 480; 80; 0.59; 13.11; 0.50; 11.11. 40; 0.46; 10.22; 0.37; 8.22. 20; 0.40; 8.89; 0.31; 6.89. 960; 80; 0.40; 17.78; 0.38; 16.89. 40; 0.28; 12.44; 0.26; 11.56. 20; 0.22; 9.78; 0.20; 8.89. 960; 120; 160; 2.80; 15.56; 2.80; 15.56. 480; 80; 0.52; 11.56; 0.43; 9.56. 40; 0.40; 8.89; 0.31; 6.89. 20; 0.33; 7.33; 0.24; 5.33. 960; 80; 0.33; 14.67; 0.32; 14.22. 40; 0.21; 9.33; 0.19; 8.44. 20; 0.15; 6.67; 0.13; 5.78. SSB availability for the UE for Te requirements Observation 11: A small XSSB implies in small period between SSBs large overhead and inefficient network resource usage and should be avoided. Proposal 14: Adopt at least SSB availability of XSSB=80 ms. Proposal 15: Adopt at SSB availability of XSSB = [80 40] ms for Te requirements with 480 kHz and 960 kHz SCS. MRTD requirements Proposal 16: RAN4 to agree on MRTD value for non-contiguous intra-band CA of [260 ns] for FR2-2. Proposal 17: For Inter-band synchronous NR carrier aggregation between FR1 and FR2-2 reuse the existing MRTD requirements for FR1 and FR2-1 as - Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2-1; 8 note1. Between FR1 and FR2-1; 25. Between FR1 and FR2-2; 25. Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA.. Proposal 18: For Inter-band synchronous NR-DC between FR1 and FR2-2 reuse the existing MRTD requirements for FR1 and FR2-1 as: Table 7.6.6-1: Maximum receive timing difference requirement for inter-band synchronous NR DC Frequency Range; Maximum receive timing difference (µs). Cell in MCG; Cell in SCG. FR1; FR1; 33. FR2-1; FR2-1; 8. FR1; FR2-1; 33. FR1; FR2-2; 33. |
| R4-2204191 | Mediatek | Observation 1: For the higher SCS, the timing error is dominated by the RF mismatch and margin. Observation 2: The timing error budget for RF mismatch and margin will be reduced to ~2.1 Ts and ~1 Ts for UL SCS of 480k Hz and 960 kHz, respectively Proposal 1: Specify 2 sets of UE transmit timing error requirements, wherein one set is made by assuming the RF margin of 2 Ts = 2*64 Tc. Proposal 2: Introduce a new UE capability to indicate which set of UE transmit timing error requirements can be applied. |
| R4-2204875 | Huawei | Proposal 1: It is suggested to define two sets of Te requirements when SSB is available in 20/40 ms and 80 ms. Table I. Te when SSB is available in last 20ms SSB SCS (kHz); UL SCS (kHz); Te; Te/CP. 120; 480; 1.35*64*Tc; 30%. 480; 120; 1.6*64*Tc; 8.8%. 480; 1*64*Tc; 22.3%. 960; 0.65*64*Tc; 29.0%. 960; 120; 1.4*64*Tc; 7.7%. 480; 0.8*64*Tc; 17.8%. 960; 0.65*64*Tc; 29.0%. Table II. Te when SSB is available in last 80ms SSB SCS (kHz); UL SCS (kHz); Te; Te/CP. 120; 480; 1.6*64*Tc; 35.7%. 480; 120; 1.9*64*Tc; 10.5%. 480; 1.2*64*Tc; 26.7%. 960; 0.85*64*Tc; 37.9%. 960; 120; 1.6*64*Tc; 8.8%. 480; 1*64*Tc; 22.3%. 960; 0.85*64*Tc; 37.9%. Proposal 2: Define Te requirements as Table I and Table II. Proposal 3: Discuss whether to have smaller RF margin and finer granularity of timing drift for gradual adjustment in FR2-2 based on conclusion on Te requirements. Proposal 4: Legacy requirements of MRTD for intra-band non-contiguous CA can apply to FR2-2. Proposal 5: Legacy requirements of MRTD for inter-band CA and synchronous DC can apply to FR1+FR2-2, which is capped by 0.5 slot length of corresponding SCS. Proposal 6: Define MTTD requirements with conclusion of MRTD and Te based on following rules: - MTTD = MRTD + (TA resolution error cc1 + TA adjustment accuracy cc1 + UL transmit error cc1) + (TA resolution error cc2 + TA adjustment accuracy cc2 + UL transmit error cc2) |
| R4-2205403 | ZTE | - Proposal 1: Define at least two sets of Te requirements based on SSB periodicity for UL SCS of 480/960 kHz. Two Te can be defined based on the assumption that if an SSB is available in the last 40 ms and 80 ms. Proposal 2: Define test cases for both sets of requirements under different SSB periodicities. |
| R4-2205417 | Ericsson | Proposal 1: Adopt Te for FR2-2 according to: SSB SCS (kHz); UL SCS (kHz); Te (Ts). 120; 480; 1,29. 120; 960; 0,45. 480; 120; 2,00. 480; 480; 1,00. 480; 960; 0,45. 960; 120; 1,50. 960; 480; 1,00. 960; 960; 0,45. Proposal 2: MRTD = 0.26 µs for non-contiguous intra band CA in FR2-2. Observation 1: It is an important observation that to be able to reuse FR2-1 sites, for FR2-2 sites, then MRTD has to be unchanged, since the new sites will have the same distance to FR1 site as FR2-1. Proposal 3: MRTD = 8 µs for FR2-2 CA inter band. Proposal 4: MRTD = 25 µs for inter band carrier aggregation between FR1 and FR2-2. Proposal 5: MTTD = 8.5 µs for FR2-2 CA inter band. Proposal 6: MRTD = 26.1 µs for inter band carrier aggregation between FR1 and FR2-2. Proposal 7: MRTD = 33 µs for inter-band asynchronous NR-DC, for SCG SCS = 480 kHz and SCG SCS = 960 kHz. Proposal 8: MTTD = 34.1 µs for inter-band asynchronous NR-DC, for SCG SCS = 480 kHz and SCG SCS = 960 kHz. |
| R4-2206005 | Intel | Proposal 1: RAN4 to define the Te requirements as 1.34*64*Tc and 0.67*64*Tc for UL SCSs 480kHz and 960kHz respectively Proposal 2: RAN4 to define timing requirements considering that at least one SSB is available at the UE during the last 40 ms. Proposal 3: RAN4 to change the definition of receive timing difference between carriers in case of NR CA to address the case when TAE + ΔT is larger than one slot. For instance, RTD can be considered between the sub-frame boundaries Proposal 4: In case of NR DC RAN4 to consider the receive timing difference between carriers as the timing difference between the closest slot boundaries. Proposal 5: RAN4 to change the definition of transmit timing difference between carriers in case of NR CA to address the case when MRTD is larger than one slot Proposal 6: In case of NR DC RAN4 to consider the transmit timing difference between carriers as the timing difference between the closest slot boundaries. Proposal 7: Agree on MRTD for intra-band CA during RAN4 #102-e meeting directly applying TAE value agreed in RF to MRTD requirement Proposal 8: RAN4 to define MRTD and MTTD values for FR1 + FR2-2 inter-band CA equal to MRTD and MTTD values for FR1 + FR2-1 inter-band CA, which are 25us and 26.1us for MRTD and MTTD respectively. |
| R4-2206115 | Qualcomm | Observation 1: RAN4 specifies the UE transmit timing based on the SSB bandwidth. Observation 2: An additional margin in the timing error is specified to account for UE artifacts related to DL to UL switching. This additional margin is needed to account for various artifacts such as - timing drift from SSB measurement to UL transmission, DAC clock uncertainty, RF calibration error on both UE and TE side. Observation 3: The upper limit on the timing error is half CP length on the uplink transmission. Observation 4: A frequency offset of 0.1ppm leads to a timing drift of 16/8/4/2ns for SSB periodicity of 160/80/40/20ms respectively. Observation 5: A margin of around 16ns is needed to account for DAC clock uncertainty to handle worst case scenario in a typical implementation for 480/960 UL SCS. Observation 6: Additional margin is needed to account for RF calibration errors due to various group delays on both the UE and the TE side. A margin of around 16ns can be considered for RAN4 requirement purpose. Proposal 1: A total margin of around 40ns is needed for TSSB of 80ms, 36ns for TSSB of 40ms and 32ns for TSSB of 20ms Observation 8: Based on the above analysis, the table below shows the minimum Te/CP ratio needed for various SSB periodicities: SSB SCS; UL SCS; T_SSB = 20ms; T_SSB = 40ms; T_SSB = 80ms. Te/CP ratio; Margin (ns); Te/CP ratio; Margin (ns); Te/CP ratio; Margin (ns). 120; 480; 0.35; 33; 0.37; 36; 0.40; 40. 480; 480; 0.26; 33; 0.28; 36; 0.31; 40. 480; 960; 0.5; 32; 0.5; 32; 0.50; 32. 960; 480; 0.25; 34; 0.26; 35; 0.30; 41. 960; 960; 0.5; 34; 0.5; 34; 0.50; 34. Proposal 2: Define Te requirements considering the above Te/CP ratio and TSSB combinations |
| R4-2203533 | Nokia | Draft CR adding timing requirements for FR2-2 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: UE transmit timing error

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1-1: Basic principles for defining Te

- Proposal 1 (Nokia): Choose Te such that the condition eRS+eDRIFT < Te < (TCP-TCH-TAC,Q)/2  holds.

  - where eRS is the DL timing estimation accuracy

  - eDRIFT is the clock drift for the RS periodicity

  - TCP is the CP length

  - TCH is the channel delay spread (as discussed in Issue 2-1-2)

  - TAC,Q is the timing advance command step

- Proposal 2 (Nokia): Increase of the SSB SCS should be translated in the calculation of the Te requirements.

- Proposal 3 (Nokia): Reduction on the availability of SSB should be translated in the calculation of the Te requirements.

- Proposal 4 (Nokia): Determine the Te requirements using the formula

Te=α144×κ×2-μ×Tc+10007.2×2μSSB -10007.2×2μSSB,min +0.1×XSSB-0.1×XSSB,max

  - Κ = 64

  - μ and μSSB are the numerologies for UL signals and SSB;

  - XSSB is the SSB availability in ms;

  - μSSB,min is the minimum numerology used for the SSB for μ

  - XSSB,max is the maximum SSB availability specified for μ;

  - α controls the baseline Te value in relation to the CP length

  - Tc is the basic timing unit defined in TS 38.211

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Nokia | As proponent company, we are fine with proposals 1 to 4. These are meant for helping us to determine how the Te requirements should be defined. Additions to the previous discussion is how the Te requirements should be adapted depending on the SSB availability and SCS, and a calculation formula for Te that helps us to define values following those principles. Please notice there was a typo in the definition of TCH. |
| Qualcomm | For proposal 1, we think other parameters such as DAC clock uncertainty, RF calibration error etc also need to be considered. We can define the following condition: eRS + eDRIFT + eDAC + eCAL < Te < (TCP-TCH-TAC,Q)/2, Where, - eRS is the DL timing estimation accuracy - eDRIFT is the clock drift for the RS periodicity - eDAC is the DAC clock uncertainty ~1-2 DAC clock cycles - eCAL is the RF calibration error - TCP is the CP length - TCH is the channel delay spread - TAC,Q is the timing advance command step Proposal 2 and 3 are not very clear to us. Isn’t proposal 1 already taking these parameters into consideration? We cannot agree with Proposal 4 as it doesn’t consider other timing error sources such as DAC clock uncertainty, RF calibration error etc. |
| Huawei | The formula can be taken as baseline as discussed in previous meeting, but it can not be used to derive the value of Te requirement directly as the implementation margin as discussed in issue 2-1-6 can not be reflected. |
| Ericsson | Proposal 1-4 are fine. |
| CATT | For proposal 1, we support the revised condition proposed by Qualcomm. And we agree with proposal 2 and 3. |
| Intel | In general, we agree with the proposed condition and its revision from Qualcomm. But for Te definition we prefer another approach. To our understanding the lower limit of eRS + eDRIFT + eDAC + eCAL is inevitable error and Te can never be lower than that value. At the same time the limitation (TCP-TCH-TAC,Q)/2 is not that strict – going beyond this value will just result in UL performance degradation. It is difficult to estimate the lower limit since only eRS + eDRIFT can be estimated accurately and eDAC + eCAL is mostly implementation dependent, while it can be the most significant part. Our approach is to define Te as high as possible limiting it by acceptable UL performance degradation. In that sense Proposals 2 and 3 are not reasonable for us. |
| vivo | For Proposal 1, we understand that it may be not enough to only consider the eDRIFT for timing margin, e.g., RF calibration error for both DL and UL, and UL clock jitter etc. |

Issue 2-1-2: Channel delay spread for defining Te

- Proposal 1 (Nokia): Consider TCH as the delay of the tap with 90% accumulated energy of TDLA30 for 480 kHz and TDLA20 for 960 kHz SCS.

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Nokia | We agree with Proposal 1 From the previous discussion we agreed to consider a channel with delay spread [20] ns for 960 kHz SCS and [30] ns for 480 kHz. SCS. But it was not clear from the agreement how exactly that should related to TCH. In our view we can consider TDLA30 and TDLA20 as typical for this scenario, and consider the accumulated tap energy for determining the value TCH. Ideally, we would use the largest tap of TDLA channel to determine TCH. Here we are willing to compromise and allow 10% of the energy of the channel taps causing some level of inter-symbol interference. |
| Qualcomm | Cannot agree to Proposal 1. We’d like to keep the agreement from the previous meeting on using delay spready of 20ns and 30ns for 960 and 480kHz SCS respectively which got agreed as a compromise. The actual delay spread is likely to be less than these values by using narrow beamforming. |
| Huawei | We think per the agreement in last meeting, 30/20 ns is considered as the max delay spread. In proposal 1, it seems 30/20 ns is considered as RMS DS and actual TCH used are larger than 30/20ns. We would like to hear more views from companies. |
| Ericsson | We have used [30 ns] and [20 ns] for the channel and not the 90% cumulative tap energies at 65 ns and 43 ns. If we had used the 90% values then the upper bound would have been lower. In the end it is percentage of UL CP for Te that matters. We are fine with our proposal in R4-2205417. |
| CATT | Agree with Qualcomm, that is, keep the agreement from the previous meeting on using delay spready of 20ns and 30ns for 960 and 480kHz SCS respectively. |
| Intel | As we commented in Issue 2-1-1, we prefer not to follow the proposed expression for the upper limit of Te requirements. Answering the comment from Huawei, we consider previous agreement on delay spread as RMS DS. The values were derived based on TR38.808 where they refer to RMS DS. |
| vivo | We prefer to follow the agreements from the last meeting, i.e., 30ns for 480kHz and 20ns for 960kHz is considered as the max delay spread. |

Issue 2-1-3: SSB periodicity

- Proposal 1:  For UL SCS of 480/960 kHz, a UE is required to meet the UL timing accuracy requirements if an SSB is available in the last X ms.

  - Option 1 (Qualcomm): X=80/40ms for 480/960 kHz respectively

  - Option 2 (Apple, Huawei): X=20ms

  - Option 3 (CATT, Vivo, Qualcomm, Intel): X=40ms

  - Option 4 (Huawei, ZTE): Two set of requirements, X=40ms and X = 80ms

  - Option 5 (Ericsson): one set X = 80 ms.

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Nokia | We are fine with Option 4 and Option 5. We would like to keep at least 80 ms, in the requirements, otherwise there will be a very harsh restriction on the network configuration, that would lead to non-efficient resource utilization and large power consumption on cells that are not being actively used. |
| Qualcomm | Support Option 1. We can define separate SSB periodicity for 960 and 480kHz SCS. For instance, 480kHz UL SCS requirements can be defined with a periodicity of 80ms and 960kHz UL SCS requirements can be defined with a periodicity of 40ms. We don’t see much value in defining two separate set of requirements. |
| Huawei | We support option 4. We would like to know if requirements are only defined for 40ms, what is the expected behavior when SSB is available in 80 ms? Or it is not expected that SSB is available more than 40ms? |
| Ericsson | Option 5, to give some Te improvement and to conserve UE energy and DL SSB blocks. |
| CATT | In our opinion, option 3, option 4 and option 5 are ok. We think the timing drift difference between 20ms and 40ms SSB periodicity is only 2ns, the saved 2ns does not play a decisive role in Te requirements, and X=20ns means that a large number of reference signal resources will be used. For Qualcomm： For option 1, the timing drift difference between 40ms and 80ms SSB period is 4ns, which contributes to reducing the lower limit of Te. However, as discussed in issue 2-1-1, in addition to the timing drift, the lower limit of Te also includes the DL timing estimation accuracy, the DAC clock uncertainty and the RF calibration error, so for higher UL SCS, it is hard to say how much the saved 4ns plays in defining Te requirements, which needs to be calculated according to different combinations. If the contribution of these 4ns is very small, it is still difficult to define unified Te requirements for some combinations. If we want to define Te for these combinations, we need to define a second set of requirements |
| Intel | Support Option 3. Option 1 also seems reasonable. 480kHz doesn’t have that crucial issue with Te since CP length is twice longer. So longer SSB periodicity (80ms) can be used. |
| ZTE | Support Option 4 to have 2 sets of requirements. Same question as Huawei, if we only have one requirement does it means SSB can only be configured with such a periodicity? |
| vivo | Support Option 3. Defining one set of Te requirement for FR2-2 based on SSB periodicity of 40ms is enough. |

Issue 2-1-4: One/Two set of requirements

- Proposal 1 (CATT, Vivo, Intel, Nokia, Qualcomm, Apple, Ericsson): One set of requirements.

- Proposal 2 (Huawei, ZTE, Mediatek): Define two set of UL timing accuracy requirements

  - Option 2a (Huawei): One for SSB periodicity of 20ms, other for 80ms

  - Option 2b (ZTE): One for SSB periodicity of 40ms, other for 80ms

    - Define test cases for both sets of requirements under different SSB periodicities

  - Option 2c (Mediatek): One set correspond to Te/CP occupancy of 50%

- Proposal 2a (Mediatek): Introduce a new UE capability to indicate which set of UE transmit timing error requirements can be applied

- Proposal 3 (Nokia): If two set of requirements are defined, Te requirements with multiple SSB availability periods are applicable to all the UEs.

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Nokia | We agree with Proposal 1, and proposal 3. For proposal 2, we can compromise with it as long as we have requirements for 80 ms, we are fine with 1st preference: Proposal 1: one set of requirements 2nd preference: Option 2a: 20 ms + 80 ms with ZTE addition: Define test cases for both sets of requirements under different SSB periodicities 3rd preference: Option 2b: 40 ms + 80 ms with ZTE addition: Define test cases for both sets of requirements under different SSB periodicities We do not agree with Option 2c. We prefer to discuss principles instead of CP percentages for the definition of Te requirements. We do not agree with Proposal 2a: Te requirements are essential to the 5G network performance. If UEs are not applying Te requirements than there is no clear expected behaviour from the network side. We agree with Proposal 3, and think it is somehow related to the addition on Option 2b (Define test cases for both sets of requirements under different SSB periodicities). |
| Qualcomm | We support proposal 1. A single set of requirements should be defined. If, at all, a second set of requirements are defined, they must be defined as in proposal 2c. Otherwise we don’t see any value in defining a second set of requirements. |
| Huawei | As commented in above issue, what is the expected behavior when SSB is available less frequent than the condition of the single set of requirements? Regarding the test cases, we think only one set of requirements need to be tested, as the main difference results from timing drift. Regarding new capability, we have similar question. What is the expected UE behavior when UE does not support the capability? Is UE allowed to transmit UL? |
| Ericsson | Proposal 1, one set. |
| CATT | Agree with option 1, but can compromise to option 2b. |
| MTK | Fine with proposal 1.To our understanding, if the SSB is available less frequent than the condition, then the Te requirement is not applicable. For 2 requirements, support Option 2c and proposal 2a. In proposal 2a regarding the new capability, UE is still required to meet one set of Te requirement as indicated by the UE capability. |
| Intel | Support Proposal 1 |
| ZTE | Option 2b. First of all TCs are needed for both requirements. Secondly comparing 2a and 2b, we think that the requirements for 40ms can somehow “cover” that of 20ms. |
| vivo | Support proposal 1. Defining one set of Te requirement for FR2-2 based on SSB periodicity of 40ms is enough. |

Issue 2-1-5: SSB and UL SCS combinations

- Proposal 1:  Define requirements for the following (SSB SCS, UL SCS) cases:

  - 120, 480

    - Yes – Nokia, Qualcomm, CATT

    - FFS: Depends on UE implementation challenges – Apple

  - 480, 960

    - Yes – CATT, Nokia

    - No – Vivo

    - FFS: Depends on UE implementation challenges – Apple, Qualcomm

  - 120, 960

    - No – Nokia, Qualcomm, Apple, CATT, Vivo,

- Recommended WF

  - Can we agree to:

    - Define the requirements for 120, 480.

    - Do not define the requirements for 120, 960.

  - Further discuss the other cases of combinations of SSB SCS and UL SCS for SCS = 120, 480 and 960 kHz.

| Company | Comments |
| --- | --- |
| Nokia | We agree to compromise on our initial preference and preclude SSB with 120 kHz and UL SCS of 960 kHz. As for the other options, we believe we should not preclude them, in order to keep more network configuration options for scenarios with UEs with mixed capabilities. |
| Qualcomm | Fine with the recommended WF. The case of 480,960 seems quite challenging as sufficient UE implementation margin is needed to meet the requirements |
| Huawei | Fine the recommended WF. And also fine with (480,960) |
| Ericsson | We can exclude (120,960). |
| CATT | We support to define Te requirements for the case of (120, 480) and (480, 960) to keep as many scenarios as possible, and agree to preclude the case of (480, 960). |
| Intel | Ok with Recommended WF |
| vivo | Fine with the recommended WF. For the combination of (120kHz SSB SCS, 960kHz uplink signal SCS) and (480kHz SSB SCS, 960kHz uplink signal SCS), the margin is very small which is challenging for UE implementation. |

Issue 2-1-6: UE implementation margin to be considered in Te requirements for 480/960kHz UL SCS

- Option 1 (Qualcomm):  A total UE implementation margin of around 40ns is needed for TSSB of 80ms, 36ns for TSSB of 40ms and 32ns for TSSB of 20ms

- Option 2 (Nokia): eRS+eDRIFT where

  - eRS is the DL timing estimation accuracy,

  - eDRIFT is the clock drift for the reference signal periodicity

- Option 3 (new) A total UE implementation margin of around eDRIFT + eDAC + eRF_Calibration is needed where:

  - eDRIFT is the clock drift for the reference signal periodicity, and is 8 ns, 4 ns, and 2 ns for 80 ms, 40 ms, and 20 ms SSB availability

  - eDAC is the DAC clock uncertainty and is:

    - Option 3a (Qualcomm): 16 ns, equivalent to 2 x clock cycle length with Fs/8, FS=998.4 MHz

    - Option 3b (Nokia): [8] ns, equivalent to 1 x clock cycle length with [Fs/8], FS=998.4 MHz

    - Option 3c FFS

  - eRF_Calibration is the additional margin is needed to account for RF calibration errors due to various group delays on both the UE and the TE side

    - Option 3d (Qualcomm): eRF_Calibration =16 ns

    - Option 3e (Nokia): leave eRF_Calibration margin for RAN5 discussion

- Recommended WF

  - Discuss the options

| Company | Comments |
| --- | --- |
| Nokia | Thanks Qualcomm for the proposal. We think it is important to discuss the individual components of Option 1, therefore we propose Option 3 with sub-options. We would be fine with Option 3b and Option 3e. As for the DAC error we believe that 1x clock sampling time is enough. Since the error may be +- sampling time, the UE can already compensate for the 8x undersampled clock uncertainty. For the RF calibration margin, the RAN5 requirements already include additional margin for the RRM tests. If you check Table 5.4.1.1.5-4 in 38.533 (v17.1.0), the requirement used in the test for 240 kBz SSB and 120 kHz UL signals is 3.75*64*Tc instead of 3*64*Tc that is specified in Table 7.1.2-1 of 38.133. |
| Qualcomm | Thanks, Nokia, for the new proposal. We prefer option 3a and 3d. |
| Huawei | We think is option 1 is more related to the implementation margin. Option 2 are more like the inevitable error. The value can be discussed together in issue 2-1-8 |
| CATT | Agree with Huawei, and we think eRS and eDRIFT are more like at least factors to consider, and option 1 is more related to the implementation margin. |
| Nokia | Just to clarify, option 3 basically discussing the parts that compose Option 1. |
| Intel | We consider implementation margin as eDAC + eRF_Calibration. For eDAC we support Option 3b and for eRF_Calibration we don’t have any preferred value. |
| vivo | We support a total UE implementation margin include eDRIFT + eDAC + eRF_Calibration. For eDAC, Option 3c is fine. For eRF_Calibration, we support Option 3d. |

Issue 2-1-7: Calculation of the Te values

- Proposal 1 (Nokia): Define Te requirements considering the Te calculation formula in Issue 2-1-1, Proposal 4 with the parameters below:

|  | 120 kHz SCS | 480 kHz SCS |  | 960 kHz SCS |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Non statistical nature | Statistical nature | Non statistical nature | Statistical nature |
| α | 0.182 | 0.22 | 0.20 | 0.18 | 0.16 |
| μSSB,min | 3 | 3 | 3 | 5 | 5 |
| XSSB,max | 160 | 80 | 80 | 80 | 80 |

- Recommended WF

  - Discuss the Proposal

| Company | Comments |
| --- | --- |
| Nokia | We agree with Proposal 1. This proposal is in line with the basic principles of Issue 2-1-1. |
| Qualcomm | We don’t agree with the proposal. First, we don’t think that we need to specify stringent requirements for statistical tests. The whole idea behind introducing statistical tests is to define a method to test the challenging requirements. The procedure should not make it even more challenging for the UE to test the requirements. |
| Ericsson | We agree that if statistical nature is the way to define requirement then we should state a stricter Te value, that is a smaller one (like in table, alpha becomes lower). |

Issue 2-1-8: Percentage of UL CP length Te can occupy for UL SCS of 480/960

- With TSSB = 80ms

| SSB SCS | UL SCS | Qualcomm | Nokia | Mediatek | Huawei | Ericsson | Intel |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 480 | 0.40 | 0.22 |  | 0.36 | 0.29 | 0.30 |
| 480 | 480 | 0.31 | 0.13 | 0.50 | 0.27 | 0.22 | 0.30 |
| 960 | 480 | 0.30 | 0.12 |  | 0.22 | 0.22 | 0.30 |
| 480 | 960 | 0.50 | 0.18 | 0.50 | 0.38 | 0.20 | 0.30 |
| 960 | 960 | 0.50 | 0.15 |  | 0.38 | 0.20 | 0.30 |

- With TSSB = 40ms

| SSB SCS | UL SCS | Qualcomm | Nokia | Mediatek | Intel | Vivo |
| --- | --- | --- | --- | --- | --- | --- |
| 120 | 480 | 0.37 | 0.19 |  | 0.30 | 0.38 |
| 480 | 480 | 0.28 | 0.10 | 0.50 | 0.30 | 0.38 |
| 960 | 480 | 0.26 | 0.09 |  | 0.30 | 0.38 |
| 480 | 960 | 0.50 | 0.12 | 0.50 | 0.30 |  |
| 960 | 960 | 0.50 | 0.09 |  | 0.30 | 0.31 |

- With TSSB = 20ms

| SSB SCS | UL SCS | Qualcomm | Nokia | Apple | Mediatek | Huawei | Intel |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 480 | 0.35 | 0.18 |  |  | 0.30 | 0.30 |
| 480 | 480 | 0.26 | 0.09 | 0.40 | 0.50 | 0.22 | 0.30 |
| 960 | 480 | 0.25 | 0.07 |  |  | 0.18 | 0.30 |
| 480 | 960 | 0.50 | 0.10 |  | 0.50 | 0.29 | 0.30 |
| 960 | 960 | 0.50 | 0.07 | 0.40 |  | 0.29 | 0.30 |

- Recommended WF

  - Discuss the options

| Company | Comments |
| --- | --- |
| Nokia | We think we should discuss the basic principles first. |
| Huawei | For issue 2-1-8, we see the proposed value are quite diverse. May be we can first converge to some range. For instance for 20 ms SSB, it is reasonable that value should be something within (25%-35% CP length), and for 80 ms SSB, the range is (35% - 50%) |
| Ericsson | The Nokia Te values for SSB period of 80 ms are fine for us, besides our own proposal. |
| CATT | Suggest to discuss the basic principles first. |
| Intel | Since, as we commented for Issue 2-1-1, we define Te by the upper limit of acceptable UL performance degradation, our proposed value doesn’t depend on the SSB periodicity. Based on that we added Intel’s column to tables of each SSB periodicity |
| Nokia | Considering the GTW discussion, we had: - Tentative agreement - Note: Set of supported SSB SCS and UL SCS combinations will be decided separately - For X = 80ms SSB SCS; UL SCS; Proposal 1. 120; 480; [0.35]. 480; 480; [0.30]. 960; 480; [0.25]. Support: Huawei, Intel, QC Object: Nokia - For X = 40ms SSB SCS; UL SCS; Proposal 1; Proposal 2. 480; 960; [0.40]; [0.43]. 960; 960; [0.40]; [0.38]. Session chair: continue discussion in the 2nd round.. For the 480 kHz we would like to compromise with the tentative agreement that was proposed in the GTW. For 960 kHz, the values of Proposal 1 and Proposal 2 violate the condition that we discussed in RAN4 #101, TCP - TCH - 2 ( Te + TAC,Q /2 ) > 0 If we consider the formula, with - TCP = 73.2 ns - TCH = 20 ns - TAC,Q = 8.1 ns The maximum Te value that can be achieved is - Te,max = 22.5 ns, which is 30.7% of the CP length for 960 kHz - TCP - TCH - 2 ( Te,max + TAC,Q /2 ) = 73.2 – 20 – 2*(22.6 + 8.1/2) = 0.1 ns Therefore, since we would like to propose the following values for 960 kHz SCS: - 480 kHz SSB and 960 kHz UL – Te with 30.7% of the CP - 960 kHz SSB and 960 kHz UL – Te with 30.7% of the CP |

Issue 2-1-9: Initial transmit timing accuracy test

- Proposal 1 (Apple):  Test cases for Te requirements for FR2-2 will be designed as having statistical nature, i.e., something like “The rate of UE meeting the Te requirement observed during repeated tests shall be at least 90%.”.

- Proposal 2 (Nokia): Te requirements and classification of statistical test should be discussed together

  - If RAN4 agrees that Te requirements are classified as statistical, smaller Te values should be specified.

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Nokia | We agree with Proposals 1 and 2 together. It is fair to discuss the nature of the statistical classification of Te RRM tests. We also understand that classification of statistical tests is an implicit relaxation of the Te requirements, therefore both discussions belong together. |
| Qualcomm | Support proposal 1. Cannot agree with proposal 2. |
| Huawei | Fine with proposal 1. |
| Ericsson | If statistical nature is the way to define requirement then we should state a stricter Te value, that is a smaller one |
| Nokia | Second reply after the GTW of 23 of February: We don’t agree with Proposal 1-2. As commented in the 1st round, we believe that Proposal 1 is a relaxation in the Te values. Considering that the Te values are on the boundaries of the maximum values discussed in the Te principles, we think that the proposals are not agreeable. |

### Sub-topic 2-2: Gradual timing adjustment

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-2-1: Gradual timing adjustment

- Proposal 1 (Huawei): Discuss whether to have smaller RF margin and finer granularity of timing drift for gradual adjustment in FR2-2 based on conclusion on Te requirements.

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Nokia | Fine with the proposal. |
| Huawei | Current Tp Tq requirements is too large for FR2-2, which may have similar impact to UL performance with too relaxed Te requirements. It can only be discussed based on conclusion of Te. |
| Ericsson | We need to settle Te first. |
| Intel | Agree with Proposal 1 |
| vivo | Fine with proposal 1. |

### Sub-topic 2-3: Timing advance

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-3-1: Timing advance adjustment accuracy

- Proposal 1 (Apple): The UE timing advance adjustment accuracy for 480/960kHz SCS is defined as shown below:

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc | [±16 Tc] | [±12 Tc] |

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Nokia | We prefer to keep the existing agreement. It is not clear the justification for increasing TA accuracy. |
| Ericsson | Like Nokia we prefer to keep the existing agreement WF on R17 NR_ext_to_71GHz_RRM_1 Meeting # 100-e R4-2115351 Agreements: The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in the below table. Note: Revisit if certain implementation issues are identified. UL Sub Carrier Spacing(kHz); 15; 30; 60; 120; 480; 960. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc; [±8 Tc]; [±4 Tc]. |

### Sub-topic 2-4: MRTD

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-4-1: MRTD definition

- Proposal 1a (Intel): RAN4 to change the definition of receive timing difference between carriers in case of NR CA to address the case when TAE + ΔT is larger than one slot. For instance, RTD can be considered between the sub-frame boundaries:

- Proposal 1b (Intel): In case of NR DC RAN4 to consider the receive timing difference between carriers as the timing difference between the closest slot boundaries

- Proposal 2 (CATT): Change the definition of MRTD for NR DC in legacy spec as below, so that it could be larger than 0.5 slot

  - A UE shall be capable of handling a relative receive timing difference between slot timing boundary of a cell belonging to MCG and slot timing boundary from the same slot index of a cell belonging to the SCG to be aggregated for NR DC operation. A UE shall be capable of handling a relative receive timing difference among the closest slot timing boundaries of different carriers to be aggregated in NR carrier aggregation

- Proposal 3 (new): RAN4 to change the definition of receive timing difference between carriers in case of NR CA to address the case when MRTD is larger than one slot. For instance, RTD can be considered between the slot boundaries

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Huawei | As analyzed in our paper, we prefer not to extend of usage of MRTD to sub-frame/slot/frame alignment. Another approach is to cap the MRTD by 0.5 slot length of corresponding SCS to avoid ambiguities as discussed in previous meeting. |
| Intel | Support Proposals 1a, 1b and 2. We think it is necessary to have requirements on whether UE can handle timing difference which results in a slot index misalignment between carriers in NR CA. |
| Nokia | We agree with Proposal 1a, but it should be “slot” instead of subframe, we would prefer to refer to MRTD instead of TAE + ΔT, therefore we made proposal 3 with the correction. We are not sure about Proposal 2, since it also changes legacy requirements. Can we have a text proposal that only impacts FR2-2? On proposal 1b, we understand the MRTD is specified as the closest slot timing between MCG and SCG, but why do we need to change to the carriers? We understand that the spec is already clear in that case and no change is needed. |
| Ericsson | ### IN WF R4- 2202657 1.2.2 MRTD we agreed: Basic principles - Define MRTD requirements in FR2-2 based on the following rules in: - For Async cases: MRTD = 0.5 slot - For sync cases: MRTD = TAE + propagation delay difference - FFS whether and how to change the MRTD definition, so that it could be larger than 0.5 slot. Carrier Aggregation: Based on WF agreement from RAN4#101-bis, we agree with Intel to change the definition of receive timing difference between carriers in case of NR CA to address the case when TAE + ΔT is larger than one slot. We think that frame boundaries (10 ms) can be used in definition of definition of RTD. (Subframe (1 ms), could also work, but frame boundaries have been used before, for instance in definition of Cell Phase Sync in TS 38.133 definition 7.4.1). Dual Connectivity: The same agreement (MRTD = TAE + propagation delay difference) applies to synchronous Dual Connectivity. We can use that frame boundaries (10 ms) can be used in definition of definition of RTD. (Subframe (1 ms), could also work). |
| Huawei | ### Further comment: We are fine to update the defination of CA, but we think there is no need to change it for sync DC as option 1b. Regarding using frame timing, according to TS 38.300, there could be slot offset for CA. Thus, is it better to say it is the timing differece excluding the slot offset if configured to UE? |

Issue 2-4-2: MRTD for intra-band non-contiguous CA

- Proposal 1 (Nokia, Ericsson, Intel): MRTD = 0.26 µs for non-contiguous intra band CA in FR2-2

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| Nokia | This issue is being further discussed in RF room and maybe TA. Therefore, we think that we should keep consistency with the RF agreements. We can try a tentative agreement conditional to the RF decisions as - Proposal 2: MRTD = [0.26] µs for non-contiguous intra band CA in FR2-2 provided that RF defined TAE requirements for intra-band non-contiguous CA. |
| LGE | We are fine with [0.26]us. After final confirmation in RF, [ ] can be removed. |
| Ericsson | Proposal 1. TAE = [260 ns] for non-contiguous intra band has been approved in BS RF section, in brackets in R4-2203016, WF on BS RF Tx requirements and since non-contiguous intra band CA is assumed to be collocated we keep that assumption in FR2-2. Then MRTD = TAE = 260 ns. |
| Intel | Ok with both Proposal 1 and 2. We can strictly follow RF agreement and keep brackets, it should not be a problem to remove it later. |

Issue 2-4-3: MRTD for FR1 and FR2-2 inter-band CA

- Proposal 1 (Nokia, Vivo, Huawei, Ericsson, Intel): The existing MRTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2, i.e., MRTD = 25 µs

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Nokia | We agree with Proposal 1 |
| Ericsson | We agree with Proposal 1 |
| CATT | Ok with Proposal 1. |
| Intel | Issue depends on the outcome of Issue 2-4-1. If new definition for MRTD will not be agreed, then MRTD should be defined as 0.5 slot |

Issue 2-4-4: MRTD for FR1 and FR2-2 NR DC - Synchronous

- Proposal 1 (Nokia, CATT, Vivo, Huawei, Ericsson): The existing MRTD requirements for FR1 and FR2-1 synchronous NR-DC can be reused for FR1 and FR2-2, i.e., MRTD = 33 µs

- Proposal 3 (Intel):

- Recommended WF

  - Discuss the proposals.

| Company | Comments |
| --- | --- |
| Nokia | We agree with Proposal 1. |
| Ericsson | We agree with Proposal 1 |
| CATT | Support proposal 1. |
| Intel | Issue depends on the outcome of Issue 2-4-1. If current definition for MRTD will be kept, then MRTD should be defined as 0.5 slot |

### Sub-topic 2-5: MTTD

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-5-1: Basic principles

- Proposal 1 (CATT, Huawei): Define MTTD requirements in FR2-2 based on the following rule:

  - MTTD = MRTD + (TA step size / 2+ TA adjustment accuracy + Te) in cc1 + (TA step size / 2 + TA adjustment accuracy +Te) in cc2

- Proposal 2a (Intel): RAN4 to change the definition of transmit timing difference between carriers in case of NR CA to address the case when MRTD is larger than one slot.

- Proposal 2b (Intel): In case of NR DC RAN4 to consider the transmit timing difference between carriers as the timing difference between the closest slot boundaries.

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Huawei | Support proposal 1. Regarding the case when MRTD is larger than one slot, similar comments as issue 2-4-1. |
| CATT | Support proposal 1. |
| Intel | Ok with Proposal 1 and the same comment for MTTD definition as for MRTD definition. |

Issue 2-5-2: MTTD for FR1 and FR2-2 inter-band CA

- Proposal 1 (Vivo, Ericsson, Intel): The existing MTTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2, i.e., MTTD = 26.1 us

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| Nokia | We are fine with proposal 1. |
| Huawei | No strong views. However, considering TA and Te requirements for FR2-2, simply reusing the same value may contradict with the principle of defining MTTD. |
| LGE | Similar view with Huawei |
| Ericsson | We are fine with proposal 1. |
| Intel | Concern from Huawei sounds reasonable |
| vivo | We are fine with proposal 1. We are also OK with considering TA and Te requirements for FR2-2 which is commented by Huawei. |

Issue 2-5-3: MTTD for FR1 and FR2-2 NR DC - Synchronous

- Proposal 1 (Vivo): The existing MTTD requirements for inter-band synchronous NR DC for FR1 and FR2-1 can be reused for inter-band synchronous NR DC for FR1 and FR2-2, i.e., MTTD = 34.1 us

- Recommended WF

  - Discuss the proposal.

| Company | Comments |
| --- | --- |
| Nokia | We are fine with proposal 1. |
| Huawei | No strong views. However, considering TA and Te requirements for FR2-2, simply reusing the same value may contradict with the principle of defining MTTD. |
| LGE | Similar view with Huawei |
| Ericsson | We are fine with proposal 1. |
| Intel | Concern from Huawei sounds reasonable |
| vivo | We are fine with proposal 1. We are also OK with considering TA and Te requirements for FR2-2 which is commented by Huawei. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2203533 Draft CR adding timing requirements for FR2-2 | Huawei: Depends on the issues under discussion. |
|  | Ericsson: Prefer one set 80 ms. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 UE transmit timing error | Issue 2-1-1: Basic principles for defining Te Companies’ views: Although some companies are okay with proposal 1, it was mentioned that these are not the only factors. Other proposals didn’t get many comments. Tentative agreements: No agreement Candidate options: Recommendations for 2nd round: No need to further discuss this issue. Directly discuss the Te values in Issue 2-1-8, as suggested by the chair. |
|  | Issue 2-1-2: Channel delay spread for defining Te Companies’ views: Many companies prefer to keep the agreement from previous meeting Tentative agreements: No agreement Candidate options: Recommendations for 2nd round: No need to further discuss this issue. Directly discuss the Te values in Issue 2-1-8, as suggested by the chair. |
|  | Issue 2-1-3: SSB periodicity & Issue 2-1-4: One/Two set of requirements Companies’ views: GTW agreements: - For UL SCS of 480/960 kHz, a UE is required to meet the UL timing accuracy requirements if an SSB is available in the last X ms - X=80 ms for UL SCS of 480 kHz - X=40 ms for UL SCS of 960 kHz - Note: test cases will be defined for both cases - Note: the agreement can be revisited in case no feasible Te requirements values are identified. Candidate options: Recommendations for 2nd round: No further discussion needed. |
|  | Issue 2-1-5: SSB and UL SCS combinations Companies’ views: Most companies agree with the recommended WF Tentative agreements: - Define the requirements for 120, 480. - Do not define the requirements for 120, 960. Candidate options: Recommendations for 2nd round: Further discuss whether the combination of (480, 960) can be supported. |
|  | Issue 2-1-6: UE implementation margin to be considered in Te requirements for 480/960kHz UL SCS Companies’ views: Most companies agree that a UE implementation margin of at-least eDRIFT + eDAC + eRF_Calibration is needed in addition to the DL timing estimation uncertainty of eRS. No consensus on the exact values for eDAC and eRF_Calibration Tentative agreements: A UE implementation margin of at-least eDRIFT + eDAC + eRF_Calibration is needed in addition to the DL timing estimation uncertainty of eRS Candidate options: Recommendations for 2nd round: No further discussion needed. Directly discuss the Te values in Issue 2-1-8, as suggested by the chair. |
|  | Issue 2-1-7: Calculation of the Te values Companies’ views: Very few comments were received. One company doesn’t agree with the proposal Tentative agreements: No agreement Candidate options: Recommendations for 2nd round: No further discussion needed. Directly discuss the Te values in Issue 2-1-8, as suggested by the chair. |
|  | Issue 2-1-8: Percentage of UL CP length Te can occupy for UL SCS of 480/960 Companies’ views: GTW (Tentative) agreements: - For UL SCS of 480/960 kHz, a UE is required to meet the UL timing accuracy requirements if an SSB is available in the last X ms. - For X = 80ms SSB SCS; UL SCS; Proposal 1. 120; 480; [0.35]. 480; 480; [0.30]. 960; 480; [0.25]. - For X = 40ms SSB SCS; UL SCS; Proposal 1; Proposal 2. 480; 960; [0.40]; [0.43]. 960; 960; [0.40]; [0.38]. Candidate options: - For X = 40ms SSB SCS; UL SCS; Option 1; Option 2; Option 3. 480; 960; [0.40]; [0.43]; [0.31]. 960; 960; [0.40]; [0.38]; [0.31]. Recommendations for 2nd round: Consider agreeing to proposal 1 for X=80ms and further discuss the options for X=40ms |
|  | Issue 2-1-9: Initial transmit timing accuracy test Companies’ views: Diverse views. Proponent of proposal 2 withdrew their proposal. Tentative agreements: No agreements Candidate options: - Proposal 1: Test cases for Te requirements for FR2-2 will be designed as having statistical nature, i.e., something like “The rate of UE meeting the Te requirement observed during repeated tests shall be at least 90%.”. Recommendations for 2nd round: Continue discussion in the second round. Since proponent of proposal 2 has withdrawn their proposal. Only discuss Proposal 1. |
| Sub-topic#2-2 Gradual timing adjustment | Issue 2-2-1: Gradual timing adjustment Companies’ views: Some companies want to re-visit this after some conclusions on Te values. Tentative agreements: No agreement Candidate options: Recommendations for 2nd round: Re-visit after conclusion on Te values. No further discussion needed |
| Sub-topic#2-3 Timing advance | Issue 2-3-1: Timing advance adjustment error Companies’ views: Two companies do not agree with the proposal Tentative agreements: No agreement Candidate options: Recommendations for 2nd round: Can the proponent company compromise to the previous agreement? |
| Sub-topic#2-4 MRTD | Issue 2-4-1: MRTD definition Companies’ views: Diverse views Tentative agreements: No agreement Candidate options: Recommendations for 2nd round: Continue discussion in the second round |
|  | Issue 2-4-2: MRTD for intra-band non-contiguous CA Companies’ views: To be aligned with the RF session, some companies suggested to use [] values Tentative agreements: MRTD = [0.26] µs for non-contiguous intra band CA in FR2-2 Candidate options: Recommendations for 2nd round: No further discussion |
|  | Issue 2-4-3: MRTD for FR1 and FR2-2 inter-band CA and Issue 2-4-4: MRTD for FR1 and FR2-2 NR DC - Synchronous Companies’ views: Some companies are fine with the proposal. One company suggested to revisit after a conclusion on Issue 2-4-1. Tentative agreements: Candidate options: - Proposal 1: The existing MRTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2, i.e., MRTD = 25 µs - Proposal 2: The existing MRTD requirements for FR1 and FR2-1 synchronous NR-DC can be reused for FR1 and FR2-2, i.e., MRTD = 33 µs Recommendations for 2nd round: Continue discussion considering Issue 2-4-1. |
| Sub-topic#2-5 MTTD | Issue 2-5-1: Basic principles Companies’ views: Some companies are fine with the proposal 1. Some companies suggested to take Issue 2-4-1 into consideration for proposal 2 Tentative agreements: - MTTD = MRTD + (TA step size / 2+ TA adjustment accuracy + Te) in cc1 + (TA step size / 2 + TA adjustment accuracy +Te) in cc2 Candidate options: - Proposal 2a: RAN4 to change the definition of transmit timing difference between carriers in case of NR CA to address the case when MRTD is larger than one slot. - Proposal 2b: In case of NR DC RAN4 to consider the transmit timing difference between carriers as the timing difference between the closest slot boundaries. Recommendations for 2nd round: Continue discussion on proposal 2 considering Issue 2-4-1. |
|  | Issue 2-5-2: MTTD for FR1 and FR2-2 inter-band CA and Issue 2-5-3: MTTD for FR1 and FR2-2 NR DC - Synchronous Companies’ views: Some companies are fine with the proposal 1. Some companies suggested to take Te and TA values into consideration. Tentative agreements: Candidate options: - FFS: The existing MTTD requirements for inter-band CA for FR1 and FR2-1 can be reused for inter-band CA for FR1 and FR2-2, i.e., MTTD = 26.1 us - FFS: The existing MTTD requirements for inter-band synchronous NR DC for FR1 and FR2-1 can be reused for inter-band synchronous NR DC for FR1 and FR2-2, i.e., MTTD = 34.1 us Recommendations for 2nd round: No further discussion needed since Te values are not finalized yet. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2203533 Draft CR adding timing requirements for FR2-2 | To be revised |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on NR extension to 71 GHz RRM requirements (Part 1) | Qualcomm | WF is supposed to capture the agreements and open issues |
| Draft CR on deriveSSB-IndexFromCell tolerance | Qualcomm | Draft CR based on agreements from GTW and first round |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2204189 | Scheduling restriction due to L3 measurements for FR2-2 | Mediatek | To be revised |  |
| R4-2204190 | Scheduling restriction due to L1 measurements for FR2-2 | Mediatek | To be revised |  |
| R4-2203533 | Draft CR adding timing requirements for FR2-2 | Nokia | To be revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm | Prashant Sharma | prasshar@qti.qualcomm.com |
| Nokia | Rafael Paiva | Rafael.paiva@nokia.com |
| Huawei | Zhongyi Shen | shenzhongyi3@huawei.com |
| Intel | Ilya Bolotin | ilya.bolotin@intel.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)