3GPP TSG-RAN WG4 Meeting # 102-e 												R4-2206771

Electronic Meeting, February 21 – March 3, 2022

Agenda item:			10.20.3, 10.20.3.1, parts of 10.20.3.4

Source:	Moderator (Ericsson)

Title:						Email discussion summary for [102-e][228] NR_redcap_RRM_1

Document for:	Information

# Introduction

The discussion covers NR RedCap RRM AI within 10.20.3, 10.20.3.1, and parts of 10.20.3.4.

When updating this document, please remember to:

- use track changes while adding your comments in this document (only updates marked with change marks will be taken into the next version),

- change the file name, adding your company name, according to the instructions from RAN4 chair:

- Length of file names shall be reduced, e.g.

At the beginning of first round, moderators share / ftp / tsg_ran / WG4_Radio / TSGR4_98_e / Inbox / Drafts / [98e][101] NR_NewRAT_SysParameters\Summary_101_1st round_v01.docx

After update by company A: Summary_101_1st round_v02_companyA

After update by company B: Summary_101_1st round_v03_companyA_companyB

After update by company C: Summary_101_1st round_v04_companyB_companyC

# Topic #1: General

Contributions from AI 10.20.3.1.1 are discussed here.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203591 | ZTE Corporation | Reduce the number of identified SSBs for RedCap UEs with 1 Rx. SDT requirements for RedCap UE with 2 Rx are reused from those defined for legacy NR UEs under Rel-17 SDT WI. |
| R4-2204249 | Xiaomi | Proposal 1: There is no impact on the requirement for maximum interruption in paging reception, i.e. Rel-15 requirements shall apply to RedCap. Proposal 2: No need to introduce scheduling availability restriction on 5G NR RedCap UEs performing measurements in HD-FDD bands for IDLE/INACTIVE mode. Proposal 3: Idle/inactive mobility is only based on measurements on the CD-SSB. |
| R4-2204281 | OPPO | Proposal 1: Support reducing the number of cells and number of SSBs for RedCap in FR1 and FR2, e.g., 5 identified cells and 10 SSBs. Proposal 2: There is no impact on the requirement for maximum interruption in paging reception, i.e. Rel-15 requirements shall apply to RedCap. Proposal 3: No need to introduce specific scheduling restriction on RedCap UEs performing measurements in HD-FDD bands. |
| R4-2204282 | OPPO | CR: draft CR on measurements requirements for inactivate state Redcap UE |
| R4-2204321 | vivo | Proposal 1: Prefer to consider reducing the number of cells and number of SSBs for RedCap in FR1 and FR2, if agreed, the starting time of the related investigation work needs be determined. Proposal 2: For paging reception, option 1 can be used for 2RX case. For 1RX, suggest to consider option 2. Proposal 3: For SDT for RedCap with 2 Rx, the current SDT requirements for 2Rx can be reused for Redcap UE with 2RX, i.e., option 1. For the SDT for RedCap with 1 Rx, the existing time window definition can be reused for Redcap UE with 1RX, i.e., option 1. |
| R4-2204902 | Huawei, Hisilicon | Proposal 1: Reuse the legacy number of cells and number of SSBs for RedCap with 1RX in FR1 and FR2. Proposal 2: For RedCap with 1RX, the interruption time shall not exceed TSI-NR + 3*Ttarget_cell_SMTC_period ms. Proposal 3: No need to introduce scheduling availability restriction on NR RedCap UEs performing measurements in HD-FDD bands. |
| R4-2204992 | CMCC | Proposal 1: Do not reduce the number of cells and number of SSBs for RedCap in FR1 and FR2. Proposal 2: There is no impact on the requirement for maximum interruption in paging reception, i.e. Rel-15 requirements shall apply to RedCap. Proposal 3: No need to introduce scheduling availability restriction on 5G NR RedCap UEs performing measurements in HD-FDD bands. Proposal 4: The RedCap UE operating in HD-FDD mode shall receive paging (if transmitted) in a serving cell provided that at least one SSB is available at the UE in the serving cell during the last 160ms duration. Proposal 5: SDT requirements for RedCap UE with 2 Rx are reused from those defined for legacy NR UEs under Rel-17 SDT WI. Proposal 6: Discuss the time windows for 1Rx RedCap UE after the 2Rx requirements are agreed. |
| R4-2205622 | Ericsson | CR: Draft CR on RRC_IDLE mode requirements for RedCap for TS 36.133 |
| R4-2205625 | Ericsson | CR: Draft CR on RRC_IDLE mode requirements for RedCap for TS 38.133 |
| R4-2205628 | Ericsson | Proposal #1: The Rel-15 CONNECTED mode intra- and inter-frequency measurement capability (number of cells and SSBs) to monitor is reused. Proposal #2: RAN4 to confirm the Rel-15 NR requirements for maximum paging rinterruption for paging reception are reeused for RedCap FD-FDD and TDD UEs. Proposal #3: No need to introduce scheduling availability restriction on 5G NR RedCap UEs performing measurements in HD-FDD bands. Proposal #4: The NR frequency bands grouping for FR1 RedCap is defined as in Table 1. Proposal #5: The NR frequency bands grouping for FR2 RedCap is defined as in Table 2. Proposal #6: Time windows defining the valid measurements used for TA validation are reused from Rel-17 SDT discussions but need to be updated to reflect the RedCap 1 Rx measurement times. Proposal #7: SDT requirements for RedCap UE with 2 Rx are reused from those defined for legacy NR UEs under Rel-17 SDT WI. |
| R4-2206078 | MediaTek inc. | - Support reducing the number of cells and the number of SSBs for the rel-17 RedCap UEs in FR1 And FR2 as it was done for reduction of carriers. - The RedCap UE shall be capable to perform measurements of at least 5 identified cells and 10 SSBs in FR1. - The maximum interruption paging reception requirements of existing 5G NR rel-15 shall apply to RedCap rel-17, hence support option 1. - Support reusing the requirements from SDT rel-17 WI to SDT for RedCap in rel-17 for the case of 2Rx RedCap. - Support reusing the TA validation requirements from SDT rel-17 WI to SDT for RedCap in rel-17 with relaxed accuracy performance for the case of 1Rx RedCap. - Support that RAN4 to capture the high speed measurements requirements in the RedCap rel-17 specification. |
| R4-2205626 | Ericsson | Draft CR on RRC_INACTIVE mode requirements for RedCap for TS 38.133 |
| R4-2204908 | Huawei, Hisilicon | Proposal 1: Legacy measurement requirements in IDLE/INACTIVE state can be reused for RedCap UE. |
| R4-2203792 | Apple | Proposal 1: Idle/inactive mobility is only based on measurements on the CD-SSB. |
| R4-2203891 | CATT | Proposal 1: In Idle/inactive mode, only CD-SSB based RRM measurement is supported. |
| R4-2204327 | vivo | Proposal 1: Idle/inactive mobility is feasible based on measurements on both CD-SSB and NCD-SSB. |
| R4-2205623 | Ericsson | Draft CR on RRC_INACTIVE mode requirements for RedCap for TS 36.133 |
| R4-2205000 | CMCC | Proposal 1: Idle/inactive mobility is only based on measurements on the CD-SSB |
| R4-2206114 | Qualcomm Incorporated | Proposal 2: Idle/inactive mobility is based only on measurements on the CD-SSB. (RAN Plenary agreement) |
| R4-2204798 | vivo | Draft CR for maximum interruption in paging reception for Redcap |
| R4-2204800 | vivo | Draft CR for Definitions, symbols and abbreviations for Redcap |

## Open issues summary

### Sub-topic 1-1: Measurement capability

Issue 1-1-1: Whether to reduce the number of cells and number of SSB for FR1

- Proposals

  - Option 1 (ZTE, OPPO, vivo, MTK): Yes.

    - Option 1a: The starting time of the related investigation needs to be determined.

  - Option 2 (HW, CMCC, E///): No

- Recommended WF

Given that it is the last meeting to complete core-part, no time for further investigation. It is recommended to make a decision in this meeting.

Issue 1-1-2: If number of cells and number of SSB are reduced, how much to reduce for FR1

- Proposals

  - Option 1 (OPPO, Vivo, MediaTek): Support reducing the number of cells and number of SSBs for RedCap in FR1, e.g., 5 identified cells and 10 SSBs.

Issue 1-1-3: Whether to reduce the number of cells and number of SSB for FR2

- Proposals

  - Option 1 (ZTE, OPPO, vivo, MTK): Yes.

    - Option 1a: The starting time of the related investigation needs to be determined.

  - Option 2 (HW, CMCC, E///): No

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no need to discuss reduction of cells and SSBs for FR2.

Issue 1-1-4: If number of cells and number of SSB are reduced, how much to reduce for FR2

- Proposals

  - Option 1 (OPPO): Support reducing the number of cells and number of SSBs for RedCap in FR2, e.g., 5 identified cells and 10 SSBs.

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no need to discuss reduction of cells and SSBs for FR2.

Sub topic 1-1

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1: Whether to reduce the number of cells and number of SSB for FR1 Issue 1-1-2: If number of cells and number of SSB are reduced, how much to reduce for FR1 |
| Huawei | Issue 1-1-1: Whether to reduce the number of cells and number of SSB for FR1 Option2. For redcap UE with 1RX, the side condition for intra-frequency and inter-frequency core requirements are the same as that for 2RX UE, i.e, -6dB for intra-frequency measurement and -4dBfor inter-frequency measurement. It means that the DL coverage for normal UE and 1RX UE is the same. Moreover Redcap UE is supposed to support same level of mobility support as a Rel-15 NR UE. Therefore the number of cells and SSB for redcap UE can be the same as legacy NR UE. Issue 1-1-3: Whether to reduce the number of cells and number of SSB for FR2 Same comments as issue 1-1-1. |
| Ericsson | Issue 1-1-1: Whether to reduce the number of cells and number of SSB for FR1 As pointed out in one of the contributions supporting option 1, investigation is needed to determine whether the number of cells and SSBs can be reduced for RedCap UE in CONNECTED mode. We believe the mobility performance should be carefully studied before deciding whether to reduce the number of cells and SSBs. As pointed out in earlier meetings, the RedCap UE is to support same level of mobility as a Rel-15 NR UE. Since RedCap doesn’t support CA/DC, the measurement capability in terms of number carriers were reduced. However, the number of cells/SSBs to monitor on each of those carriers should not be impacted. System study would be needed to study the mobility performance, and since it is the last meeting to complete the core, we don’t think there is time for that. Issue 1-1-2: If number of cells and number of SSB are reduced, how much to reduce for FR1 No reduction as commented in previous issue. |
| Apple | Issue 1-1-1: Whether to reduce the number of cells and number of SSB for FR1 Option 2 is same as our proposal in previous meeting, and we proposed to handle the measurement capability in a same way as cat-1bis UE in LTE (no reduction for measurement capability). But we could compromise to option 1 if the reduction number is agreeable to all companies. Agree with moderator that this is the last meeting, and if we could not decide on the reduced number we may go with option 2. Issue 1-1-2: If number of cells and number of SSB are reduced, how much to reduce for FR1 |
| CMCC | Issue 1-1-1: Whether to reduce the number of cells and number of SSB for FR1 Option2. Given the number of carriers are already reduced, we don’t think number of cells or SSBs can be further reduced considering the limited time and mobility performance. Issue 1-1-3 and issue1-1-4 Support recommended WF. No reduction. |
| MediaTek | Issue 1-1-1: Whether to reduce the number of cells and number of SSB for FR1 Based on our SLS simulation performance for both 1Rx and 2Rx, we support option 1. Issue 1-1-2: If number of cells and number of SSB are reduced, how much to reduce for FR1 Support Option 1. |
| OPPO | Issue 1-1-1: Whether to reduce the number of cells and number of SSB for FR1 Prefer option 1. But considering this is the last meeting, if we could not decide on the reduced number, we can also compromise to keep the same number of cells or SSBs to support same level of mobility as a Rel-15 NR UE. Issue 1-1-2: If number of cells and number of SSB are reduced, how much to reduce for FR1 Same comments as issue 1-1-1. |
| ZTE | Issue 1-1-1: Whether to reduce the number of cells and number of SSB for FR1 Prefer option 1 while can also compromise to keep the current number. Issue 1-1-3 Prefer option 1 while can also compromise to keep the current number. |
| Nokia | Issue 1-1-1: Whether to reduce the number of cells and number of SSB for FR1 We support option 2. Side conditions for intra-frequency and inter-frequency measurement requirements can remain same as for legacy UE, hence number of cells and SSBs for legacy UE can remain. Issue 1-1-3: Whether to reduce the number of cells and number of SSB for FR2 We support the recommended WF. |
| vivo | Issue 1-1-1: Whether to reduce the number of cells and number of SSB for FR1 Option 1. OK with moderator’s recommended WF. |

### Sub-topic 1-2: SSB type for IDLE/INACTIVE mode mobility

Issue 1-2-1: Type of SSB to be used for IDLE/INACTIVE mode mobility

- Proposals

  - Option 1 (Xiaomi, HW, Apple, CATT, vivo, CMCC, QC): Idle/inactive mobility is only based on measurements on the CD-SSB.

- Recommended WF

  - RAN4 to follow the RAN plenary agreement, thus option 1 is agreeable. No more discussions needed.

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Issue 1-2-1: Type of SSB to be used for IDLE/INACTIVE mode mobility Support recommended WF. |
| Apple | Issue 1-2-1: Type of SSB to be used for IDLE/INACTIVE mode mobility Fine with recommended WF. |
| CMCC | Issue 1-2-1: Type of SSB to be used for IDLE/INACTIVE mode mobility Support recommended WF |
| OPPO | Issue 1-2-1: Type of SSB to be used for IDLE/INACTIVE mode mobility Support recommended WF. |
| Xiaomi | Issue 1-2-1: Type of SSB to be used for IDLE/INACTIVE mode mobility Support recommended WF. |
| Nokia | Issue 1-2-1: Type of SSB to be used for IDLE/INACTIVE mode mobility We support the recommended WF. |
| vivo | Issue 1-2-1: Type of SSB to be used for IDLE/INACTIVE mode mobility Support Recommended WF. |

### Sub-topic 1-3: Impact on paging reception requirements

Issue 1-3-1: Impact on paging reception requirements for FD-FFD/TDD UEs for 1 Rx

- Proposals

  - Option 1 (Xiaomi, OPPO, CMCC, E///, MTK): For 1 Rx, there is no impact on the requirement for maximum interruption in paging reception, i.e. Rel-15 requirements shall apply to RedCap.

  - Option 2 (vivo, HW):	For RedCap with 1RX, the interruption time shall not exceed TSI-NR + 3*Ttarget_cell_SMTC_period ms.

- Recommended WF

  - Check if companies can compromise to option 1 unless it is critical to extend?

Issue 1-3-2: Impact on paging reception requirements for FD-FFD/TDD UEs for 2 Rx

- Proposals

  - Option 1 (vivo): For 2 Rx, there is no impact on the requirement for maximum interruption in paging reception, i.e. Rel-15 requirements shall apply to RedCap.

- Recommended WF

  - It has been agreed in R4-2115358 that for 2 Rx capable RedCap UEs, RAN4 to use the Rel-15 NR UE requirements as baseline. Thus no further discussion is needed.

Sub topic 1-3

| Company | Comments |
| --- | --- |
| XXX | Issue 1-3-1: Impact on paging reception requirements for FD-FFD/TDD UEs for 1 Rx |
| Huawei | Issue 1-3-1: Impact on paging reception requirements for FD-FFD/TDD UEs Option 1. For RedCap UE, reducing 2RX to 1RX would impact the T/F synchronization performance. At least 2 SSB are required for pure T/F synchronization for 1RX @ SNR=-6dB. Also one additional SSB shall also be considered as UE may be not able to decode one complete PBCH when it starts to monitor the target cell at first. Therefore totally TSI-NR + 3*Ttarget_cell_SMTC_period ms. |
| Ericsson | Issue 1-3-1: Impact on paging reception requirements for FD-FFD/TDD UEs for 1 Rx We support the recommended WF. |
| CMCC | Issue 1-3-1: Impact on paging reception requirements for FD-FFD/TDD UEs for 1 Rx Support recommended WF |
| MediaTek | Issue 1-3-1: Impact on paging reception requirements for FD-FFD/TDD UEs for 1 Rx Option 1 can be supported. To our understanding, there is no need to extend this requirement because: - The duration for UE to synchronize to the target cell because the cell is already detected and measured. Margin of one more period to address SMTC alignment uncertainty (1 SMTC). |
| OPPO | Issue 1-3-1: Impact on paging reception requirements for FD-FFD/TDD UEs for 1 Rx We support the recommended WF. |
| Xiaomi | Issue 1-3-1: Impact on paging reception requirements for FD-FFD/TDD UEs for 1 Rx We support the recommended WF. |
| vivo | Issue 1-3-1: Impact on paging reception requirements for FD-FFD/TDD UEs for 1 Rx Option 2. Based on our simulation results, for synchronize, one more sample need to be increased for 1Rx UE compared with 2Rx. We can compromise to Option 1. |

### Sub-topic 1-4: HD-FDD operation

Issue 1-4-1: Paging reception for HD-FDD UE

- Proposals

  - Option 1 (CMCC): The RedCap UE operating in HD-FDD mode shall receive paging (if transmitted) in a serving cell provided that at least one SSB is available at the UE in the serving cell during the last 160ms duration.

- Recommended WF

  - This has already been agreed in R4-2202670. Thus no further discissions needed.

Issue 1-4-2: Scheduling restriction for HD-FDD UE in IDLE/INACTIVE mode

- Proposals

  - Option 1 (Xiaomi, OPPO, HW, CMCC, E///): No need to introduce scheduling availability restriction on 5G NR RedCap UEs performing measurements in HD-FDD bands.

- Recommended WF

Option 1 is recommended to be agreed.

Sub topic 1-4

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | Issue 1-4-2: Scheduling restriction for HD-FDD UE in IDLE/INACTIVE mode Support recommended WF. |
| Apple | Issue 1-4-2: Scheduling restriction for HD-FDD UE in IDLE/INACTIVE mode Fine with recommended WF. |
| CMCC | Issue 1-4-2: Scheduling restriction for HD-FDD UE in IDLE/INACTIVE mode Support recommended WF |
| MediaTek | Issue 1-4-2: Scheduling restriction for HD-FDD UE in IDLE/INACTIVE mode Support recommended WF. |
| OPPO | Issue 1-4-2: Scheduling restriction for HD-FDD UE in IDLE/INACTIVE mode Support recommended WF. |
| Xiaomi | Issue 1-4-2: Scheduling restriction for HD-FDD UE in IDLE/INACTIVE mode Support recommended WF. |
| Nokia | Issue 1-4-2: Scheduling restriction for HD-FDD UE in IDLE/INACTIVE mode We support the recommended WF. |
| vivo | Issue 1-4-2: Scheduling restriction for HD-FDD UE in IDLE/INACTIVE mode Support recommended WF. |

### Sub-topic 1-5: RedCap bandgroups

Issue 1-5-1: NR frequency band grouping for FR1 RedCap

- Proposals

  - Option 1 (E///): The NR frequency bands grouping for FR1 RedCap is defined as in Table 1.

Table 1: NR frequency band groups for FR1 for RedCap

| Group |  | NR FDD |  |  | NR TDD |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Band group notation |  | Operating bands | Band group notation |  | Operating bands |
| A | NR_FDD_RC_FR1_A |  | n1, n18, n24, n70, n744, n91, n92, n93, n94 | NR_TDD_ RC_FR1_A |  | n34, n389, n39, n40, n50, n51, n53 |
| B | NR_FDD_RC_FR1_B |  | n65, n66, n743 |  |  |  |
| C | NR_FDD_ RC_FR1_C |  | n30 | NR_TDD_ RC_FR1_C |  | n48, n771, n78 |
| D | NR_FDD_ RC_FR1_D |  | n28 | NR_TDD_ RC_FR1_D |  | n772 |
| E | NR_FDD_ RC_FR1_E |  | n2, n5, n7 | NR_TDD_ RC_FR1_E |  | n41, n90 |
| F | NR_FDD_ RC_FR1_F |  | n266 | NR_TDD_ RC_FR1_F |  | - |
| G | NR_FDD_ RC_FR1_G |  | n3, n8, n12, n13, n14, n20, n71, | NR_TDD_ RC_FR1_G |  | - |
| H | NR_FDD_ RC_FR1_H |  | n25 | NR_TDD_ RC_FR1_H |  | - |
| I | NR_FDD_ RC_FR1_I |  | - | NR_TDD_ RC_FR1_I |  |  |
| J | NR_FDD_ RC_FR1_J |  | - | NR_TDD_ RC_FR1_J |  | - |
| NOTE 1: Except 3.8 GHz to 4.2 GHz. NOTE 2: Only 3.8 GHz to 4.2 GHz. NOTE 3: Except 1475.9 MHz to 1510.9 MHz. NOTE 4: Only when the band is confined in 1475.9 MHz to 1510.9 MHz. NOTE 5: These bands are used only in NR carrier aggregation with other NR bands according to NR CA band combinations specified in TS 38.101-1 [18] and TS 38.101-3 [20]. NOTE 6: The minimum Io condition is reduced by 0.5 dB when the carrier frequency of the assigned NR channel bandwidth is within 865-894 MHz. NOTE 9: When this band is only used for WAN service. |  |  |  |  |  |  |

- Recommended WF

  - Discuss if option 1 can be agreed.

Issue 1-5-2: NR frequency band grouping for FR2 RedCap

- Proposals

  - Option 1 (E///): The NR frequency bands grouping for FR2 RedCap is defined as in Table 2.

- Table 2: NR frequency band groups for FR1 for RedCap

| Group | Band group notation | Operating bands |
| --- | --- | --- |
| A | NR_TDD_FR2_A | n2571, n2581, n2611 |
| B | NR_TDD_FR2_B | n2574, n2584, n2614 |
| C | NR_TDD_FR2_C |  |
| D | NR_TDD_FR2_D |  |
| E | NR_TDD_FR2_E |  |
| F | NR_TDD_FR2_F | n2604 |
| G | NR_TDD_FR2_G | n2601 |
| H | NR_TDD_FR2_H |  |
| I | NR_TDD_FR2_I |  |
| J | NR_TDD_FR2_J |  |
| K | NR_TDD_FR2_K |  |
| L | NR_TDD_FR2_L | n2572, n2582, n2612 |
| M | NR_TDD_FR2_M |  |
| N | NR_TDD_FR2_N |  |
| O | NR_TDD_FR2_O |  |
| P | NR_TDD_FR2_P |  |
| Q | NR_TDD_FR2_Q |  |
| R | NR_TDD_FR2_R |  |
| S | NR_TDD_FR2_S |  |
| T | NR_TDD_FR2_T | n2573, n2583, n2613 |
| U | NR_TDD_FR2_U |  |
| V | NR_TDD_FR2_V |  |
| W | NR_TDD_FR2_W |  |
| X | NR_TDD_FR2_X |  |
| Y | NR_TDD_FR2_Y | n2603 |
| Z | NR_TDD_FR2_Z |  |
| AA | NR_TDD_FR2_AA | n2593 |
| NOTE 1: UE power class 1. NOTE 2: UE power class 2. NOTE 3: UE power class 3. NOTE 4: UE power class 4. |  |  |

- Recommended WF

  - Discuss if option 1 can be agreed.

Sub topic 1-5

| Company | Comments |
| --- | --- |
| XXX | Issue 1-5-1: NR frequency band grouping for FR1 RedCap Issue 1-5-2: NR frequency band grouping for FR2 RedCap |
| Huawei | Issue 1-5-1: NR frequency band grouping for FR1 RedCap Disagree. To our knowledge, there is still no conclusion in RF session. Issue 1-5-2: NR frequency band grouping for FR2 RedCap Disagree. To our knowledge, only n257, n258, n261 are example bands in RF. N259, n260 are not example band for FR2 in RF so far. |
| Ericsson | Issue 1-5-1: NR frequency band grouping for FR1 RedCap We support option 1. The exact bands are agreed in RF group and our view is that there is no need to spend time on discussing the exact bands in RRM group. RRM group can simply reuse the agreements form RF group. Issue 1-5-2: NR frequency band grouping for FR2 RedCap We support option 1. The exact bands are agreed in RF group and our view is that there is no need to spend time on discussing the exact bands in RRM group. RRM group can simply reuse the agreements form RF group. |
| Apple | Issue 1-5-1: NR frequency band grouping for FR1 RedCap Same view as HW, more RF conclusion is needed. Issue 1-5-2: NR frequency band grouping for FR2 RedCap Same view as HW, needs to align with RF conclusions and more progress in RF is needed. |
| CMCC | Issue 1-5-1: NR frequency band grouping for FR1 RedCap Issue 1-5-2: NR frequency band grouping for FR2 RedCap Wait for agreements of Main session |
| Mediatek | Issue 1-5-1: NR frequency band grouping for FR1 RedCap Fine with the current table in option 1. Also, we believe note 5 should be removed. Issue 1-5-2: NR frequency band grouping for FR2 RedCap To our knowledge power class are still in discussion in RF session so we should wait for RF session outcome. |
| Nokia | Issue 1-5-1: NR frequency band grouping for FR1 RedCap This aspect should be confirmed by RF session. We propose to send this to RF session as a formal input during this meeting and request feedback prior to final draft CR submission. Issue 1-5-2: NR frequency band grouping for FR2 RedCap This aspect should be confirmed by RF session. We propose to send this to RF session as a formal input during this meeting and request feedback prior to final draft CR submission. |
| vivo | Issue 1-5-1: NR frequency band grouping for FR1 RedCap Wait for RF session outcome. Issue 1-5-2: NR frequency band grouping for FR2 RedCap Wait for RF session outcome. |

### Sub-topic 1-6: Small data transmission for RedCap

Issue 1-6-1: SDT for RedCap with 2 Rx

- Proposals

  - Option 1 (ZTE, vivo, CMCC, E///, MTK): SDT requirements for RedCap UE with 2 Rx are reused from those defined for legacy NR UEs under Rel-17 SDT WI.

- Recommended WF

  - Option 1 is recommended to be agreed.

  - Moderator: Note that HD-FDD is not considered in Rel-17 SDT WI. The HD-FDD requirements needs to be addressed in Rel-17 RedCap WI.

Issue 1-6-2: SDT for RedCap with 1 Rx – time window

- Proposals

  - Option 1 (vivo, E///, MTK): Time windows defining the valid measurements used for TA validation are reused from Rel-17 SDT discussions but need to be updated to reflect the RedCap 1 Rx measurement times (compared to 2 Rx measurement times).

    - Option 1a (MediaTek): Support reusing the TA validation requirements from SDT rel-17 WI to SDT for RedCap in rel-17 with relaxed accuracy performance for the case of 1Rx RedCap.

  - Option 2 (CMCC): Discuss the time windows for 1Rx RedCap UE after the 2Rx requirements are agreed.

- Recommended WF

  - Since both RedCap and SDT are Rel-17 WI, CRs are being prepared in this meeting. Discuss if the RedCap CRs can be updated during the meeting based on the progress of SDT CRs.

Sub topic 1-6

| Company | Comments |
| --- | --- |
| XXX | Issue 1-6-1: SDT for RedCap with 2 Rx Issue 1-6-2: SDT for RedCap with 1 Rx – time window |
| Huawei | Issue 1-6-1: SDT for RedCap with 2 Rx RAN2 is still discussing SDT and Redcap interworking issue. RAN4 can wait for SDT progress. Issue 1-6-2: SDT for RedCap with 1 Rx – time window Option 2. In SDT time window discussion is on-going (assuming 2RX). Issue 1-6-3: SDT for RedCap with 1 Rx – relaxed accuracy In RAN4 SDT discussion, whether TA validation requirements are related with measurement accuracy has no conclusion. Option 1 is premature. |
| Ericsson | Issue 1-6-1: SDT for RedCap with 2 Rx Regarding HD-FDD, our understanding is that availability of SSBs for HD-FDD to meet the SDT requirements are discussed in SDT WI. Issue 1-6-2: SDT for RedCap with 1 Rx – time window We support option 1. Option 1 means UE shall meet measurement requirements associated with the 1 Rx UE instead of 2 Rx UE. Thus, RAN4 shall reuse the agreement from 2 Rx SDT requirements and update it to refer to the 1 Rx requirements. |
| Apple | Issue 1-6-1: SDT for RedCap with 2 Rx Fine with option 1. For HD-FDD, could wait for the conclusion from SDT WI. Issue 1-6-2: SDT for RedCap with 1 Rx – time window Option 2. |
| CMCC | Issue 1-6-1: SDT for RedCap with 2 Rx Option1 Issue 1-6-2: SDT for RedCap with 1 Rx – time window Compared to agree CRs in parallel and update further, we prefer to postpone the CRs. |
| MediaTek | Issue 1-6-1: SDT for RedCap with 2 Rx Support recommended WF. Issue 1-6-2: SDT for RedCap with 1 Rx – time window We believe that reusing the same time window from 2Rx requirements for 1Rx is sufficient, however, the accuracy for 1Rx requirements has to be relaxed compared to that of 2Rx. Hence, we support option 1a. |
| ZTE | Issue 1-6-1: SDT for RedCap with 2 Rx Support the recommended WF. |
| Nokia | Issue 1-6-1: SDT for RedCap with 2 Rx We support the recommended WF. Issue 1-6-2: SDT for RedCap with 1 Rx – time window We agree to proceed along option 1, i.e. to define time window first for 2Rx UE aligned to current SDT assumption. Then, requirements can be defined for 1Rx UE based on longer measurement times. Regarding CRs, we agree to the recommended WF. |
| vivo | Issue 1-6-1: SDT for RedCap with 2 Rx Support recommended WF. Issue 1-6-2: SDT for RedCap with 1 Rx – time window Option 1. We understand the for SDT for RedCap with 2 Rx, the current SDT requirements for 2Rx can be reused for Redcap UE with 2Rx. For the SDT for RedCap with 1 Rx, we think the definition of the timing window defining valid measurement will not be impacted by the number of antenna hence the existing time window definition can be reused for Redcap UE with 1RX. |

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Company A |  |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204282 (OPPO) | Title: draft CR on measurements requirements for inactivate state Redcap UE |
|  | Ericsson: In 5.1B.2.5, it shall be referred to inter-RAT requirements in 5.1B.2.5. CR form should be 12-2. |
|  | OPPO: Inter-RAT requirements for inactive state can refer to those for idle mode but the clause number may be further checked. OK to be revised. |
|  | Nokia: We wonder why content in clause 5.1B.2.5 points to idle mode clause for legacy UE rather than to idle mode clause 4.2B for RedCap UE. |
| R4-2205622 (Ericsson) | Title: Draft CR on RRC_IDLE mode requirements for RedCap for TS 36.133 |
|  | Nokia: Change 1: “The parameter NNR_carrier” should read “The parameter NNR_carrier_RedCap”. Applicability of Table 4.2.2.5.8-3 for 1Rx RedCap UE in FR2 is missing. Change 2: It should read: 6 NR inter-RAT carriers, 6 FDD E-UTRA inter-frequency carriers, 6 TDD E-UTRA inter-frequency carriers. |
| R4-2205625 (Ericsson) | Title: Draft CR on RRC_IDLE mode requirements for RedCap for TS 38.133 |
|  | Nokia: Change 1 – terminology for 1 Rx RedCap UE and 2 Rx RedCap UE to be agreed and aligned in all draft CRs. RAN1 uses the terms RedCap UE with 1 Rx branch, RedCap UE with 2 Rx branches in their specs. Change 2: Applicability of Table 4.2B.2.2-2 for 2Rx RedCap UE in FR1 is missing. Change 3: Table referencing errors in description for Tables 4.2B.2.3-1 and 4.2B.2.3-3. Change 4: Applicability of Table 4.2B.2.4-2 for RedCap UE in FR1 as well as applicability of Table 4.2B.2.4-3 for RedCap UE in FR2 is missing. Change 6: editorial comment: “when the when the”. In 4.2B.2.2, the timer T is still under discussion, so that should be indicated. In Table 4.2B.2.2-2, PTW length for eDRX cycle equal to 0.32 should also be in [ ]. In Table 4.2B.2.3-2, the formatting of the cell in which the note is included should be fixed. Our impression is that the values should also be in []’s, since they are still being discussed in thread 229. |
| R4-2205626 (Ericsson) | Title: Draft CR on RRC_INACTIVE mode requirements for RedCap for TS 38.133 |
|  | Nokia: It is unclear, why the draft CR only deals with configured RRM measurement relaxation. The case RRM measurement relaxations are not configured (i.e. subclauses 4.2B.2.3 to 4.2B.2.5 for idle mode) should also be referred for this case. |
| R4-2205623 (Ericsson) | Title: Draft CR on RRC_INACTIVE mode requirements for RedCap for TS 36.133 |
|  | Nokia: Wrong reference to idle mode clauses. The clause on UE measurement capability should point to clause 4.2.2.14 and the clause on inter-RAT NR measurements should point to clause 4.2.2.5.8. |
| R4-2204798 (vivo) | Title: Draft CR for maximum interruption in paging reception for Redcap |
|  | Ericsson: Needs to be revised to capture the agreements for HD-FDD UE when paging reception overlaps with SDT transmission, see R4-2202670. |
|  | Nokia: The draft CR is agreeable. |
| R4-2204800 (vivo) | Title: Draft CR for Definitions, symbols and abbreviations for Redcap |
|  | Ericsson: - In general, clause 3.6.11.6 refers to 3.6.9 which has scheduling availability for UE supporting more than one cell (e.g. CA, DC etc). But this is not the scenario for RedCap which has only one cell. Therefore not all these references are valid for RedCap. We should exclude those text that has scheduling restriction for PSCell, Scell, etc. - 3.6.11.2 refers to 3.6.3 which is related to Scell. But Scell does not apply to RedCap. Thus do we really need this section? perhaps not. - 3.6.11.5 refers to 3.6.9. But we will have separate section for RA requirements. For examples, the RedCap RA requirements are not exactly identical to existing RA requirements. We have HD-FDD and we have also few HD-FDD conditions. If we have separate section for RA that includes 2-step, do we really need this section? - 3.6.11.6 refers to 3.6.9 which is contains scheduling restriction (8.1.7.3) that applies due to radio link monitoring on an FR2 serving PCell and/or PSCell. However, for RedCap there is only a single cell therefore this section with scheduling restriction should not be applicable. - 3.6.11.7 refers to 3.6.10 which has following text: "The network configures mixed numerology on two CCs if the UE does not have the capability of supporting simultaneous reception with different numerologies between the two CCs in DL." This is not relevant for RedCap as there is no multiple CC. |
|  | Nokia: Besides the definition of RedCap UE, in our view definition terms for RedCap UE with 1 Rx branch and UE with 2 Rx branches should be added, e.g. 1Rx RedCap and 2Rx RedCap, that are used throughout 38.133. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic #1-1 | Issue 1-1-1: Whether to reduce the number of cells and number of SSB for FR1 - Option 1 (MTK): Yes. - Option 1a: The starting time of the related investigation needs to be determined. - Option 2 (HW, CMCC, E///, Apple, OPPO, ZTE, Nokia, vivo): No Tentative agreements: [The number of cells and number of SSB for FR1 is reused for release 15 NR requirements.] |
| Sub-topic #1-2 | Issue 1-2-1: Type of SSB to be used for IDLE/INACTIVE mode mobility Tentative agreements: RAN4 to follow the RAN plenary agreement, i.e._: - Idle/inactive mobility is only based on measurements on the CD-SSB. |
| Sub-topic #1-3 | Issue 1-3-1: Impact on paging reception requirements for FD-FFD/TDD UEs for 1 Rx - Option 1 (Xiaomi, OPPO, CMCC, E///, MTK, vivo): For 1 Rx, there is no impact on the requirement for maximum interruption in paging reception, i.e. Rel-15 requirements shall apply to RedCap. - Option 2 (HW): For RedCap with 1RX, the interruption time shall not exceed TSI-NR + 3*Ttarget_cell_SMTC_period ms. Recommendation for the 2nd round: Check if companies can compromise to option 1 which has the most support. |
| Sub-topic #1-4 | Issue 1-4-2: Scheduling restriction for HD-FDD UE in IDLE/INACTIVE mode Tentative agreements: In IDLE/INACTIVE states, no need to introduce scheduling availability restriction on 5G NR RedCap UEs performing measurements in HD-FDD bands. |
| Sub-topic #1-5 | Issue 1-5-1: NR frequency band grouping for FR1 RedCap Recommendations for 2nd round: Companies are encouraged to check the agreements from RF group. Based on that continue the discussions. Issue 1-5-2: NR frequency band grouping for FR2 RedCap Recommendations for 2nd round: Companies are encouraged to check the agreements from RF group. Based on that continue the discussions. |
| Sub-topic #1-5 | Issue 1-6-1: SDT for RedCap with 2 Rx - Option 1 (ZTE, vivo, CMCC, E///, MTK): SDT requirements for RedCap UE with 2 Rx are reused from those defined for legacy NR UEs under Rel-17 SDT WI. - Option 2 (HW): Wait for RAN2 progress Tentative agreement from 1st round:: For FDD and TDD, SDT requirements for RedCap UE with 2 Rx are reused from those defined for legacy NR UEs under Rel-17 SDT WI. Recommendations for 2nd round: Discuss the SDT requirements for HD-FDD based on the progress of SDT WI. Issue 1-6-2: SDT for RedCap with 1 Rx – time window - Option 1 (vivo, E///, MTK): Time windows defining the valid measurements used for TA validation are reused from Rel-17 SDT discussions but need to be updated to reflect the RedCap 1 Rx measurement times (compared to 2 Rx measurement times). - Option 1a (MediaTek): Support reusing the TA validation requirements from SDT rel-17 WI to SDT for RedCap in rel-17 with relaxed accuracy performance for the case of 1Rx RedCap. - Option 2 (CMCC, HW, Apple): Discuss the time windows for 1Rx RedCap UE after the 2Rx requirements are agreed. Recommendations for 2nd round: Given that this is the last meeting to complete the the core part, try to update the CR for 1 Rx UE based on progress and agreements of the 2 RX SDT requirements from SDT WI. |

## Discussion on 2nd round (if applicable)

Companies are encouraged to provide their view on the issues recommended for 2nd round discussions in 1.4.1.

Topic #1

| Company | Comments |
| --- | --- |
| Company X | Issue x-y |

# Topic #2: Mobility requirements

Contributions from AI 10.20.3.1.2 are discussed here.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203584 | ZTE Corporation | - Any agreement can be captured in the spec in a way that describes the applicability of the requirements, instead of mandating specific UE/NW behavior. The UE shall meet HO requirements provided the following is met: - SSB is available at the UE once every SMTC period during Tsearch. - One SSB is available during T∆ - One SSB is available during Tiu - Early identification doesn’t impact RA requirements. |
| R4-2203787 | Apple | Proposal 1: RAN4 to relax the Tsearch for RedCap HO delay requirement as followings when SINR=-2dB and 1Rx is used - For HO to FR1, Tsearch = 2*Trs for intra-frequency HO and Tsearch = 5* Trs for inter-frequency HO - For HO to FR2, Tsearch = 8*2*Trs for intra-frequency HO and Tsearch = 8*5* Trs for inter-frequency HO Proposal 2: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in HO requirement. Proposal 3: the lower boundary in Max function for reestablishment delay requirement shall not be changed for RedCap UE with 1Rx. Proposal 4: RAN4 to relax the Tidentify for RedCap reestablishment delay requirement with 1Rx by: - 2 more samples for FR1 for intra-frequency and inter-frequency cases Proposal 5: RAN4 to relax the Tidentify-NR for RedCap RRC redirection delay requirement with 1Rx by: - 2 more samples for FR1 |
| R4-2204322 | vivo | Proposal 1: For Tsearch (in HO) for 1Rx for FR1, support option 1. For FR2 Tsearch (in HO) with 1Rx, we propose for an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, Tsearch = 8*2*Trs ms and for an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch =8* 5* Trs ms, i.e., option 1. Proposal 2: For the issue of impact on Tsearch (in reestablishment) for 1Rx in FR1, one sample need to be increased when SINR=-8dB and 1Rx is used for unknown NR cell, i.e., option 1. Proposal 3: For the lower bound in Max function in reestablishment delay, support that the lower boundary in Max function for reestablishment delay requirement shall not be changed for RedCap UE with 1Rx, i.e., option 1. Proposal 4: One sample need to be increased for RedCap RRC redirection delay requirement with 1Rx for FR1. Proposal 5: Prefer that there is no need to define separate threshold for 1 Rx RedCap UE, i.e., option 2. |
| R4-2204539 | Nokia, Nokia Shanghai Bell | Proposal 1: Tsearch for RedCap UE with 1 RX extended by 1 sample in FR1. For intra-frequency handover, Tsearch=2*Trs and for inter-frequency handover, Tsearch=4*Trs Proposal 2: The lower boundary in Max function for reestablishment delay requirement shall not be changed for RedCap UE with 1Rx. Proposal 3: For RRC re-establishment for RedCap UEs with 1 Rx, relax Tidentify by 1 sample for the unknown NR cell case in FR1. Proposal 4: For RRC connection release with redirection, for RedCap UEs with 1 RX extend Tidentify_NR by 1 sample in FR1. |
| R4-2204903 | Huawei, Hisilicon | - Proposal 1: 3 samples are required for intra-frequency Tsearch (in HO) and 5 samples are required for inter-frequency for Tsearch (in HO) for Redacap UE with 1Rx. Proposal 2: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in HO requirement. Proposal 3: For RedCap UE with 1Rx, when serving cell SSB SINR≥ -8dB the re-establishment requirements are defined: -Tidentify_intra_NR for unknown case is 11 x TSMTC for FR1. Proposal 4: For RedCap UE, the lower boundary in Max function for reestablishment delay requirement shall not be changed. Proposal 5: For RedCap UE with 1Rx, Tidentify-NR for RRC connection release with redirection is 12 x TSMTC for FR1. Proposal 6: For RedCap UE, the lower boundary in Max function for RRC connection release with redirection shall not be changed. Proposal 7: Early identification doesn’t impact RA requirements and can directly refer to RAN2 spec. |
| R4-2204904 | Huawei, Hisilicon | Draft CR: Draft CR on mobility requirements for Redcap UE |
| R4-2204905 | Huawei, Hisilicon | Draft CR on E-UTRAN - NR Handover for Redcap UE |
| R4-2204993 | CMCC | Proposal 1: For HD-FDD, UE shall meet HO requirements provided the following is met: - SSB is available at the UE once every SMTC period during Tsearch. - One SSB is available during T∆ - One SSB is available during Tiu Proposal 2: Following applies to CBRA: - The RedCap UE operating in HD-FDD mode is not expected to perform PRACH transmission on a PRACH resource of a cell if UE has not received at least one SSB associated with that PRACH resource during the last Tp period in the cell, where - Option 1: Tp=160 ms. - The RedCap UE operating in HD-FDD mode shall meet the PRACH requirements when performing PRACH transmission on a PRACH resource of a cell provided that the UE has received at least one SSB associated with that PRACH resource during the last Tp period before the PRACH transmission, where - Option 1: Tp=160 ms. Proposal 3: Introduce separate RSRP/RSRQ thresholds for RedCap UE with 1Rx. |
| R4-2205409 | ZTE Corporation | Proposal 1: Any agreement can be captured in the spec in a way that describes the applicability of the requirements, instead of mandating specific UE/NW behavior. Proposal 2: The UE shall meet HO requirements provided the following is met:  SSB is available at the UE once every SMTC period during Tsearch.  One SSB is available during T∆  One SSB is available during Tiu Proposal 3: Early identification doesn’t impact RA requirements. |
| R4-2206038 | Ericsson | Draft CR: RRC connection release with redireciton for redcap in TS 36.133 |
| R4-2206079 | MediaTek inc. | Proposal 1: Support Option 1: intra-frequency HO: Tsearch = 2*Trs and inter-frequency HO: Tsearch = 5* Trs. Proposal 2: Support Option 2: The UE shall meet HO requirements provided the following is met: (i) SSB is available at the UE once every SMTC period during Tsearch, (ii) One SSB is available during T∆, and (iii) One SSB is available during Tiu. Proposal 3: Support Option 2: Support extending the lower bound for known cell to be 240 ms, while for the unknown cell to be 860 ms. Proposal 4: Support Option 1: Support relaxing the Tidentify for re-establishment with an additional sample. Proposal 5: Support Option 2: Support relaxing the Tidentify-NR for RRC connection release with redirection with an additional sample. Proposal 6: Support Option 1: Early identification doesn’t impact RA requirements. Proposal 7: Support Option 2: No need to define separate threshold for 1 Rx RedCap UE. |
| R4-2206111 | Qualcomm Incorporated | Proposal 1: For Handover delay requirements, specify Tsearch as: - If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 3*Trs ms. - If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 5* Trs ms Proposal 2: For RRC re-establishment delay requirements, increase the Tidentify_intra_NR and Tidentify_inter_NR for FR1 by two more samples. Proposal 3: Increase the Tidentify_NR in Tconnection_release_redirect_NR for FR1 by two more samples. Proposal 4: RAN4 to study the impact of configuring ROs in the redcap specific BWP which doesn’t contain any SSB on mobility requirements. |
| R4-2205629 | Ericsson | Proposal 1: TSearch in HO requirements for 1 Rx in FR1 is defined as: - For Intra-frequency HO: Tsearch = 2*Trs - For inter-frequency HO: Tsearch = 5* Trs Proposal 2: The UE shall meet HO requirements provided the following is met: - SSB is available at the UE once every SMTC period during Tsearch. - One SSB is available during T∆ - One SSB is available during Tiu Proposal 3: The UE is not expected to perform HO to the target cell when: - It has done measurement in target cell on SSB which is different than the SSB associated with BWP where RACH is configured and if the separation between initial BWP and Redcap BWPs is larger than 20 MHz (for FR1) or 100 MHz (for FR2). Proposal 4: When both CD-SSB and NCD-SSB are transmitted on initial BWP and RedCap specific BWP respectively, handover directly to a RedCap specific BWP where only NCD-SSB is transmitted is supported when: - the difference of center frequency between NCD-SSB of target BWP and CD-SSB of initial BWP of target cell is no larger than 20MHz in FR1 and 100MHz in FR2, - the difference of reception power between NCD-SSB and CD-SSB is less than 3dB, - the periodicity of NCD-SSB and CD-SSB is the same Proposal 5: Legacy NR handover requirements can be reused for handover to a target RedCap specific BWP where NCD-SSB is transmitted when the periodicity of NCD-SSB and CD-SSB is same. Proposal 6: When there is no SSB transmission in a target RedCap specific BWP, handover to that RedCap specific BWP is supported when: - target RedCap specific BWP and initial BWP of target cell are not separated by larger than 20MHz in FR1 and 100MHz in FR2. Proposal 7: The lower boundary in max function for reestablishment delay requirement shall not be changed for RedCap UE with 1Rx. Proposal 8: Tsearch for 1 Rx RedCap UE in FR1 is extended by 1 sample for the target unknown cell. Proposal 9: The time period Tp during which the UE has received at least one SSB associated with the PRACH resource to be able to meet the RA requirements is 160 ms. Proposal 10: Introduce separate RSRP thresholds for RedCap UE with 1Rx and 2Rx in procedures that depend on RSRP based thresholds such as RA in IDLE/INACTIVE. RAN4 discuss further two options: - Option 1: Introduce additional RSRP thresholds for RedCap 1Rx UE - Option 2: Consider the offset, ΔSS-RSRP, for SS-RSRP based selection for RedCap 1Rx UE, e.g., ‘With SS-RSRP above msgA-RSRP-ThresholdSSB + ΔSS-RSRP for single Rx, and with SS-RSRP above msgA-RSRP-ThresholdSSB for RedCap UE other than single Rx’. RAN4 discuss whether ΔSS-RSRP can be signaled from the network or defined in TS38.133. Proposal 11: RAN4 capture the RedCap UE early indication procedure in TS38.133 6.2.2 for 4-step RA type and 6.2.3 for 2-step RA type according to TS38.321/331. Proposal 13: Tsearch from RRC reestablishment requirements for 1Rx RedCap UE in FR1 is reused for Tsearch for 1 Rx UE RRC connection release with redirection requirements in FR1. |

## Open issues summary

### Sub-topic 2-1 Handover

Issue 2-1-1: Impact on Tsearch (in HO) for 1Rx for FR1

Proposals: RAN4 to relax the Tsearch for RedCap HO delay requirement as followings when SINR=-2dB and 1Rx is used

  - Option 1 (Apple, vivo, MTK, E///):

      - For HO to FR1, Tsearch = 2*Trs for intra-frequency HO and Tsearch = 5* Trs for inter-frequency HO

  - Option 2 (HW, QC):

      - For HO to FR1, Tsearch = 3*Trs for intra-frequency HO and Tsearch = 5* Trs for inter-frequency HO

  - Option 3 (Nokia):

      - For HO to FR1, Tsearch = 2*Trs for intra-frequency HO and Tsearch = 4* Trs for inter-frequency HO

- Recommended WF

  - Discuss if following compromise option can be agreed:

    - For HO to FR1,

      - Tsearch = 2*Trs for intra-frequency HO

      - Tsearch = 5* Trs for inter-frequency HO

Issue 2-1-2: Impact on Tsearch (in HO) for FR2

- Proposals

  - Option 1 (Apple, vivo): RAN4 to relax the Tsearch for RedCap HO delay requirement as followings when SINR=-2dB and 1Rx is used

      - For HO to FR2, Tsearch = 8*2*Trs for intra-frequency HO and Tsearch = 8*5* Trs for inter-frequency HO

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no need to discuss requirements for 1 Rx UE in FR2.

Issue 2-1-3: Requirements for HD-FDD in HO

- Proposals

  - Option 1 (Apple, HW):

    - RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in HO requirement.

  - Option 2 (ZTE, CMCC, MTK, E///): The UE shall meet HO requirements provided the following is met:

    - SSB is available at the UE once every SMTC period during Tsearch.

    - One SSB is available during T∆

    - One SSB is available during Tiu

    - Option 2a (ZTE): Any agreement can be captured in the spec in a way that describes the applicability of the requirements, instead of mandating specific UE/NW behavior.

- Recommended WF

  - Check if companies can compromise to option 2.

Issue 2-1-4: HO to target cell when SSB used for measurement and SSB associated with RACH BWP is different

- Proposals

  - Option 1 (E///): The UE is not expected to perform HO to the target cell when:

    - It has done measurement in target cell on SSB which is different than the SSB associated with BWP where RACH is configured and if the separation between initial BWP and Redcap BWPs is larger than 20 MHz for FR1 and 100 MHz for FR2

- Recommended WF

  - Discuss the option.

Issue 2-1-5: HO to a RedCap specific BWP with NCD-SSB (no CD-SSB)

- Proposals

  - Option 1 (E///): Handover directly to a RedCap specific BWP where only NCD-SSB is transmitted is supported when:

    - the difference of center frequency between NCD-SSB of target BWP and CD-SSB of initial BWP of target cell is not larger than 20MHz in FR1 and 100MHz in FR2,

    - the difference of reception power between NCD-SSB and CD-SSB is less than 3dB,

    - the periodicity of NCD-SSB and CD-SSB is the same

- Recommended WF

  - Discuss the option.

Issue 2-1-6: Requirements for handover to RedCap specific BWP with NCD-SSB (no CD-SSB)

- Proposals

  - Option 1 (E///): Legacy NR handover requirements can be reused for handover to a target RedCap specific BWP where NCD-SSB is transmitted when the periodicity of NCD-SSB and CD-SSB is same.

- Recommended WF

  - Discuss the option.

Issue 2-1-7: Handover to RedCap specific BWP with no SSB

- Proposals

  - Option 1 (E///): When there is no SSB transmission in a target RedCap specific BWP, handover to that RedCap specific BWP is supported when:

    - target RedCap specific BWP and initial BWP of target cell are not separated by larger than 20MHz in FR1 and 100MHz in FR2.

- Recommended WF

  - Discuss the option.

Sub topic 2-1

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1-1: Impact on Tsearch (in HO) for 1Rx for FR1 Issue 2-1-2: Impact on Tsearch (in HO) for FR2 Issue 2-1-3: Requirements for HD-FDD in HO Issue 2-1-4: HO to target cell when SSB used for measurement and SSB associated with RACH BWP is different Issue 2-1-5: HO to a RedCap specific BWP with NCD-SSB (no CD-SSB) Issue 2-1-6: Requirements for handover to RedCap specific BWP with NCD-SSB (no CD-SSB) Issue 2-1-7: Handover to RedCap specific BWP with no SSB |
| Huawei | Issue 2-1-1: Impact on Tsearch (in HO) for 1Rx for FR1 Option 2. At least 3 samples are required for FR1 handover for intra-frequency HO based on our simulation results. 5 samples for inter-frequency HO is a compromise. Issue 2-1-2: Impact on Tsearch (in HO) for FR2 Support to reuse 8 scaling factor in FR2. The concrete value depends on issue 2-1-1. Issue 2-1-3: Requirements for HD-FDD in HO Option 1. In RAN1, there is agreement that the SSB is prioritized over dynamically scheduled UL transmission： - Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS. Agreement: - For Case 5 of dynamically scheduled UL transmission vs. SSB, support Option 2 at least for dynamically scheduled UL transmission other than Msg3 (re)transmission and PUCCH for Msg4 - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission. The similar principle can be applied for neighbour cell handover: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in HO requirement. If option 1 can not be approved, option 2 is the second priority. |
| Ericsson | Issue 2-1-1: Impact on Tsearch (in HO) for 1Rx for FR1 Compromise proposal from moderator is acceptable. Issue 2-1-3: Requirements for HD-FDD in HO We support the recommend WF from moderator, i.e. to compromise to option 2. Issue 2-1-4: HO to target cell when SSB used for measurement and SSB associated with RACH BWP is different We support option 1. If BWP where the UE has performed the measurement triggering the HO and BWP where the UE is about to perform the RACH are different and placed far apart in the frequency domain, then the radio conditions may be different compared to each other. This means the DL measurement which was used for triggering the HO may not reflect the radio conditions of the BWP where RACH is performed i.e. when the two BWPs are far part in frequency. RedCap UE can be served by BS using very large BW e.g. 100 MHz in FR1. Therefore in practice the frequency separation between the BWPs can be very large. This may require the RedCap UE to acquire again AGC (e.g. to receive Msg2), time tracking etc., before transmitting the RACH at HO. Therefore the maximum frequency separation to meet the HO requirements can be limited to certain value i.e. same as the UE BW of 20 MHz in FR1 and 100 MHz in FR2. Otherwise, the UE is not required to meet the HO requirements. Also note that a similar condition also exists in LTE for NB-IoT where the anchor and non-anchor carriers are within 20 MHz, see section 6.9.1 in TS 36.133. Issue 2-1-5: HO to a RedCap specific BWP with NCD-SSB (no CD-SSB) For the reasons explained in issue 2-1-4, we support option 1. In addition, if periodicity of NCD-SSB and CD-SSB is assumed to be the same, then no specific new requirements need to be developed. Issue 2-1-6: Requirements for handover to RedCap specific BWP with NCD-SSB (no CD-SSB) We support option 1. For simplicity, the handover requirements can be defined assuming that the periodicity of NCD-SSB and CD-SSB are same which makes current requirements reusable. Otherwise, if NCD-SSB transmission periodicity is longer then an extension to the HO delay is needed since UE may need additional time to perform the AGC etc. Issue 2-1-7: Handover to RedCap specific BWP with no SSB For the reasons explained in issue 2-1-4, we support option 1. |
| Apple | Issue 2-1-1: Impact on Tsearch (in HO) for 1Rx for FR1 Fine with recommended WF. Issue 2-1-3: Requirements for HD-FDD in HO Option 1. The HO delay is an interruption to the source cell (no UL traffic is expected after HO triggered) and we don’t understand why not prioritize DL measurement during interruption, and we also think the same principle for scheduling restriction in TDD band on FR1 could be reused for HD-FDD case, i.e., RRM DL measurement is prioritized over the UL transmission of HD-FDD. Issue 2-1-4: HO to target cell when SSB used for measurement and SSB associated with RACH BWP is different Issue 2-1-5: HO to a RedCap specific BWP with NCD-SSB (no CD-SSB) We think more discussion is needed for option 1. We are wondering if the target cell synchronization and measurement could be performed on NCD-SSB, then HO could be rely on NCD-SSB without option 1. Issue 2-1-6: Requirements for handover to RedCap specific BWP with NCD-SSB (no CD-SSB) Depends on whether NCD-SSB could be used for cell synchronization/search or not during HO. Issue 2-1-7: Handover to RedCap specific BWP with no SSB We think no requirement shall be applied if no any SSB exists on the specific BWP to HO to. We think it’s a more likely case that target specific BWP has NCD-SSB. |
| CMCC | Issue 2-1-3: Requirements for HD-FDD in HO Support option2 Issue 2-1-4: HO to target cell when SSB used for measurement and SSB associated with RACH BWP is different Issue 2-1-5: HO to a RedCap specific BWP with NCD-SSB (no CD-SSB) Issue 2-1-6: Requirements for handover to RedCap specific BWP with NCD-SSB (no CD-SSB) Need to understand more about the scenario. We don’t think there is any problem if HO is performed based on NCD-SSB. Why the frequency separation and periodicity of NCD-SSB and CD-SSB matters? |
| MediaTek | Issue 2-1-1: Impact on Tsearch (in HO) for 1Rx for FR1 Support recommended WF. Issue 2-1-3: Requirements for HD-FDD in HO Support recommended WF (Option 2). Issue 2-1-4: HO to target cell when SSB used for measurement and SSB associated with RACH BWP is different We disagree with Option 1 and we believe the above scenario is valid. Issue 2-1-5: HO to a RedCap specific BWP with NCD-SSB (no CD-SSB) RAN2 is currently discussing on whether the handover with NCD-SSB is a valid scenario or not. With first bullet: it is not clear to us where we are going to perform the RACH. We agree on bullets 2 and 3. Issue 2-1-6: Requirements for handover to RedCap specific BWP with NCD-SSB (no CD-SSB) RAN2 is currently discussing on whether the handover with NCD-SSB is a valid scenario or not. Issue 2-1-7: Handover to RedCap specific BWP with no SSB We suggest to add another Option 2: When there is no SSB transmission in a target RedCap specific BWP, handover to that RedCap then the UE shall switch back to CD-SSB. |
| ZTE | Issue 2-1-3: Requirements for HD-FDD in HO Support Option 2. Don’t think it is necessary to mandate any UE behavior, especially those which are hard to test. How can Option 1 be tested? |
| Nokia | Issue 2-1-1: Impact on Tsearch (in HO) for 1Rx for FR1 Fine with the compromise option. Issue 2-1-3: Requirements for HD-FDD in HO We support Option 1. Issue 2-1-4: HO to target cell when SSB used for measurement and SSB associated with RACH BWP is different Support Option 1. For this scenario, it can also be beneficial to keep the existing requirements for HO delay. Issue 2-1-5: HO to a RedCap specific BWP with NCD-SSB (no CD-SSB) We need more discussions on this scenario. In our view, this is a RAN2 topic, since current RRC signalling does not allow using NCD-SSB for mobility. We copy below RAN2 understanding of the current situation (RAN2 #116e). The issue is still being discussed in RAN2. In connected mode, current RRC signalling allows configuring SSB-based RRM measurements on any (CD- or NCD-) SSB, but it does not allow using an NCD-SSB for RLM, BFD, link recovery, RO selection, mobility (mobility here refers to the frequency indicated in FreqDLInfo in HO command), in TCI-states or for any other functionality (other than RRM measurements). Issue 2-1-6: Requirements for handover to RedCap specific BWP with NCD-SSB (no CD-SSB) Same as above. Issue 2-1-7: Handover to RedCap specific BWP with no SSB We need more discussion in this scenario. |
| vivo | Issue 2-1-1: Impact on Tsearch (in HO) for 1Rx for FR1 Support the Recommended WF. Issue 2-1-3: Requirements for HD-FDD in HO Recommended WF is fine. |
| Qualcomm | Issue 2-1-1: Impact on Tsearch (in HO) for 1Rx for FR1 We support Option 2. Two additional samples are needed based on our simulations. Issue 2-1-3: Requirements for HD-FDD in HO Option 2 is acceptable to us. Issue 2-1-4: HO to target cell when SSB used for measurement and SSB associated with RACH BWP is different We cannot support option 1. There should be no such restriction. Issue 2-1-5: HO to a RedCap specific BWP with NCD-SSB (no CD-SSB) Cannot agree to 1st bullet. We are fine with 2nd and 3rd bullet. Issue 2-1-6: Requirements for handover to RedCap specific BWP with NCD-SSB (no CD-SSB) This scenario needs more discussion. The target cell search has to be performed on CD-SSB only. The UE may use the NCD-SSB in the redcap specific initial BWP for time/frequency tracking and will need to perform RA in this BWP. UE needs to perform a BWP switch from non-Redcap specific initial BWP (which contains the CD-SSB) to Redcap specific initial BWP (for RA). So, we propose to update the handover delay as below: Proposal 2 (Qualcomm): When the Redcap specific initial DL BWP is configured for RA only, define the interruption time as - Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin + TBWP-switching-delay ms - Where, TBWP-switching-delay is the BWP switching delay Issue 2-1-7: Handover to RedCap specific BWP with no SSB The above scenario holds for this issue as well. However, in this case the BWP switch shall be initiated just before the UE transmits the RACH Proposal 2 (Qualcomm): When the Redcap specific initial DL BWP is configured for RA only, and there is no SSB available in the redcap specific initial BWP, define the interruption time as - Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin + TBWP-switching-delay ms - Where, TBWP-switching-delay is the BWP switching delay |
| MediaTek2 | Further comments on issues 2-14, 2-1-5, 2-16, and 2-1-7: Based on the recent agreement from the current RAN2 GTW meeting: ‘NCD-SSB should not be indicated in the handover command, i.e., network sets ServingCellConfigCommon => downlinkConfigCommon => frequencyInfoDL => absoluteFrequencySSB to the frequency of the CD-SSB (not the NCD-SSB)’. We suggest that there is no need to further discuss these issues (2-1-4, 2-1-5, 2-1-6, and 2-1-7) and we don’t think there should be any restrictions. |

### Sub-topic 2-2 RRC re-establishment

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-2-1: Lower bound in Max function in reestablishment delay

- Proposals

  - Option 1 (Apple, vivo, Nokia, HW, E///): The lower boundary in Max function for reestablishment delay requirement shall not be changed for RedCap UE with 1Rx.

  - Option 2 (MTK): Support extending the lower bound

    - for known cell to be 240 ms,

    - for the unknown cell to be 860 ms.

- Recommended WF

  - Check if supporting company of option 2 can compromise to option 1.

Issue 2-2-2: Impact on Tidentify (in reestablishment) for 1Rx in FR1

- Proposals:

RAN4 to relax the Tidentify- for RedCap reestablishment delay requirement with 1Rx by:

  - Option 1 (Apple, QC): 2 more samples for intra-frequency and inter-frequency

  - Option 2 (HW, Nokia, MTK, E///, vivo):

    - 1 more sample for unknown target cell (total of 11 x TSMTC) for intra-frequency and inter-frequency

- Recommended WF

  - Check if companies can compromise to option 2.

Sub topic 2-2

| Company | Comments |
| --- | --- |
| XXX | Issue 2-2-1: Lower bound in Max function in reestablishment delay Issue 2-2-2: Impact on Tidentify (in reestablishment) for 1Rx in FR1 |
| Huawei | Issue 2-2-1: Lower bound in Max function in reestablishment delay Option 1. Issue 2-2-2: Impact on Tidentify (in reestablishment) for 1Rx in FR1 Option 2 based on our simulation results. |
| Ericsson | Issue 2-2-1: Lower bound in Max function in reestablishment delay We support the recommended WF from moderator. Issue 2-2-2: Impact on Tidentify (in reestablishment) for 1Rx in FR1 We support the recommended WF from moderator. |
| Apple | Issue 2-2-1: Lower bound in Max function in reestablishment delay Option 1. Issue 2-2-2: Impact on Tidentify (in reestablishment) for 1Rx in FR1 Option 1 based on our simulation, but if all the companies support option 2 we can also compromise. |
| MediaTek | Issue 2-2-1: Lower bound in Max function in reestablishment delay We can compromise to support recommended WF. Issue 2-2-2: Impact on Tidentify (in reestablishment) for 1Rx in FR1 Support WF. (Option 2) |
| Nokia | Issue 2-2-1: Lower bound in Max function in reestablishment delay We support the WF. Issue 2-2-2: Impact on Tidentify (in reestablishment) for 1Rx in FR1 We support the WF. |
| vivo | Issue 2-2-1: Lower bound in Max function in reestablishment delay Support the Recommended WF. Issue 2-2-2: Impact on Tidentify (in reestablishment) for 1Rx in FR1 Support the Recommended WF. |
| Qualcomm | Issue 2-2-1: Lower bound in Max function in reestablishment delay Support proposal 2. Exact extension depends on the number of samples We think the lower bound has to be extended depending on the number of samples we are increasing. Issue 2-2-2: Impact on Tidentify (in reestablishment) for 1Rx in FR1 Option 1. We think more samples are needed for inter-frequency case, where cell search also needs to be performed and two additional samples are needed. We can accept the following compromise: Option 3 (Compromise): 2 more samples for inter-frequency. No extension for intra-frequency. Also, similar to our comment in Handover, we think RRC re-establishment delay should also be extended by BWP switching delay when the RA has to be performed in Redcap specific initial BWP. We request the moderator to create a separate issue for discussion in the second round. |

### Sub-topic 2-3 RRC Connection release with redirection

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-3-1: Lower bound in Max function in RRC connection release with redirection delay

- Proposals

  - Option 1 (HW) The lower boundary in Max function for RRC redirection delay requirement shall not be changed for RedCap UE with 1Rx.

- Recommended WF

  - There was already an agreement at previous meeting to follow the corresponding agreement from RRC reestablishment, see R4-2202670. Thus no more discussion needed.

Issue 2-3-2: Impact on Tidentify-NR (in RRC connection release with redirection) for 1Rx in FR1

- Proposals

RAN4 to relax the Tidentify-NR  for RedCap RRC redirection delay requirement with 1Rx by:

  - Option 1 (Apple, QC): 2 more samples

  - Option 2 (vivo, Nokia, HW, MTK, E///): 1 more sample

- Recommended WF

  - Check if corresponding agreement from RRC reestablishment can be reused. If so, no need to discuss separately.

Sub topic 2-3

| Company | Comments |
| --- | --- |
| XXX | Issue 2-3-2: Impact on Tidentify-NR (in RRC connection release with redirection) for 1Rx in FR1 |
| Huawei | Issue 2-3-1: Lower bound in Max function in RRC connection release with redirection delay Fine with the agreement from last meeting is: follow the agreement from the related issue for RRC reestablishment. Issue 2-3-2: Impact on Tidentify-NR (in RRC connection release with redirection) for 1Rx in FR1 Option 2 |
| Ericsson | Issue 2-3-2: Impact on Tidentify-NR (in RRC connection release with redirection) for 1Rx in FR1 Although the side condition is defined at SSB Ês/Iot -4 dB and -8 dB for RRC connection release with redirection and RRC reestablishment respectively, we think the overall conclusion on how many samples to relax the Tidentify-NR with can be reused the from RRC reestablishment issue 2-2-2. |
| Apple | Issue 2-3-2: Impact on Tidentify-NR (in RRC connection release with redirection) for 1Rx in FR1 Option 1 based on our simulation, but if all the companies support option 2 we can also compromise. |
| MediaTek | Issue 2-3-2: Impact on Tidentify-NR (in RRC connection release with redirection) for 1Rx in FR1 Support recommended WF (Option 2). |
| vivo | Issue 2-3-2: Impact on Tidentify-NR (in RRC connection release with redirection) for 1Rx in FR1 Support Option 2. |
| Qualcomm | Issue 2-3-2: Impact on Tidentify-NR (in RRC connection release with redirection) for 1Rx in FR1 Support Option 1. Also, similar to our comment in Handover, we think RRC connection release with re-direction delay should also be extended by BWP switching delay when the RA has to be performed in Redcap specific initial BWP. We request the moderator to create a separate issue for discussion in the second round. |

### Sub-topic 2-4 Random access

Sub-topic description

Issue 2-4-1: RA requirements for HD-FDD

Background:  Following was agreed at last meeting [R4-2202670]:

“Following applies to CBRA:

    - The RedCap UE operating in HD-FDD mode is not expected to perform PRACH transmission on a PRACH resource of a cell if UE has not received at least one SSB associated with that PRACH resource during the last Tp period in the cell, where

      - Option 1: Tp=TBD ms.

    - The RedCap UE operating in HD-FDD mode shall meet the PRACH requirements when performing PRACH transmission on a PRACH resource of a cell provided that the UE has received at least one SSB associated with that PRACH resource during the last Tp period before the PRACH transmission, where

      - Option 1: Tp=TBD ms.”

Value of Tp needs to be confirmed.

- Proposals

  - Option 1 (CMCC, E///):

      - Option 1: Tp=160 ms.

- Recommended WF

Option 1 is recommended to be agreed.

Issue 2-4-2: Separate RSRP threshold for RedCap

- Proposals

  - Option 1 (CMCC, E///):

    - Introduce separate RSRP/RSRQ thresholds for RedCap UE with 1Rx.

  - Option 2 (vivo, MTK): No need to define separate threshold for 1 Rx RedCap UE

- Recommended WF

  - Discuss the options.

Issue 2-4-3: Alternative to separate RSRP threshold: offset to apply for 1 Rx UE

- Proposals

  - Option 1 (E///):

    - 1 Rx UE to apply an offset, ΔSS-RSRP, for SS-RSRP based selection for RedCap 1Rx UE,

      - e.g., ‘With SS-RSRP above msgA-RSRP-ThresholdSSB + ΔSS-RSRP for single Rx, and with SS-RSRP above msgA-RSRP-ThresholdSSB for RedCap UE other than single Rx’

      - Offset is predefined or configurable.

- Recommended WF

  - Discuss the option.

Issue 2-4-4: Impact due to early identification in RA

- Proposals

  - Option 1 (ZTE, HW, MTK): Early identification doesn’t impact RA requirements.

  - Option 2 (E///):	 RAN4 capture the RedCap UE early indication procedure in TS38.133 6.2.2 for 4-step RA type and 6.2.3 for 2-step RA type according to TS38.321/331.

- Recommended WF

  - Check if option 1 can be agreed.

Sub topic 2-4

| Company | Comments |
| --- | --- |
| XXX | Issue 2-4-2: Separate RSRP threshold for RedCap Issue 2-4-3: Alternative to separate RSRP threshold: offset to apply for 1 Rx UE Issue 2-4-4: Impact due to early identification in RA |
| Huawei | Issue 2-4-4: Impact due to early identification in RA Option 1. The random access defined in clause 6.2.2 are general description of 4step/2 step RACH procedure. As RAN2 has agreements on RedCap UE early identification and would define their procedure, RAN4 can directly refer to RAN2 spec without changing on RACH requirements. |
| Ericsson | Issue 2-4-2: Separate RSRP threshold for RedCap We support to introduce additional RSRP threshold for 1Rx on top of the existing RSRP threshold used for 2Rx UE. The dedicated threshold can reduce the risk, for example, 1Rx UE chooses 2-step RA type due to the overestimation of RSRP level. As we argued in our paper, in the current specification, gNB should configure the common threshold for UEs in IDLE/INACTIVE. Therefore we still it is beneficial to configure two threshold for 1Rx UE and 2Rx UE, which gives more flexibility to the network. Issue 2-4-3: Alternative to separate RSRP threshold: offset to apply for 1 Rx UE If companies have concern to define new signalling, we could propose to specify the UE behaviour as shown in Option 1. Delta value can be set based on the accuracy relaxation from 2Rx to 1Rx, i.e., 2.5-3.0dB as discussed in L1-RSRP measurement accuracy relaxation. Issue 2-4-4: Impact due to early identification in RA We are fine with Option 1. |
| Apple | Issue 2-4-1: RA requirements for HD-FDD Fine with recommended WF Issue 2-4-2: Separate RSRP threshold for RedCap Fine with option 2. But we could compromise to some middle ground, e.g., network configures one threshold for legacy 2Rx case as well as a margin for 1Rx case, and clarify in spec that UE may adjust this extra margin based on UE implementation and channel conditions. Issue 2-4-3: Alternative to separate RSRP threshold: offset to apply for 1 Rx UE Fine with option 1 if offset is predefined in spec. Issue 2-4-4: Impact due to early identification in RA - Option 1. |
| CMCC | Issue 2-4-1: RA requirements for HD-FDD Support recommended WF Issue 2-4-2: Separate RSRP threshold for RedCap Issue 2-4-3: Alternative to separate RSRP threshold: offset to apply for 1 Rx UE OK with option1, offset is better to be configurable. Issue 2-4-4: Impact due to early identification in RA OK with option1 |
| MediaTek | Issue 2-4-1: RA requirements for HD-FDD Support WF. Issue 2-4-2: Separate RSRP threshold for RedCap Support Option 2. To our understanding, there is no need to define a separate threshold, also, this issue should be discussed in RAN2 rather than RAN4. Yet, this issue was discussed in RAN2 already and consensus was reached and hence we support option 2 and suggest closing this issue. Issue 2-4-3: Alternative to separate RSRP threshold: offset to apply for 1 Rx UE We support another option as mentioned in CMCC contribution paper: Option 2: Use similar approach as cell ranking margin in idle mode, i.e. use different margins for 1Rx and 2Rx based on RSRP accuracy difference. Issue 2-4-4: Impact due to early identification in RA Support WF, Option 1. |
| Xiaomi | Issue 2-4-1: RA requirements for HD-FDD Fine with the recommended WF. Issue 2-4-2: Separate RSRP threshold for RedCap Issue 2-4-3: Alternative to separate RSRP threshold: offset to apply for 1 Rx UE Fine with Option 1, predefined offset based on accuracy difference between 1RX and 2RX is preferred. Issue 2-4-4: Impact due to early identification in RA Fine with Option 1. |
| ZTE | Issue 2-4-4: Impact due to early identification in RA Option 1. Early identification is a network behavior / function, doesn’t impact UE behaviors. |
| vivo | Issue 2-4-2: Separate RSRP threshold for RedCap Support Option 2. To our understanding the legacy system also has UE with different number of antenna and single threshold is used for this scenario hence we think the necessity to introduce extra threshold is not strong. Issue 2-4-3: Alternative to separate RSRP threshold: offset to apply for 1 Rx UE Open for option 1 if offset is predefined in spec. Issue 2-4-4: Impact due to early identification in RA Support Recommended WF. |
| Qualcomm | Issue 2-4-2: Separate RSRP threshold for RedCap Support Option 2. This was agreed in RAN2 and was not agreed. If needed, it must be done by RAN2. Issue 2-4-3: Alternative to separate RSRP threshold: offset to apply for 1 Rx UE This can be considered but needs further study. Can be FFS. Issue 2-4-4: Impact due to early identification in RA Support Recommended WF. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204904 (Huawei, Hisilicon) | Title: Draft CR on mobility requirements for Redcap UE |
|  | Ericsson: - in section 6.2.2B.2, the HD-FDD part looks good. But we need to clarify that those conditions are not applicable for FDD and TDD UEs. Perhaps we can clarify as follows: "The requirements for the 4-step RA type procedure described in clause 6.2.2.2 and the requirements for the 2-step RA type procedure described in the clause 6.2.2.3 are applicable for FDD and TDD RedCap UEs. The 4-step and 2-step RA requirements defined in clause 6.2.2.2 and 6.2.2.3 respectively apply to HD-FD UE with following conditions:”. - Requirements related to 1 Rx in FR2 are not needed since number of receive branches is not supported as per RF group agreement. - To better align with other sections, please use “Random access for RedCap” in all titles. - For RRC connection release with redirection, we need to include the changes from R4-2202670. - Current CR form version is 12-2 - NR FR1 HO for the Tsearch number and NR FR2-FR2 HO should follow the discussion Issue2-1-1 and Issue2-1-2 - The CR needs to be further updated to capture the agreements from issues 2-1-4 to 2-1-7. |
|  | Nokia: The clause title should be aligned with other CRs Some clause numbers should be corrected: 6.1.2C.1: it should be 6.1C.2.1, 6.2.3.2A.1 should be 6.2.3A.2.1 and the same for 6.2.3.2A.2. The table number should also be corrected. There is typo: RecCap UE (in 6.1.2C.1 and other occurrences) |
| R4-2204905 (Huawei, Hisilicon) | Title: Draft CR on E-UTRAN - NR Handover for Redcap UE |
|  | Ericsson: - Current CR form version is 12-2 - Requirements related to 1 Rx in FR2 are not needed since number of receive branches is not supported as per RF group agreement. Therefore the section on FR2 requirements should contain requirements for 2 Rx UE only. - Conditions for HD-FDD UE needs to be added depending on the outcome of issue 2-1-3. - To better align with other sections, please use “…for RedCap” in all titles. - The CR needs to be further updated to capture the agreements from issues 2-1-4 to 2-1-7. |
| R4-2206038 (Ericsson) | Title: RRC connection release with redireciton for redcap in TS 36.133 |
|  | Nokia: On table 6.3.2.6-1, the FR2 requirements are not needed, following the agreements from RF session. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic#2-1 | Issue 2-1-1: Impact on Tsearch (in HO) for 1Rx for FR1 - Option 2 (HW, QC): - For HO to FR1, Tsearch = 3*Trs for intra-frequency HO and Tsearch = 5* Trs for inter-frequency HO - Compromise option from moderator (E///, Apple, MTK, Nokia, vivo): - For HO to FR1, - Tsearch = 2*Trs for intra-frequency HO - Tsearch = 5* Trs for inter-frequency HO Recommendation for 2nd round: Check if companies can compromise to the compromise proposal from the moderator above which has the most support. Issue 2-1-3: Requirements for HD-FDD in HO - Option 1 (Apple, HW, Nokia): - RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in HO requirement. - Option 2 (ZTE, CMCC, MTK, E///, HW, QC): The UE shall meet HO requirements provided the following is met: - SSB is available at the UE once every SMTC period during Tsearch. - One SSB is available during T∆ - One SSB is available during Tiu - Option 2a (ZTE): Any agreement can be captured in the spec in a way that describes the applicability of the requirements, instead of mandating specific UE/NW behavior. Recommendations for 2nd round: Moderator: Please note that SSB is received in both options to meet the HO. The difference is that in option 1, DL is prioritized over UL during the entire HO requirement while in option 2 states the availability of SSB during Tsearch, T∆ and Tiu. Therefore they are not conflicting. With this clarification, can companies compromise to option 2 which has the most support? Issue 2-1-4: HO to target cell when SSB used for measurement and SSB associated with RACH BWP is different - Option 1 (E///, Nokia): The UE is not expected to perform HO to the target cell when: - It has done measurement in target cell on SSB which is different than the SSB associated with BWP where RACH is configured and if the separation between initial BWP and Redcap BWPs is larger than 20 MHz for FR1 and 100 MHz for FR2 - Option 2 (MTK, QC): Scenario is invalid/no restriction needed. Recommendations for 2nd round: Questions to discuss in 2nd round: “We are wondering if the target cell synchronization and measurement could be performed on NCD-SSB, then HO could be rely on NCD-SSB without option 1.” “Why the frequency separation and periodicity of NCD-SSB and CD-SSB matters?” Companies are encouraged to check the RAN2 agreements and provide further view. Issue 2-1-5: HO to a RedCap specific BWP with NCD-SSB (no CD-SSB) - Option 1 (E///): Handover directly to a RedCap specific BWP where only NCD-SSB is transmitted is supported when: - the difference of center frequency between NCD-SSB of target BWP and CD-SSB of initial BWP of target cell is not larger than 20MHz in FR1 and 100MHz in FR2, - (MTK, QC): the difference of reception power between NCD-SSB and CD-SSB is less than 3dB, - (MTK, QC): the periodicity of NCD-SSB and CD-SSB is the same - Option 2 (MTK, Nokia): Depends on whether the scenario is supported in RAN2. Recommendations for 2nd round: Companies to check if the scenario is supported by RAN2. Questions to discuss in 2nd round: “We are wondering if the target cell synchronization and measurement could be performed on NCD-SSB, then HO could be rely on NCD-SSB without option 1.” “Why the frequency separation and periodicity of NCD-SSB and CD-SSB matters?” Companies are encouraged to check the RAN2 agreements and provide further view. Issue 2-1-6: Requirements for handover to RedCap specific BWP with NCD-SSB (no CD-SSB) - Option 1 (E///): Legacy NR handover requirements can be reused for handover to a target RedCap specific BWP where NCD-SSB is transmitted when the periodicity of NCD-SSB and CD-SSB is same. - Option 2 (, MTK, Nokia): Depends on whether the scenario is supported in RAN2. - New Option 3 (Qualcomm): When the Redcap specific initial DL BWP is configured for RA only, define the interruption time as - Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin + TBWP-switching-delay ms - Where, TBWP-switching-delay is the BWP switching delay Recommendations for 2nd round: Questions to discuss in 2nd round based on the updated proposals: “We are wondering if the target cell synchronization and measurement could be performed on NCD-SSB, then HO could be rely on NCD-SSB without option 1.” “Why the frequency separation and periodicity of NCD-SSB and CD-SSB matters?” Companies are encouraged to check the RAN2 agreements and provide further view. Issue 2-1-7: Handover to RedCap specific BWP with no SSB - Option 1 (E///): When there is no SSB transmission in a target RedCap specific BWP, handover to that RedCap specific BWP is supported when: - target RedCap specific BWP and initial BWP of target cell are not separated by larger than 20MHz in FR1 and 100MHz in FR2. - Option 2 (Apple): When there is no SSB transmission in a target RedCap specific BWP, handover to that RedCap specific BWP is supported but no requirement applies. - Option 3 (MTK): When there is no SSB transmission in a target RedCap specific BWP, handover to that RedCap then the UE shall switch back to CD-SSB. - New Option 4 (Qualcomm): When the Redcap specific initial DL BWP is configured for RA only, and there is no SSB available in the redcap specific initial BWP, define the interruption time as - Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin + TBWP-switching-delay ms - Where, TBWP-switching-delay is the BWP switching delay Recommendations for 2nd round: Discuss the updated options above. Companies are encouraged to check the RAN2 agreements and provide further view. |
| Sub-topic#2-2 | Issue 2-2-1: Lower bound in Max function in reestablishment delay - Option 1 (Apple, vivo, Nokia, HW, E///, MTK): The lower boundary in Max function for reestablishment delay requirement shall not be changed for RedCap UE with 1Rx. - Option 2 (QC): Support extending the lower bound - for known cell to be 240 ms, - for the unknown cell to be 860 ms. Tentative agreements: [The lower boundary in Max function for reestablishment delay requirement shall not be changed for RedCap UE with 1Rx.] Issue 2-2-2: Impact on Tidentify (in reestablishment) for 1Rx in FR1 RAN4 to relax the Tidentify- for RedCap reestablishment delay requirement with 1Rx by: - Option 1 ( QC): 2 more samples for intra-frequency and inter-frequency - Option 2 (HW, Nokia, MTK, E///, vivo, Apple): - 1 more sample for unknown target cell (total of 11 x TSMTC) for intra-frequency and inter-frequency Tentative agreements: [RAN4 to relax the Tidentify- for RedCap reestablishment delay requirement with 1Rx by: 1 more sample for unknown target cell (total of 11 x TSMTC) for intra-frequency and inter-frequency] |
| Sub-topic#2-3 | Issue 2-3-2: Impact on Tidentify-NR (in RRC connection release with redirection) for 1Rx in FR1 RAN4 to relax the Tidentify-NR for RedCap RRC redirection delay requirement with 1Rx by: - Option 1 (QC): 2 more samples - Option 2 (vivo, Nokia, HW, MTK, E///, Apple): 1 more sample Tentative agreements: [RAN4 to relax the Tidentify-NR for RedCap RRC redirection delay requirement with 1Rx by 1 more sample.] |
| Sub-topic#2-4 | Issue 2-4-1: RA requirements for HD-FDD Tentative agreements: Following applies to CBRA: - The RedCap UE operating in HD-FDD mode is not expected to perform PRACH transmission on a PRACH resource of a cell if UE has not received at least one SSB associated with that PRACH resource during the last Tp period in the cell, where - Option 1: Tp=160 ms. - The RedCap UE operating in HD-FDD mode shall meet the PRACH requirements when performing PRACH transmission on a PRACH resource of a cell provided that the UE has received at least one SSB associated with that PRACH resource during the last Tp period before the PRACH transmission, where - Option 1: Tp=160 ms Issue 2-4-2: Separate RSRP threshold for RedCap Issue 2-4-3: Alternative to separate RSRP threshold: offset to apply for 1 Rx UE Moderator: Given that this issue been discussed for last several meetings and very little time left to complete the core part, agreeing on FFS shall be avoided at this stage. Tentative agreements: Network configures one RSRP/RSRQ threshold for 2 Rx RedCap UE (same as for legacy 2 Rx UE), and 1 Rx RedCap UE applies a margin to that threshold. Recommendations for 2nd round: Discuss whether the margin is: - Option 1: configurable by the NW - Option 2: predefined in the specification. Issue 2-4-4: Impact due to early identification in RA - Option 1 (ZTE, HW, MTK, E///): Early identification doesn’t impact RA requirements. - Option 2 (E///): RAN4 capture the RedCap UE early indication procedure in TS38.133 6.2.2 for 4-step RA type and 6.2.3 for 2-step RA type according to TS38.321/331. Tentative agreements: Early identification doesn’t impact RA requirements for RedCap UE. |

## Discussion on 2nd round (if applicable)

Companies are encouraged to provide their view on the issues recommended for 2nd round discussions in 2.3.1.

Topic #2

| Company | Comments |
| --- | --- |
| Company X | Issue x-y |

# Topic #3: Timing requirements

Contributions from AI 10.20.3.1.3 are discussed here.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203585 | ZTE Corporation | - Proposal 1: Redcap UE should meet the existing Te and Tq requirements provided that the SSB is available at the UE at least once every 160 ms, where SSB can be CD-SSB or NCD-SSB. Proposal 2: From RAN4 perspective, CSI-RS can be used for RedCap UEs to acquire the reference cell timing depending on UE capability. |
| R4-2206037 | Ericsson | Proposal #1: UE shall meet the existing Te and Tq requirements for corresponding FR and SCS defined in section 7.1 of TS 38.133 provided that: - the CD-SSB or NCD-SSB is within the UE’s active BWP or - the initial BWP and the RedCap BWP are within 20 MHz for FR1 or 100 MHz for FR2 if the NCD-SSB is not within the UE’s RedCap active BWP. According to agreements captured in the WF in R4-2202774 and in LS reply to RAN1 in R4-2202773, it was agreed that RAN4 will not define UE transmit timing requirements based on CSI-RS in Rel-17. |
| R4-2204250 | Xiaomi | Proposal 1: For UE transmit timing requirements, SSB refers to both CD-SSB and NCD-SSB. Proposal 2: CSI-RS/TRS is not needed to acquire the reference cell timing for RedCap UE in Rel-17. |
| R4-2204906 | Huawei, Hisilicon | Proposal 1: Redcap UE should meet the existing Te and Tq requirements provided that the SSB is available at the UE at least once every 160 ms: - SSB refers to CD-SSB or any of CS- and NCD-SSB regardless whether SSB is in active BWP. |
| R4-2204994 | CMCC | Proposal 1: Redcap UE should meet the existing Te and Tq requirements provided that the SSB is available at the UE at least once every 160 ms. No further condition is needed. Proposal 2: There is no need to continue discuss whether CSI-RS/TRS can be used for meeting the timing requirements, this can be up to UE implementation. |
| R4-2206080 | MediaTek inc. | - Support Option 2: SSB has to be in active BWP, and Option 3: SSB refers to both CD-SSB and NCD-SSB and SSB shall be in the active BWP. - Don’t support the use of CSI-RS/TRS to acquire the reference cell timing in RedCap UEs. - Support Option 1: CSI-RS/TRS is not needed to acquire the reference cell timing in Rel-17. |
| R4-2204248 | Xiaomi | Draft CR on timing requirements for RedCap UE |
| R4-2204913 | Huawei, Hisilicon | Draft CR: Clarification on transmit timing before Msg1 or MsgA retransmission |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1 Timing

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 3-1-1: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements

Background: Following was agreed in R4-2120418: “RedCap UE shall meet the existing transmit timing requirements defined in section 7.1 in TS 38.133.”

In section 7.1 in 38.133, there is following statement: “The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms.”

- Proposals

- Option 1 (MTK): 	SSB has to be in active BWP.

- Option 2 (ZTE, HW, CMCC):	 Redcap UE should meet the existing Te and Tq requirements provided that the SSB is available at the UE at least once every 160 ms regardless whether SSB is in active BWP.

- Option 3 (E///): Te requirements are met under any of the following scenarios:

  - SSB is in the UE’s active BWP, or

  - SSB is not in the UE’s active BWP (RedCap BWP) but the following condition is met:

    - UE’s active BWP(RedCap BWP) and initial BWP are within 20 MHz for FR1, or within 100 MHz for FR2.

- Recommended WF

  - Discuss the options.

Issue 3-1-2: Type of SSB to use for meeting the UE transmit timing requirements

- Proposals

- Option 1 (MTK, CMCC, HW, E///, ZTE, Xiaomi): SSB refers to both CD-SSB or NCD-SSB

- Recommended WF

  - Option 1 recommended to be agreed.

Issue 3-1-3: Whether CSI-RS/TRS can be used for meeting the timing requirements

- Proposals

  - Option 1 (ZTE):  From RAN4 perspective, CSI-RS can be used for RedCap UEs to acquire the reference cell timing depending on UE capability.

  - Option 2 (Xiaomi, MTK): CSI-RS/TRS is not needed to acquire the reference cell timing for RedCap UE in Rel-17.

  - Option 3 (CMCC, E///): Previous agreement is sufficient, no further discussions needed.

- Recommended WF

  - Given the agreements from last meeting in the WF in R4-2202774 and in LS reply to RAN1 in R4-2202773 that RAN4 will not define any UE transmit timing requirements based on CSI-RS in Rel-17, no more discussions needed.

Sub topic 3-1

| Company | Comments |
| --- | --- |
| XXX | Issue 3-1-1: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements |
| Huawei | Issue 3-1-1: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements Option 2. From UE implementation perspective UE can perform T/F tracking based on CD-SSB or NCD-SSB or CSI-RS and the existing Te, Tq requirements can be satisfied. In legacy requirements, the condition for timing requirements don’t limit the SSB shall be in the active BWP. Even for the UE in SSB-less BWP, the timing requirements still need to be satisfied. How UE implement timing is up to UE implementation. Issue 3-1-2: Type of SSB to use for meeting the UE transmit timing requirements Recommended WF is fine, as it is agreed that RAN4 will not define any UE transmit timing requirements based on CSI-RS in Rel-17, we don’t discuss CSI-RS based timing anymore and focus on CD-/NCD-SSB. Issue 3-1-3: Whether CSI-RS/TRS can be used for meeting the timing requirements Agree with the recommended WF as it is agreed that RAN4 will not define any UE transmit timing requirements based on CSI-RS in Rel-17 |
| Ericsson | Issue 3-1-1: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements We support Option 3. This is compromise between Option 1 and Option 2. RedCap UE will be served by legacy BS which can operate in very larger cell BW. Therefore, the RedCap BWP may be configured anywhere within the cell BW wrt the frequency location of the initial BWP. It may be more challenging for the RedCap UE to meet timing requirements if there is no NCD-SSB in RedCap BWP, which is more than RedCap BW (20 MHz/100 MHz for FR1/FR2) away from initial BWP. Otherwise if RedCap BWP and initial BWP are close to each other (within the RedCap BW) then should be able to meet the timing requirements by switching to CD-SSB e.g. once every 160 ms. Issue 3-1-2: Type of SSB to use for meeting the UE transmit timing requirements We support Option 1. If SSB is available once every 160 ms then it does not matter which it is CD-SSB or NCD-SSB. Issue 3-1-3: Whether CSI-RS/TRS can be used for meeting the timing requirements Agree with the moderator’s recommended WF. |
| Apple | Issue 3-1-1: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements Support option 2. Issue 3-1-2: Type of SSB to use for meeting the UE transmit timing requirements Fine with recommended WF. Issue 3-1-3: Whether CSI-RS/TRS can be used for meeting the timing requirements Fine with recommended WF. |
| CMCC | Issue 3-1-1: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements Option2 Issue 3-1-2: Type of SSB to use for meeting the UE transmit timing requirements Option1 Issue 3-1-3: Whether CSI-RS/TRS can be used for meeting the timing requirements Support recommended WF |
| MediaTek | Issue 3-1-1: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements We believe it is necessary to support SSB withing active BWP, hence we support option 1. Issue 3-1-2: Type of SSB to use for meeting the UE transmit timing requirements Recommended WF is agreeable. Issue 3-1-3: Whether CSI-RS/TRS can be used for meeting the timing requirements Recommended WF is agreeable. |
| Xiaomi | Issue 3-1-1: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements Support option 2. Issue 3-1-2: Type of SSB to use for meeting the UE transmit timing requirements Support the recommended WF. Issue 3-1-3: Whether CSI-RS/TRS can be used for meeting the timing requirements Support the recommended WF. |
| ZTE | Issue 3-1-1: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements Option 2. SSB available in the last X (X = 160) ms shall be considered. Not fully understand the suggested proposal from Ericsson, why do we need to judge the distance on frequency domain between the initial BWP and active BWP? Is there any requirement / capability on that? Why 20 MHz as the threshold? Issue 3-1-2: Type of SSB to use for meeting the UE transmit timing requirements Support the recommended WF. Issue 3-1-3: Whether CSI-RS/TRS can be used for meeting the timing requirements Option 3, but considering the time left for R17 completion it might not be possible to define anything related to CSI-RS, so also fine with the recommended WF. |
| Nokia | Issue 3-1-1: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements We have a preference for option 2. However, as option 2 may increase UE complexity, in particular for 1Rx UE, we are fine with option 3 as a compromise between option 1 and option 2. Issue 3-1-2: Type of SSB to use for meeting the UE transmit timing requirements We support option 1 and the recommended WF. Issue 3-1-3: Whether CSI-RS/TRS can be used for meeting the timing requirements We support the recommended WF. |
| vivo | Issue 3-1-1: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements Support option 1. For legacy UE, the assumption is that there is SSB available in serving cell active BWP in our understanding. At least the PCell timing is based on available SSB within active BWP. Otherwise, there will be interruptions/scheduling restrictions due to RF retuning, which is not specified in the spec. The mandatory basic BWP operation requires that active BWP includes SSB. Issue 3-1-2: Type of SSB to use for meeting the UE transmit timing requirements Fine with recommended WF. Issue 3-1-3: Whether CSI-RS/TRS can be used for meeting the timing requirements Fine with recommended WF. |
| Qualcomm | Issue 3-1-1: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements Support option 1. SSB must be within the active BWP. Issue 3-1-2: Type of SSB to use for meeting the UE transmit timing requirements Recommended WF is agreeable. Issue 3-1-3: Whether CSI-RS/TRS can be used for meeting the timing requirements Recommended WF is agreeable. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204248 Xiaomi | Title: Draft CR on timing requirements for RedCap UE |
|  | Ericsson: - The following should be removed in Table 7.1A.2-2since there is only PCell: In case of multiple UL carriers in the same TAG, UE expects that the same value of n-TimingAdvanceOffset is provided for all the UL carriers according to clause 4.2 in TS 38.213 [3] and the value 39936 of can also be provided for a FDD serving cell. - SA also covers NR CA. Therefore we suggest to replace the following in section 7.3A.1: The timing advance is initiated from gNB to UE in NR SA operation mode By: The timing advance is initiated from gNB to UE configured with only PCell. Another approach is to capture it in the applicability CR. |
|  | Nokia: The wording in section 7.1A.2 in the second paragraph needs to take into account the conclusion for issue 3-1-1 (currently option 2 is implemented). Editorial comment: In section 7.3A.2.1, it reads “UE shall adjust the timing of its uplink transmission timing at time slot…”. The last occurrence of “timing” can be removed. |
| R4-2204913 (Huawei, Hisilicon) | Title: Clarification on transmit timing before Msg1 or MsgA retransmission |
|  | Ericsson: - If there is NO SSB then obviously no Te requirement is met regardless of any RRC state and type of UL transmission as covered in section 7.1.2. - The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. - Section 7.1.1 is for legacy NR UE (non-redcap UE). There should be NO impact of Redcap in existing section. In summary: following change is not needed in section 7.1.2 or even in RedCap specific new timing section. This may cause even confusion based on 1) above: Note: If a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, UE is not required to meet Te requirements before Msg1/A retransmission. |
|  | Nokia: The change should be included in the above draft CR in the new section for RedCap. It depends on the conclusion for issue 3-1-1. This draft CR defines an exception for option 2 in issue 3-1-1, but may not be needed in case option 3 is agreed. |
|  | vivo: Whether to add the note needs further discussion. The requirements should be based on SSB availability in active BWP. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic#3-1 | Issue 3-1-1: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements Background: Following was agreed in R4-2120418: “RedCap UE shall meet the existing transmit timing requirements defined in section 7.1 in TS 38.133.” In section 7.1 in 38.133, there is following statement: “The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms.” - Proposals - Option 1 (MTK, vivo, QC): SSB has to be in active BWP. - Option 2 (ZTE, HW, CMCC, Apple, Xiaomi): Redcap UE should meet the existing Te and Tq requirements provided that the SSB is available at the UE at least once every 160 ms regardless whether SSB is in active BWP. - Option 3 (E///, Nokia): Te requirements are met under any of the following scenarios: - SSB is in the UE’s active BWP, or - SSB is not in the UE’s active BWP (RedCap BWP) but the following condition is met: - UE’s active BWP(RedCap BWP) and initial BWP are within 20 MHz for FR1, or within 100 MHz for FR2. Recommendations for 2nd round: Discuss the questions raised in the 1st round: - “why do we need to judge the distance on frequency domain between the initial BWP and active BWP? Is there any requirement / capability on that? Why 20 MHz as the threshold?” Please note that option 1 is different from legacy requirements. Check if companies can compromise to option 3 (combination of option 1 and 2). Issue 3-1-2: Type of SSB to use for meeting the UE transmit timing requirements Tentative agreement: Type of SSB used to meet the UE transmit timing requirements can be CD-SSB or NCD-SSB. |

## Discussion on 2nd round (if applicable)

Companies are encouraged to provide their view on the issues recommended for 2nd round discussions in 3.3.1.

Topic #3

| Company | Comments |
| --- | --- |
| Company X | Issue x-y |

# Topic #4: Signalling characteristics

Contributions from AI 10.20.3.1.4 are discussed here.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203586 | ZTE Corporation | - Proposal 1: Reuse legacy BWP switching delay when only center frequency is changed. |
| R4-2203788 | Apple | Proposal 1: For RedCap UE with 1Rx, to follow the OOS evaluation period of RLM, RAN4 to double the evaluation period for SSB based and CSI-RS based BFD in both FR1 and FR2. Proposal 2: For RedCap UE with 1Rx, RAN4 to double the lower bound for SSB based and CSI-RS based RLM OOS evaluation period in both FR1 and FR2. Proposal 3: For RedCap UE with 1Rx, no need to extend the evaluation period for SSB based and CSI-RS based CBD in both FR1 and FR2. Proposal 4: For RedCap UE with 1Rx, SSB-based L1-RSRP measurement period can be unchanged for both FR1 and FR2. Proposal 5: For RedCap UE with 1Rx, CSI-RS-based L1-RSRP measurement period can be unchanged for both FR1 and FR2. Proposal 6: the SSB-based and CSI-RS-based L1-RSRP measurement accuracy requirement shall be defined: - For FR1 RedCap with 1Rx - Relax the current absolute L1-RSRP accuracy by 3dB - Relax the current relative L1-RSRP accuracy by 3dB - For FR2 RedCap with 1Rx - Relax the current absolute L1-RSRP accuracy by 2dB - Relax the current relative L1-RSRP accuracy by 2dB |
| R4-2204251 | Xiaomi | Proposal 1: RAN4 to reuse the legacy BWP switching delay for RedCap UE in Rel-17. |
| R4-2204323 | vivo | Proposal 1: The lower bound extension for RLM is by a factor of 2. Proposal 2: Regarding remaining issues for BFD evaluation period, the measurement period of SS-SINR for legacy UE can be reused for BFD when 1 Rx is used, i.e., option 1. The measurement period of CSI-SINR for legacy UE can be reused for BFD when 1 Rx is used, i.e., option 1. Proposal 3: For absolute accuracy in FR1 for SSB-based L1-RSRP, we support to relax 2.5dB or 3dB, i.e., Option 3 and Option 2. For absolute accuracy in FR2 for SSB-based L1-RRP, we support to relax 2dB or 3dB, i.e., Option 2 and Option 3. For absolute accuracy in FR1 for CSI-RS based L1-RSRP, we support to relax 3dB, i.e., Option 3. For absolute accuracy in FR2 for CSI-RS based L1-RSRP, we support to relax 2dB, i.e., Option 2. Proposal 4: Support option 2 for this issue, i.e., RAN4 to reuse the legacy BWP switching delay for RedCap UE in Rel-17. Proposal 5: Support to reuse the legacy requirements for MAC-CE and RRC based TCI state switch delay with target TCI unknown when 1Rx is used. Proposal 6: Support to reuse the legacy requirements for MAC-CE and RRC-based spatial relation switch delay with target spatial relation associated with DL RS is unknown when 1Rx is used. |
| R4-2204536 | Nokia, Nokia Shanghai Bell | - Proposal 1: Extend the lower bound of the out-of-sync evaluation period for SSB-based and CSI-RS based RLM by a factor of 2. Proposal 2: In order to compensate for the SINR degradation due to decreasing the number of Rx branches, for SSB based and CSI-RS RLM in-sync evaluation relax the SINR in the test case by 0.5 dB in FR1 and FR2. Proposal 3: In order to compensate for the SINR degradation due to decreasing the number of Rx branches, for SSB based RLM out-of-sync evaluation relax the SINR in the test case by 0.5 dB in FR1 and FR2. Proposal 4: In order to compensate for the SINR degradation due to decreasing the number of Rx branches, for CSI-RS based RLM in-sync evaluation relax the SINR in the test case by 1.0 dB in FR1 and FR2. Proposal 5: In order to compensate for the SINR degradation due to decreasing the number of Rx branches, for CSI-RS based RLM out-of-sync evaluation relax the SINR in the test case by 1.0 dB in FR1 and FR2. |
| R4-2204907 | Huawei, Hisilicon | Proposal 1: Double the lower bound for SSB and CSI-RS based RLM OOS evaluation period. Proposal 2: Legacy SSB and CSI-RS based BFD evaluation period can be reused for 1 RX Redcap. Proposal 3: For RedCap UE with 1RX, the measurement accuracy shall be based on one sample. - The absolute accuracy for both SSB based and CSI-RS based L1-RSRP can be relaxed by 3dB; - The relative accuracy for both SSB based and CSI-RS based L1-RSRP can be relaxed by 3dB. |
| R4-2204995 | CMCC | Proposal 1: Define new BWP switching delay involving only changing of the center-frequency of the BWP without changing its BW, SCS or any other parameter for RF retuning as follows: Frequency Range; Type 1 Delay (us); Type 2 Delay (us). 1; 200; 1050. 2; 200; 1050. Proposal 2: It is proposed to define a new UE capability to indicate the support of new BWP switching delay when only center-frequency is changed. Proposal 3: Do not introduce L1 measurement gaps. Proposal 4: TCI state switching &UL spatial relation switch delay: - For MAC-CE based and RRC-based delay requirements: outcome of L1-RSRP delay requirements from CBD evaluation requirements is reused. |
| R4-2204996 | CMCC | Draft CR to 38.133 for introducing RedCap requirements on active BWP switch delay, active TCI state switching delay and UE specific CBW change |
| R4-2205630 | Ericsson | Proposal #1: - Lower bound in the Qout evaluation period is extended by factor 2. - Lower bound in Qin evaluation period is unchanged. Proposal #2: SSB based RLM in-sync evaluation period for 1 Rx UE is reused for SSB based BFD evaluation period for 1 Rx UE, including lower bound. Proposal #3: CSI-RS based RLM in-sync evaluation period for 1 Rx UE is reused for CSI-RS based BFD evaluation period for 1 Rx UE, including lower bound. Proposal #4: Relax the SSB based L1-RSRP measurement absolute and relative accuracy requirements by 3.0dB for 1 Rx UE compared to the 2 Rx UE requirements for FR1. This relaxation is applicable for both absolute and relative accuracy requirements. Proposal #5: Relax the CSI-RS based L1-RSRP measurement absolute accuracy requirements by 3.0dB for 1 Rx UE compared to 2 Rx UE requirements in FR1. Proposal #6: Define BWP switching delay between non-initial DL BWP for RedCap and initial BWP for RedCap involving only changing of the center-frequency of the BWP. Proposal #7: Define BWP switching delay between non-initial DL BWP for RedCap and initial BWP for RedCap involving only changing of the center-frequency of the BWP based on the on previous agreement in R4-1803283: Frequency Range; Type 1 Delay (us); Type 2 Delay (us). 1; 200; 1050. 2; 200; 1050. Proposal #8: Active BWP switching delay involving only changing of the center-frequency of the BWP without changing its BW, SCS or any other parameter for RF retuning between non-initial DL BWP for RedCap and initial BWP for RedCap can be expressed in slots as follows: SCS; Slot length (ms); Type 1 Delay (slots); Type 2 Delay (slots). 15 kHz; 1; 1; 2. 30 kHz; 0.5; 1; 3. 60 kHz; 0.25; 1; 5. 120 kHz; 0.125; 2; 9. Proposal #9: The UE is not required to transmit UL signals or receive DL signals while tuning between initial BWP and the RedCap specific non-initial BWP provided that the DRX cycle is less than 640 ms. No scheduling restriction is allowed for DRX cycle of 640 ms or longer. Proposal #10: Do not introduce L1 measurement gaps for acquiring SSB outside active BWP. Proposal #11: - For MAC-CE based TCI state switch delay with target TCI unknown: legacy requirements are reused for FR1. - For RRC-based TCI state switch delay with target TCI unknown: legacy requirements are reused for FR1. Proposal #12: No need to discuss the TCI state requirements for 1 Rx for FR2 given the RF agreement that number of Rx branches is not reduced in FR2. Proposal #13: - For MAC-CE based spatial relation switch delay with target spatial relation associated with DL RS is unknown: legacy requirements are reused for FR1. - For RRC-based spatial switch delay with target spatial relation associated to DL RS is unknown: legacy requirements are reused for FR1. Proposal #14: No need to discuss the UL spatial relation switch delay requirements for 1 Rx for FR2 given the RF agreement that number of Rx branches is not reduced in FR2. |
| R4-2206085 | MediaTek Inc. | DraftCR on reduced capability Ues for RLM for RedCap |
| R4-2206112 | Qualcomm Incorporated | Proposal 1: RAN4 to not introduce any faster BWP switching delays than the current Type1 and Type2 BWP switching delays as there is no strong use case to enable fast BWP switching for Redcap UEs. |
| R4-2206081 | MediaTek inc. | Proposal 1: Support Option 1: extending the lower bound of the evaluation period of 5G NR RedCap RLM requirement by two compared to the existing general 5G NR RLM requirements. Proposal 2: Support following the agreement of out-of-sync from RLM for BFD. Proposal 3: Support extending the evaluation period for BFD by a factor of two in comparison to the existing NR. Proposal 4: Support clarifying whether the issues of relative/absolute accuracy are related to intra-frequency and/or inter-frequency. Proposal 5: Support relaxing the accuracy requirements of FR1 by the following: SSB-based L1-RSRP: absolute accuracy with measurement restriction in FR1 to relax by 3dB. SSB-based L1-RSRP: relative accuracy with measurement restriction in FR1 to relax by 2dB. CSI-RS based L1-RSRP: absolute accuracy with measurement restriction in FR1 to relax by 3dB. Proposal 6: For all performance accuracy requirements related to L1-RSRP for FR2, support further study these issues. |
| R4-2204284 | OPPO | Proposal 1: lower bound in RLM evaluation period is extended by factor 2. Proposal 2: No need to extend the evaluation period for BFD in FR1 and FR2. Proposal 3: RAN4 to reuse the legacy BWP switching delay for RedCap UE in Rel-17. Proposal 4: Do not introduce L1 measurement gaps. |
| R4-2204799 | vivo | Draft CR for Link Recovery Procedures for Redcap |

## Open issues summary

### Sub-topic 4-1 RLM

Issue 4-1-1: Whether to extend the lower bound for SSB-based and CSI-RS based RLM OOS evaluation period for FR1 and FR2

- Proposals

  - Option 1 (Apple, vivo): Yes

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements.

For 1 Rx UE in FR1, it was already agreed at last meeting to extend the lower bound in RLM evaluation period, see R4-2202670. Thus no more discussion needed on this issue.

Issue 4-1-2: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qout evaluation period for FR1 and FR2

Background: At last meeting it was agreed at last meeting to extend the lower bound in RLM evaluation period, see R4-2202670. Under this issue it is discuss how much to relax.

- Proposals

  - Option 1 (Apple, vivo, Nokia, HW, E///, MTK, OPPO): By factor 2

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements.

For 1 Rx, option 1 is recommended to be agreed.

Issue 4-1-3: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qin evaluation period for FR1 and FR2

Background: At last meeting it was agreed at last meeting to extend the lower bound in RLM evaluation period, see R4-2202670. Under this issue it is discuss how much to relax.

- Proposals

  - Option 1 (E///): By factor 1

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements.

Discussions needed only for 1 Rx UE in FR1.

Issue 4-1-4: Test cases:

- Proposals

  - Option 1 (Nokia):

    - In order to compensate for the SINR degradation due to decreasing the number of Rx branches, for SSB based and CSI-RS RLM in-sync evaluation relax the SINR in the test case by 0.5 dB in FR1 and FR2.

    - In order to compensate for the SINR degradation due to decreasing the number of Rx branches, for SSB based RLM out-of-sync evaluation relax the SINR in the test case by 0.5 dB in FR1 and FR2.

    - In order to compensate for the SINR degradation due to decreasing the number of Rx branches, for CSI-RS based RLM in-sync evaluation relax the SINR in the test case by 1.0 dB in FR1 and FR2.

    - In order to compensate for the SINR degradation due to decreasing the number of Rx branches, for CSI-RS based RLM out-of-sync evaluation relax the SINR in the test case by 1.0 dB in FR1 and FR2.

- Recommended WF

Focus on completing the remaining issues of core-part in this meeting. The proposals are related to test cases and can be discussed under performance part. No discussions needed.

Sub topic 4-1

| Company | Comments |
| --- | --- |
| XXX | Issue 4-1-3: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qin evaluation period for FR1 and FR2 |
| Huawei | Issue 4-1-1: Whether to extend the lower bound for SSB-based and CSI-RS based RLM OOS evaluation period for FR1 and FR2 Agee with the recommended WF. Issue 4-1-2: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qout evaluation period for FR1 and FR2 Agee with the recommended WF. Issue 4-1-3: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qin evaluation period for FR1 and FR2 Option 1 as the sample numbers for SSB-based and CSI-RS based RLM Qin are not increased. Issue 4-1-4: Test cases: Agee with the recommended WF. |
| Ericsson | Issue 4-1-3: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qin evaluation period for FR1 and FR2 In previous issue, we supported the option of extending lower bound by factor 2 for RLM Qout evaluation period. But we think the lower bound for RLM Qin can be unchanged since the RLM Qin evaluation period is not changed as agreed at last meeting. Therefore we support option 1. |
| Apple | Issue 4-1-2: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qout evaluation period for FR1 and FR2 Fine with recommended WF. Issue 4-1-3: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qin evaluation period for FR1 and FR2 Option 1 for FR1. Issue 4-1-4: Test cases: Fine with recommended WF. |
| MediaTek | Issue 4-1-2: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qout evaluation period for FR1 and FR2 Support recommended WF. Issue 4-1-3: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qin evaluation period for FR1 and FR2 Support recommended WF. Issue 4-1-4: Test cases: Support recommended WF. |
| OPPO | Issue 4-1-2: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qout evaluation period for FR1 and FR2 Fine with recommended WF. Issue 4-1-4: Test cases: Fine with recommended WF. |
| Nokia | Issue 4-1-3: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qin evaluation period for FR1 and FR2 Given that the Qin evaluation period is not extended, we support Option 1. |
| vivo | Issue 4-1-1: Whether to extend the lower bound for SSB-based and CSI-RS based RLM OOS evaluation period for FR1 and FR2 Fine with the recommended WF. Issue 4-1-2: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qout evaluation period for FR1 and FR2 Fine with the recommended WF. Issue 4-1-3: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qin evaluation period for FR1 and FR2 Option 1. The sample number for SSB-based and CSI-RS based RLM Qin is not increased for 1Rx. Issue 4-1-4: Test cases: Fine with the recommended WF. |
| Qualcomm | Issue 4-1-2: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qout evaluation period for FR1 and FR2 Fine with recommended WF. Issue 4-1-3: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qin evaluation period for FR1 and FR2 Option 1 for FR1. Issue 4-1-4: Test cases: Fine with recommended WF. |

### Sub-topic 4-2 BFD

Issue 4-2-1: SSB-based based BFD: evaluation period for FR1 and FR2

- Proposals

  - Option 1 (HW, E///, OPPO): No need to extend the evaluation period for BFD compared to legacy evaluation period.

  - Option 2 (Apple, MTK): Set SSB based BFD evaluation period based on 10 samples for 1Rx UE.

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements. Discussions needed only for 1 Rx UE in FR1.

Issue 4-2-2: CSI-RS-based BFD: evaluation period for FR1 and FR2

- Proposals

  - Option 1 (HW, E///): No need to extend the evaluation period compared to legacy evaluation period.

  - Option 2 (Apple): Set CSI-RS based BFD evaluation period based on 20 samples for 1Rx UE.

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements. Discussions needed only for 1 Rx UE in FR1.

Sub topic 4-2

| Company | Comments |
| --- | --- |
| XXX | Issue 4-2-1: SSB-based based BFD: evaluation period for FR1 and FR2 Issue 4-2-2: CSI-RS-based BFD: evaluation period for FR1 and FR2 |
| Huawei | Issue 4-2-1: SSB-based based BFD: evaluation period for FR1 and FR2 Option 1. The side condition for BFD Qout is observed at -8dB which is higher than Qout for RLM (note: Qout for RLM is observed @-10dB). At higher side condition, the evaluation samples for BFD Qout for RedCap with 1RX can be the same as legacy. Issue 4-2-2: CSI-RS-based BFD: evaluation period for FR1 and FR2 Option 1. Same comments as above. |
| Ericsson | Issue 4-2-1: SSB-based based BFD: evaluation period for FR1 and FR2 Rel-15 beam failure detection requirements assume the same number of SSB/CSI-RS samples as RLM In-sync to define evaluation period, that is, 5 samples for SSB based BFD and 10 samples for CSI-RS based BFD (density=3, 24RBs). At last meeting it was agreed to not the RLM in-sync evaluation period. Therefore, the same should apply also for BFD. Thus, we support option 1. Issue 4-2-2: CSI-RS-based BFD: evaluation period for FR1 and FR2 For same reason as for issue 4-2-1, we support option 1. |
| Apple | Issue 4-2-1: SSB-based based BFD: evaluation period for FR1 and FR2 Option 2 to reuse the same extension from RLM OOS, but we can compromise to option 1 since option 1 was our previous proposal. Issue 4-2-2: CSI-RS-based BFD: evaluation period for FR1 and FR2 Option 2 to reuse the same extension from RLM OOS, but we can compromise to option 1 since option 1 was our previous proposal. |
| MediaTek | Issue 4-2-1: SSB-based based BFD: evaluation period for FR1 and FR2 Based on our LLS performance, we can observe that we need to increase the number of samples in the time evaluation by a factor of 2. Also, this requirement is defined as Qout_LR in the specs and given that we have already agreed to extend the evaluation period for out-of-sync in RLM, thus we support extending the evaluation period by a period of 2. Therefore, we support option 2. Also, we support to modify option 2 to be extended by a factor 2. This is because in the existing requirements different DRX cycles has different number of samples. Thus, Option 2 can be written as: Option 2: Set SSB based BFD evaluation period to be extended by a factor of 2 for 1Rx UE. Issue 4-2-2: CSI-RS-based BFD: evaluation period for FR1 and FR2 Based on our LLS performance, we can observe that we need to increase the number of samples in the time evaluation by a factor of 2. Also, given that we have already agreed to extend the evaluation period for out-of-sync in RLM, thus we support extending the evaluation period by a period of 2. Therefore, we support option 2. |
| OPPO | Issue 4-2-1: SSB-based based BFD: evaluation period for FR1 and FR2 Option 1. The observations from Huawei is agreeable and no need to extend the evaluation period. |
| Nokia | Issue 4-2-1: SSB-based based BFD: evaluation period for FR1 and FR2 We support Option 1. Issue 4-2-2: CSI-RS-based BFD: evaluation period for FR1 and FR2 We support Option 1. |
| vivo | Issue 4-2-1: SSB-based based BFD: evaluation period for FR1 and FR2 Option 1. Based on our simulation results, there is no need to extend the evaluation period for BFD for 1Rx UE. Issue 4-2-2: CSI-RS-based BFD: evaluation period for FR1 and FR2 Option 1. Based on our simulation results, there is no need to extend the evaluation period for BFD for 1Rx UE. |
| Qualcomm | Issue 4-2-1: SSB-based based BFD: evaluation period for FR1 and FR2 We support Option 2. The procedure for BFD evaluation is similar to RLM OOS and hence the delay should be extended. Issue 4-2-2: CSI-RS-based BFD: evaluation period for FR1 and FR2 We support Option 2. Follow the conclusion from Issue 4-2-1 |

### Sub-topic 4-3 CBD including L1-RSRP measurements

Issue 4-3-1: Whether relaxed L1-RSRP requirements applies to both intra- and inter-frequency

- Proposals

  - Option 1 (MTK): Clarify whether the issues of relative/absolute accuracy are related to intra-frequency and/or inter-frequency.

- Recommended WF

  - Companies to provide their view on whether the relaxed L1 RSRP measurement requirements for 1 Rx apply to intra-frequency measurements or both intra- and inter-frequency measurements.

Issue 4-3-2: SSB-based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1

- Proposals

  - Option 1 (Apple, vivo, HW, E///, MTK):

    - Relax the current absolute L1-RSRP accuracy by 3 dB when 1Rx is used compared with the legacy UE.

  - Option 2 (vivo):

    - relaxed by 2.5dB when 1Rx is used compared with the legacy UE.

- Recommended WF

  - Check if supporting company of option 2 can compromise to option 1.

Issue 4-3-3: SSB-based L1-RSRP measurements: relative accuracy with measurement restriction in FR1

- Proposals

  - Option 1 (Apple, HW, E///):

    - Relax the relative L1-RSRP accuracy by 3dB

  - Option 2 (MTK):

    - Relax the relative L1-RSRP accuracy by 2dB

- Recommended WF

  - Check if supporting company of option 2 can compromise to option 1.

Issue 4-3-4: SSB-based L1-RSRP measurements: absolute accuracy with measurement restriction in FR2

- Proposals

  - Option 1 (Apple, vivo):

    - Relax the current absolute accuracy by 2dB when 1Rx is used compared with the legacy UE.

  - Option 2 (vivo):

    - Relaxed by 3 dB when 1Rx is used compared with the legacy UE.

  - Option 3 (MTK): Further study needed.

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements.

Issue 4-3-5: SSB-based L1-RSRP measurements: relative accuracy with measurement restriction in FR2

- Proposals

  - Option 1 (Apple):

    - Relax the current relative accuracy by 2 dB

  - Option 2 (MTK): Further study needed.

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements.

Issue 4-3-6: CSI-RS based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1

- Proposals

  - Option 1 (Apple, vivo, HW, E///, MTK):

    - By 3 dB when 1Rx is used compared with the legacy UE.

- Recommended WF

  - Option 1 is recommended to be agreed.

Issue 4-3-7: CSI-RS based L1-RSRP measurements: relative accuracy with measurement restriction in FR1

- Proposals

  - Option 1 (Apple, HW):

    - Relax the relative L1-RSRP accuracy by 3dB

- Recommended WF

  - At last meeting it was already agreed to relax the relative accuracy by 3 dB for CSI-RS based L1-RSRP, see R4-2202670. Thus, no more discussions needed.

Issue 4-3-8: CSI-RS based L1-RSRP measurements: absolute accuracy with measurement restriction in FR2

- Proposals

  - Option 1 (Apple, vivo):

    - Relax the current absolute accuracy by 2dB

  - Option 2 (MTK): Further study needed.

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements.

Issue 4-3-9: CSI-RS based L1-RSRP measurements: relative accuracy with measurement restriction in FR2

- Proposals

  - Option 1 (Apple):

    - Relax the current relative accuracy by 2dB

  - Option 2 (MTK): Further study needed.

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements.

Issue 4-3-10: SSB-based CBD: evaluation period for FR1

- Proposals

  - Option 1 (Apple): No need to extend the evaluation period for SSB based CBD.

- Recommended WF

  - At last meeting it was agreed that evaluation period for SSB based CBD is unchanged for 1 Rx UE for FR1 compared to legacy requirements, see R4-2202670. Thus, no more discussions needed.

Issue 4-3-11: CSI-based CBD: evaluation period for FR1

- Proposals

  - Option 1 (Apple): No need to extend the evaluation period for CSI-RS based CBD

- Recommended WF

  - At last meeting it was agreed that evaluation period for CSI-RS based CBD is unchanged for 1 Rx UE for FR1 compared to legacy requirements, see R4-2202670. Thus, no more discussions needed.

Issue 4-3-12: SSB-based L1-RSRP in CSI reporting for FR1 without measurement restriction

Background: In legacy requirements 3 samples are used without measurement restriction, otherwise 1 sample.

- Proposals

  - Option 1 (Apple, vivo): For RedCap UE with 1Rx, SSB-based L1-RSRP measurement period can be unchanged.

- Recommended WF

  - Check if option 1 can be agreed.

Issue 4-3-13: CSI-based L1-RSRP in CSI reporting for FR1 without measurement restriction

- Proposals

  - Option 1 (Apple, vivo): For RedCap UE with 1Rx, CSI-RS-based L1-RSRP measurement period can be unchanged.

- Recommended WF

  - Check if option 1 can be agreed.

Sub topic 4-3

| Company | Comments |
| --- | --- |
| XXX | Issue 4-3-1: Whether relaxed L1-RSRP requirements applies to both intra- and inter-frequency Issue 4-3-2: SSB-based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 Issue 4-3-3: SSB-based L1-RSRP measurements: relative accuracy with measurement restriction in FR1 Issue 4-3-12: SSB-based L1-RSRP in CSI reporting for FR1 without measurement restriction Issue 4-3-13: CSI-based L1-RSRP in CSI reporting for FR1 without measurement restriction |
| Huawei | Issue 4-3-1: Whether relaxed L1-RSRP requirements applies to both intra- and inter-frequency Yes, herein the relative/absolute accuracy are related to intra-frequency and/or inter-frequency. Issue 4-3-2: SSB-based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 Option 1. Issue 4-3-3: SSB-based L1-RSRP measurements: relative accuracy with measurement restriction in FR1 Option 1. Issue 4-3-6: CSI-RS based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 Agree with the recommended WF. Issue 4-3-10: SSB-based CBD: evaluation period for FR1 Fine with the recommended WF. Issue 4-3-11: CSI-based CBD: evaluation period for FR1 Fine with the recommended WF. Issue 4-3-12: SSB-based L1-RSRP in CSI reporting for FR1 without measurement restriction Fine with Option 1. Issue 4-3-13: CSI-based L1-RSRP in CSI reporting for FR1 without measurement restriction Fine with Option 1. |
| Ericsson | Issue 4-3-1: Whether relaxed L1-RSRP requirements applies to both intra- and inter-frequency Our understanding is that the L1-RSRP requirements being discussed for 1 Rx UE for CBD applies to both intra- and inter-frequency measurements. Issue 4-3-12: SSB-based L1-RSRP in CSI reporting for FR1 without measurement restriction We support option 1. Given that accuracy levels are relaxed for 1 Rx UE, current measurement period can be reused. Issue 4-3-13: CSI-based L1-RSRP in CSI reporting for FR1 without measurement restriction We support option 1. Given that accuracy levels are relaxed for 1 Rx UE, current measurement period can be reused. |
| Apple | Issue 4-3-1: Whether relaxed L1-RSRP requirements applies to both intra- and inter-frequency Yes, the relaxation shall be applied to both intra- and inter-frequency cases. Issue 4-3-2: SSB-based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 Fine with recommended WF. Issue 4-3-3: SSB-based L1-RSRP measurements: relative accuracy with measurement restriction in FR1 Fine with recommended WF. Issue 4-3-6: CSI-RS based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 Fine with recommended WF. Issue 4-3-12: SSB-based L1-RSRP in CSI reporting for FR1 without measurement restriction Option 1 Issue 4-3-13: CSI-based L1-RSRP in CSI reporting for FR1 without measurement restriction Option 1 |
| MediaTek | Issue 4-3-2: SSB-based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 Support Option 1. Issue 4-3-3: SSB-based L1-RSRP measurements: relative accuracy with measurement restriction in FR1 Support recommended WF. Issue 4-3-6: CSI-RS based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 Support recommended WF. Issue 4-3-12: SSB-based L1-RSRP in CSI reporting for FR1 without measurement restriction Support recommended WF. Issue 4-3-13: CSI-based L1-RSRP in CSI reporting for FR1 without measurement restriction Support recommended WF. |
| Nokia | Issue 4-3-1: Whether relaxed L1-RSRP requirements applies to both intra- and inter-frequencyYes, our understanding is that the relaxed L1-RSRP requirements would apply to both intra and inter-frequency measurements. Issue 4-3-2: SSB-based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 We support Option 1. Issue 4-3-3: SSB-based L1-RSRP measurements: relative accuracy with measurement restriction in FR1 We support Option 1 Issue 4-3-6: CSI-RS based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 We support the recommended WF. Issue 4-3-12: SSB-based L1-RSRP in CSI reporting for FR1 without measurement restriction We support the recommended WF. Issue 4-3-13: CSI-based L1-RSRP in CSI reporting for FR1 without measurement restriction We support the recommended WF. |
| vivo | Issue 4-3-2: SSB-based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 Agree with the recommended WF. Issue 4-3-3: SSB-based L1-RSRP measurements: relative accuracy with measurement restriction in FR1 Agree with the recommended WF. Issue 4-3-6: CSI-RS based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 Agree with the recommended WF. Issue 4-3-12: SSB-based L1-RSRP in CSI reporting for FR1 without measurement restriction Support Option 1. Issue 4-3-13: CSI-based L1-RSRP in CSI reporting for FR1 without measurement restriction Support Option 1. |
| Qualcomm | Issue 4-3-1: Whether relaxed L1-RSRP requirements applies to both intra- and inter-frequency Yes, the requirements should apply to both intra-frequency and inter-frequency requirements Issue 4-3-2: SSB-based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 We think the legacy requirements were defined with AWGN channel. If that’s the case, then why do we need to consider the relaxations for fading channel. Note that the test cases are defined with AWGN channels only. If you look at the following note in Table 10.1.19.1.1-1 NOTE 1: Io is assumed to have constant EPRE across the bandwidth. Fading channels won’t have constant EPRE across bandwidth, and hence they should be precluded. We think the accuracy requirements should be defined with AWGN channels We propose to relax the accuracy requirements by +-1db as was done in LTE CAT1-bis requirements. Issue 4-3-3: SSB-based L1-RSRP measurements: relative accuracy with measurement restriction in FR1 Same comment as in Issue 4-3-2 Issue 4-3-6: CSI-RS based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 Same comment as in Issue 4-3-2 Issue 4-3-12: SSB-based L1-RSRP in CSI reporting for FR1 without measurement restriction Fine with Option 1. Issue 4-3-13: CSI-based L1-RSRP in CSI reporting for FR1 without measurement restriction Fine with Option 1. |

### Sub-topic 4-4 BWP switching

Issue 4-4-1: Whether to introduce new UE capability to indicate support of new BWP switching delay when only center-frequency is changed

- Proposals

  - Option 1 (CMCC):  RAN4 to define a new UE capability to indicate the support of new BWP switching delay when only center-frequency is changed.

- Recommended WF

  - Discuss the option.

Issue 4-4-2: Whether to define BWP switching delay when only center-frequency is changed in Rel-17

- Proposals

  - Option 1 (CMCC, E///):  Define BWP switching delay between non-initial DL BWP for RedCap and initial BWP for RedCap involving only changing of the center-frequency of the BWP.

  - Option 2 (ZTE, Xiaomi, vivo, QC, OPPO): Reuse the legacy BWP switching delay when only center frequency is changed in release 17.

- Recommended WF

  - Discuss the options.

Issue 4-4-3: If new BWP switching delay when only center-frequency is changed is introduced, how to express those:

- Proposals

  - Option 1 (CMCC, E///):  Define BWP switching delay between non-initial DL BWP for RedCap and initial BWP for RedCap involving only changing of the center-frequency of the BWP based on the on previous agreement in R4-1803283:

| Frequency Range | Type 1 Delay (us) | Type 2 Delay (us) |
| --- | --- | --- |
| 1 | 200 | 1050 |
| 2 | 200 | 1050 |

In terms of slots, the delay is expressed as:

| SCS | Slot length (ms) | Type 1 Delay (slots) | Type 2 Delay (slots) |
| --- | --- | --- | --- |
| 15 kHz | 1 | 1 | 2 |
| 30 kHz | 0.5 | 1 | 3 |
| 60 kHz | 0.25 | 1 | 5 |
| 120 kHz | 0.125 | 2 | 9 |

- Recommended WF

  - Discuss the option.

Issue 4-4-4: If reduced BWP switching delay when only center-frequency is changed, scheduling restriction:

- Proposals

  - Option 1 (E///):  The UE is not required to transmit UL signals or receive DL signals while tuning between initial BWP and the RedCap specific non-initial BWP provided that the DRX cycle is less than 640 ms. No scheduling restriction is allowed for DRX cycle of 640 ms or longer. Recommended WF

- Recommended WF

  - Discuss the option.

Issue 4-4-5: Whether to introduce L1 measurement gaps for performing receiving SSB outside active BWP in Rel-17

- Proposals

  - Option 1 (CMCC, E///, OPPO): Do not introduce L1 measurement gaps for acquiring SSB outside active BWP.

- Recommended WF

  - Option 1 is recommended to be agreed.

Sub topic 4-4

| Company | Comments |
| --- | --- |
| XXX | Issue 4-4-1: Whether to introduce new UE capability to indicate support of new BWP switching delay when only center-frequency is changed Issue 4-4-2: Whether to define BWP switching delay when only center-frequency is changed in Rel-17 Issue 4-4-3: If new BWP switching delay when only center-frequency is changed is introduced, how to express those: Issue 4-4-4: If reduced BWP switching delay when only center-frequency is changed, scheduling restriction: Issue 4-4-5: Whether to introduce L1 measurement gaps for performing receiving SSB outside active BWP in Rel-17 |
| Huawei | Issue 4-4-5: Whether to introduce L1 measurement gaps for performing receiving SSB outside active BWP in Rel-17 Support option 1. UE can perform L1-measurement based on CSI-RS or retuning RF if UE supporting FG 6-1a. There is no need to define L1 measurement gap. |
| Ericsson | Issue 4-4-1: Whether to introduce new UE capability to indicate support of new BWP switching delay when only center-frequency is changed The “new BWP switching delay when only center-frequency is changed” is a UE requirement which support by supported by RedCap capable UE. In our view requirements should be mandatory for UE supporting the related feature i.e. RedCap in this case. We do not see the need to define new UE capability to indicate support of new BWP switching delay when only center-frequency is changed. Issue 4-4-2: Whether to define BWP switching delay when only center-frequency is changed in Rel-17 RAN4 should define the requirements new BWP switching delay when only center-frequency is changed. In RedCap, the BWP switching when only center-frequency is changed will be very common case because other parameters like MIMO layers, SCS are not likely to change due to BWP switching. As also explained in our paper in R4-2205630, RAN4 can reuse the previous agreement in R4-1803283 and thus effort is not significant. We therefore support Option 1. Issue 4-4-3: If new BWP switching delay when only center-frequency is changed is introduced, how to express those: We support option 1. The BWP switching delay between non-initial DL BWP for RedCap and initial BWP for RedCap involving only changes in the center-frequency of the BWP can be based on the on previous agreement in R4-1803283. We prefer that the new BWP switching delay is expressed in slots like in legacy requirements. Issue 4-4-4: If reduced BWP switching delay when only center-frequency is changed, scheduling restriction: We support option 1. If the UE autonomously switches between initial BWP and non-initial BWP then in case of longer DRX cycle (e.g. 640 ms or longer) there should be no scheduling restriction since the UE can switch between them during the OFF duration of the DRX cycle. But scheduling restriction is needed for the shorter DRX cycles e.g. < 640 ms. Issue 4-4-5: Whether to introduce L1 measurement gaps for performing receiving SSB outside active BWP in Rel-17 We support option 1. Following Rel-15 approach, the RLM/BFD/CBD/L1-RSRP requirements are applicable if the SSB is within the active BWP. The same approach should apply for RedCap. Therefore, for RedCap the RLM/BFD/CBD/L1-RSRP requirements are applicable if the SSB is within the active BWP i.e. if NCD-SSB is within RedCap BWP. In summary there is no need for any L1 measurement gaps for RLM/BFD/CBD/L1-RSRP measurements. |
| Apple | Issue 4-4-1: Whether to introduce new UE capability to indicate support of new BWP switching delay when only center-frequency is changed We cannot agree on option 1. We already have type 1 and type2 for BWP switching and don’t see necessity to have a new one for RedCap UE. Issue 4-4-2: Whether to define BWP switching delay when only center-frequency is changed in Rel-17 Support option 2. Issue 4-4-5: Whether to introduce L1 measurement gaps for performing receiving SSB outside active BWP in Rel-17 We don’t have such L1 MG for legacy UE L1 measurement from R15 to R17 and don’t need to discuss it for RedCap UE. Fine with recommended WF. |
| CMCC | Issue 4-4-1: Whether to introduce new UE capability to indicate support of new BWP switching delay when only center-frequency is changed Define UE capability is a comproise solution we provide in order to resolve the concern from UE vendors. We are also fine if this is mandatory. Issue 4-4-2: Whether to define BWP switching delay when only center-frequency is changed in Rel-17 Support option1. We already explained the reasons many times. The BWP switching delay came from previous RAN4 agreement. Issue 4-4-3: If new BWP switching delay when only center-frequency is changed is introduced, how to express those: Support option 1 Issue 4-4-5: Whether to introduce L1 measurement gaps for performing receiving SSB outside active BWP in Rel-17 Option1 |
| MediaTek | Issue 4-4-1: Whether to introduce new UE capability to indicate support of new BWP switching delay when only center-frequency is changed We don’t support this issue. However, it can wait for outcome from issue 4-4-2. Issue 4-4-2: Whether to define BWP switching delay when only center-frequency is changed in Rel-17 From [R4-1803283], Scenario 1: The reconfiguration involves changing the center frequency of the BWP without changing its BW. The reconfiguration may or may not involve changing the SCS. Scenario 2: The reconfiguration involves changing the BW of the BWP without changing its center frequency. The reconfiguration may or may not involve changing the SCS. Scenario 3: The reconfiguration involves changing both the BW and the center frequency of the BWP. The reconfiguration may or may not involve changing the SCS. Scenario 4: The reconfiguration involves changing only the SCS, where the center frequency and BW of the BWP remain unchanged. Table 1: BWP switching delay parameters Frequency Range; Scenario; Type 1 Delay (us); Type 2 Delay (us). 1; 1; 600; 2000. 2; 600; 2000. 3; 600; 2000. 4; 400; 950. Now, given that this issue is to whether to define a new BWP switching delay when only centre frequency is changed, hence we compare the delay from scenario 1 and scenario 3. From which, it is clear that when only the centre frequency is changed the delay is equal to that with changing centre frequency and BW. Thus, we support Option 2: Reuse the legacy BWP switching delay when only center frequency is changed in release 17. Furthermore, in CMCC contribution paper [R4-2204995] the proposal is based on scenario 4, however, it is clear from scenario 4 description it is clear that the center frequency remains unchanged. Thus, we don’t think their proposal is valid. Please clarify if there is misunderstanding or typo in CMCC contribution paper?? In addition, this issue has been open for at least 3 meeting and no new technical arguments have been provided in the proponent contribution papers, hence we suggest that the proponent companies to compromise to option 2. Issue 4-4-3: If new BWP switching delay when only center-frequency is changed is introduced, how to express those: We don’t support this issue. However, it can wait for outcome from issue 4-4-2. Issue 4-4-4: If reduced BWP switching delay when only center-frequency is changed, scheduling restriction: We don’t support this issue. However, it can wait for outcome from issue 4-4-2. Issue 4-4-5: Whether to introduce L1 measurement gaps for performing receiving SSB outside active BWP in Rel-17 Support recommended WF. |
| OPPO | Issue 4-4-1: Whether to introduce new UE capability to indicate support of new BWP switching delay when only center-frequency is changed Pending on issue 4-4-2. Issue 4-4-2: Whether to define BWP switching delay when only center-frequency is changed in Rel-17 Support option 2. There may exist some UE implementation which could enable faster BWP switching than type 2 but less than type 1 and option 1 was our previous preference. Due to the limited time, we suggest to keep BWP switching delay requirements as the legacy. Open to further discuss in later release for optimization. Issue 4-4-5: Whether to introduce L1 measurement gaps for performing receiving SSB outside active BWP in Rel-17 Fine with recommended WF. |
| Xiaomi | Issue 4-4-1: Whether to introduce new UE capability to indicate support of new BWP switching delay when only center-frequency is changed Wait for outcome from issue 4-4-2. Issue 4-4-2: Whether to define BWP switching delay when only center-frequency is changed in Rel-17 Support option 2. Issue 4-4-5: Whether to introduce L1 measurement gaps for performing receiving SSB outside active BWP in Rel-17 Fine with recommended WF. |
| ZTE | Issue 4-4-2: Whether to define BWP switching delay when only center-frequency is changed in Rel-17 Option 2. We think that this is a corner case and should be considered as enhancements. What’s more, the scenario can also happen for legacy UEs, but we don’t have any enhancements for them. |
| Nokia | Issue 4-4-5: Whether to introduce L1 measurement gaps for performing receiving SSB outside active BWP in Rel-17 Option 1. |
| vivo | Issue 4-4-1: Whether to introduce new UE capability to indicate support of new BWP switching delay when only center-frequency is changed We don’t support option 1. There already have two types BWP switching delay and no need to introduce a new one. Issue 4-4-2: Whether to define BWP switching delay when only center-frequency is changed in Rel-17 Option 2. Issue 4-4-5: Whether to introduce L1 measurement gaps for performing receiving SSB outside active BWP in Rel-17 Fine with recommended WF. |
| Qualcomm | Issue 4-4-2: Whether to define BWP switching delay when only center-frequency is changed in Rel-17 Support option 2. We have already commented that there is no use case when only centre frequency changes during the BWP switch. There is no clear motivation and this issue must be closed. |

### Sub-topic 4-5 Active TCI state switching and UL spatial relation switch delay

Issue 4-5-1: Active TCI state switching

- Proposals

  - Option 1 (CMCC):	For Rel-17 TCI state switch delay requirements for Redcap:

    - For MAC-CE based TCI state switch delay with target TCI unknown: outcome of L1-RSRP delay requirements from CBD evaluation requirements is reused.

    - For RRC-based TCI state switch delay with target TCI unknown: outcome of L1-RSRP delay requirements from CBD evaluation requirements is reused.

  - Option 2 (vivo, E///):

    - The MAC-CE based TCI state switch requirements in Rel-15 are reused for Rel-17 1 Rx Redcap UE when the TCI state in unknown.

    - the RRC based TCI state switch requirements in Rel-15 are reused for Rel-17 1 Rx Redcap UE when the TCI state is unknown.

- Recommended WF

Discuss the options.

Issue 4-5-2: UL spatial relation switch delay

- Proposals

  - Option 1 (vivo, E///): Support to reuse the legacy requirements for MAC-CE and RRC-based spatial relation switch delay with target spatial relation associated with DL RS is unknown when 1Rx is used.

  - Option 2 (CMCC):

    - For UL spatial delay switch requirements for RedCap UE with 1 Rx:

      - For MAC-CE based spatial relation switch delay with target spatial relation associated with DL RS is unknown: outcome of L1-RSRP delay requirements from CBD evaluation requirements is reused.

      - For RRC-based spatial switch delay with target spatial relation associated to DL RS is unknown: outcome of L1-RSRP delay requirements from CBD evaluation requirements is reused.

- Recommended WF

  - Discuss the options.

Sub topic 4-5

| Company | Comments |
| --- | --- |
| XXX | Issue 4-5-1: Active TCI state switching Issue 4-5-2: UL spatial relation switch delay |
| Huawei | Issue 4-5-1: Active TCI state switching It is agreed in previous meeting, SSB-based L1-RSRP: measurement period without measurement restriction For RedCap UE with 1RX SSB-based L1-RSRP measurement period can be unchanged for FR1 and FFS for FR2. CSI-RS based L1-RSRP (excluding 60 kHz SCS): measurement period without measurement restriction For RedCap UE with 1RX CSI-RS based L1-RSRP measurement period can be unchanged for FR1 and FFS for FR2. Therefore option 1 and option 2 seem the same. Issue 4-5-2: UL spatial relation switch delay Same comments in issue 4-5-1. |
| Ericsson | Issue 4-5-1: Active TCI state switching In our view, option 1 and option 2 are similar. Since we support the proposal of reusing the existing measurement period for L1 RSRP for CBD, our view is that legacy TCI state switching requirements can be reused. Obviously, it depends on the outcome of issue 4-3-12 and 4-3-13. Issue 4-5-2: UL spatial relation switch delay In our view, option 1 and option 2 are similar. Since we support the proposal of reusing the existing measurement period for L1 RSRP for CBD, our view is that the legacy TCI state switching requirements can be reused. Obviously, it depends on the outcome of issue 4-3-12 and 4-3-13. |
| Apple | Issue 4-5-1: Active TCI state switching Option 1 and option 2 are same since L1-RSRP measurement period is unchanged for 1Rx case. Issue 4-5-2: UL spatial relation switch delay UL spatial relation is only used for FR2, and number of Rx branches is not reduced for FR2 based on RF conclusion. Thus, no need to discuss this issue. |
| CMCC | Issue 4-5-1: Active TCI state switching Issue 4-5-2: UL spatial relation switch delay Option 1 and 2 are the same, we can discuss the CR directly. No further discussion is needed. |
| Nokia | Issue 4-5-1: Active TCI state switching We agree with Huawei and Apple. Both options lead to the same requirement. Issue 4-5-2: UL spatial relation switch delay Same comments as in the issue above. |
| vivo | Issue 4-5-1: Active TCI state switching Support Option 1. L1-RSRP measurement period is unchanged for 1Rx case. Issue 4-5-2: UL spatial relation switch delay Support Option 1. L1-RSRP measurement period is unchanged for 1Rx case. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204996 (CMCC) | Title: Draft CR to 38.133 for introducing RedCap requirements on active BWP switch delay, active TCI state switching delay and UE specific CBW change |
|  | Ericsson sPCell should be replaced with PCell since sPCell term is used when UE supports DC. As commented on issue 4-4-3, we prefer to express BWP switching delay in slots. Following sentence says MR-DC: “The requirements in this clause apply for a RedCap UE configured with one or more TCI state configurations on serving cell in MR-DC or standalone”. Since there is no support for RedCap, this text needs to be updated. |
| R4-2206085 (MediaTek Inc.) | Title: DraftCR on reduced capability Ues for RLM for RedCap |
|  | Ericsson: - No track changes. - For PDCCH parameters, it is better to add ‘Note: SCS=60kHz is not applicable for FR1’ in Table 8.1B.2.1-1, Table 8.1B.2.1-2, Table 8.1B.3.1-1, and Table 8.1B.3.1-2 - For PDCCH parameters, aggregation level and BW should not be affected by ‘antenna port is used for cell-specific reference signal transmission by the PCell’, We suggest to specify like ‘16 for 1Rx capable UE; 8 for 2Rx capable UE’ - RLM evaluation period for 1Rx may need to be updated according to the conclusion in 4-1. It is good to have the separate table between 2Rx UE and 1Rx UE. - According to the conclusion FR2 1Rx table (e.g. Table 8.1B.2.2-4:) should be removed. |
| R4-2204799 (vivo) | Title: Draft CR for Link Recovery Procedures for Redcaps |
|  | Ericsson - For PDCCH parameters, it is better to add ‘Note: SCS=60kHz is not applicable for FR1’ in Table 8.5B.2.1-1, Table 8.5B.3.1-1 - BFD evaluation period for 1Rx may need to be updated according to the conclusion on 4-2. It is good to have the separate table between 2Rx UE and 1Rx UE. Also note that there are no requirements for 1 Rx FR2. For FR2 2 Rx, we could simply refer to existing requirements. - we have to add conditions for HD-FDD as agreed in the WF. - in the title, it is good have RedCap, e.g. “Link Recovery Procedures for RedCap” to be aligned with the rest. - ‘.’ is missing at the end of the very first sentence. - better to use Qout_LR_RedCap instead of Qout_LR . We should follow same approach for all other parameters in the same section. For example for Qin parameter, Tevaluate parameters etc. - in 8.5B.2 title, add “for RedCap”, and same comment applies to all other clauses. - Following sentence need to be removed since there is only one cell for RedCap: “The requirements in this clause could not be applicable if UE is required to perform beam failure detection on more than 1 serving cell per band.” Also similar statement referring to more than 1 cell needs to be removed in entire CR. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic#4-1 | Issue 4-1-2: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qout evaluation period for FR1 and FR2 Tentative agreements: The lower bound for SSB-based and CSI-RS based RLM Qout evaluation period for FR1 is extended by factor 2. Issue 4-1-3: How much to extend the lower bound for SSB-based and CSI-RS based RLM Qin evaluation period for FR1 and FR2 Background: At last meeting it was agreed at last meeting to extend the lower bound in RLM evaluation period, see R4-2202670. Under this issue it is discuss how much to relax. - Option 1 (E///, HW, Apple, MTK, Nokia, vivo, QC): By factor 1 Tentative agreements: The lower bound for SSB-based and CSI-RS based RLM Qin evaluation period for FR1 is not extended. |
| Sub-topic#4-2 | Issue 4-2-1: SSB-based based BFD: evaluation period for FR1 and FR2 - Option 1 (HW, E///, OPPO, Nokia, vivo): No need to extend the evaluation period for BFD compared to legacy evaluation period. - Option 2 (Apple, MTK, QC): Set SSB based BFD evaluation period to be extended by a factor of 2 (10 samples) for 1Rx UE. Recommendations for 2nd round: Continue the discussions. Please take into the account the comments from HW and Ericsson regarding the SNR levels and number of samples. - “The side condition for BFD Qout is observed at -8dB which is higher than Qout for RLM (note: Qout for RLM is observed @-10dB). At higher side condition, the evaluation samples for BFD Qout for RedCap with 1RX can be the same as legacy.” - “Rel-15 beam failure detection requirements assume the same number of SSB/CSI-RS samples as RLM In-sync to define evaluation period, that is, 5 samples for SSB based BFD and 10 samples for CSI-RS based BFD (density=3, 24RBs). At last meeting it was agreed to not the RLM in-sync evaluation period. Therefore, the same should apply also for BFD.” Issue 4-2-2: CSI-RS-based BFD: evaluation period for FR1 and FR2 - Option 1 (HW, E///): No need to extend the evaluation period compared to legacy evaluation period. - Option 2 (Apple): Set CSI-RS based BFD evaluation period based on 20 samples for 1Rx UE. Tentative agreements: Follow the corresponding agreement from SSB based BFD evaluation period. |
| Sub-topic#4-3 | Issue 4-3-1: Whether relaxed L1-RSRP requirements applies to both intra- and inter-frequency Tentative agreements: The relaxed L1-RSRP requirements (absolute and relative accuracy) apply to both intra- and inter-frequency measurements. Issue 4-3-2: SSB-based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 - Option 1 (Apple, vivo, HW, E///, MTK, Nokia): - Relax the current absolute L1-RSRP accuracy by 3 dB when 1Rx is used compared with the legacy UE. Recommended for 2nd round: To confirm following agreement: [For SSB-based L1-RSRP measurements, the absolute accuracy with measurement restriction in FR1: Relax the current absolute L1-RSRP accuracy by 3 dB when 1Rx is used compared with the legacy UE. ] Issue 4-3-3: SSB-based L1-RSRP measurements: relative accuracy with measurement restriction in FR1 - Option 1 (Apple, HW, E///, MTK, Nokia, vivo): - Relax the relative L1-RSRP accuracy by 3dB Recommended for 2nd round: To confirm following agreement: [For SSB-based L1-RSRP measurements, the relative accuracy with measurement restriction in FR1: Relax the relative L1-RSRP accuracy by 3dB when 1Rx is used compared with the legacy UE. ] Issue 4-3-6: CSI-RS based L1-RSRP measurements: absolute accuracy with measurement restriction in FR1 - Option 1 (Apple, vivo, HW, E///, MTK, Nokia): - By 3 dB when 1Rx is used compared with the legacy UE. Recommended for 2nd round: To confirm following agreement: [For CSI-RS based L1-RSRP measurements, the absolute accuracy with measurement restriction in FR1: Relax the relative L1-RSRP accuracy by 3dB when 1Rx is used compared with the legacy UE. ] Issue 4-3-12: SSB-based L1-RSRP in CSI reporting for FR1 without measurement restriction - Option 1 (Apple, vivo, HW, E///, MTK, Nokia, QC): For RedCap UE with 1Rx, SSB-based L1-RSRP measurement period can be unchanged. Tentative agreements: For RedCap UE with 1Rx, SSB-based L1-RSRP measurement period can be unchanged compared to legacy Release 15 NR UE requirements. Issue 4-3-13: CSI-based L1-RSRP in CSI reporting for FR1 without measurement restriction - Option 1 (Apple, vivo, HW, E///, MTK, Nokia, QC): For RedCap UE with 1Rx, CSI-RS-based L1-RSRP measurement period can be unchanged. Tentative agreements: For RedCap UE with 1Rx, CSI-RS-based L1-RSRP measurement period can be unchanged compared to legacy Release 15 NR UE requirements |
| Sub-topic#4-4 | Issue 4-4-1: Whether to introduce new UE capability to indicate support of new BWP switching delay when only center-frequency is changed - Option 1 (CMCC): RAN4 to define a new UE capability to indicate the support of new BWP switching delay when only center-frequency is changed. - Option 2 (E///, CMCC): The “new BWP switching delay when only the center-frequency is changed” is a requirements for RedCap UE. - Option 3 (Apple, MTK, vivo): No capability to be introduced. Tentative agreements: No consensus to introduce new UE capability to indicate the support of new BWP switching delay when only center-frequency is changed. Issue 4-4-2: Whether to define BWP switching delay when only center-frequency is changed in Rel-17 - Option 1 (CMCC, E///): Define BWP switching delay between non-initial DL BWP for RedCap and initial BWP for RedCap involving only changing of the center-frequency of the BWP. - Option 2 (ZTE, Xiaomi, vivo, QC, OPPO, MTK): Reuse the legacy BWP switching delay when only center frequency is changed in release 17. Tentative agreements: No consensus to define BWP switching delay when only center-frequency is changed in Rel-17 for RedCap. Issue 4-4-5: Whether to introduce L1 measurement gaps for performing receiving SSB outside active BWP in Rel-17 Tentative agreements: RAN4 agrees to not introduce L1 measurement gaps for acquiring SSB outside active BWP for RedCap UE in Release 17. |
| Sub-topic#4-5 | Issue 4-5-1: Active TCI state switching Recommendation for 2nd round: No further discussions needed. Provide feedback directly to the CR. Issue 4-5-2: UL spatial relation switch delay Recommendation for 2nd round: No further discussions needed. Provide feedback directly to the CR. |

## Discussion on 2nd round (if applicable)

Companies are encouraged to provide their view on the issues recommended for 2nd round discussions in 4.3.1.

Topic #4

| Company | Comments |
| --- | --- |
| Company X | Issue x-y |

# Topic #5: Measurement procedure

Contributions from AI 10.20.3.1.5 are discussed here.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2204249 | Xiaomi | Proposal 4: RRM measurements on serving cell and neighbour cell can be based on NCD-SSB in connected mode. Proposal 5: The reference SSB for UE to perform intra-frequency measurement should be the SSB configured in the active BWP, which could be either CD-SSB or NCD-SSB. Proposal 6: The legacy definition of intra-frequency and inter-frequency based on the reference SSB could be reused. |
| R4-2206078 | MediaTek inc. | Proposal 4: Support introducing scheduling availability restriction on 5G NR RedCap UEs performing measurements in HD-FDD bands in CONNECTED mode. Proposal 5: Support clarifying that the scheduling restriction is for intra-frequency and inter-frequency measurements in CONNECTED mode in HD-FDD bands. |
| R4-2204283 | OPPO | Proposal 1: RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability in Rel-17. Proposal 2: CSSFoutside_gap,i = 1 for RedCap UE measurement outside gap based on Rel-15 requirement. Proposal 3: The current design for CSSF within gap could be reused for RedCap UE. Proposal 4: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in cell identification and measurement requirement. |
| R4-2203587 | ZTE Corporation | Proposal 1: RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability. Proposal 2: The searcher will be exclusively used by intra-frequency without gap measurement provided that RAN4 agrees that RedCap UE does NOT support ‘Inter-frequency without gap’ measurement capability in Rel-17. Proposal 3: CSSFoutside_gap,i = 1 for RedCap UE measurement outside gap based on Rel-15 requirement. Proposal 4: The current design for CSSF within gap could be reused for RedCap UE. Proposal 5: Keep measurement period same as Rel-15. |
| R4-2203789 | Apple | Proposal 1: RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability. Proposal 2: CSSFoutside_gap,i = 1 for RedCap UE measurement outside gap based on Rel-15 requirement. Proposal 3: The current design for CSSF within gap could be reused for RedCap UE. Proposal 4: the baseline intra-frequency PSS/SSS detection requirement without DRX shall be defined: • In FR1, extend the PSS/SSS detection delay by 2 SMTC without changing the lower boundary ‘600ms’ - non-DRX delay requirement: max( 600ms, ceil( 7 x Kp) x SMTC period ) x CSSFintra Proposal 5: the baseline FR1 intra-frequency time index detection requirement without DRX shall be defined: • In FR1, extend the time index detection delay by 1 SMTC with changing the lower boundary to ‘160 ms’ - non-DRX delay requirement: max(160ms, ceil( 4 x Kp ) x SMTC period)x CSSFintra Proposal 6: the baseline SSB based intra-frequency RSRP measurement requirement without DRX shall be defined: - Extend the lower bound of measurement delay to 400ms for FR1 and 800ms for FR2 for longer duty cycle (like in LTE cat1-bis) without increasing the sample number - Delay is max(400ms, ceil( 5 x Kp) x SMTC period) x CSSFintra for FR1 - Delay is max(800ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period) x CSSFintra for FR2 Proposal 7: the baseline SSB based intra-frequency RSRP measurement accuracy requirement shall be defined: - For FR1 RedCap with 1Rx - Relax the current absolute and relative RSRP accuracy by 1dB - For FR2 RedCap with 1Rx - Relax the current absolute and relative accuracy by 1dB Proposal 8: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in cell identification and measurement requirement. |
| R4-2204252 | Xiaomi | - Proposal 1: RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability in Rel-17. Proposal 2: CSSFoutside_gap,i = 1 for RedCap UE measurement outside gap based on Rel-15 requirement. Proposal 3: The current design for CSSF within gap could be reused for RedCap UE. Proposal 4: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in cell identification and measurement requirement. |
| R4-2204997 | CMCC | Proposal 1: RAN4 needs to consider ‘inter-frequency without MG’ capability when define RedCap RRM requirements. Proposal 2: The searcher is shared by intra-frequency without gap and inter-frequency without gap measurement for RedCap UE: CSSFoutside_gap,I for PCC=2 if configured inter-frequency MOs without MG when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap that are being measured outside of MG for RedCap UE; otherwise, CSSFoutside_gap,I for PCC=1 CSSFoutside_gap,i for inter-frequency MO with no measurement gap=2*Y, Y is the number of configured inter-frequency MOs without MG when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap that are being measured outside of MG for RedCap UE; otherwise, it is 0. Proposal 3: The current design for CSSF within gap is reused for RedCap UE. Consider to add additional two values of measGapSharingScheme factor for RedCap UE, e.g. 85%, 95%. Proposal 5: For HD-FDD RedCap UE, no measurement period relaxation or prioritization between measurement and dynamically scheduled UL transmission are needed. Clarification on available samples can be considered: - “The UE shall meet the current requirements on cell identification (PSS/SSS detection defined in Table x.y.z for FR1 and FR2) provided that at least 5 SMTC windows are available at the UE during cell identification time.” - “The UE shall meet the current requirements on time index detection (defined in Table x.y.z for FR1 and FR2) provided that at least 3 SMTC windows are available at the UE during time index detection.” - “The UE shall meet the current requirements intra-frequency/inter-frequency measurement (defined in Table x.y.z for FR1 and FR2) provided that at least 5 SMTC windows are available at the UE during measurement period.” |
| R4-2206087 | MediaTek Inc. | DraftCR on reduced capability Ues for general measurements and intra-frequency |
| R4-2206088 | MediaTek Inc. | DraftCR on reduced capability Ues for inter-frequency and inter-RAT measurements |
| R4-2204537 | Nokia, Nokia Shanghai Bell | - Draft CR - Introducing L1-RSRP requirements for RedCap UEs |
| R4-2205511 | Ericsson | - draftCR on inter-RAT NR measurement for RedCap |
| R4-2205938 | Nokia, Nokia Shanghai Bell | - DraftCR: Introduction of RedCap UE in clause 9.11A |
| R4-2204324 | vivo | Proposal 1: For the issue “Inter-frequency without gap”, prefer that RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability in Rel-17, i.e., option 1. Proposal 2: For the issue “CSSF outside gap”, prefer that CSSFoutside_gap,i = 1 for RedCap UE measurement outside gap based on Rel-15 requirement, i.e., option 3; For the issue “CSSF within gap”, prefer that the current design for CSSF within gap could be reused for RedCap UE, i.e., option 2. Proposal 3: One sample need to be increased for PSS/SSS detection when 1Rx is used for FR1. Proposal 4: There is no need to extend the lower bound in PSS/SSS detection delay for FR1 and FR2. Proposal 5: One sample need to be increased for time index detection when 1Rx is used for FR1, i.e., total 4 samples are needed for 1Rx for FR1. Proposal 6: Three samples need to be increased for PSS/SSS detection when 1Rx is used for FR2, i.e., total 8 samples are needed for 1Rx for FR2. Proposal 7: The RSRP accuracy for SSB based measurement for FR1 and FR2 needs to be relaxed about 1.5dB when 1Rx is used. Proposal 8: There is no need to extend the lower bound for SSB based measurement when 1Rx is used. Proposal 9: The legacy RF margin can be reused for RedCap for FR1 when 1Rx is used. Proposal 10: For the priority between UL and DL during cell identification for HD-FDD, support option 1, i.e., RRM DL measurement is prioritized over the UL transmission of HD-FDD for Redcap UE in cell identification and measurement requirement. |
| R4-2204538 | Nokia, Nokia Shanghai Bell | Proposal 1: Time period for PSS/SSS detection, TPSS/SSS_sync_intra and TPSS/SSS_sync_inter, for RedCap UEs with 1 RX is extended by 1 sample in FR1. Proposal 2: Do not extend the lower bound for PSS/SSS detection delay in FR1 and FR2 Proposal 3: Define the time index detection delay in FR1 for RedCap UEs based on the side condition of -6dB Proposal 4: Define the time index detection delay in FR1 for RedCap UEs for the worst-case scenario (within the proposed channel profiles) Proposal 5: Extend the time index detection delay in FR1 by 2 samples at for RedCap UE with 1 RX Proposal 6: For the requirements of SSB based measurements for RedCap UEs with 1 RX, keep the same measurement period as in Rel-15. Proposal 7: For the requirements of SSB based measurements for RedCap UEs with 1 RX, do not extend the lower bound. Proposal 8: According to our simulation results, the accuracy degradation due to reducing the number of RX branches is up to 0.74 dB in FR1 and 0.70 dB in FR2. Proposal 9: For SSB-based RRM measurements, relax the accuracy level by 1 dB in both FR1 and FR2 for RedCap UEs with 1 RX. |
| R4-2204908 | Huawei, Hisilicon | Proposal 2: Support RRM measurement on serving cell based on NCD-SSB in connected state. Proposal 3: For RedCap UE, RRM measurement on neighbour cell is supposed to be performed on CD-SSB in connected mode. Proposal 4: The intra-frequency measurement definition for RedCap UE can be specified as, “A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the CD-SSB of the serving cell indicated for measurement and the centre frequency of the CD-SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same.” Proposal 5: RedCap UE needs to consider “inter-frequency without gap” when defining CSSF outside gap. Proposal 6: For RedCap UE CSSFoutside_gap,i= A+B, where A is the intra-frequency without gap and B is the number of configured inter-frequency MOs without MG. Proposal 7: For intra-frequency PSS/SSS detection for RedCap UE with 1RX, at least 6 samples are needed for FR1. Proposal 8: Not to extend the lower bound in PSS/SSS detection delay. Proposal 9: For time index acquisition for RedCap UE with 1RX, - 6 samples are needed in FR1; - 11 samples (without considering scaling factor due to RX sweeping) are needed in FR2 (for inter-frequency measurement). Proposal 10: PBCH decoding delay for CGI reading for RedCap UE: -The MIB decoding delay requirement in FR1 can be the same as non-RedCap UE for SNR=-3Db; -The MIB decoding delay requirement in FR2 shall be increased to 5 samples (without considering RX beam sweeping); Proposal 11: For RedCap UE with 1RX, RRM measurement period can be unchanged and RRM measurement accuracy is relaxed by 1dB. Proposal 12: Measurement period relaxation purely due to HD-FDD is not considered and some clarification on available samples can be made. Proposal 13: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in cell identification and measurement requirement. |
| R4-2206113 | Qualcomm Incorporated | Proposal 1a: At least for FR1, considering a duty cycle of 50%, prefer to extend the PSS/SSS detection period to a total of 10 samples (2-3 for AGC and 8 samples for detection) Proposal 1b: At least for FR1, extend the lower bound for PSS/SSS detection period by a factor of X - FFS: X=2, i.e., increase the lower bound from 600ms to 1200ms. Proposal 2: For FR1, specify SSB time index identification requirements based on 7 SMTC periods for 1 Rx UEs Proposal 3: For FR1, relax the measurement accuracy requirements for SSB based L3 measurements by +/- 1.5db. |
| R4-2206082 | MediaTek inc. | Proposal 1: We support the following: For Inter-frequency without gap: support option 1. For Assumption on searcher: support option 2. For CSSF outside gap: support option 3. For CSSF within gap: support option 1 and option 2. For Type of measurement gaps: support option 1. Proposal 2: We support the following: For If number of attempts are increased, how much to increase for FR1: support option 2. For Whether to extend the lower bound in PSS/SSS detection delay: support option 1. Proposal 3: We support the following: Method for defining 1 Rx requirements for SSB based measurement, FR1 and FR2: support option 1. For How much to relax? FR1 and FR2: support option 3 (1.5~2dB). For If measurement period is extended, how much to extend? FR1 and FR2: need justification. For Whether legacy RF margin can be considered for RedCap for FR1. Don’t support because the RF margin is already captured in the absolute accuracy. Proposal 4: We support Option 1: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in cell identification and measurement requirement. |
| R4-2205631 | Ericsson | Proposal 1: RAN4 to discuss RedCap UE’s behaviour based on the following scenarios: - Case A: Serving cell active BWP with CD-SSB - Case B: Serving cell active BWP with NCD-SSB - Case B-1: All neighbour cells with NCD-SSB - Case B-2: Some neighbour cells with NCD-SSB, some neighbour cells without NCD-SSB - Case C: Serving cell active BWP without SSB Proposal 2: RAN4 to define requirement for both CD-SSB and NCD-SSB neighbour cell measurements. The detail signalling design is up to RAN2. Proposal 3: Network indicates the reference SSB to UE to perform intra-frequency measurements by serving cell MO, such as NCD-SSB or CD-SSB. Proposal 4: For RedCap UE, a measurement is defined as an SSB based intra-frequency measurement provided the - If NW configures NCD-SSB of the serving cell indicated for measurement, the centre frequency of the NCD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same. - Otherwise, the centre frequency of the CD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same. - The subcarrier spacing of the two SSBs are also the same. Proposal 5: When both CD-SSB and NCD-SSB serving cell measurement are configured, UE can perform serving cell measurements based on NCD-SSB within active BWP provided that - the difference of center frequency between NCD-SSB and CD-SSB is no larger than 20MHz in FR1 and 100MHz in FR2 - the difference of reception power between NCD-SSB and CD-SSB is less than 3dB - the periodicity of NCD-SSB and CD-SSB is the same Otherwise, UE should perform serving cell measurements based on both NCD-SSB and CD-SSB. Proposal 6: When both CD-SSB and NCD-SSB neighbour cell measurement are configured, UE should follow NW’s configuration to perform measurements on both SSBs. Proposal 7: RedCap UE needs to report the RRM measurement result together with the type of RS, either NCD-SSB or CD-SSB. Proposal 8: RAN4 to introduce the new measurement delay requirements for RedCap UE. - (Case B-1) Cell identification and measurement by NCD-SSB - (Case B-2) Cell identification and measurement when both NCD-SSB and CD-SSB are configured Proposal 9: In Rel-17, Non-RedCap UE may support the new capability of NCD-SSB measurement. Proposal 10: RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability in Rel-17 provided that the intra-frequency measurement is defined based on the NCD-SSB in active BWP, if NW transmits the NCD-SSB. Proposal 11: RAN4 should add additional values of measGapSharingScheme factor to promote PCell’s measurement for RedCap UE. Proposal 12: When RAN4 defines CGI identification of an E-UTRA cell with autonomous gaps, RAN4 to reuse existing cat-1bis CGI reading requirement in LTE. Proposal 13: The MIB decoding delay requirement of 1Rx RedCap UE can be the same as non-RedCap UE for SNR=-3dB. Proposal 14: 6 samples are needed for 1Rx RedCap UE to achieve the SIB1 90% successful rate. Proposal 15: If indicated by network, UE will further report the NCD-SSB information (such as SS/PBCH block center frequency, SCS etc.) together with global cell ID when UE reporting the CGI. Proposal 16: Cell detection requirements from Rel-15 NR are extended by 1 sample (6 samples in total) for RedCap UE with 1 Rx in FR1. Proposal 17: Lower bound in legacy PSS/SSS detection delay requirements are reused for 1 Rx RedCap UE. Proposal 18: Measurement period for FR1 RedCap 1 Rx UE is reused from release 15 NR requirements based on 5 samples. Proposal 19: The lower bound of SSB based RSRP measurement delay requirement is extended to 400 ms for FR1. Proposal 20: Measurement accuracy for SSB based RedCap UE with 1 Rx is relaxed by 1 dB in FR1 compared to Rel-15 SSB based RSRP measurement accuracy requirements with 2 Rx. Proposal 21: The RF margin from Rel-15 NR is used for defining the measurement accuracy requirements for RedCap UEs. Proposal 22: Time index detection delay in FR1 is extended from 1 attempt (legacy) to 2 attempts at -8 dB SNR for RedCap UE with 1 Rx. Proposal 23: Given the RF group agreement to not reduce the number of Rx branch for FR2, no need to discuss the requirements for FR2 for 1 Rx RedCap UE. Proposal 24: - The 1 Rx RedCap UE shall meet the current requirements on cell identification (PSS/SSS detection defined in Table x.y.z for FR1) provided that at least 5 SMTC windows are available at the UE during cell identification time. - The 1 Rx RedCap UE shall meet the current requirements on time index detection (defined in Table x.y.z for FR1) provided that at least 3 SMTC windows are available at the UE during time index detection. - The 1 Rx RedCap UE shall meet the current requirements intra-frequency/inter-frequency measurement (defined in Table x.y.z for FR1) provided that at least 5 SMTC windows are available at the UE during measurement period. Proposal 25: - The 1 Rx RedCap UE shall meet the current requirements on cell identification (PSS/SSS detection defined in Table x.y.z for FR2) provided that at least X1 SMTC windows are available at the UE during cell identification time, where X1 is the number of SMTC windows used in release 15 legacy NR requirements used for cell identification. - The 1 Rx RedCap UE shall meet the current requirements on time index detection (defined in Table x.y.z for FR2) provided that at least X2 SMTC windows are available at the UE during time index detection, where X2 is the number of SMTC windows used in the release 15 legacy NR requirements for time index detection. - The 1 Rx RedCap UE shall meet the current requirements intra-frequency/inter-frequency measurement (defined in Table x.y.z for FR2) provided that at least X3 SMTC windows are available at the UE during measurement period, where X3 is the number of SMTC windows used in the legacy NR requirements for intra-frequency/inter-frequency measurements. |
| R4-2203792 | Apple | Proposal 2: In connected mode, RRM measurement on neighbour cell can be based on CD-SSB or NCD-SSB. Proposal 3: The neighbor cell’s NCD-SSB information (frequency/SCS) shall be provided to UE if UE is configured to perform cell identification/measurement on neighbor cell’s NCD-SSB, i.e., UE is not required to read neighbor cell SIB to figure out the neighbor cell’s NCD-SSB by itself. Proposal 4: The reference SSB of serving cell for UE to perform intra-frequency measurements could be CD-SSB or NCD-SSB in corresponding serving cell MO. No extra network indication is needed. Proposal 5: For RedCap UE supporting NCD-SSB measurement, a measurement is defined as a SSB based intra-frequency measurement provided: - the centre frequency of the CD-SSB or NCD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same, and - the subcarrier spacing of the two SSBs are also the same. Proposal 6: if both CD-SSB and NCD-SSB measurement are configured to RedCap UE for serving cell but both of them need MG, UE could choose to perform CD-SSB measurement only. Proposal 7: When NW configures MOs with both CD-SSB and NCD-SSB measurements on neighbor cell, UE should follow NW’s configuration to perform measurements. Proposal 8: ssb-PositionsInBurst for NCD-SSB could be a equal-set of ssb-PositionsInBurst for CD-SSB from the same serving cell. Proposal 9: NCD-SSB should be configured off sync raster. Proposal 10: NCD-SSBs and CD-SSB from the same serving cell shall be transmitted in different half-frames when NCD-SSB and CD-SSB are contained in the same SFN. Proposal 11: RAN4 to assume that periodicity of NCD-SSB is same as periodicity of corresponding CD-SSB from the same cell. |
| R4-2203891 | CATT | Proposal 2: In connected mode, RRM measurement on neighbour cell can be based on CD-SSB or NCD-SSB. Proposal 3: It is supported the following scenarios mentioned in R4-2201780. - The RedCap UE shall support RRM measurement on neighbour cells for the following scenarios: - The UE is operating in a BWP that includes CD-SSB. Hence, no need for measurement gaps. - The UE is operating in a BWP that includes NCD-SSB and NCD-SSBs are transmitted by all neighbour cells. Hence, no need for measurement gap. - The UE is operating in a BWP that includes NCD-SSB, however, no NCD-SSB is provided by other neighbour cell, yet the UE is configured with measurement gaps to re-tune to different frequency to measure CD-SSB. - The RedCap UE shall not support RRM measurement on neighbour cells for the following scenarios: - The UE is operating in a BWP that does not include CD-SSB and does not include NCD-SSB and the network did not provide the UE with measurement gaps. Proposal 4: If RRM measurement on neighbour cell can be based on NCD-SSB, support intra-frequency and inter-frequency measurement definition for NCD-SSB as follow: - For RedCap UE supporting NCD-SSB measurement, a measurement is defined as a SSB based intra-frequency measurement provided - the centre frequency of the CD-SSB or NCD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same. - the subcarrier spacing of the two SSBs are also the same. Proposal 5: In principle, NCD-SSB should be used for all possible purposes. |
| R4-2204327 | vivo | Proposal 2: In connected mode, RRM measurement on neighbour cell can be based on CD-SSB or NCD-SSB, i.e., option 2. For the reference SSB to UE to perform intra-frequency measurement, option 3 is preferred (CD-SSB or NCD-SSB) Proposal 3: - For RedCap UE supporting NCD-SSB measurement, a measurement is defined as a SSB based intra-frequency measurement provided - the centre frequency of the CD-SSB or NCD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same. - the subcarrier spacing of the two SSBs are also the same. Proposal 4: If both CD-SSB and NCD-SSB measurement are configured to RedCap UE, UE should perform serving cell measurements based on SSB with active BWP. Proposal 5: NCD-SSB should be configured off sync raster. Proposal 6: No change wrt to legacy NR, i.e. up to NW implementation Proposal 7: FFS for requirements when NCD-SSB is configured and NCD-SSB impact on non-RedCap UE |
| R4-2205000 | CMCC | Proposal 2: In connected mode, RRM measurement on neighbour cell can be based on CD-SSB or NCD-SSB. Proposal 3: The reference SSB of serving cell for UE to perform intra-frequency measurements should be CD-SSB. Proposal 4: For RedCap UE supporting NCD-SSB measurement, a measurement is defined as a SSB based intra-frequency measurement provided - the centre frequency of the CD-SSB of the serving cell and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same. - the subcarrier spacing of the two SSBs are also the same. Proposal 5: If both CD-SSB and NCD-SSB measurement are configured to RedCap UE, UE should perform serving cell measurements based on SSB with active BWP. If both of them need MG, UE choose to perform CD-SSB measurement only. |
| R4-2206114 | Qualcomm Incorporated | Proposal 1: The periodicity of NCD-SSB cannot be configured to be greater than 160ms. Proposal 3: If configured with NCD-SSB, UE should perform serving cell measurements based on the SSB that is available within the active BWP, i.e., NCD-SSB (assuming CD-SSB is outside the active BWP). - FFS: Can the network configure NCD-SSB if there is a CD-SSB in the active BWP? - Note 1: If the UE is also required to measure CD-SSB (configured outside the active BWP), measurement gaps should be configured to perform such measurement - Note 2: Measurement gaps are not needed to measure NCD-SSBs, as they are always configured within the active BWP. Proposal 4: In connected mode, RRM measurement on neighbour cell can be based on CD-SSB or NCD-SSB. Proposal 5: The neighbour cell’s NCD-SSB information (frequency/SCS) shall be provided to UE if UE is configured to perform cell identification/measurement on neighbour cell’s NCD-SSB. Proposal 6: Network indicates the reference SSB (CD-SSB or NCD-SSB) of the serving cell to determine the intra-frequency measurement of the neighbour cell. Proposal 7: For RedCap UE supporting NCD-SSB measurement, a measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of reference SSB of serving cell and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same, and the subcarrier spacing of the two SSBs are also the same. Proposal 8: When the network configures Mos with both CD-SSB and NCD-SSB measurements, and: - When both, NCD-SSB and CD-SSB of the neighbour cell lie within the UE active BWP, UE should be required to measure only one SSB. - When both, NCD-SSB and CD-SSB of the neighbour cell lie outside the UE active BWP, UE should be required to measure only CD-SSB. - When either NCD-SSB or CD-SSB lie within the UE active BWP, UE should be required to measure the SSB that lies within the UE active BWP. |
| R4-2206078 | MediaTek inc. | - Support the discussion that RAN4 to decide whether to capture the naming of CD-SSB and/or NCD-SSB or the existing SSB shall apply to both CD-SSB and NCD-SSB. |

## Open issues summary

### Sub-topic 5-1 Use of NCD-SSB for CONNECTED mode measurements

Issue 5-1-1: The measurement scenarios for NCD-SSB and CD-SSB

- Proposals

  - Option 1 (Ericsson): RAN4 to discuss RedCap UE’s measurement based on the following scenarios:

- Case A: Serving cell active BWP includes CD-SSB

- Case B: Serving cell active BWP includes NCD-SSB

  - Case B-1: All neighbour cells include NCD-SSB

  - Case B-2: Some neighbour cells include NCD-SSB, and some neighbour cells without NCD-SSB

- Case C: Serving cell active BWP without SSB

- Case D: Serving cell active BWP includes both CD-SSB and NCD-SSB

- Recommended WF

  - [Moderator’s comments] Case D is additionally added based on QC’s proposal which is discussed in issue 5-1-7.

  - RAN4 agrees to discuss whether to define the RedCap UE’s measurement requirement for all of the above scenarios.

Issue 5-1-2: Whether to define neighbour cell measurement requirements for NCD-SSB

- Proposals

  - Option 1 (E///, Apple, CATT, vivo, CMCC, QC, Xiaomi): RRM measurement on neighbour cell can be based on CD-SSB or NCD-SSB.

  - Option 2 (HW): RRM measurement on neighbour cell is supposed to be performed on CD-SSB in connected mode.

- Recommended WF

  - Given the large support for option 1, can the supporting company of option 2 compromise to option1? Please note that the specific conditions are discussed further below.

Issue 5-1-3: Reference SSB to decide measurement type (intra- or inter-frequency)

- Proposals

  - Option 1 (E///, QC): NW indicates the reference SSB (CD-SSB or NCD-SSB)

  - Option 2 (Apple): The SSB type indicated in serving cell MO is used as reference SSB (CD-SSB or NCD-SSB)

    - Option 2a (Xiaomi): SSB in the active BWP is used (CD-SSB or NCD-SSB)

  - Option 3 (HW, CMCC): CD-SSB of the serving cell

- Recommended WF

  - Discuss the options.

Issue 5-1-4: Definition of SSB based intra-frequency measurement

- Proposals: A measurement is defined as an SSB based intra-frequency when:

  - Option 1 (E///, QC):

    - If NW configures NCD-SSB of the serving cell indicated for measurement:

      - the centre frequency of the NCD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same and the subcarrier spacing of the two SSBs are also the same.

    - Otherwise:

      - the centre frequency of the CD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same and the subcarrier spacing of the two SSBs are also the same.

  - Option 2 (Apple, CATT, vivo, MTK):

    - the centre frequency of the CD-SSB or NCD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same, and

    - the subcarrier spacing of the two SSBs are also the same.

  - Option 3 (CMCC):

    - the centre frequency of the CD-SSB of the serving cell and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same.

    - the subcarrier spacing of the two SSBs are also the same.

  - Option 4 (HW):

    - the centre frequency of the CD-SSB of the serving cell indicated for measurement and the centre frequency of the CD-SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same.

  - Option 5 (Xiaomi):

    - The legacy definition of intra-frequency and inter-frequency based on the reference SSB could be reused.

- Recommended WF

  - Following is agreeable:

“At least the subcarrier spacing of the two SSBs need to be same to are also the same.”

All options are similar, but the definition depends on the reference SSB which is discussed in previous issue (issue 5-1-3). Once that is agreed, it is easier to agree on the measurement definition. Therefore, it is recommended to reach an agreement on the reference SSB first.

Issue 5-1-5: When both CD-SSB and NCD-SSB are configured for serving cell measurements

When both CD-SSB and NCD-SSB serving cell measurement are configured and one of SSBs within active BWP:

- Proposals

  - Option 1 (E///):	 UE can perform serving cell measurements based on NCD-SSB within active BWP provided that

    - the difference of center frequency between NCD-SSB and CD-SSB is no larger than 20MHz in FR1 and 100MHz in FR2

    - the difference of reception power between NCD-SSB and CD-SSB is less than 3dB

    - the periodicity of NCD-SSB and CD-SSB is the same.

Otherwise, UE should perform serving cell measurements based on both NCD-SSB and CD-SSB.

  - Option 2 (vivo, CMCC, QC):  UE should perform serving cell measurements based on SSB with active BWP.

- Recommended WF

  - Discuss the options.

Issue 5-1-6: When both CD-SSB and NCD-SSB are configured for serving cell measurements and both require MG

When both CD-SSB and NCD-SSB serving cell measurement are configured and both need MG:

- Proposals

  - Option 1 (Apple, CMCC):  UE could choose to perform CD-SSB only if both NCD-SSB and CD-SSB measurements need gap.

- Recommended WF

  - Discuss the options.

Issue 5-1-7: When both CD-SSB and NCD-SSB are configured for neighbour cell measurements

When both CD-SSB and NCD-SSB neighbour cell measurement are configured:

- Proposals

  - Option 1 (E///, Apple):	 UE should follow NW’s configuration to perform measurements on both SSBs.

  - Option 2 (QC):

    - When both, NCD-SSB and CD-SSB of the neighbour cell lie within the UE active BWP, UE should be required to measure only one SSB.

    - When both, NCD-SSB and CD-SSB of the neighbour cell lie outside the UE active BWP, UE should be required to measure only CD-SSB.

    - When either NCD-SSB or CD-SSB lie within the UE active BWP, UE should be required to measure the SSB that lies within the UE active BWP.

- Recommended WF

  - Discuss the options.

Issue 5-1-8: Neighbour cell’s NCD-SSB information

- Proposals

  - Option 1 (Apple, QC):

    - The neighbor cell’s NCD-SSB information (frequency/SCS) shall be provided to UE if UE is configured to perform cell identification/measurement on neighbor cell’s NCD-SSB, i.e., UE is not required to read neighbor cell SIB to figure out the neighbor cell’s NCD-SSB by itself.

- Recommended WF

  - Discuss the option.

Issue 5-1-9: Delay requirements for NCD-SSB based measurement

Measurement delay requirements are introduced for RedCap UE when:

- Proposals

  - Option 1 (E///):

    - Case B-1: Cell identification and measurement by NCD-SSB

    - Case B-2: Cell identification and measurement when both NCD-SSB and CD-SSB are configured

- Recommended WF

  - Discuss the option.

Issue 5-1-10: NCD-SSB configuration on frequency

- Proposals

  - Option 1 (Apple, vivo):	 NCD-SSB should be configured off sync raster.

- Recommended WF

  - No need to discuss this issue since it’s not relevant to define the requirements.

Issue 5-1-11: Periodicity of NCD-SSB

- Proposals

  - Option 1 (Apple):	RAN4 to assume that periodicity of NCD-SSB is same as periodicity of corresponding CD-SSB from the same cell.

  - Option 2 (QC): The periodicity of NCD-SSB cannot be configured to be greater than 160ms.

- Recommended WF

  - Recommended to agree on option 2. Since the options are not mutually exclusive, discuss if option 1 can also be agreed.

Issue 5-1-12: NCD-SSB configuration on time

- Proposals

  - Option 1 (Apple): NCD-SSBs and CD-SSB from the same serving cell shall be transmitted in different half-frames when NCD-SSB and CD-SSB are contained in the same SFN.

  - Option 2 (vivo): Up to NW implementation

- Recommended WF

  - No discussions needed since it is a RAN1 issue and it not relevant to define NW requirements.

Issue 5-1-13: Other NCD-SSB properties

- Proposals

  - Option 1 (Apple):

    - ssb-PositionsInBurst for NCD-SSB could be a equal-set of ssb-PositionsInBurst for CD-SSB from the same serving cell.

  - Option 2 (vivo): Up to NW implementation

- Recommended WF

  - No need to discuss the issue since RAN2 has reached following agreements:

“The network may provide absoluteFrequencySSB and ssb-periodicity explicitly for NCD-SSB, i.e., other properties such as PCI, ssb-PBCH-BlockPower, ssb-PositionsInBurst are configured with the same values from serving cell's CD-SSB.”

Issue 5-1-14: Reporting of RS type (NCD-SSB or CD-SSB) as part of RRM measurement reporting

- Proposals

  - Option 1 (E///):	 RedCap UE needs to report the RRM measurement result together with the type of RS, either NCD-SSB or CD-SSB.

  - Option 2 (vivo): Up to RAN1/RAN2

- Recommended WF

  - Discuss the option.

Issue 5-1-15: New NCD-SSB capability for non-RedCap UE

- Proposals

  - Option 1 (E///):	 In Rel-17, Non-RedCap UE may support the new capability of NCD-SSB measurement.

- Recommended WF

  - Discuss the option.

Issue 5-1-16: Terminology in specification

- Proposals

  - Option 1 (MTK):	RAN4 to decide whether to capture the naming of CD-SSB and/or NCD-SSB or the existing SSB shall apply to both CD-SSB and NCD-SSB.

- Recommended WF

  - Moderator: If measurement based on NCD-SSB is supported, then CD-SSB and NCD-SSB need to be captured in the specification to differentiate the SSB type. NCD-SSB is also specifically used in other WG specification. Thus, no further discussion is needed.

Issue 5-1-17: Whether to define serving cell measurement requirements based on NCD-SSB

- Proposals

  - Option 1 (HW): Support RRM measurement on serving cell based on NCD-SSB in connected state.

- Recommended WF

  - Following was agreed at last meeting, see R4-2202774:

“RRM measurement on serving cell can be based on CD-SSB or NCD-SSB.”

Thus, no discussions needed.

Sub topic 5-1

| Company | Comments |
| --- | --- |
| XXX | Issue 5-1-1: The measurement scenarios for NCD-SSB and CD-SSB Issue 5-1-2: Whether to define neighbour cell measurement requirements for NCD-SSB Issue 5-1-3: Reference SSB to decide measurement type (intra- or inter-frequency) Issue 5-1-4: Definition of SSB based intra-frequency measurement Issue 5-1-5: When both CD-SSB and NCD-SSB are configured for serving cell measurements Issue 5-1-6: When both CD-SSB and NCD-SSB are configured for serving cell measurements and both require MG Issue 5-1-7: When both CD-SSB and NCD-SSB are configured for neighbour cell measurements Issue 5-1-8: Neighbour cell’s NCD-SSB information Issue 5-1-9: Delay requirements for NCD-SSB based measurement Issue 5-1-11: Periodicity of NCD-SSB Issue 5-1-13: Reporting of RS type (NCD-SSB or CD-SSB) as part of RRM measurement reporting Issue 5-1-15: New NCD-SSB capability for non-RedCap UE |
| Huawei | Issue 5-1-2: The measurement scenarios for NCD-SSB and CD-SSB Support option2. We are afraid the issue that UE may only be able to observe partial neighbour cells based on NCD-SSB. To our knowledge, from network NCD-SSB configuration perspective, it is not possible to always guarantee all NCD-SSB in serving cell and neighbour cells are aligned in frequency domain. One example is presented in Figure 1 where NCD-SSB1 is in active BWP1, neighbour cell 1 has NCD-SSB in the same frequency position but neighbour cell 2 doesn’t have. If UE performs cell detection and measurement on NCD-SSB1, although no measurement gap is needed, neighbour cell2 is not able to be identified. It means that the cells observed on NCD-SSB are only a sub-set of all neighbour cells. If UE could not identify a full set of the neighbour cells, there is risk for mobility performance. For example, if UE is moving to neighbour cell2, but this cell is not detectable by the UE, the handover would face enormous risk, and the case shall be highly not expected. Therefore to guarantee UE can observe full set of neighbour cells and avoid mobility performance degradation, RRM measurement on neighbour cell is supposed to be performed on CD-SSB. Issue 5-1-3: Reference SSB to decide measurement type (intra- or inter-frequency) Support option 3, the main motivation is to avoid that one target MO is regarded as intra-f and inter-f dynamically. For RedCap UE, although the servingcellMO design is not stable in RAN2, there is a possibility that the servingcellMO may include CD-SSB and NCD-SSB configuration. If CD-SSB and NCD-SSB configuration parameters are configured in servingcellMO, the actual used SSB would dynamic change with active BWP. According to the current intra-frequency measurement definition, as the serving cell reference SSB is dynamic changing, one target MO may be regarded as intra-frequency or inter-frequency measurement dynamically as well. As we know, the required sample numbers for intra-frequency and inter-frequency measurement are different (5 samples for intra-frequency and 8 samples for inter-frequency). The dynamic changes between intra-frequency and inter-frequency measurement would complicate UE implementation. In order to use the samples efficiently, the better way is to remain one MO measurement type unchanged (i.e., always intra-frequency or inter-frequency). To address this issue, fix the reference point of serving cell SSB is needed. We propose that the reference centre frequency of SSB of the serving cell is CD-SSB. Issue 5-1-4: Definition of SSB based intra-frequency measurement We support option 4 where both reference SSB and target neighbor cell SSB are CD-SSB. For the to-be-measurement neighbor cell SSB, as we proposed in issue 5-1-2, L3 mobility on neighbor cell shall be CD-SSB. Issue 5-1-5: When both CD-SSB and NCD-SSB are configured for serving cell measurements For serving cell measurement Option 2 is agreeable. UEs can directly evaluate the serving cell quality based on NCD-SSB without retuning to CD-SSB. The typical scenario is that UE is in the center cell. Issue 5-1-6: When both CD-SSB and NCD-SSB are configured for serving cell measurements and both require MG For serving cell measurement, we can agree option1. Issue 5-1-7: When both CD-SSB and NCD-SSB are configured for neighbour cell measurements As we commented in issue 5-1-2, RRM measurement on neighbour cell is supposed to be performed on CD-SSB in connected mode. Issue 5-1-8: Neighbour cell’s NCD-SSB information Wait for RAN2 conclusion Issue 5-1-9: Delay requirements for NCD-SSB based measurement As we commented in issue 5-1-2, RRM measurement on neighbour cell is supposed to be performed on CD-SSB in connected mode. Issue 5-1-11: Periodicity of NCD-SSB The issue is confused. Both RAN1 and RAN4 have conclusion on the periodicity of NCD-SSB. Taking together the below agreements, the periodicity of NCD-SSB shall be larger than periodicity of CD-SSB. RAN4#101e agreement - Periodicities of NCD-SSB are up to network configuration and can be same or different from those of CD-SSB, if both NCD-SSB and CD-SSB are transmitted on the serving cell of RedCap UE. Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB.. RAN1#107b-e Agreement: Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB Issue 5-1-10: NCD-SSB configuration on time Fine with option 2. Under some scenario, to avoid peforming power boosting for NCD-SSBs and CD-SSB simultaneously for network, NCD-SSBs and CD-SSB can be transmitted in different half-frames. It is up to NW configuration. Issue 5-1-13: Reporting of RS type (NCD-SSB or CD-SSB) as part of RRM measurement reporting Wait for RAN2 discussion on the MO configuration. If RAN2 agreed that a MO is related with a specific SSB (CD or NCD), UE is no need to report the types as network knows the relation between MO and SSB. Issue 5-1-14: New NCD-SSB capability for non-RedCap UE Disagree with option1. This is out of RedCap scope. Issue 5-1-15: Terminology in specification Depends on multiple issues listed in above. Issue 5-1-16: Whether to define serving cell measurement requirements based on NCD-SSB Fine with the recommended WF. |
| Ericsson | Issue 5-1-1: The measurement scenarios for NCD-SSB and CD-SSB Option 1. We’re also fine to down-select the scenarios to define the measurement requirement in Rel-17. Issue 5-1-2: Whether to define neighbour cell measurement requirements for NCD-SSB Option 1. Since Rel-15 the network can already request UEs to perform RRM measurements on “NCD-SSBs”. There is no reason to preclude NCD-SSB neighbour cell measurement for RedCap. Issue 5-1-3: Reference SSB to decide measurement type (intra- or inter-frequency) Option 1. One of the possible methods is to reuse the definition as non-RedCap UE to use CD-SSB to define the intra-frequency measurements. However, the definition is not reasonable when both serving cell and neighbour cells have NCD-SSB in the active BWP (scenario B-2).Another possible method is to use the SSB in RedCap active BWP to define the intra-frequency measurements. However, RedCap UE is similar as non-RedCap UE which may not have SSB in active BWP. Thus, we propose NW to indicate the SSB to define the intra-frequency. In this case, both NW and UE have the common understanding on which SSB/frequency layer belongs to intra-frequency measurement, which SSB/frequency layer belongs to inter-frequency measurement. How to indicate the reference SSB to define intra-frequency is up to RAN2. From our understanding, option 2 is one of possible solutions to indicate the intra-frequency by ServingCellMO, but it should up to RAN2’s design. Issue 5-1-4: Definition of SSB based intra-frequency measurement Option 1. Option 1 is better than option 2 which is clearer to define the intra-frequency measurement. Option 2 is unclear when NW configures both NCD-SSB and CD-SSB for serving cell measurement. Issue 5-1-5: When both CD-SSB and NCD-SSB are configured for serving cell measurements Option 1. When NW configures both CD-SSB and NCD-SSB measurement, UE shall follow NW’s configuration to perform both SSB’s measurement. The only exception case is the measurement metric of both CD-SSB and NCD-SSB will bring the same information as the pre-condition mentioned in option 1. Issue 5-1-6: When both CD-SSB and NCD-SSB are configured for serving cell measurements and both require MG Not support option 1. When NW configures both CD-SSB and NCD-SSB measurement, UE shall follow NW’s configuration to perform both SSB’s measurement. The only exception case is the measurement metric of both CD-SSB and NCD-SSB will bring the same information as the pre-condition mentioned in option 1. We add option 2 as follow: When both NCD-SSB and CD-SSB measurements need gap, UE could choose to perform any SSB only provided that - the difference of center frequency between NCD-SSB and CD-SSB is no larger than 20MHz in FR1 and 100MHz in FR2 - the difference of reception power between NCD-SSB and CD-SSB is less than 3dB - the periodicity of NCD-SSB and CD-SSB is the same. Otherwise, UE should perform serving cell measurements based on both NCD-SSB and CD-SSB. Issue 5-1-7: When both CD-SSB and NCD-SSB are configured for neighbour cell measurements Option 1. When NW configures neighbour cell measurement with both CD-SSB and NCD-SSB measurements, the UE shall measure on both SSBs provided that UE can do so without gaps or the NW configured required measurement gaps. The UE shall not stop doing configured measurements. Issue 5-1-8: Neighbour cell’s NCD-SSB information Agree the proposal. Issue 5-1-9: Delay requirements for NCD-SSB based measurement Option 1. We’re also fine to postpone this issue until RAN4 has a clear intra-frequency definition and the NCD-SSB measurement. Issue 5-1-10: NCD-SSB configuration on frequency Agree with the recommended WF. This is not a RAN4 issue and not impact the RAN4’s requirement. Issue 5-1-11: Periodicity of NCD-SSB We support option 2, but not support option 1. Based on RAN1’s Working Assumption, the periodicity of NCD-SSB shall be no less than periodicity of CD-SSB. Issue 5-1-12: NCD-SSB configuration on time Agree with the recommended WF. No discussions needed since it is a RAN1 issue and it not relevant to define NW requirements. Issue 5-1-13: Other NCD-SSB properties Agree with the recommended WF. RAN2 had already agreed that ‘ssb-PositionsInBurst are configured with the same values from serving cell's CD-SSB’. Issue 5-1-14: Reporting of RS type (NCD-SSB or CD-SSB) as part of RRM measurement reporting If UE only performs measurement on one of SSB, UE should report the type of SSB together with measurement reporting. We’re fine to postpone this issue until RAN4 has a clear intra-frequency definition and the NCD-SSB measurement procedure. Issue 5-1-15: New NCD-SSB capability for non-RedCap UE Option 1. When non-RedCap UE is configured to measure the signal strength(RSRP/RSRQ/RSSI etc.) of a neighbour cell, UE can use NCD-SSB transmission instead of CD-SSB if CD-SSB is out of active BWP. Issue 5-1-16: Terminology in specification Agree with the recommended WF. Issue 5-1-17: Whether to define serving cell measurement requirements based on NCD-SSB Agree with the recommended WF. Do not need to discuss again. |
| Apple | Issue 5-1-1: The measurement scenarios for NCD-SSB and CD-SSB Agree to down-select scenarios from option 1, e.g., define requirement when serving cell active BWP include CD-SSB or NCD-SSB and neighbor cell NCD-SSB could also be covered by serving cell active BWP. If the neighbor cell NCD-SSB cannot be covered by serving cell active BWP, that means MG is always needed, and then we think neighbor cell CD-SSB could be used for cell detection/measurement with MG. More discussion on down-selection is needed. Issue 5-1-2: Whether to define neighbour cell measurement requirements for NCD-SSB Option 1 based on network configuration. Issue 5-1-3: Reference SSB to decide measurement type (intra- or inter-frequency) Option 2. We think there would be two kinds of SSBs from one target cell, and also its serving cell may also have two kinds of SSBs. There might be following scenarios: - Serving cell CD-SSB has same SCS and center-frequency as neighbor cell CD-SSB (legacy intra-frequency) - Serving cell NCD-SSB has same SCS and center-frequency as neighbor cell CD-SSB (intra-frequency for neighbor cell CD-SSB measurement) - Serving cell CD-SSB has same SCS and center-frequency as neighbor cell NCD-SSB (intra-frequency for neighbor cell NCD-SSB measurement) - Serving cell NCD-SSB has same SCS and center-frequency as neighbor cell NCD-SSB (intra-frequency for neighbor cell NCD-SSB measurement) The serving cell MO (servingCellMO: This field is mandatory present for the SpCell if the UE has a measConfig, and it is optionally present, Need M, for SCells) is configured by network, and in such MO network could configure UE to perform measurement on either CD-SSB or NCD-SSB. There is no need to additionally indicate which is the reference SSB for intra-frequency measurement. Issue 5-1-4: Definition of SSB based intra-frequency measurement Option 2. Issue 5-1-5: When both CD-SSB and NCD-SSB are configured for serving cell measurements Option 2. To measurement serving cell without MG is the benefit by introducing NCD-SSB (locate NCD-SSB in the active BWP). So that means UE could choose to use NCD-SSB or CD-SSB for serving cell measurement depending on which SSB is within active BWP. Issue 5-1-6: When both CD-SSB and NCD-SSB are configured for serving cell measurements and both require MG Option 1. If MG is needed for both CD-SSB and NCD-SSB measurement, there is no benefit to use NCD-SSB to avoid RF tuning/retuning and therefore in this case UE could choose to use CD-SSB for measurement. Issue 5-1-7: When both CD-SSB and NCD-SSB are configured for neighbour cell measurements Option 1 and we could compromise to option 2 for UE flexibility. Issue 5-1-8: Neighbour cell’s NCD-SSB information Option 1. For intra-frequency measurement object, we think placing neighbor cell NCD-SSB colliding with serving cell NCD-SSB (they have same SCS) would benefit both UE and network since MG (RF tuning/retuning) is not needed. If different SCS is used for neighbor cell’s NCD SSB and serving cell’s NCD-SSB, it’s a kind of inter-frequency measurement and MG is needed. However, it also requires network to provide the NCD-SSB information (frequency/SCS information) to UE for neighbor cell detection/measurement. Issue 5-1-9: Delay requirements for NCD-SSB based measurement Fine with option 1. Issue 5-1-11: Periodicity of NCD-SSB Option 1 and 2. RAN2 agreed that, For connected mode operation NCD-SSB has the same properties (e.g., ssb-PositionsInBurst, PCI, ssb-periodicity, ssb-PBCH-BlockPower) as the corresponding CD-SSB. FFS if an additional property needs to be specified. Issue 5-1-12: NCD-SSB configuration on time Option 1 to consider the power limitation from NW side. It would be beneficial to predefine such time relation, otherwise after detecting CD-SSB UE still needs to blindly detect NCD-SSB of the same cell which does not make much sense. Issue 5-1-13: Other NCD-SSB properties Option 1 since RAN2 had agreement. Issue 5-1-14: Reporting of RS type (NCD-SSB or CD-SSB) as part of RRM measurement reporting Option 2. Issue 5-1-15: New NCD-SSB capability for non-RedCap UE It’s not scope of RedCap WI. No need to discuss. Issue 5-1-16: Terminology in specification Fine with recommended WF. |
| CMCC | Issue 5-1-1: The measurement scenarios for NCD-SSB and CD-SSB In our view, RAN4 requirements do not need to differenciate NCD-SSB and CD-SSB. Issue 5-1-2: Whether to define neighbour cell measurement requirements for NCD-SSB Option 1 Issue 5-1-3: Reference SSB to decide measurement type (intra- or inter-frequency) We do not support to indicate the reference of SSB. Considering the limited time, use CD-SSB of serving cell as reference is the simpliest solution. Issue 5-1-4: Definition of SSB based intra-frequency measurement OK with the recommended WF Issue 5-1-5: When both CD-SSB and NCD-SSB are configured for serving cell measurements Option 2 Issue 5-1-6: When both CD-SSB and NCD-SSB are configured for serving cell measurements and both require MG Option 1 Issue 5-1-7: When both CD-SSB and NCD-SSB are configured for neighbour cell measurements Option1. Follow network configuration. Issue 5-1-8: Neighbour cell’s NCD-SSB information This is up to RAN2 discussion. Issue 5-1-9: Delay requirements for NCD-SSB based measurement Requirements should be the same for CD-SSB and NCD-SSB. No additional requirements are needed Issue 5-1-11: Periodicity of NCD-SSB NCD-SSB periodicity configuration should up to network. No further discussion is needed. Issue 5-1-14: Reporting of RS type (NCD-SSB or CD-SSB) as part of RRM measurement reporting MOs are configured by network, why UE need to inform network about the type of RS？ Issue 5-1-15: New NCD-SSB capability for non-RedCap UE Don’t think we need to discuss this case. Network can configure legacy UE with BWP with CD-SSB. No restriction on the bandwidth for legacy UE Issue 5-1-16: Terminology in specification Whether to capture depends on whether additional requirements are introduced for NCD-SSB |
| MediaTek | Issue 5-1-1: The measurement scenarios for NCD-SSB and CD-SSB Case A, B and C make sense; however, Case D is unnecessary because no NW is going to configure both NCD-SSB and CD-SSB in the same BWP. Issue 5-1-2: Whether to define neighbour cell measurement requirements for NCD-SSB Support Option 1. Issue 5-1-3: Reference SSB to decide measurement type (intra- or inter-frequency) Support Option 2a. Issue 5-1-4: Definition of SSB based intra-frequency measurement To our understanding, if we agree on the reference SSB as in the previous issue to be the SSB associated with the active BWP. Then, Options 1, 2 and 5 are the same. Yet, we support option 2 because it is clearer. Issue 5-1-5: When both CD-SSB and NCD-SSB are configured for serving cell measurements Support Option 2. Issue 5-1-6: When both CD-SSB and NCD-SSB are configured for serving cell measurements and both require MG Option 1. UE can choose to measure on CD-SSB only. Issue 5-1-7: When both CD-SSB and NCD-SSB are configured for neighbour cell measurements The scenario from option 2: ‘When both, NCD-SSB and CD-SSB of the neighbour cell lie within the UE active BWP, UE should be required to measure only one SSB.’ is unlikely to happen hence we partially support option 2 after being modified to: - When both, NCD-SSB and CD-SSB of the neighbour cell lie outside the UE active BWP, UE should be required to measure only CD-SSB. - When either NCD-SSB or CD-SSB lie within the UE active BWP, UE should be required to measure the SSB that lies within the UE active BWP. Issue 5-1-8: Neighbour cell’s NCD-SSB information Agree with Option 1. Issue 5-1-9: Delay requirements for NCD-SSB based measurement We don’t support this proposal. Also, we don’t understand what have been changed since rel-15/16 that need to change these requirements. Issue 5-1-10: NCD-SSB configuration on frequency Support Option 1. Issue 5-1-11: Periodicity of NCD-SSB We support Options 1 and 2 as this shall simplify the requirements. Issue 5-1-13: Reporting of RS type (NCD-SSB or CD-SSB) as part of RRM measurement reporting Option 2, leave it to RAN1/2. Issue 5-1-15: New NCD-SSB capability for non-RedCap UE We don’t support this proposal, we shall focus on RedCap UE behaviour in this work item. |
| Xiaomi | Issue 5-1-1: The measurement scenarios for NCD-SSB and CD-SSB Case D is not a valid case from our understanding. Issue 5-1-2: Whether to define neighbour cell measurement requirements for NCD-SSB Support Option 1. Issue 5-1-3: Reference SSB to decide measurement type (intra- or inter-frequency) Support Option 2a. For the case that active BWP without SSB, CD-SSB of the serving cell could be used as reference SSB which follows the legacy method. For other options, there exist the case where UE could perform intra-frequency measurement based on the SSB inside the active BWP but have to return to the specified SSB to perform inter-freq meas. In this way, additional UE effort is needed. Issue 5-1-4: Definition of SSB based intra-frequency measurement Depend on the conclusion on issue 5-1-3. Issue 5-1-5: When both CD-SSB and NCD-SSB are configured for serving cell measurements Support Option 2. Issue 5-1-6: When both CD-SSB and NCD-SSB are configured for serving cell measurements and both require MG Fine with Option 1. Issue 5-1-7: When both CD-SSB and NCD-SSB are configured for neighbour cell measurements - Option 1, we think UE have to perform neighbour cell measurement based on MW configuration. Issue 5-1-8: Neighbour cell’s NCD-SSB information Agree with Option 1. Issue 5-1-9: Delay requirements for NCD-SSB based measurement We think the legacy requirement could be reused for NCD-SSB Issue 5-1-10: NCD-SSB configuration on frequency Support the recommended WF Issue 5-1-11: Periodicity of NCD-SSB Support Option 1 to follow RAN2 agreement Issue 5-1-14: Reporting of RS type (NCD-SSB or CD-SSB) as part of RRM measurement reporting Option 2, leave it to RAN1/2. |
| Nokia | Issue 5-1-1: The measurement scenarios for NCD-SSB and CD-SSB We agree to downselect the scenarios. At least Case A and Case B-1 should have requirements. In our view, the requirements of Case A and Case B-1 would be the same. There is no reason to differentiate between CD-SSB and NCD-SSB in the specification (this is also our understanding from RAN2 agreements in the last meeting) if neighbor cells include CD-SSB or NCD-SSB. We don’t see scenario D as typical. Sending NCD-SSB is an overhead to the network, therefore we see no gain in sending both CD-SSB and NCD-SSB in the same BWP. We agree with Apple that more discussion on the down-selection is needed. Issue 5-1-2: Whether to define neighbour cell measurement requirements for NCD-SSB Option 1. NCD-SSB measurements are already possible for non-RedCap UEs. RAN2 understanding (RAN2 #116e): In connected mode, current RRC signalling allows configuring SSB-based RRM measurements on any (CD- or NCD-) SSB, but it does not allow using an NCD-SSB for RLM, BFD, link recovery, RO selection, mobility (mobility here refers to the frequency indicated in FreqDLInfo in HO command), in TCI-states or for any other functionality (other than RRM measurements). Issue 5-1-3: Reference SSB to decide measurement type (intra- or inter-frequency) We prefer Option 2 or Option 2a wording. In any case, no other indication is needed from the network. Issue 5-1-4: Definition of SSB based intra-frequency measurement We prefer Option 2 or Option 5, which in our view would lead to the same definition. Issue 5-1-5: When both CD-SSB and NCD-SSB are configured for serving cell measurements Option 2: UE shall perform serving cell measurements based on the SSB within the active BWP. Issue 5-1-6: When both CD-SSB and NCD-SSB are configured for serving cell measurements and both require MG Not support Option 1. UE shall perform all measurements configured by the network. Issue 5-1-7: When both CD-SSB and NCD-SSB are configured for neighbour cell measurements Option 1, for the same reason as in issue 5-1-6. Issue 5-1-8: Neighbour cell’s NCD-SSB information Agree to Option 1. Issue 5-1-9: Delay requirements for NCD-SSB based measurement Do not agree to Option 1. We agree with CMCC. There is no need to differentiate between NCD-SSB and CD-SSB requirements (for RRM measurements). In our understanding NCD-SSB based measurements are already supported in the specification, so nothing is needed. Issue 5-1-11: Periodicity of NCD-SSB This issue was already agreed in RAN2, as mentioned by Apple. Issue 5-1-12: NCD-SSB configuration on time Agree with the recommended WF. Issue 5-1-14: Reporting of RS type (NCD-SSB or CD-SSB) as part of RRM measurement reporting Again, we agree with CMCC. The MO is configured by the network, why should the UE report the type of RS? Furthermore, NCD-SSB measurements are already possible for non RedCap UEs, without reporting the RS type. Anyway, this should be a RAN1 / RAN2 decision. Issue 5-1-15: New NCD-SSB capability for non-RedCap UE We see no need to discuss a new capability. |
| vivo | Issue 5-1-1: The measurement scenarios for NCD-SSB and CD-SSB These scenarios in option 1 can be discussed as starting point. Case A and Case B-1 should be high priority and supported in Rel-17. RRM requirements for the two scenarios should be defined. Case B-2 needs further discussion. The requirements could be quite complicated. It is not expected at least from UE side. Case C is not needed to be considered at least in R17, in our view. For legacy UE, the assumption is that there is SSB available in serving cell active BWP. At least the PCell timing is based on available SSB within active BWP. The basic BWP operation requires active BWP includes SSB. For RedCap UE, it is not mandated to support advanced BWP operation without SSB within active BWP. We don’t think Case D could be typical deployment. Especially, the maximum BW for RedCap UE in FR1 in 20MHz. It is difficult to understand why CD-SSB and NCD-SSB is deployed in such narrow frequency range. Issue 5-1-2: Whether to define neighbour cell measurement requirements for NCD-SSB Support option 1. It is obvious for case B (at least Case B-1), the neighbour cell measurement should be based on NCD-SSB. Issue 5-1-3: Reference SSB to decide measurement type (intra- or inter-frequency) It would be relevant to RAN2 design. Both CD-SSB and NCD-SSB can be used as reference SSB as long as it can be identified by UE that they belong to the same serving cell. We would like to propose option 4. Option 4: Measurement based on reference SSB of both CD-SSB and NCD-SSB of the same serving cell are intra-frequency measurements. Note that NCD-SSB association with CD-SSB or serving cell may be needed. Issue 5-1-4: Definition of SSB based intra-frequency measurement Option 2 would be fine in general. To avoid different interpretation, we would like to propose a revised option 2a based on comments to Issue 5-1-3. - the centre frequency of the CD-SSB and/or NCD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same, and - the subcarrier spacing of the two SSBs are also the same. Issue 5-1-5: When both CD-SSB and NCD-SSB are configured for serving cell measurements Option 2. Issue 5-1-6: When both CD-SSB and NCD-SSB are configured for serving cell measurements and both require MG. Firstly, it depends on scenario discussion for Issue 5-1-1 (Case C). In our view, the case of serving cell measurement with MG should be deprioritized in Rel-17. In RAN1, there is relevant discussions. There are following proposal being discussed. High Priority Proposal 4-1-1a: A RedCap UE supports NCD-SSB based operation (including NCD-SSB based measurements) as a mandatory feature in an RRC-configured DL BWP that does not include CD-SSB. Considering limited time in Rel-17, requirements for mandatory feature should be prioritized. In addition, comments to Issue 5-1-1 Case C should also be taken into account. Therefore, in our view case in Issue 5-1-6 should not be considered in Rel-17. Issue 5-1-7: When both CD-SSB and NCD-SSB are configured for neighbour cell measurements In our understanding, UE should follow NW configuration as there may be reasons measurements based on both NCD-SSB and CD-SSB are necessary. Issue 5-1-8: Neighbour cell’s NCD-SSB information Agree with option 1. Issue 5-1-9: Delay requirements for NCD-SSB based measurement In general option 1 is fine. Maybe some clarification is needed. In our understanding, applicability of existing requirements should need to be specified. There would be no additional new delay requirements. Issue 5-1-11: Periodicity of NCD-SSB We support both option 1 and 2. Issue 5-1-14: Reporting of RS type (NCD-SSB or CD-SSB) as part of RRM measurement reporting Option 2. Issue 5-1-15: New NCD-SSB capability for non-RedCap UE It is not in the scope of this WI. |
| Qualcomm | Issue 5-1-1: The measurement scenarios for NCD-SSB and CD-SSB We are fine with Case A and B. Given the limited amount of time, case C can pe de-prioritized as the optional feature – ‘Not-need-NCD-SSB’ may not be supported by many UEs. We agree that Case D is very atypical but can be considered. Issue 5-1-2: Whether to define neighbour cell measurement requirements for NCD-SSB Support option 1. Issue 5-1-3: Reference SSB to decide measurement type (intra- or inter-frequency) We can support option 2a. The reference SSB should be the one that is in the active BWP. Our proposal in Option 1 is more related to case D in Issue 5-1-1. When both CD-SSB and NCD-SSB are configured within the active BWP, network should indicate the reference SSB. But if we are the deprioritize case D, then we support option 2a. We don’t see any incentive in defining an SSB outside the active BWP as reference SSB. Issue 5-1-4: Definition of SSB based intra-frequency measurement We support Option 1 but based on our comment in Issue 5-1-3, if case D is deprioritized and both CD-SSB and NCD-SSB cannot be configured in the active BWP, then we are fine with defining the intra-frequency measurement as below: Option 6: A measurement is defined as an SSB based intra-frequency measurement when - one of the NCD-SSB or CD-SSB of the serving cell indicated for measurement lies in the active BWP of the UE and - the centre frequency of this SSB (that lies within the active BWP of the UE) and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same and - the subcarrier spacing of the two SSBs are also the same. Issue 5-1-5: When both CD-SSB and NCD-SSB are configured for serving cell measurements Option 2. Issue 5-1-6: When both CD-SSB and NCD-SSB are configured for serving cell measurements and both require MG. NCD-SSB of the serving cell is always configured in the active BWP and should not require measurement gaps. No need to discuss this issue. Issue 5-1-7: When both CD-SSB and NCD-SSB are configured for neighbour cell measurements Support option 2 Issue 5-1-8: Neighbour cell’s NCD-SSB information Support option 1. Issue 5-1-9: Delay requirements for NCD-SSB based measurement We think current requirements are sufficient and additional requirements are not needed. Issue 5-1-11: Periodicity of NCD-SSB Option 1 includes option 2. So we can support Option 1. Issue 5-1-14: Reporting of RS type (NCD-SSB or CD-SSB) as part of RRM measurement reporting Option 2. We don’t see a need to define such capability. Upto RAN1/RAN2 to decide. Issue 5-1-15: New NCD-SSB capability for non-RedCap UE It is not in the scope of this WI. |

### Sub-topic 5-2 CSSF, gap related issues

Issue 5-2-1: Inter-frequency without gap

- Proposals

  - Option 1 (ZTE, Apple, Xiaomi, vivo, MTK, OPPO):	 RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability in Rel-17.

  - Option 1a (E///):

    - RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability in Rel-17 provided that the intra-frequency measurement is defined based on the NCD-SSB in active BWP, if NW transmits the NCD-SSB.

  - Option 2 (CMCC, HW): 	 RAN4 needs to consider ‘inter-frequency without MG’ capability when defining RedCap RRM requirements.

- Recommended WF

  - Discuss the option.

Issue 5-2-2: Assumption on searcher

- Proposals

  - Option 1 (CMCC):	The searcher is shared by intra-frequency without gap and inter-frequency without gap measurement for RedCap UE:

  - Option 2 (ZTE, MTK): The searcher will be exclusively used by intra-frequency without gap measurement provided that RAN4 agrees that RedCap UE does NOT support ‘Inter-frequency without gap’ measurement capability in Rel-17.

  - Option 3 (MTK):

    - The searcher will be exclusively used by intra-frequency without gap measurement provided that RAN4 agrees that RedCap UE does NOT support ‘Inter-frequency without gap’ measurement capability in Rel-17.

- Recommended WF

  - Discuss the options

Issue 5-2-3: CSSF outside gap

- Proposals

  - Option 1 (ZTE, Apple, Xiaomi, vivo, MTK, OPPO): CSSFoutside_gap,I = 1 for RedCap UE measurement outside gap based on Rel-15 requirement.

  - Option 2 (HW): CSSFoutside_gap,I = A+B, where A is the intra-frequency without gap and B is the number of configured inter-frequency MOs without MG.

  - Option 3 (CMCC):

    - CSSFoutside_gap,I  PCC= 2, if configured inter-frequency mOs without MG when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap that are being measured outside of MG for RedCap UE.

      - CSSFoutside_gap,I  PCC = 1 otherwise

    - CSSFoutside_gap,I  PCC = 2*Y, for inter-frequency MO with no measurement gap, Y is the number of configured inter-frequency mOs without MG when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap that are being measured outside of MG for RedCap UE;

      - CSSFoutside_gap,I  PCC = 0 otherwise

- Recommended WF

  - It depends on conclusion of whether inter-frequency without gap is supported. Therefore, it is recommended to reach a conclusion on issue 5-2-1 first in 1st round.

Issue 5-2-4: CSSF within gap

- Proposals

  - Option 1 (ZTE, Apple, Xiaomi, vivo, CMCC, OPPO): The current design for CSSF within gap could be reused for RedCap UE.

  - Option 1a (MTK): The current design for CSSF within gap could be reused for RedCap UE provided that the definition of intra-frequency measurement for RedCap UE is as follow.

- the centre frequency of the CD-SSB or NCD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same.

- the subcarrier spacing of the two SSBs are also the same.

  - Option 2 (CMCC, E///): Consider to add additional two values of measGapSharingScheme factor for RedCap UE, e.g. 85%, 95%.

- Recommended WF

  - Check if companies can compromise to option 1.

Issue 5-2-5: Type of measurement gaps (if considered)

- Proposals

  - Option 1 (MTK):	 If MG is needed, RAN4 to specify per-UE MG based cell identification/measurement requirement regardless of independentGapConfig.

- Recommended WF

  - Discuss the option

Sub topic 5-2

| Company | Comments |
| --- | --- |
| XXX | Issue 5-2-1: Inter-frequency without gap Issue 5-2-2: Assumption on searcher Issue 5-2-3: CSSF outside gap Issue 5-2-4: CSSF within gap Issue 5-2-5: Type of measurement gaps (if considered) |
| Huawei | Issue 5-2-1: Inter-frequency without gap Support option 2. Issue 5-2-2: Assumption on searcher Support option 1. Issue 5-2-3: CSSF outside gap Support option 3, option 3 is more accurate compared with option 2. Issue 5-2-4: CSSF within gap Support option 1. Issue 5-2-5: Type of measurement gaps (if considered) It depends on UE type. If a RedCap UE supports both FR1 and FR2, the independentGapConfig capability seems still needed. |
| Ericsson | Issue 5-2-1: Inter-frequency without gap Option 1 or 1a RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability in Rel-17 provided the intra-frequency measurement is defined by NCD-SSB when NW transmits NCD-SSB. Issue 5-2-2: Assumption on searcher Option 2. Issue 5-2-3: CSSF outside gap Option 1 provided only one of NCD-SSB and CD-SSB within the serving cell’s active BWP will be measured. Issue 5-2-4: CSSF within gap Option 2. As we mentioned before, if no NCD-SSB transmission, the intra-frequency measurement will be defined based on CD-SSB. Thus, RedCap UE may highly perform intra-frequency measurement with measurement gap because CD-SSBs being measured may be outside the active BWP due to BW reduction for RedCap UE. In Rel-15, CSSF is always set to 1 for PCell measurement to guarantee the mobility of PCell. The current gap sharing indication can be ‘equal splitting’ or ‘25%’ which implies intra-frequency measurement has lower/equal priority with inter-frequency within the measurement gap. However, to guarantee UE’s mobility, PCell measurement shall be prioritized which means only two values can be used in RedCap. Thus, we suggest to add other options in gap sharing factor. Issue 5-2-5: Type of measurement gaps (if considered) Not support option 1. Firstly, whether supports per-FR gap is optional, RedCap UE can choose whether supporting it or not. In addition, for the RedCap UE supporting per-FR gap and serving cell in FR1, the MOs belongs to FR2 is also possible. We see the benefits for the RedCap UE which supports per-FR gap in this scenario. |
| Apple | Issue 5-2-1: Inter-frequency without gap Option 1. Issue 5-2-2: Assumption on searcher Option 2. Issue 5-2-3: CSSF outside gap Option 1 based on conclusion in issue 5-2-1. Issue 5-2-4: CSSF within gap Option 1. Issue 5-2-5: Type of measurement gaps (if considered) Support option 1 based on single carrier capability for RedCap UE. |
| CMCC | Issue 5-2-1: Inter-frequency without gap Option 2 Issue 5-2-4: CSSF within gap OK with option 1 Issue 5-2-5: Type of measurement gaps (if considered) OK with option1 |
| MediaTek | Issue 5-2-1: Inter-frequency without gap Support Option 1. Issue 5-2-2: Assumption on searcher Support Option 2. Issue 5-2-3: CSSF outside gap Support Option 1. Issue 5-2-4: CSSF within gap Support Option 2. Issue 5-2-5: Type of measurement gaps (if considered) Support Option 1. |
| Xiaomi | Issue 5-2-1: Inter-frequency without gap Option 1. Issue 5-2-2: Assumption on searcher Option 2. Issue 5-2-3: CSSF outside gap Option 1. Issue 5-2-4: CSSF within gap Option 1. |
| ZTE | Issue 5-2-1: Inter-frequency without gap Support option 1. don’t consider necessary for RedCap UEs to support an advanced capability. Issue 5-2-2: Assumption on searcher Support option 2, this is actually reasonable under the given assumption. Issue 5-2-3: CSSF outside gap Support option 1, derived from Option 1 in issue 5-2-1. Issue 5-2-4: CSSF within gap Option 1 is preferred. |
| Nokia | Issue 5-2-1: Inter-frequency without gap We support option 1. Support of inter-frequency measurements without gap results in complexity increase for single searcher. Issue 5-2-2: Assumption on searcher We support option 2. Issue 5-2-3: CSSF outside gap We support option 1. Issue 5-2-4: CSSF within gap We support option 1. Issue 5-2-5: Type of measurement gaps (if considered) In our view, RAN4 can prioritize per-UE gap support and consider per-FR gap support afterwards for R17. |
| vivo | Issue 5-2-1: Inter-frequency without gap Support Option 1. Issue 5-2-3: CSSF outside gap Support Option 1. Issue 5-2-4: CSSF within gap Support Option 1. |

### Sub-topic 5-3 PSS/SSS detection with 1 Rx

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 5-3-1: If number of attempts are increased, how much to increase for FR1

Background: At last meeting it was agreed to increase the number of attempts for PSS/SSS detection for 1 Rx RedCap UE in FR1, see R4-2202670. Under this issue it is discussed how much (nr of attempts) to relax.

- Proposals

  - Option 1 (Apple): By 2 SMTC window as follows:

    - non-DRX delay requirement: max( 600ms, ceil( 7 x Kp) x SMTC period ) x CSSFintra

  - Option 2 (vivo, Nokia, HW, MTK, E///): 1 sample

  - Option 3 (QC): Double the number of attempts, i.e.  a total of 10 samples

    - (2-3 for AGC and 8 samples for detection)

- Recommended WF

  - Check if companies can compromise to option 2.

Issue 5-3-2: Whether to extend the lower bound in PSS/SSS detection delay in FR1 and FR2

- Proposals

  - Option 1 (Apple, vivo, Nokia, HW, MTK, E///): No

  - Option 2 (QC): Yes for FR1

    - increase the lower bound from 600ms to 1200ms

Recommended WF

- Given that number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements.

- For FR1, check if supporting company of option 2 can compromise to option 1.

Sub topic 5-3

| Company | Comments |
| --- | --- |
| XXX | Issue 5-3-1: If number of attempts are increased, how much to increase for FR1 Issue 5-3-2: Whether to extend the lower bound in PSS/SSS detection delay in FR1 and FR2 |
| Huawei | Issue 5-3-1: If number of attempts are increased, how much to increase for FR1 Option 2 based on our simulation results. Issue 5-3-2: Whether to extend the lower bound in PSS/SSS detection delay in FR1 and FR2 Option 1. |
| Ericsson | Issue 5-3-1: If number of attempts are increased, how much to increase for FR1 We support the recommend WF from the moderator. Issue 5-3-2: Whether to extend the lower bound in PSS/SSS detection delay in FR1 and FR2 We support the recommend WF from the moderator. |
| Apple | Issue 5-3-1: If number of attempts are increased, how much to increase for FR1 Option 1 based on our simulation, but if all companies agree on option 2 we can compromise to option 2. Issue 5-3-2: Whether to extend the lower bound in PSS/SSS detection delay in FR1 and FR2 Option 1 for FR1. |
| MediaTek | Issue 5-3-1: If number of attempts are increased, how much to increase for FR1 Support recommended WF. Issue 5-3-2: Whether to extend the lower bound in PSS/SSS detection delay in FR1 and FR2 Support recommended WF. |
| Nokia | Issue 5-3-1: If number of attempts are increased, how much to increase for FR1 We are fine with the recommended WF. Issue 5-3-2: Whether to extend the lower bound in PSS/SSS detection delay in FR1 and FR2 Option 1 |
| vivo | Issue 5-3-1: If number of attempts are increased, how much to increase for FR1 Option 2 based on our simulation results. Issue 5-3-2: Whether to extend the lower bound in PSS/SSS detection delay in FR1 and FR2 Option 1. |
| Qualcomm | Issue 5-3-1: If number of attempts are increased, how much to increase for FR1 We support Option 3, we still believe a duty cycle for cell detection is needed to save power. Issue 5-3-2: Whether to extend the lower bound in PSS/SSS detection delay in FR1 and FR2 We support option 2. Lower bound should be doubled which is in-line with our proposal for number of samples. We can compromise on one of these but not on both. Either number of samples or the lower bound needs to be doubled. |

### Sub-topic 5-4 time index detection with 1 Rx

Issue 5-4-1: Side condition for defining requirements in FR1

- Proposals

  - Option 1 (Nokia):

    - Define the time index detection delay in FR1 for RedCap UEs based on the side condition of -6dB

    - Define the time index detection delay in FR1 for RedCap UEs for the worst-case scenario (within the proposed channel profiles)

- Recommended WF

  - Discuss the option

Issue 5-4-2: Whether to extend time index delay in FR1 (PBCH-DMRS detection)

- Proposals

  - Option 1 (Apple, vivo, Nokia, HW, QC, E///): Yes

- Recommended WF

  - Option is recommended to be agreed.

Issue 5-4-3: If extended, how much to extend time index delay in FR1

- Proposals

  - Option 1 (Apple, vivo, HW, E///): by 1 SMTC compared to legacy requirements

  - Option 2 (Nokia): by 2 samples/SMTC

  - Option 3 (QC): by 4 samples/SMTC, i.e. 7 attempts in total

- Recommended WF

  - Discuss the options.

Issue 5-4-4: If extended, how much to extend time index delay in FR2

Background: at last meeting it was agreed that time index delay in FR2 is extended, see R4-2202670:

- Proposals

  - Option 1 (vivo): 3 samples

  - Option 2 (HW): 11 samples needed in total

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements. Thus, no discussions needed.

Issue 5-4-5: Whether to extend the lower bound in time index detection delay

- Proposals

  - Option 1 (Apple): Yes

    - Lower boundary changed to 160 ms as follows:

      - non-DRX delay requirement: max(160ms, ceil( 4 x Kp ) x SMTC period)x CSSFintra

- Recommended WF

  - Discuss the option.

Sub topic 5-4

| Company | Comments |
| --- | --- |
| XXX | Issue 5-4-1: Side condition for defining requirements in FR1 Issue 5-4-3: If extended, how much to extend time index delay in FR1 Issue 5-4-5: Whether to extend the lower bound in time index detection delay |
| Huawei | Issue 5-4-1: Side condition for defining requirements in FR1 Support option 1. Issue 5-4-2: Whether to extend time index delay in FR1 (PBCH-DMRS detection) Agree with the recommended WF. Issue 5-4-3: If extended, how much to extend time index delay in FR1 Our position is not option 1. At least 6 samples are needed for time index decoding for RedCap UE with 1Rx. As legacy required samples are 3, we support option 3. Issue 5-4-4: If extended, how much to extend time index delay in FR2 If RF has the agreement, we are fine not to discuss this issue. Issue 5-4-5: Whether to extend the lower bound in time index detection delay Depends on conclusion of above issues. |
| Ericsson | Issue 5-4-1: Side condition for defining requirements in FR1 We can compromise to option 1. Issue 5-4-3: If extended, how much to extend time index delay in FR1 Although we proposed option 1, we can also compromise to option 2. Issue 5-4-5: Whether to extend the lower bound in time index detection delay We are fine to extend the lower bound to align with the number of samples extended for the delay requirement. |
| Apple | Issue 5-4-1: Side condition for defining requirements in FR1 Fine with option 1. Issue 5-4-2: Whether to extend time index delay in FR1 (PBCH-DMRS detection) Option 1. Issue 5-4-3: If extended, how much to extend time index delay in FR1 Option 1 based on our simulation. Issue 5-4-5: Whether to extend the lower bound in time index detection delay Option 1. |
| MediaTek | Issue 5-4-2: Whether to extend time index delay in FR1 (PBCH-DMRS detection) Support recommended WF. Issue 5-4-5: Whether to extend the lower bound in time index detection delay It can be extended to follow the agreement from issue 5-4-3. Fine with Option 1. |
| Nokia | Issue 5-4-1: Side condition for defining requirements in FR1 Option 1, our intention with the proposal is that the results from different companies are evaluated under the same conditions. Issue 5-4-3: If extended, how much to extend time index delay in FR1 Option 2, based on our simulation results. We have a question to Qualcomm on their contribution. In FR1, our understanding is that only the PBCH DMRS is needed to detect the time index. Looking at Table 1 in R4-2206113 (and observation 5), only 5 attempts would be needed for 99% PBCH decoding rate at -6dB. The question is: why is the proposal considering PBCH decoding rate for the time index detection in FR1? If we consider Observation 5, 5 attempts would be needed, and only 2 samples/SMTC extensions compared to current requirements, which would be the same as Option 2. Issue 5-4-5: Whether to extend the lower bound in time index detection delay It depends on the conclusion of Issue 5-4-3, but we would be OK with Option 1 proposed by Apple. |
| vivo | Issue 5-4-1: Side condition for defining requirements in FR1 Agree with option 1. Issue 5-4-2: Whether to extend time index delay in FR1 (PBCH-DMRS detection) Agree with the recommended WF. Issue 5-4-3: If extended, how much to extend time index delay in FR1 Option 1 based on our simulation results. |
| Qualcomm | Issue 5-4-1: Side condition for defining requirements in FR1 Fine with option 1. Issue 5-4-3: If extended, how much to extend time index delay in FR1 Support option 3 based on our simulation Issue 5-4-5: Whether to extend the lower bound in time index detection delay Support option 1. But the extension should correspond to the number of samples increased. |

### Sub-topic 5-5 SSB based L3 measurement with 1 Rx

Sub-topic description

All simulation results show that measurement performance is impacted with 1 Rx compared to 2 Rx. Discuss how to define the 1 Rx requirements.

Issue 5-5-1: Measurement period (# of samples) for 1 Rx requirements for SSB based L3 measurement in FR1

- Proposals

  - Option 1 (ZTE, vivo, Nokia, MTK, E///): Keep measurement period same as Rel-15

- Recommended WF

  - Option 1 is recommended to be agreed.

Issue 5-5-2: Measurement period for 1 Rx requirements for SSB based L3 measurement in FR2

- Proposals

  - Option 1 (ZTE, HW): Keep measurement period same as Rel-15

  - Option 2 (Apple): Only lower bound is extended while keeping the same number of samples.

    - Lower bound extended to 800 ms as follows:

      - max(800ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period) x CSSFintra for FR2

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements.

Issue 5-5-3: Whether lower bound in measurement period for 1 Rx requirements is extended in FR1

- Proposals

  - Option 2 (Apple): Only lower bound is extended while keeping the same number of samples.

    - Lower bound extended to 400 ms as follows:

      - max(400ms, ceil( 5 x Kp) x SMTC period) x CSSFintra

- Recommended WF

  - Discuss the option.

Issue 5-5-4: Relaxation of accuracy levels for FR1

- Proposals

  - Option 1 (vivo, QC, MTK): 1.5 dB compared to legacy requirements for 2 Rx UE

  - Option 2 (Nokia, HW, E///, Apple):

    - Absolute accuracy by 1 dB compared to legacy requirements for 2 Rx UE

    - Relative accuracy by 1 dB compared to legacy requirements for 2 Rx UE

- Recommended WF

  - Discuss the option.

Issue 5-5-5: Relaxation of accuracy levels for FR2

- Proposals

  - Option 2 (Nokia): 1 dB compared to legacy requirements for 2 Rx UE

  - Option 3 (vivo): 1.5 dB compared to legacy requirements for 2 Rx UE

  - Option 5 (Apple):

    - Absolute accuracy by 1 dB compared to legacy requirements for 2 Rx UE

    - Relative accuracy by 1 dB compared to legacy requirements for 2 Rx UE

- Recommended WF

Given that number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements.

Issue 5-5-6: Whether legacy RF margin can be considered for RedCap for FR1

- Proposals

  - Option 1 (vivo, E///): Use same RF margin as in Rel-15 NR for RSRP accuracy requirements.

    - Option 1a (MTK): Don’t support because the RF margin is already captured in the absolute accuracy.

- Recommended WF

  - The options for issue 5-5-4 includes relaxation proposals for 1 Rx UE compared to legacy requirements for 2 Rx UE. This means RF margin assumed for 1 Rx UE is same as for 2 Rx. Following this method for defining accuracy requirements for 1 Rx UE, there no need to discuss RF margin for 1 Rx UE separately.

Sub topic 5-5

| Company | Comments |
| --- | --- |
| XXX | Issue 5-5-3: Whether lower bound in measurement period for 1 Rx requirements is extended in FR1 Issue 5-5-4: Relaxation of accuracy levels for FR1 |
| Huawei | Issue 5-5-1: Measurement period (# of samples) for 1 Rx requirements for SSB based L3 measurement in FR1 Fine with recommended WF. Issue 5-5-2: Measurement period for 1 Rx requirements for SSB based L3 measurement in FR2 If RF has the agreement, we are fine not to discuss this issue. Issue 5-5-3: Whether lower bound in measurement period for 1 Rx requirements is extended in FR1 As the sample number keep unchanged for L3 measurement, it is straight forward not to extend the lower bound. We understand the motivation of option 1 is to power saving, we are open to this. Issue 5-5-4: Relaxation of accuracy levels for FR1 Support option 2. |
| Ericsson | Issue 5-5-3: Whether lower bound in measurement period for 1 Rx requirements is extended in FR1 If measurement period is unchanged, then we don’t see any reason to extend the lower bound. Issue 5-5-4: Relaxation of accuracy levels for FR1 Our simulation results show that 1 dB relaxation compared to 2 Rx UE is sufficient for 1 Rx UE. |
| Apple | Issue 5-5-1: Measurement period (# of samples) for 1 Rx requirements for SSB based L3 measurement in FR1 Agree with option 1. Issue 5-5-3: Whether lower bound in measurement period for 1 Rx requirements is extended in FR1 Option 2 to consider the relaxed duty cycle like what RAN4 did in LTE cat-1bis. Issue 5-5-4: Relaxation of accuracy levels for FR1 Option 2 based on our simulation. Issue 5-5-6: Whether legacy RF margin can be considered for RedCap for FR1 Fine with recommended WF. |
| MediaTek | Issue 5-5-1: Measurement period (# of samples) for 1 Rx requirements for SSB based L3 measurement in FR1 Support recommended WF. Issue 5-5-3: Whether lower bound in measurement period for 1 Rx requirements is extended in FR1 Fine to support option 2. Issue 5-5-4: Relaxation of accuracy levels for FR1 Support Option 1: relax absolute and relative accuracy by 1.5 dB compared to legacy requirements for 2 Rx UE. |
| ZTE | Issue 5-5-1: Measurement period (# of samples) for 1 Rx requirements for SSB based L3 measurement in FR1 Fine with recommended WF (Option 1). Issue 5-5-2: Measurement period for 1 Rx requirements for SSB based L3 measurement in FR2 Option 1, support the recommended WF. |
| Nokia | Issue 5-5-3: Whether lower bound in measurement period for 1 Rx requirements is extended in FR1 Our proposal is not to extend the measurement period, so we do not see reason for extending the lower bound in measurement period for 1Rx requirements in FR1. Issue 5-5-4: Relaxation of accuracy levels for FR1 Option 1, based on our simulation results. |
| vivo | Issue 5-5-1: Measurement period (# of samples) for 1 Rx requirements for SSB based L3 measurement in FR1 Fine with recommended WF. Issue 5-5-3: Whether lower bound in measurement period for 1 Rx requirements is extended in FR1 We understand that the sample number for L3 measurement is not increased for 1Rx. Therefore, it may be no need to extend the lower bound for SSB based measurement. Issue 5-5-4: Relaxation of accuracy levels for FR1 Support option 1 based on our simulation results. Issue 5-5-6: Whether legacy RF margin can be considered for RedCap for FR1 Support recommended WF. |
| Qualcomm | Issue 5-5-1: Measurement period (# of samples) for 1 Rx requirements for SSB based L3 measurement in FR1 Fine with option 1. Issue 5-5-3: Whether lower bound in measurement period for 1 Rx requirements is extended in FR1 Fine with option 2 Issue 5-5-4: Relaxation of accuracy levels for FR1 Support option 2. Also, we think the legacy requirements were defined with AWGN channel. If that’s the case, then why do we need to consider the relaxations for fading channel. Note that the test cases are defined with AWGN channels only. If you look at the following note in Table 10.1.19.1.1-1 NOTE 1: Io is assumed to have constant EPRE across the bandwidth. Fading channels won’t have constant EPRE across bandwidth, and hence they should be precluded. We think the accuracy requirements should be defined with AWGN channels Issue 5-5-6: Whether legacy RF margin can be considered for RedCap for FR1 Fine with recommended WF. |

### Sub-topic 5-6 Measurement conditions for HD-FDD UE

Issue 5-6-1: Priority between UL and DL during cell identification and measurement for HD-FDD

- Proposals

  - Option 1 (Apple, Xiaomi, vivo, HW, MTK, OPPO): RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in cell identification and measurement requirement.

  - Option 2 (CMCC, E///): No measurement period relaxation or prioritization between measurement and dynamically scheduled UL transmission are needed. Clarification on available samples can be considered:

      - “The UE shall meet the current requirements on cell identification (PSS/SSS detection defined in Table x.y.z for FR1 and FR2) provided that at least 5 SMTC windows are available at the UE during cell identification time.”

      - “The UE shall meet the current requirements on time index detection (defined in Table x.y.z for FR1 and FR2) provided that at least 3 SMTC windows are available at the UE during time index detection.”

      - “The UE shall meet the current requirements intra-frequency/inter-frequency measurement (defined in Table x.y.z for FR1 and FR2) provided that at least 5 SMTC windows are available at the UE during measurement period.”

- Recommended WF

  - Check if companies can compromise to option 1.

Issue 5-6-2: Scheduling availability for HD-FDD UE in CONNECTED mode

Proposals

  - Option 1 (MTK):

      - Support introducing scheduling availability restriction on 5G NR RedCap UEs performing measurements in HD-FDD bands in CONNECTED mode.

      - Support clarifying that the scheduling restriction is for intra-frequency and inter-frequency measurements in CONNECTED mode in HD-FDD bands.

- Recommended WF

Discuss the option.

Issue 5-6-3: Measurement period relaxation for HD-FDD

Proposals

  - Option 1 (HW): Measurement period relaxation purely due to HD-FDD is not considered and some clarification on available samples can be made.

- Recommended WF

Following was agreed last meeting, see R4-2202670:

“Measurement period relaxation purely due to HD-FDD is not considered and some clarification on available samples can be made.”. Thus, no more discussions needed.

Sub topic 5-6

| Company | Comments |
| --- | --- |
| XXX | Issue 5-6-1: Priority between UL and DL during cell identification and measurement for HD-FDD Issue 5-6-2: Scheduling availability for HD-FDD UE in CONNECTED mode |
| Huawei | Issue 5-6-1: Priority between UL and DL during cell identification and measurement for HD-FDD Option 1. In RAN1 the following agreement is made, Agreements: - For HD-FDD operation for RedCap UEs, collisions may be addressed or alleviated with proper scheduling. The following cases of potential collisions can be further studied to see if any change to the current specs is necessary: - Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS. In RAN1#107e meeting, the following agreement was made, Agreement: - For Case 5 of dynamically scheduled UL transmission vs. SSB, support Option 2 at least for dynamically scheduled UL transmission other than Msg3 (re)transmission and PUCCH for Msg4 - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission. It shall be noted that in RAN1 discussion the SSB refers to the SSB in serving cell. From RRM point of view, measurements are based on SMTC which is configured per frequency layer. It is suggested that RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in cell identification and measurement requirement. Issue 5-6-3: Measurement period relaxation for HD-FDD Fine with the recommended WF. |
| Ericsson | Issue 5-6-1: Priority between UL and DL during cell identification and measurement for HD-FDD We don’t understand why the method of defining HD-FDD requirements from LTE cannot be reused for RedCap HD-FDD. In HD-FDD requirements for cat-M in CONNECTED, the requirements are identical to FDD with some conditions of DL subframe availability as follows [TS 36.133 V17.4.0]: “The requirements defined in clause 8.13.2.1.1.1 also apply for this section provided the following conditions are met: - at least downlink subframe # 0 or downlink subframe # 5 per radio frame of an intra-frequency cell to be identified by the UE is available at the UE over Tidentify_intra_UE cat M1; - at least one downlink subframe per radio frame of measured cell is available at the UE for RSRP measurement assuming measured cell is identified cell over Tmeasure_intra_UE cat M1.” Except for the conditions above, rest is up to UE implementation. We haven’t heard any technical reason why this approach cannot be reused instead of option 1 which is very strict in the sense DL is always prioritize during the entire cell identification period and measurement period. Issue 5-6-2: Scheduling availability for HD-FDD UE in CONNECTED mode If one of the options in issue 5-6-1 is agreed, then we don’t see any need to introduce scheduling availability. |
| Apple | Issue 5-6-1: Priority between UL and DL during cell identification and measurement for HD-FDD Option 1. Like scheduling restriction of TDD case in FR1, the RRM measurement is prioritized over UL data/control. Issue 5-6-2: Scheduling availability for HD-FDD UE in CONNECTED mode Some question to confirm before agreeing on option 2: - For 1st bullet, is this scheduling restriction used to DL data/control reception when performing DL measurement? e.g., SSB and DL data has different SCS and UE cannot support mixed numerology. - For 2nd bullet, if inter-frequency measurement is always with MG (up to Issue 5-2-1), no scheduling restriction shall be considered. Issue 5-6-3: Measurement period relaxation for HD-FDD Fine with recommended WF. |
| CMCC | Issue 5-6-1: Priority between UL and DL during cell identification and measurement for HD-FDD OK with option1 Issue 5-6-2: Scheduling availability for HD-FDD UE in CONNECTED mode No additional restriction due to HD-FDD is needed |
| MediaTek | Issue 5-6-1: Priority between UL and DL during cell identification and measurement for HD-FDD Support Option 1. Issue 5-6-2: Scheduling availability for HD-FDD UE in CONNECTED mode This issue to follow the scheduling handling principles of rel-15/16 for NR TDD, which can be reused in here. Also, please refer to the RAN1 agreement provided by Huawei in issue 5-6-1. Hence, we support Option 1. |
| Xiaomi | Issue 5-6-1: Priority between UL and DL during cell identification and measurement for HD-FDD Option 1. |
| Nokia | Issue 5-6-1: Priority between UL and DL during cell identification and measurement for HD-FDD We are fine with the recommended WF proposed by the moderator. Issue 5-6-2: Scheduling availability for HD-FDD UE in CONNECTED mode |
| vivo | Issue 5-6-1: Priority between UL and DL during cell identification and measurement for HD-FDD Support Option 1. Issue 5-6-3: Measurement period relaxation for HD-FDD Agree with Recommended WF. |

### Sub-topic 5-7 CGI reading

Issue 5-7-1: Whether to define CGI identification of an E-UTRAN cell with autonomous gaps for RedCap

- Proposals

  - Option 1 (E///): 	Yes

- Recommended WF

  - Check if option 1 can be agreed.

Issue 5-7-2: If CGI identification of an E-UTRAN cell with autonomous gaps for RedCap, the requirements:

- Proposals

  - Option 1 (E///): 	RAN4 to reuse existing cat-1bis CGI reading requirement in LTE.

- Recommended WF

  - Check if option 1 can be agreed.

Issue 5-7-3: PBCH decoding delay for CGI reading for 1 Rx in FR1

- Proposals

  - Option 1 (HW, E///): The MIB decoding delay requirement of 1Rx RedCap UE can be the same as non-RedCap UE for SNR=-3dB.

- Recommended WF

  - Option 1 is recommended to be agreed.

Issue 5-7-4: PBCH decoding delay for CGI reading for 1 Rx in FR2

- Proposals

  - Option 1 (HW): 	The MIB decoding delay requirement in FR2 shall be increased to 5 samples (without considering RX beam sweeping);

- Recommended WF

  - Given that RAN4 RF session had agreed that the number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements.

Issue 5-7-5: SIB1 decodign delay for CGI reading

- Proposals

  - Option 1 (E///): 	6 samples are needed for 1Rx RedCap UE to achieve the SIB1 90% successful rate.

- Recommended WF

  - Check if option 1 can be agreed.

Issue 5-7-6: Assistance information for CGI reading

- Proposals

  - Option 1 (E///): 	If indicated by network, UE will further report the NCD-SSB information (such as SSB-frequency, SCS etc.) together with global cell ID when UE reporting the CGI.

- Recommended WF

  - Check if option 1 can be agreed.

Sub topic 5-7

| Company | Comments |
| --- | --- |
| XXX | Issue 5-7-1: Whether to define CGI identification of an E-UTRAN cell with autonomous gaps for RedCap Issue 5-7-2: If CGI identification of an E-UTRAN cell with autonomous gaps for RedCap, the requirements: Issue 5-7-5: SIB1 decodign delay for CGI reading Issue 5-7-6: Assistance information for CGI reading |
| Huawei | Issue 5-7-1: Whether to define CGI identification of an E-UTRAN cell with autonomous gaps for RedCap Fine with option 1. Issue 5-7-2: If CGI identification of an E-UTRAN cell with autonomous gaps for RedCap, the requirements: Fine with option 1. Issue 5-7-3: PBCH decoding delay for CGI reading for 1 Rx in FR1 Support option 1. Issue 5-7-5: SIB1 decoding delay for CGI reading More samples are required. Based on our below simulation, 12 samples are needed for 1Rx RedCap UE to achieve the SIB1 90% successful rate. Table 1. SIB1 decoding performance for 1Rx Channel; SIB1 decoding success rate. 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12. 1Rx AWGN (single sample); 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. 1Rx TDLC300 (single sample); 0.05; 0.10; 0.15; 0.19; 0.23; 0.27; 0.31; 0.35; 0.38; 0.41; 0.44; 0.47. 1Rx AWGN (Soft-combining every 2 samples); 0.98; 1; 1; 1; 1; 1. 1Rx TDLC300 (Soft-combining every 2 samples); 0.34; 0.56; 0.71; 0.81; 0.87; 0.91. Issue 5-7-6: Assistance information for CGI reading Depends on RAN2 conclusion. |
| Ericsson | Issue 5-7-1: Whether to define CGI identification of an E-UTRAN cell with autonomous gaps for RedCap Option 1. In Rel-16, RAN4 defined the following two type of CGI reading: - CGI identification of an NR cell with autonomous gaps - CGI identification of an E-UTRA cell with autonomous gaps Thus, it’s naturally to define the similar requirements for CGI reading. Issue 5-7-2: If CGI identification of an E-UTRAN cell with autonomous gaps for RedCap, the requirements: Option 1, Issue 5-7-3: PBCH decoding delay for CGI reading for 1 Rx in FR1 Option 1, Issue 5-7-4: PBCH decoding delay for CGI reading for 1 Rx in FR2 Agree with recommended WF. RAN4 RF session had agreed that the number of Rx branches is not reduced for FR2, no discussions needed for FR2 1 Rx requirements. Issue 5-7-5: SIB1 decoding delay for CGI reading Option 1 based our simulation results. Issue 5-7-6: Assistance information for CGI reading Option 1. NCD-SSB will be used for L3 measurement, L1 measurement and mobility for RedCap UE. It’s important for serving cell to know the information to further configure the L3 measurements or handover command with NCD-SSB to UEs. In addition, when serving cell manages the newly detected neighbour cells for ANR, it’s also beneficial to know whether NCD-SSB is transmitted in the neighbour cells. Therefore, we propose that if indicated by network, UE will further report the NCD-SSB information (such as SSB-frequency, SCS etc.) together with global cell ID when UE reporting the CGI, such as in the same SIB1 message. |
| Apple | Issue 5-7-6: Assistance information for CGI reading We think such SON functionality of blindly searching NCD-SSB of neighbor cell is not in the scope of RedCap WI. |
| CMCC | Issue 5-7-1: Whether to define CGI identification of an E-UTRAN cell with autonomous gaps for RedCap OK with recommended WF Issue 5-7-2: If CGI identification of an E-UTRAN cell with autonomous gaps for RedCap, the requirements: OK with recommended WF Issue 5-7-3: PBCH decoding delay for CGI reading for 1 Rx in FR1 OK with recommended WF Issue 5-7-6: Assistance information for CGI reading Need further check |
| MediaTek | Issue 5-7-1: Whether to define CGI identification of an E-UTRAN cell with autonomous gaps for RedCap Support recommended WF. Issue 5-7-2: If CGI identification of an E-UTRAN cell with autonomous gaps for RedCap, the requirements: Support recommended WF. Issue 5-7-6: Assistance information for CGI reading This should be left to RAN2 discussion. |
| Nokia | Issue 5-7-1: Whether to define CGI identification of an E-UTRAN cell with autonomous gaps for RedCap We support option 1. Issue 5-7-2: If CGI identification of an E-UTRAN cell with autonomous gaps for RedCap, the requirements: We support option 1. Issue 5-7-5: SIB1 decodign delay for CGI reading We support option 1. Issue 5-7-6: Assistance information for CGI reading This aspect is in the RAN2 scope. |
| Qualcomm | Issue 5-7-1: Whether to define CGI identification of an E-UTRAN cell with autonomous gaps for RedCap We support option 1. Issue 5-7-2: If CGI identification of an E-UTRAN cell with autonomous gaps for RedCap, the requirements: We support option 1. Issue 5-7-3: PBCH decoding delay for CGI reading for 1 Rx in FR1 We think more samples may be needed. Suggest FFS Issue 5-7-5: SIB1 decoding delay for CGI reading We think more samples may be needed. Suggest FFS Issue 5-7-6: Assistance information for CGI reading Upto RAN2 |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2206087 (MediaTek Inc.) | Title: DraftCR on reduced capability Ues for general measurements and intra-frequency |
|  | Ericsson: - Change marks should be enabled in the CR to better understand the changes. - Text related to Nfreq configured by LPP should be removed since there is no support for LPP. - Good to add “_RedCap” in all parameter names to differentiate from legacy parameters. - Why don’t we use instead of ? The former one is used in legacy requirements. - in 9.1A.5, not sure whether there is any agreement to have CSI-RS based L3 measurement. - The neighbour cell measurement based on NCD-SSB needs to be updated based on the outcome of discussion in the meeting. - Following sentence is not needed since there is no agreement to support SRS carrier based switching: “The requirements in this clause shall also apply, when the UE is configured to perform SRS carrier based switching and using measurement gaps.” - All changes related to 1 Rx in FR2 can be removed since number fo Rx branches is not reduced for FR2. - Better to use “intra-frequency” instead of “intrafrequency” - Conditions related to HD-FDD UE to meet the requirements need to be added. |
| R4-2206088 (MediaTek Inc.) | Title: DraftCR on reduced capability Ues for inter-frequency and inter-RAT measurements |
|  | Ericsson - Change marks should be enabled. - The neighbour cell measurement based on NCD-SSB needs to be updated based on the outcome of discussion in the meeting. - Also inter-frequency without gap needs to be updated based on the outcome - Text related to SRS based switching needs to be removed since there is no agreement to support those - All changes related to 1 Rx in FR2 can be removed since number fo Rx branches is not reduced for FR2. FOR FR2, requirements can be referred to existing requirements but conditions related to HD-FDD needs to be updated. - Section 9.4A.4, it depends on whether to support those as being discussed under CGI. As per work split, if agreed it should be covered by Nokia’s CR. |
|  | Nokia: On top of Ericsson’s comments, we have an editorial comment: some occurrences of Mmeas_period with_gaps (and others) in the tables the RedCap subscript is missing. |
| R4-2204537 (Nokia, Nokia Shanghai Bell) | Title: Draft CR – Introducing L1-RSRP requirements for RedCap UEs |
|  | Ericsson - All changes related to 1 Rx in FR2 can be removed since number fo Rx branches is not reduced for FR2. FOR FR2, requirements can be referred to existing requirements but conditions related to HD-FDD needs to be updated. - Separate table for 1 Rx and 2 Rx preferred. For 2 Rx, it can point to existing requirements. |
| R4-2205511 (Ericsson) | Title: draftCR on inter-RAT NR measurement for RedCap |
|  | Nokia Only a minor comment so far: In some occurrences of Mpss/sss_sync_irat (and others) in the tables the _RedCap subscript is missing |
| R4-2205938 (Nokia, Nokia Shanghai Bell) | Title: Introduction of RedCap UE in clause 9.11A |
|  | Ericsson - Good to add “_RedCap” in all parameter names to differentiate from legacy parameters. - All changes related to 1 Rx in FR2 can be removed since number fo Rx branches is not reduced for FR2. FOR FR2, requirements can be referred to existing requirements. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic#5-1 | Issue 5-1-1: The measurement scenarios for NCD-SSB and CD-SSB - Agreements - Define RedCap UE’s measurement requirements based on the following scenarios: - Case A: Serving cell active BWP includes CD-SSB - Case B: Serving cell active BWP includes NCD-SSB - Case B-1: All neighbour cells include NCD-SSB on the same frequency location as serving cell NCD-SSB/[CD-SSB] - FFS whether to support Case B-2 - Case B-2: Some neighbour cells include NCD-SSB, and some neighbour cells without NCD-SSB on the same frequency location as serving cell NCD-SSB/CD-SSB - Note: if the scenario is supported then no new requirements or minimum changes shall be introduced comparing to Case A and B-1 requirements Issue 5-1-2: Whether to define neighbour cell measurement requirements for NCD-SSB Tentative agreements: RAN4 to define requirements for RRM measurement on neighbour cell based on CD-SSB or NCD-SSB for the agreed scenarios in issue 5-1-1. Issue 5-1-3: Reference SSB to decide measurement type (intra- or inter-frequency) - Agreements - FFS: Reference SSB to decide measurement type (intra- or inter-frequency) - Option 1 (E///, QC, ZTE): NW indicates the reference SSB (CD-SSB or NCD-SSB) - Option 1a (Apple, Nokia, CMCC): The SSB indicated in serving cell MO is used as reference SSB (CD-SSB or NCD-SSB) - Option 2 (Xiaomi, MTK, Nokia, QC, vivo): SSB in the active BWP is used (CD-SSB or NCD-SSB) - Option 3 (HW, CMCC): CD-SSB of the serving cell Recommendations for 2nd round: Continue the discussions based on the agreed options above. Check also if option 1 and 1a can be merged. Issue 5-1-4: Definition of SSB based intra-frequency measurement Candidate options: - Proposals: A measurement is defined as an SSB based intra-frequency when: - Option 1 (E///): - If NW configures NCD-SSB of the serving cell indicated for measurement: - the centre frequency of the NCD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same and the subcarrier spacing of the two SSBs are also the same. - Otherwise: - the centre frequency of the CD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same and the subcarrier spacing of the two SSBs are also the same. - Option 2 (Apple, CATT, vivo, MTK): - the centre frequency of the CD-SSB or NCD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same, and - the subcarrier spacing of the two SSBs are also the same. - Option 3 (CMCC): - the centre frequency of the CD-SSB of the serving cell and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same. - the subcarrier spacing of the two SSBs are also the same. - Option 4 (HW): - the centre frequency of the CD-SSB of the serving cell indicated for measurement and the centre frequency of the CD-SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. - Option 5 (Xiaomi): - The legacy definition of intra-frequency and inter-frequency based on the reference SSB could be reused. - Option 6 (QC): - A measurement is defined as an SSB based intra-frequency measurement when - one of the NCD-SSB or CD-SSB of the serving cell indicated for measurement lies in the active BWP of the UE and - the centre frequency of this SSB (that lies within the active BWP of the UE) and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same and - the subcarrier spacing of the two SSBs are also the same. Recommendations for 2nd round: Since the definition of SSB based intra-frequency measurement depends on the reference SSB to use, it is recommended to focus on issue 5-1-3 and reach an agreement for that issue. Once that is agreed, it is easier to agree on the measurement definition. Issue 5-1-5: When both CD-SSB and NCD-SSB are configured for serving cell measurements - Option 1 (E///): UE can perform serving cell measurements based on NCD-SSB within active BWP provided that - the difference of center frequency between NCD-SSB and CD-SSB is no larger than 20MHz in FR1 and 100MHz in FR2 - the difference of reception power between NCD-SSB and CD-SSB is less than 3dB - the periodicity of NCD-SSB and CD-SSB is the same. Otherwise, UE should perform serving cell measurements based on both NCD-SSB and CD-SSB. - Option 2 (vivo, CMCC, QC, HW, Apple, MTK, Xiaomi, Nokia): UE should perform serving cell measurements based on SSB with active BWP. Recommendations for 2nd round: Further discuss the issue based on the progress in Issue 5-1-3, 5-1-4. Issue 5-1-6: When both CD-SSB and NCD-SSB are configured for serving cell measurements and both require MG Candidate options: When both CD-SSB and NCD-SSB serving cell measurement are configured and both need MG: - Proposals - Option 1 (Apple, CMCC, HW, MTK, Xiaomi): UE could choose to perform CD-SSB only if both NCD-SSB and CD-SSB measurements need gap. - Option 2 (Nokia, E///): UE shall follow the NW’s configuration to perform measurement. Recommendation for 2nd round: Continue the discussion based on the proposal above. Check if following compromise proposal based on 1st round comments is agreeable: - Option 3(possible compromise): When both NCD-SSB and CD-SSB measurements need gap, UE could choose to perform any SSB only provided that - the difference of center frequency between NCD-SSB and CD-SSB is no larger than 20MHz in FR1 and 100MHz in FR2 - the difference of reception power between NCD-SSB and CD-SSB is less than 3dB - the periodicity of NCD-SSB and CD-SSB is the same. Otherwise, UE should perform serving cell measurements based on both NCD-SSB and CD-SSB. Issue 5-1-7: When both CD-SSB and NCD-SSB are configured for neighbour cell measurements When both CD-SSB and NCD-SSB neighbour cell measurement are configured: - Proposals - Option 1 (E///, Apple, CMCC, Xiaomi, Nokia, vivo): UE should follow NW’s configuration to perform measurements on both SSBs. - Option 2 (QC): - When both, NCD-SSB and CD-SSB of the neighbour cell lie within the UE active BWP, UE should be required to measure only one SSB. - When both, NCD-SSB and CD-SSB of the neighbour cell lie outside the UE active BWP, UE should be required to measure only CD-SSB. - When either NCD-SSB or CD-SSB lie within the UE active BWP, UE should be required to measure the SSB that lies within the UE active BWP. - Option 2a (MTK): - When both, NCD-SSB and CD-SSB of the neighbour cell lie outside the UE active BWP, UE should be required to measure only CD-SSB. - When either NCD-SSB or CD-SSB lie within the UE active BWP, UE should be required to measure the SSB that lies within the UE active BWP. Recommendation for 2nd round: Check if companies can compromise to option 1: - When both CD-SSB and NCD-SSB neighbour cell measurement are configured, UE should follow NW’s configuration to perform measurements on both SSBs. Issue 5-1-8: Neighbour cell’s NCD-SSB information - Option 1 (Apple, QC, E///, MTK, Xiaomi, Nokia, vivo, QC): - The neighbor cell’s NCD-SSB information (frequency/SCS) shall be provided to UE if UE is configured to perform cell identification/measurement on neighbor cell’s NCD-SSB, i.e., UE is not required to read neighbor cell SIB to figure out the neighbor cell’s NCD-SSB by itself. - Option 2 (HW, CMCC): Wait for RAN2 conclusion Recommendation for 2nd round: Given the very limited time left until completion of core-part, waiting for RAN2 shall be avoided unless there is any strong dependency. Can supporting companies of option 2 compromise to option 1? Issue 5-1-9: Delay requirements for NCD-SSB based measurement Recommendation for the 2nd round: Given that scenarios were agreed in the issue 5-1-1, discuss the requirements for those. Measurement delay requirements are introduced for RedCap UE for the scenarios agreed in issue 5-1-1, and the options are: - Option 1 (vivo, Apple, E///): Check and define new requirements if needed for scenarios agreed in issue 5-1-1 when: - Cell identification and measurement by NCD-SSB - Cell identification and measurement when both NCD-SSB and CD-SSB are configured - Option 2 (QC, Nokia, Xiaomi, MTK, CMCC) : Current requirements apply, no addition requirements are introduced. Issue 5-1-11: Periodicity of NCD-SSB - Proposals - Option 1 (Apple, MTK, Xiaomi, vivo, QC): RAN4 to assume that periodicity of NCD-SSB is same as periodicity of corresponding CD-SSB from the same cell. - Option 2 (QC, E///, Apple, MTK, vivo): The periodicity of NCD-SSB cannot be configured to be greater than 160ms. - Option 3 (CMCC): NCD-SSB periodicity is up to NW. Since this is a common understanding among the companies: Recommendation for the 2nd round: Discuss if companies can compromise to option 2. Postpone the discussion for option 1. To Option 1, from moderator’s view, it is suggested to postpone the discussion since RAN2 is discussing the same issue and there is not relevant to define final requirements. Issue 5-1-14: Reporting of RS type (NCD-SSB or CD-SSB) as part of RRM measurement reporting - Option 1 (E///): RedCap UE needs to report the RRM measurement result together with the type of RS, either NCD-SSB or CD-SSB. - Option 2 (vivo, HW, Apple, MTK, Xiaomi, Nokia, QC): Up to RAN1/RAN2 - Option 3 (E///): Discuss after the definition of intra-frequency measurement is settled. Recommendation for 2nd round: No discussion needed until issue 5-1-4 is resolved. Issue 5-1-15: New NCD-SSB capability for non-RedCap UE - Option 1 (E///): In Rel-17, Non-RedCap UE may support the new capability of NCD-SSB measurement. - Option 2 (Apple, CMCC, MTK, Nokia, vivo, QC): No need to discuss/not in scope of RedCap WI Tentative agreement: No need to discuss whether new NCD-SSB capability is to be introduced for non-RedCap UE. |
| Sub-topic#5-2 | Issue 5-2-1: Inter-frequency without gap - Option 1 (ZTE, Apple, Xiaomi, vivo, MTK, OPPO, E///, Nokia): RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability in Rel-17. - Option 1a (E///): - RedCap UE won’t support ‘Inter-frequency without gap’ measurement capability in Rel-17 provided that the intra-frequency measurement is defined based on the NCD-SSB in active BWP, if NW transmits the NCD-SSB. - Option 2 (CMCC, HW): RAN4 needs to consider ‘inter-frequency without MG’ capability when defining RedCap RRM requirements. Recommendation for 2nd round: Check if companies can compromise to option 1. Issue 5-2-2: Assumption on searcher - Option 1 (CMCC, HW): The searcher is shared by intra-frequency without gap and inter-frequency without gap measurement for RedCap UE: - Option 2 (ZTE, MTK, Apple, Xiaomi, Nokia): The searcher will be exclusively used by intra-frequency without gap measurement provided that RAN4 agrees that RedCap UE does NOT support ‘Inter-frequency without gap’ measurement capability in Rel-17. - Option 3 (MTK): - The searcher will be exclusively used by intra-frequency without gap measurement provided that RAN4 agrees that RedCap UE does NOT support ‘Inter-frequency without gap’ measurement capability in Rel-17. Recommendation for 2nd round: Check if companies can compromise to option 2. Issue 5-2-3: CSSF outside gap - Option 1 (ZTE, Apple, Xiaomi, vivo, MTK, OPPO, Nokia): CSSFoutside_gap,I = 1 for RedCap UE measurement outside gap based on Rel-15 requirement. - Option 1a (E///): CSSFoutside_gap,I = 1 for RedCap UE measurement outside gap based on Rel-15 requirement provided that only one of NCD-SSB and CD-SSB within the serving cell’s active BWP will be measured. - Option 3 (CMCC, HW): - CSSFoutside_gap,I PCC= 2, if configured inter-frequency mOs without MG when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap that are being measured outside of MG for RedCap UE. - CSSFoutside_gap,I PCC = 1 otherwise - CSSFoutside_gap,I PCC = 2*Y, for inter-frequency MO with no measurement gap, Y is the number of configured inter-frequency mOs without MG when none of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap that are being measured outside of MG for RedCap UE; - CSSFoutside_gap,I PCC = 0 otherwise Recommendation for 2nd round: Since option 1 and 1a are not conflicting, check if companies can agree to option 1. Issue 5-2-4: CSSF within gap - Option 1 (ZTE, Apple, Xiaomi, vivo, CMCC, OPPO, HW, Nokia): The current design for CSSF within gap could be reused for RedCap UE. - Option 1a (MTK): The current design for CSSF within gap could be reused for RedCap UE provided that the definition of intra-frequency measurement for RedCap UE is as follow. - the centre frequency of the CD-SSB or NCD-SSB of the serving cell indicated for measurement and the centre frequency of the target SSB of the neighbour cell indicated for measurement are the same. - the subcarrier spacing of the two SSBs are also the same. - Option 2 (CMCC, E///, MTK): Consider to add additional two values of measGapSharingScheme factor for RedCap UE, e.g. 85%, 95%. Tentative agreement The current design for CSSF within gap could be reused for RedCap UE. Recommendation for 2nd round: Given that option 1 is a tentative agreement, companies to provide view on option 2: - Option 2 (CMCC, E///, MTK): Consider to add additional two values of measGapSharingScheme factor for RedCap UE, e.g. 85%, 95%. Issue 5-2-5: Type of measurement gaps (if considered) - Option 1 (MTK, Apple, CMCC): If MG is needed, RAN4 to specify per-UE MG based cell identification/measurement requirement regardless of independentGapConfig. - Option 2 (E///): RedCap UE supports per-FR gap. - Option 2a (HW): RedCap UE can support per-FR gap depends on UE capability. - Option 3(Nokia): Prioritize per-UE gap and consider per-FR gap support afterwards in R17 Recommendation for 2nd round: Continue the discussions. Discuss the use case for per-FR gap for RedCap. |
| Sub-topic#5-3 | Issue 5-3-1: If number of attempts are increased, how much to increase for FR1 - Option 2 (vivo, Nokia, HW, MTK, E///, Apple): 1 sample - Option 3 (QC): by 3 samples Recommended for 2nd round: Discuss the updated options. Issue 5-3-2: Whether to extend the lower bound in PSS/SSS detection delay in FR1 and FR2 - Option 1 (Apple, vivo, Nokia, HW, MTK, E///): No - Option 2 (QC): Yes for FR1 - increase the lower bound from 600ms to 960 ms Recommended for 2nd round: Discuss the updated options. |
| Sub-topic#5-4 | Issue 5-4-1: Side condition for defining requirements in FR1 - Option 1 (Nokia, HW, E///, Apple, vivo, QC): - Define the time index detection delay in FR1 for RedCap UEs based on the side condition of -6dB - Define the time index detection delay in FR1 for RedCap UEs for the worst-case scenario (within the proposed channel profiles) Tentative agreement: - Define the time index detection delay in FR1 for RedCap UEs based on the side condition of -6dB - Define the time index detection delay in FR1 for RedCap UEs for the worst-case scenario (within the proposed channel profiles) Issue 5-4-2: Whether to extend time index delay in FR1 (PBCH-DMRS detection) Tentative agreement: The time index delay in FR1 (PBCH-DMRS detection) is extended. Issue 5-4-3: If extended, how much to extend time index delay in FR1 - Option 1 (Apple, vivo, E///): by 1 SMTC compared to legacy requirements - Option 2 (Nokia, E///): by 2 samples/SMTC - Option 3 (QC, HW): by 4 samples/SMTC, i.e. 7 attempts in total Recommendation for the 2nd round: Since the extension proposed in option 2 is between option 1 and 3, can companies compromise to option 2? Issue 5-4-5: Whether to extend the lower bound in time index detection delay - Option 1 (Apple, E///, MTK, Nokia, QC): Yes - Lower boundary changed to 160 ms as follows: - non-DRX delay requirement: max(160ms, ceil( 4 x Kp ) x SMTC period)x CSSFintra Tentative agreement: [The lower bound in the time index detection delay is extended as follows: - non-DRX delay requirement: max(160ms, ceil( 4 x Kp ) x SMTC period)x CSSFintra] |
| Sub-topic#5-5 | Issue 5-5-1: Measurement period (# of samples) for 1 Rx requirements for SSB based L3 measurement in FR1 Tentative agreement: Measurement period (# of samples) for 1 Rx requirements for SSB based L3 measurement in FR1 is same as in Release 15 requirements. Issue 5-5-3: Whether lower bound in measurement period for 1 Rx requirements is extended in FR1 - Option 2 (Apple, MTK, QC): Only lower bound is extended while keeping the same number of samples. - Lower bound extended to 400 ms as follows: - max(400ms, ceil( 5 x Kp) x SMTC period) x CSSFintra - Option 3 (HW, E///, Nokia, vivo): Lower bound is not extended compared to Release 15 requirements. Recommendation for 2nd round: Given that the measurement period is not extended (in issue 5-5-1), can companies compromise to not extend the lower bound. Issue 5-5-4: Relaxation of accuracy levels for FR1 - Option 1 (vivo, MTK): 1.5 dB compared to legacy requirements for 2 Rx UE - Option 2 (Nokia, HW, E///, Apple, QC): - Absolute accuracy by 1 dB compared to legacy requirements for 2 Rx UE - Relative accuracy by 1 dB compared to legacy requirements for 2 Rx UE Recommendation for 2nd round: Check if companies can compromise to option 2? |
| Sub-topic#5-6 | Issue 5-6-1: Priority between UL and DL during cell identification and measurement for HD-FDD - Option 1 (Apple, Xiaomi, vivo, HW, MTK, OPPO, CMCC, Nokia): RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in cell identification and measurement requirement. - Option 2 (CMCC, E///): No measurement period relaxation or prioritization between measurement and dynamically scheduled UL transmission are needed. Clarification on available samples can be considered: - “The UE shall meet the current requirements on cell identification (PSS/SSS detection defined in Table x.y.z for FR1 and FR2) provided that at least 5 SMTC windows are available at the UE during cell identification time.” - “The UE shall meet the current requirements on time index detection (defined in Table x.y.z for FR1 and FR2) provided that at least 3 SMTC windows are available at the UE during time index detection.” - “The UE shall meet the current requirements intra-frequency/inter-frequency measurement (defined in Table x.y.z for FR1 and FR2) provided that at least 5 SMTC windows are available at the UE during measurement period.” Tentative agreement: RRM DL measurement is prioritized over the UL transmission of HD-FDD for RedCap UE in cell identification and measurement requirement. Issue 5-6-2: Scheduling availability for HD-FDD UE in CONNECTED mode - Option 1 (MTK): - Support introducing scheduling availability restriction on 5G NR RedCap UEs performing measurements in HD-FDD bands in CONNECTED mode. - Support clarifying that the scheduling restriction is for intra-frequency and inter-frequency measurements in CONNECTED mode in HD-FDD bands. - Option 2 (E///, CMCC): No scheduling restriction needed. Recommendation for 2nd round: Address the questions raised in the 1st round: - “For 1st bullet, is this scheduling restriction used to DL data/control reception when performing DL measurement? e.g., SSB and DL data has different SCS and UE cannot support mixed numerology. - For 2nd bullet, if inter-frequency measurement is always with MG (up to Issue 5-2-1), no scheduling restriction shall be considered.” If no consensus reached in the 2nd round then no need to continue the discussions. |
| Sub-topic#5-7 | Issue 5-7-1: Whether to define CGI identification of an E-UTRAN cell with autonomous gaps for RedCap - Option 1 (E///, HW, CMCC, MTK, Nokia, QC): Yes Tentative agreement: RAN4 to define CGI identification of an E-UTRAN cell with autonomous gaps for RedCap Issue 5-7-2: If CGI identification of an E-UTRAN cell with autonomous gaps for RedCap, the requirements: - Option 1 (E///, HW, CMCC, MTK, Nokia, QC): RAN4 to reuse existing cat-1bis CGI reading requirement in LTE. Tentative agreement: For CGI identification of an E-UTRAN cell with autonomous gaps for 1 Rx RedCap UE, RAN4 to reuse existing cat-1bis CGI reading requirement in LTE. Issue 5-7-3: PBCH decoding delay for CGI reading for 1 Rx in FR1 - Option 1 (HW, E///, CMCC): The MIB decoding delay requirement of 1Rx RedCap UE can be the same as non-RedCap UE for SNR=-3dB. Recommendation for 2nd round: Simulation assumptions was agreed at last meeting and results have been presented to this meeting. Proposals are discussed based on those results and therefore given the very short time left to complete the WI, FFS shall be avoided. Based on this situation, companies to check if they can compromise to option 1. Issue 5-7-5: SIB1 decodign delay for CGI reading - Option 1 (E///, Nokia): 6 samples are needed for 1Rx RedCap UE to achieve the SIB1 90% successful rate. - Option 2 (HW, QC): More samples are needed. - Option 2a (Huawei):12 samples are needed in FR1 for 1Rx RedCap UE to achieve the SIB1 90% successful rate. Recommendation for 2nd round: Companies to provide their views on the updated proposals. Issue 5-7-6: Assistance information for CGI reading - Option 1 (E///): If indicated by network, UE will further report the NCD-SSB information (such as SSB-frequency, SCS etc.) together with global cell ID when UE reporting the CGI. - Option 2 (HW, MTK, Nokia, QC): Depends on RAN2 conclusion. - Option 3 (Apple): Not in RedCap WI scope. Recommendation for 2nd round: Companies are encouraged to check the RAN2 conclusion and provide updated view. |

## Discussion on 2nd round (if applicable)

Companies are encouraged to provide their view on the issues recommended for 2nd round discussions in 5.3.1.

Topic #5

| Company | Comments |
| --- | --- |
| Company X | Issue x-y |

# Topic #6: Work split

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2205627 | Ericsson | Updated worksplit for RedCap for RRM |

## Open issues summary

### Sub-topic 6-1 Work split

### Work split for TS 38.133

| Section | Section for RedCap requirements | Volunteering company |
| --- | --- | --- |
| 3. Definitions, symbols and abbreviations | 3.1 Definitions, 3.2 Symbols, 3.3 Abbreviations | Vivo |
|  | 3.5 Frequency bands grouping |  |
|  | 3.6 Applicability of requirements in this specification version |  |
| 4. IDLE state mobility | 4.2B.2.1x UE measurement capability | Ericsson |
|  | 4.2B.2.2x Measurement and evaluation of serving cell |  |
|  | 4.2B.2.3x Measurements of intra-frequency NR cells |  |
|  | 4.2B.2.4 Measurements of inter-frequency NR cells |  |
|  | 4.2B.2.5 Measurements of inter-RAT E-UTRAN cells |  |
|  | 4.2B.2.6 Maximum interruption in paging reception |  |
|  | 4.2B.2.7 General requirements |  |
|  | 4.2B.2.8 Minimum requirement at transitions |  |
|  | 4.2B.2.9 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion 4.2B.2.10 Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion 4.2B.2.11 Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion |  |
| 5. INACTIVE state mobility | 5.1B.2.1 UE measurement capability | Oppo |
|  | 5.1B.2.2 Measurement and evaluation of serving cell | Huawei |
|  | 5.1B.2.3 Measurements of intra-frequency NR cells | Huawei |
|  | 5.1B.2.4 Measurements of inter-frequency NR cells | Huawei |
|  | 5.1B.2.5 Measurements of inter-RAT E-UTRAN cells | Oppo |
|  | 5.1B.2.6 Maximum interruption in paging reception | vivo |
|  | 4.2B.2.7 General requirements | Ericsson |
|  | 4.2B.2.8 Minimum requirement at transitions |  |
|  | 4.2B.2.9 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion 4.2B.2.10 Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion 4.2B.2.11 Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion |  |
|  | Small data transmission for RedCap | Ericsson |
| 6. CONNECTED state mobility | 6.1C Handover | Huawei |
|  | 6.2.1B RRC re-establishment 6.2.2B Random Access | Huawei |
|  | 6.2.3A RRC Connection Release with Redirection | Huawei |
| 7. Timing | 7.1A UE transmit timing 7.2A UE timer accuracy 7.3A Timing Advance | Xiaomi |
|  | 7.4 Cell phase synchronization accuracy | No change |
|  | 7.5 Maximum Transmission Timing Difference | N/A |
|  | 7.6 Maximum Receive Timing Difference | N/A |
|  | 7.7 deriveSSB-IndexFromCell tolerance | FFS |
| 8. Signalling characteristics | 8.1B Radio Link Monitoring | MediaTek |
|  | 8.2A Interruption | N/A |
|  | 8.3 Scell Activation and Deactivation Delay | N/A |
|  | 8.4A UE UL carrier RRC reconfiguration delay | N/A |
|  | 8.5B Link Recovery Procedures | Vivo |
|  | 8.6A Active BWP switch delay | CMCC |
|  | 8.8 NE-DC: E-UTRAN PSCell Addition and Release Delay | N/A |
|  | 8.9 NR-DC: PSCell Addition and Release Delay | N/A |
|  | 8.10B Active TCI state switching delay | CMCC |
|  | 8.11 PSCell Change | N/A |
|  | 8.12A Uplink spatial relation switch delay | FFS |
|  | 8.13A UE-specific CBW change | CMCC |
| 9. Measurement Procedure | 9.1A General measurement requirement 9.2B NR intra-frequency measurements | MediaTek |
|  | 9.3B NR inter-frequency measurements 9.4A Inter-RAT measurements | MediaTek |
|  | 9.5B L1-RSRP measurements for Reporting | Nokia |
|  | 9.6 NE-DC: Measurements | N/A |
|  | 9.7 Cross Link Interference measurements | FFS |
|  | 9.8 L1-SINR measurements for Reporting | FFS |
|  | 9.9 NR measurements for positioning | N/A |
|  | 9.10 CSI-RS based L3 measurements | N/A |
|  | 9.11A NR measurements with autonomous gaps | Nokia |
|  | 9.X CONNECTED mode measurement relaxation | FFS |
| 10. Measurement Performance requirements | 10.1 NR measurements | To be discussed in performance part |
| 12. V2X Requirements | V2X Requirements | N/A |

### Work split for TS 36.133

| Section | Type of requirements | Volunteering company |
| --- | --- | --- |
| 4. E-UTRAN RRC_IDLE state mobility | 4.1 Cell Selection | N/A |
|  | 4.2 Cell Re-selection //new subsections to be created | Ericsson |
|  | 4.6 Cell Selection and Re-selection Requirements for UE category NB1 | N/A |
|  | 4.7 Cell Selection and Re-selection Requirements for UE category M1 | N/A |
| 4A E-UTRAN RRC_INACTIVE state mobility | 4A.1.2.9 Measurements of inter-RAT NR cells for RedCap 4A.1.2.8 UE measurement capability for RedCap | Ericsson |
| 5. E-UTRAN RRC_CONNECTED state mobility | 5.3 Handover to other RATs | Huawei |
|  | 5.5 E-UTRAN Handover for Cat-M1 Ues | N/A |
| 6. RRC Connection Mobility Control | 6.3 RRC Connection Release with Redirection //new subsections to be created | Ericsson |
|  | 6.8 RRC Connection Release with Redirection for Cat-M1 UEs | N/A |
|  | 6.9 RRC Connection Redirection to Non-anchor Carrier in NB-IoT | N/A |
| 8. UE Measurements Procedures in RRC_CONNECTED State | General sections, e.g. gap applicability, Inter-RAT measurements to RedCap //new subsections to be created | Ericsson |
| 10. Measurements Performance Requirements for E-UTRAN |  | To be discussed in performance part |

# Topic #7: Feature lists for RedCap

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |

## Open issues summary

### Sub-topic 7-1 Features for RedCap in release 17

Companies are invited to provide their input also on feature not yet proposed but is needed for RedCap in release 17. The features will be included in the overall feature list under [101-bis-e][139] R17_feature_list.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-1 |  |  |  |  |  |  |  |  |  |  |  |  |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on RedCap RRM requirements | Ericsson | To capture the agreements from thread [102-e][228] NR_redcap_RRM_1 |
| LS on configuring margin for 1 Rx RedCap UEs | Ericsson | To: RAN_2, //Depends on outcome in 2nd round |
| LS on handover using NCD-SSB | Ericsson | To: RAN_2 //Depends on outcome in 2nd round |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2204282 | draft CR on measurements requirements for inactivate state Redcap UE | OPPO | Revised |  |
| R4-2205622 | Draft CR on RRC_IDLE mode requirements for RedCap for TS 36.133 | Ericsson | Revised |  |
| R4-2205625 | Draft CR on RRC_IDLE mode requirements for RedCap for TS 38.133 | Ericsson | Revised |  |
| R4-2205626 | Draft CR on RRC_INACTIVE mode requirements for RedCap for TS 38.133 | Ericsson | Revised |  |
| R4-2205623 | Draft CR on RRC_INACTIVE mode requirements for RedCap for TS 36.133 | Ericsson | Revised |  |
| R4-2204798 | Draft CR for maximum interruption in paging reception for Redcap | Vivo | Revised |  |
| R4-2204800 | Draft CR for Definitions, symbols and abbreviations for Redcap | vivo | Revised |  |
| R4-2204248 | Draft CR on timing requirements for RedCap UE | Xiaomi | Revised |  |
| R4-2204913 | Clarification on transmit timing before Msg1 or MsgA retransmission | Huawei, Hisilicon | Revised |  |
| R4-2204996 | Draft CR to 38.133 for introducing RedCap requirements on active BWP switch delay, active TCI state switching delay and UE specific CBW change | CMCC | Revised |  |
| R4-2206085 | DraftCR on reduced capability Ues for RLM for RedCap | MediaTek Inc. | Revised |  |
| R4-2204799 | Draft CR for Link Recovery Procedures for Redcap | vivo | Revised |  |
| R4-2204904 | Draft CR on mobility requirements for Redcap UE | Huawei, Hisilicon | Revised |  |
| R4-2204905 | Draft CR on E-UTRAN – NR Handover for Redcap UE | Huawei, Hisilicon | Revised |  |
| R4-2206038 | RRC connection release with redireciton for redcap in TS 36.133 | Ericsson | Revised |  |
| R4-2206087 | DraftCR on reduced capability Ues for general measurements and intra-frequency | MediaTek Inc. | Revised |  |
| R4-2206088 | DraftCR on reduced capability Ues for inter-frequency and inter-RAT measurements | MediaTek Inc. | Revised |  |
| R4-2204537 | Draft CR – Introducing L1-RSRP requirements for RedCap UEs | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2205511 | draftCR on inter-RAT NR measurement for RedCap | Ericsson | Revised |  |
| R4-2205938 | Introduction of RedCap UE in clause 9.11A | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2205627 | Updated worksplit for RedCap for RRM | Ericsson | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei | Jing Han | Hw.hanjing@huawei.com |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |
| Apple | Jie Cui | Jie_cui@apple.com |
| CMCC | Xiaoran ZHANG | zhangxiaoran@chinamobile.com |
| MediaTek | Waseem Ozan | Waseem.ozan@mediatek.com |
| Nokia | Erika Almeida | Erika.almeida@nokia.com |
| Nokia | Juergen Hofmann | Juergen.hofmann@nokia.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)