3GPP TSG-RAN WG4 Meeting # 102-e 												R4-2206836

Electronic Meeting, February 21 – March 3, 2022

Agenda item:			10.3.3, 10.3.4

Source:	Huawei, HiSilicon

Title:	WF on R17 NR FR1 RF enhancement RRM

Document for:	Approval

- RRM test case for R17 Tx switching enhancements

Issue 2-1: Test cases need to be specified

Six DL interruption test cases in SA need to be verified:

- DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors for UL inter-band CA

#1: NR UL FDD carrier 1 with 2Tx and NR UL TDD carrier 2 with 2Tx, and the two uplink carriers are in different bands;

#2: NR UL TDD carrier 1 with 2Tx and NR UL TDD carrier 2 with 2Tx, and the two uplink carriers are in different bands, and carrier 1 and carrier 2 have different UL-DL pattern.

- DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors for UL inter-band CA

#3: NR UL FDD carrier 1 in band A with 1TX, NR UL TDD carrier 2 and TDD carrier 3 in band B with 2Tx. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers;

#4:  NR UL TDD carrier 1 in band A with 1TX, NR UL TDD carrier 2 and TDD carrier 3 in band B with 2Tx. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers and with the same TDD pattern. Carrier 1 and carrier 2 have different UL-DL pattern.

- DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors for UL inter-band CA

#5: NR UL FDD carrier 1 in band A with 2TX, NR UL TDD carrier 2 and TDD carrier 3 in band B with 2Tx. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers;

#6:  NR UL TDD carrier 1 in band A with 2TX, NR UL TDD carrier 2 and TDD carrier 3 in band B with 2Tx. NR UL carrier 2 and carrier 3 are two contiguous aggregated carriers and with the same TDD pattern. Carrier 1 and carrier 2 have different UL-DL pattern.

Issue 2-2: Test configurations and UE antenna configurations, if test case #1 and #2 listed in issue 2-1 are agreeable

For test case #1 and #2, the test configurations and UE antenna configurations are listed as below,

| Test case | Description |
| --- | --- |
| 1 | NR carrier 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode; NR carrier 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; |
| 2 | NR carrier 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; Note: TDD CA combination configuration with different UL/DL patterns |

| Cells | UE antenna configuration |
| --- | --- |
| PCell | With 2Tx |
| SCell | With 2TX |

Issue 2-3: Test configurations and UE antenna configurations, if test case #3 and #4 listed in issue 2-1 are agreeable

For test case #3 and #4, the test configurations and UE antenna configurations are listed as below,

| Test case | Description |
| --- | --- |
| 3 | NR carrier 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode; NR carrier 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; Note1: Carrier 2 and carrier 3 with same UL/DL patterns. |
| 4 | NR carrier 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; Note1: Carrier 2 and carrier 3 with same UL/DL patterns. Note2: Carrier 1 and carrier 2 with different UL/DL patterns. |

| Cells | UE antenna configuration |
| --- | --- |
| PCell | With 1TX |
| SCell 1 | With 2TX |
| SCell 2 | With 2TX |

Issue 2-4: Test configurations and UE antenna configurations, if test case #5 and #6 listed in issue 2-1 are agreeable

For test case #5 and #6, the test configurations and UE antenna configurations are listed as below,

| Test case | Description |
| --- | --- |
| 5 | NR carrier 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode; NR carrier 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; Note1: Carrier 2 and carrier 3 with same UL/DL patterns. |
| 6 | NR carrier 1 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 2 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; Note1: Carrier 2 and carrier 3 with same UL/DL patterns. Note2: Carrier 1 and carrier 2 with different UL/DL patterns. |

| Cells | UE antenna configuration |
| --- | --- |
| PCell | With 2Tx |
| SCell 1 | With 2Tx |
| SCell 2 | With 2Tx |

- CR split

The following table provides test cases for FR1 Tx switching enhancement which are expected to require individual draft CR.  (Please company who would like to offer help on CR preparing put your name in the following table).

| Test case | Description | Volunteer Company |
| --- | --- | --- |
| 1 | NR carrier 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode; NR carrier 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; |  |
| 2 | NR carrier 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; Note: TDD CA combination configuration with different UL/DL patterns |  |
| 3 | NR carrier 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode; NR carrier 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; Note1: Carrier 2 and carrier 3 with same UL/DL patterns. |  |
| 4 | NR carrier 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; Note1: Carrier 2 and carrier 3 with same UL/DL patterns. Note2: Carrier 1 and carrier 2 with different UL/DL patterns. |  |
| 5 | NR carrier 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode; NR carrier 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; Note1: Carrier 2 and carrier 3 with same UL/DL patterns. |  |
| 6 | NR carrier 1 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 2 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; NR carrier 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; Note1: Carrier 2 and carrier 3 with same UL/DL patterns. Note2: Carrier 1 and carrier 2 with different UL/DL patterns. |  |