3GPP TSG-RAN WG4 Meeting # 102-e	R4-2206858

# Electronic Meeting, 21 February - 3 March 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | CRNum | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.133: Enhancements for Tq timing adjustment and one shot large UL timing adjustment requirements for FR2 NR HST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2-Core |  |  |  |  |  | Date: |  |  | 2022-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Enhancements on Tq autonomous timing adjustment and one shot large UL timing adjustment for FR2 NR HST were agreed but are not specified in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The autonomous time adjustment step Tq is enhanced to support FR2 HST in Table 7.1.2.1-1. One shot large UL timing adjustment is added as a new section in 7.1.2.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 NR HST enhancements for autonomous timing adjustment and one shot large UL timing adjustment are missing from the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 7.1.2.1	Gradual timing adjustment

<Unmodified text omitted >

Table 7.1.2.1-1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2 | 60 | K*64*Tc | 2.5*64*Tc |
|  | 120 | K*64*Tc | 2.5*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] NOTE 2: When [highSpeedMeasFlagFR2-r17] is configured for UE supporting power class 6, K = [4.5]; otherwise, when [highSpeedMeasFlagFR2-r17] is not configured K = 2.5. |  |  |  |

<End of Change 1>

<Start of Change 2>

#### 7.1.2.2	Void

Table 7.1.2.2-1: Void

#### 7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE

When [largeOneStepUL-timingFR2-r17] is enabled for UE suppporting FR2 power class 6, the requirement in clause 7.1.2.1 doesn’t apply to the first UL transmission after a TCI state switch if the new TCI state is from a new non-collocated RRH. The UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell after UE finishes the TCI state switch procedure, the same as the requirement specified in clause 7.1.2 except clause 7.1.2.1.

(NTA+NTA_offset)+2×Tnew-ToldThe UE transmit timing immediately after TCI state switch shall be Tnew-(NTA+NTA offset)+2 (Told-Tnew), where

-	Tnew (in Tc units) is the DL timing defined as the time when UE receives downlink frame with new target TCI state.

-	Told  (in Tc units) is the DL timing defined as the time when UE receives downlink frame with old source TCI state.

-

Tnew-(NTA+NTA offset)+2 (Told-Tnew)

The UE UL transmittion timing error after the TCI state switching procedure shall be less than or equal to ±9*64*Tc, and the reference point shall be the downlink timing of the new cell minus Tnew-(NTA+NTA offset)-+2 (Told-Tnew).

When [largeOneStepUL-timingFR2-r17] is not enabled, then UE transmit power shall be turned off except for PRACH transmission or message A transmission until UE has acquired UL timing. and UE is scheduled by UL grant.

<End of Change 2>