# 3GPP TSG-RAN WG4 Meeting # 102-e	R4-2207035

# Electronic Meeting, February 21 – March 3, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | Draft | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on introducing intra-frequency measurement requirements for Rel-17 NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The requirements for connected mode neighbour cell measurements before RLF should be defined. The intra-frequency measurement requirements are introduced in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce intra-frequency neighbour cell measurement requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The corresponding requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.14.5	NB-IoT neighbour cell measurements

8.14.5.1	Introduction

This clause contains requirements for the neighbour cell measurement performed by the UE category NB1 in RRC_CONNECTED state. The requirements in this clause are applicable when:

- the UE is in normal coverage or in enhanced coverage on the serving cell and

- the target cell fulfils the criteria for normal coverage.

8.14.5.2	Requirements

The UE supporting connectedModeMeasurements-r17 [31] shall measure neighbour cell on anchor carrier when criterion for triggering the neighbour cell measurement defined in [1] is fulfilled and UE is not in relaxed monitoring mode as defined in [1]. The UE may trigger the neighbour cell before or during the radio link failure depending on the fulfilment of the criterion.

The measurement quantities are defined in [4], the measurement model is defined in [22].

The requirements for intra-frequency neighbour cell measurement when the target carrier is same as serving carrier is defined in clause 8.14.5.3.

The requirements for inter-frequency neighbour cell measurement when the target carrier is different from serving carrier is defined in clause 8.14.5.4.

8.14.5.3	Intra-frequency neighbour cell measurement

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra when the criteria for intra-frequency measurement is fulfilled [1]. An intra frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.x.y for a corresponding Band.

Tidentify_intra = Tdetect_intra + Tmeasure _intra

When DRX is not used, Tdetect_intra is 1400 ms, and Tmeasure _intra is 800 ms and 1600 ms for NRS-based measurement and NSSS-based measurement respectively.

When DRX is used, Tdetect_intra and Tmeasure _intra are defined in table 8.14.5.3-1 and 8.14.5.3-2.

Table 8.14.5.3-1: Requirement for intra-frequency detection

| DRX cycle length (s) | Tdetect_intra (s) (DRX cycles) |
| --- | --- |
| 0.256<DRX-cycle≤10.24 | (6)Note 1 |
| Note1: Time depends upon the DRX cycle in use |  |

Table 8.14.5.3-2: Requirement for intra-frequency measurement

| DRX cycle length (s) | Tmeasure_intra (s) (DRX cycles) |
| --- | --- |
| 0.256<DRX-cycle≤10.24 | (5)Note 1 |
| Note1: Time depends upon the DRX cycle in use |  |

When UE is monitoring multiple carriers, Tidentify_intra = Tdetect + Tmeasure, where Tdetect = Tdetect _intra+Nfreq* Tdetect_inter and Tmeasure = Tmeasure _intra+Nfreq* Tmeasure_inter. Nfreq is number of inter-frequency carriers to be measured according to the measurement capability, and Tdetect_inter  and Tmeasure_inter are defined in clause 8.14.5.4

<End of Change 1>