3GPP TSG-RAN WG4 Meeting #102-e  		                                                R4-2207165

Electronic Meeting, 21st of February – 3rd of March, 2022

Agenda item:			10.13.1, 10.13.1.1, 10.13.1.2, 10.13.1.3, 10.13.1.4

Source:	Moderator (THALES)

Title:	Email discussion summary for [102-e][308] NTN_Solutions_Part1

Document for:	Information

# Introduction

This discussion summary document captures general issues related to RAN4 RF part Rel-17 NR NTN WI, including system parameters, NTN class/Type, and regulatory discussions, including exemplary bands. It contains a summary of the contributions under sections and subsections 10.13.1.1, 10.13.1.2, 10.13.1.3, 10.13.1.4 at TSG-RAN WG4 #102-e, together with identified key open issues and recommends topics/questions to be handled via email discussions. The goal of this document is to provide recommendation on prioritization of discussion.

Please also note the draft TSG-RAN WG4 #102-e meeting agenda with respect to NTN topic:

-------------------------------------- Items led by other working group ----------------------------------------------------

10.13	Solutions for NR to support non-terrestrial networks (NTN)	[NR_NTN_solutions]

* Incoming LS from RAN2: R2-2201883 Reply LS on Multiple SMTCs for NR NTN

* Incoming LS from RAN2: R2-2201884 Reply LS on NR NTN Neighbor Cell and Satellite Information

10.13.1	General	[NR_NTN_solutions-Core]

*T-docs related to NTN UE features for system parameters and RF can be submitted under this AI.

10.13.1.1		System parameters	[NR_NTN_solutions-Core]

10.13.1.2 	NTN Satellite Access Node Class/Type	 [NR_NTN_solutions-Core]

10.13.1.3		Regulatory information	[NR_NTN_solutions-Core]

10.13.1.4		Others 	[NR_NTN_solutions-Core]

10.13.2	Coexistence aspects	[NR_NTN_solutions-Core]

10.13.2.1		NTN coexistence scenarios and simulations	[NR_NTN_solutions-Core]

10.13.2.2		HAPS coexistence scenarios and simulations 	[NR_NTN_solutions-Core]

10.13.2.3		ACLR/ACS proposals 	[NR_NTN_solutions-Core]

10.13.3	Satellite Access Node RF requirements 	[NR_NTN_solutions-Core]

10.13.3.1		TX requirements for radiated characteristics	[NR_NTN_solutions-Core]

10.13.3.2		RX requirements for radiated characteristics 	[NR_NTN_solutions-Core]

10.13.3.3		Tx requirements for conducted characteristics	[NR_NTN_solutions-Core]

10.13.3.3		Rx requirements for conducted characteristics	[NR_NTN_solutions-Core]

10.13.4	UE RF requirements 	[NR_NTN_solutions-Core]

10.13.4.1		TX requirements	[NR_NTN_solutions-Core]

10.13.4.2		RX requirements 	[NR_NTN_solutions-Core]

10.13.5	RRM core requirements	[NR_NTN_solutions-Core]

10.13.5.1		General	[NR_NTN_solutions-Core]

10.13.5.2		GNSS-related requirements 	[NR_NTN_solutions-Core]

10.13.5.3		Mobility requirements 	[NR_NTN_solutions-Core]

10.13.5.4		Timing requirements	[NR_NTN_solutions-Core]

10.13.5.5		Measurement procedure requirements	[NR_NTN_solutions-Core]

10.13.6	Demodulation requirements	[NR_NTN_solutions-Perf]

10.13.6.1		General 	[NR_NTN_solutions-Perf]

10.13.6.2		Satellite Access Node demodulation requirements 	[NR_NTN_solutions-Perf]

10.13.6.2.1	PUSCH requirements	 [NR_NTN_solutions-Perf]

10.13.6.2.2	PUCCH requirements 	[NR_NTN_solutions-Perf]

10.13.6.2.3	PRACH requirements 	[NR_NTN_solutions-Perf]

10.13.6.3		UE demodulation requirements	 [NR_NTN_solutions-Perf]

10.13.6.3.1	PDSCH requirements	 [NR_NTN_solutions-Perf]

10.13.6.3.2	PDCCH/PBCH requirements	[NR_NTN_solutions-Perf]

10.13.6.4		CSI requirements	[NR_NTN_solutions-Perf]

*It’s FFS whether CSI requirements needed or not

For informative purpose, RAN4#102-e E-meeting Arrangements and Guidelines proposed the following schedule:

- Moderators provide initial summary (Draft) by Thursday February 17th, 5pm UTC.

- Companies can provide comments on initial summary by Friday February 18th, 5pm UTC.

- Moderators kick off email discussion (Monday 21st of February).

- Companies provide comments for the 1st round (Monday 21st of February – Thursday 8am UTC February 24th).

- Moderators summarize the status and possible proposals, recommending what decisions can be made for 1st round. A formal t-doc will be used (Thursday 23:59 UTC, February 24th).

- Moderators kick off 2nd round email discussion (no later than Friday 4am UTC February 25th).

- After receiving the summary from moderators, session chair may approve documents, make agreements or assign new CRs, WFs, LSs, etc. (Monday 5pm UTC February 28th).

- Draft WF/LS and revised CRs/TPs shall be shared by Monday 17:00 UTC February 28th.

- Companies provide comments for the 2nd	round summary (no later than Tuesday 5pm UTC March 1st).

- Moderators provide 2nd round WF draft by Tuesday 19:00 UTC, March 1st.

- Moderators provide 2nd round draft summary by Wednesday 11:59 UTC, March 2nd.

- Formal tdocs of WF/LS/CRs/TPs shall be uploaded to the Inbox by Wednesday 17:00 UTC, March 2nd.

- Moderators provide 2nd round summary with a formal tdoc by Thursday 8am UTC, March 3rd.

A total of 19+3 TDocs have been identified for discussion in [102-e][308] NTN_Solutions_Part1 (please also see the Appendix for the details, with all the observations/proposals):

| TDoc Number | TDoc Type | Title | Company | Status | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- | --- |
| R4-2203964 | discussion | UE feature for NTN | CATT | available | Discussion | 10.13.1 |
| R4-2205232 | pCR | TP TR 38.863 7.4.1 NTN UE Requirement (General) | HUGHES Network Systems Ltd | available | Approval | 10.13.1 |
| R4-2205554 | discussion | On NTN System parameters | Nokia, Nokia Shanghai Bell | available | Approval | 10.13.1.1 |
| R4-2203953 | pCR | TP for 38.108: clause 5.3&5.4 on system parameters | CATT | available | Approval | 10.13.1.1 |
| R4-2204507 | pCR | TP on TS 38.101-5 for UE channel bandwidth and channel arrangement | Qualcomm Incorporated | available | Approval | 10.13.1.1 |
| R4-2203956 | pCR Moved to [310] | TP for 38.108: clause 9.3 OTA Satellite Access Node output power | CATT | Available Moved to [310] | Approval | 10.13.1.2 |
| R4-2205673 | pCR | Draft text proposal for Clause 4.3 Requirement reference points - TS 38.108 | THALES | available | Approval | 10.13.1.2 |
| R4-2205730 | pCR | Draft text proposal for Clause 4.4 Satellite Access Node classes - TS 38.108 | THALES | available | Approval | 10.13.1.2 |
| R4-2205048 | other | NTN - SAN class | Ericsson | available | Approval | 10.13.1.2 |
| R4-2205111 | pCR | TP for TR 38.863: Regulatory aspects for NTN satellite access nodes and UEs operating in UL1626.5-1660.5 MHz and DL 1525-1559 MHz frequencies ranges | Ligado Networks | available | Approval | 10.13.1.3 |
| R4-2205314 | pCR | TP to TR 38.863 on Section 5.2 NTN Satellite band | HUGHES Network Systems Ltd | available | Approval | 10.13.1.3 |
| R4-2205733 | pCR withdrawn | Draft text proposal for Clause 4.5 Regional requirements - TS 38.108 | THALES | withdrawn | Approval | 10.13.1.3 |
| R4-2205555 | pCR | TP to TR 38.863 Regulatory aspects for HAPS | Nokia, Nokia Shanghai Bell | available | Approval | 10.13.1.3 |
| R4-2205671 | pCR | Draft text proposal for Clause 3 - TS 38.101-5 | THALES | available | Approval | 10.13.1.4 |
| R4-2205672 | pCR | Draft text proposal for Clause 4 - TS 38.101-5 | THALES | available | Approval | 10.13.1.4 |
| R4-2205667 | pCR | Draft text proposal for Annex B - TS 38.108 | THALES | available | Approval | 10.13.1.4 |
| R4-2205921 | pCR | Draft text proposal for Clauses 7, 7.1, 7.2, 7.3 in TR 38.863 | THALES | available | Approval | 10.13.1.4 |
| R4-2205051 | other | NTN - General aspect related to TS assumptions | Ericsson | available | Approval | 10.13.1.4 |
| R4-2204195 | CR | CR for TS 38.104: capturing HAPS requirements | Softbank, Deutsche Telekom, Ericsson, NTT Docomo, KDDI, Nokia, Intelsat | available | Agreement | 10.13.1.4 |
| R4-2203952 | pCR | TP for 38.108: clause 4.3 requirement reference point | CATT | available | Approval | 10.13.1.4 |
| R4-2203536 | discussion | Considerations on HAPS operating band(s) | SoftBank, Deutsche Telekom, Ericsson, NTT Docomo, KDDI, Nokia, Intelsat | available | Decision | 10.13.1.4 |
| R4-2205437 | pCR | TP to TR 38.108 on 4.5 Regional Requirement | HUGHES Network Systems Ltd | available | Approval | 10.13.3 (added for discussion in [308] as asked by the chairman) |
| R4-2205476 | Other -> pCR | TP for TS 38.108: Genera(5.1) and Operating Band(5.2) | ZTE Corporation | available | Approval | 10.13.3 (added for discussion in [308] as asked by the chairman) |
| R4-2205472 | Other -> pCR | TP for TS 38.101-5: Genera(5.1) and Operating Band(5.2) | ZTE Corporation | available | Approval | 10.13.4.1 (added for discussion in [308] as asked by the chairman) |

Moderator note1: There are 5 pCRs to TR 38.863, which the moderator proposes to discuss in the dedicated folders from 1st round and 2nd round.

Moderator note2: There are 5+2 pCRs to TR 38.108, which the moderator proposes to discuss in the dedicated folders from 1st round and 2nd round.

Moderator note3: There are 3+1 pCRs to TR 38.101-5, which the moderator proposes to discuss in the dedicated folders from 1st round and 2nd round.

Moderator note4: There is 1 CR to TR 38.104, which the moderator proposes to discuss in the dedicated folders from 1st round and 2nd round.

Moderator note5: TDoc R4-2203956 has been moved from [102-e][308] NTN_Solutions_Part1 discussion list in [102-e][310] NTN_Solutions_Part3.

Moderator note6: TDocs R4-2205437 and R4-2205476 have been moved from [102-e][310] NTN_Solutions_Part3 discussion list in [102-e][308] NTN_Solutions_Part1.

Moderator note7: TDoc R4-2205472 has been moved from [102-e][311] NTN_Solutions_Part4 discussion list in [102-e][308] NTN_Solutions_Part1.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

Identified topics and issues for the 1st round:

- Topic #1: NTN Satellite System Parameters/Features

  - Issue 1-1-1: UE feature for NTN – PDSCH

  - Issue 1-1-2: UE feature for NTN - PUSCH

- Topic #2: NTN SAN Class

  - Issue 2-1-1: Satellite Access Node Class (SAN Class)

  - Issue 2-1-2: SAN number of classes

  - Issue 2-1-3: SAN class differentiation

- Topic #3: TS (writing) assumptions

  - Issue 3-1-1: General approach to write NTN TS (i.e. TS 38.108 and TS 38.101-5)

  - Issue 3-1-2: General approach to write NTN TS (i.e. TS 38.108 and TS 38.101-5)

- Topic #4: pCRs/CRs

  - Issue 4-1-1: pCR to TR 38.863 – see R4-2205232 (HUGHES Network Systems Ltd)

  - Issue 4-1-2: pCR to TR 38.863 – see R4-2205111 (Ligado Networks)

  - Issue 4-1-3: pCR to TR 38.863 – see R4-2205314 (HUGHES Network Systems Ltd)

  - Issue 4-1-4: pCR to TR 38.863 – see R4-2205555 (Nokia, Nokia Shanghai Bell)

  - Issue 4-1-5: pCR to TS 38.863 – see R4-2205921 (THALES)

  - Issue 4-2-1: pCR to TS 38.108 – see R4-2203953 (CATT)

  - Issue 4-2-2: pCR to TS 38.108 – see R4-2205673 (THALES)

  - Issue 4-2-3: pCR to TS 38.108 – see R4-2205730 (THALES)

  - Issue 4-2-4: pCR to TS 38.108 – see R4-2205667 (THALES)

  - Issue 4-2-5: pCR to TS 38.108 – see R4-2203952 (CATT)

  - Issue 4-2-6: pCR to TS 38.108 – see R4-2205437 (HUGHES Network Systems Ltd)

  - Issue 4-2-7: pCR to TS 38.108 – see R4-2205476 (ZTE Corporation)

  - Issue 4-3-1: pCR to TS 38.101-5 – see R4-2204507 (Qualcomm Incorporated)

  - Issue 4-3-2: pCR to TS 38.101-5 – see R4-2205671 (THALES)

  - Issue 4-3-3: pCR to TS 38.101-5 – see R4-2205672 (THALES)

  - Issue 4-3-4: pCR to TS 38.101-5 – see R4-2205472 (ZTE Corporation)

  - Issue 4-4-1: CR to TS 38.104 – see R4-2204195 (Softbank, Deutsche Telekom, Ericsson, NTT Docomo, KDDI, Nokia, Intelsat)

- Topic #5: HAPS Generalities

  - Issue 5-1-1: NR operating band for HAPS

  - Issue 5-1-2: Description of HAPS operating band to TS 38.104

  - Issue 5-2-1: HAPS UE

# Topic #1: NTN Satellite System Parameters/Features

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| TDoc Number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203964 | CATT | It is proposed the following is included in the Rel-17 UE feature list for NTN 64QAM: see also Annex 8. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description: UE feature for NTN

Moderator Note: Please see Annex 8 for details.

Open issues and candidate options before e-meeting:

Issue 1-1-1: UE feature for NTN – PDSCH

- Proposals

  - Option 1: [22-1] 64QAM for PDSCH

    - Features: [22. NR_NTN_solutions]

    - Index: [22-1]

    - Feature group: 64QAM for PDSCH

    - Components: 64QAM for PDSCH

    - Prerequisite feature groups: [N/A]

    - Need for the gNB to know if the feature is supported: [Yes]

    - Applicable to the capability signalling exchange between UEs (V2X WI only): [No]

    - Consequence if the feature is not supported by the UE: [UE does not support higher modulation 64QAM for PDSCH for NTN. The network cannot schedule 64QAM for PDSCH when DL propagation condition is good enough.]

    - Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC): [Per Band]

    - Need of FDD/TDD differentiation: [FDD only]

    - Need of FR1/FR2 differentiation: [FR1 only]

    - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: [N/A]

    - Note: [N/A]

    - Mandatory /Optional: [Optional with capability signalling]

- Recommended WF

  - TBD

Question: Which is the company preference for the above option?

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 |
| --- | --- |
| Qualcomm | We understand the agreement from last meeting was that UE will support 64QAM with UE capability signalling. However, on the other hand, NTN UE is supposed to support TN functionality, and TN UE is mandatory to support 64QAM for both UL and DL. In this case, we’d better to discuss the necessity of introducing a new signalling for 64QAM for NTN UE. It seems there is no need to introduce this additional UE feature signalling for NTN UE since UE with TN and NTN functionality should naturally support 64QAM for DL and UL. |
| CATT | Our understanding is that this capability is needed. To Qualcomm. If we don’t introduce UE capability, does it mean that SAN can scheduling PUSCH and PDSCH assuming all the satellite UE support 64QAM for both UL and DL? |
| Ericsson | We would be fine with the 2 proposals but are those 2 NTN features the only ones needed? |
| Huawei | Based on the approved WF R4-2120774, we reached the following agreement. “RAN4 follow the assumption that NTN handheld User Equipment has both TN and NTN functionalities for FR1 in Rel-17 NTN WI from RAN4 requirements perspective.” For TN UE, the 64QAM is mandatory. That means the NTN handheld UE can also support 64QAM naturally since NTN handheld User Equipment has both TN and NTN functionalities for FR1. Maybe some logic issues can be observed when we specify 64QAM (DL and UL) capabilities for NTN satellite UE. Besides, it means some implementation restriction when capabilities are defined for UE. For 64QAM implementation, it’s better to check with UE vendors what the implemental restriction is for satellite UE. |
| ZTE | It’s fine for us to have UE capability, however it should be per UE instead per band from our understanding. |
| Hughes/EchoStar | We support the WF R4-2120774 “RAN4 follow the assumption that NTN handheld User Equipment has both TN and NTN functionalities for FR1 in Rel-17 NTN WI from RAN4 requirements perspective.”. We understand TN UE is mandatory to support 64QAM for both UL and DL. NTN UE is supposed to support TN functionality and vice-versa. |
| Omnispace | We agree Hughes/EchoStar views. This issue was agreed in WF-2120774. |
| THALES | We sustain the approved WF R4-2120774 ([101-e][309] NTN_Solutions_Part1, THALES) (GTW 12/11/2021): RAN4 follow the assumption that NTN handheld User Equipment has both TN and NTN functionalities for FR1 in Rel-17 NTN WI from RAN4 requirements perspective. However, we also agree that 64QAM may not be supported for all NTN scenarios (depending on orbit, elevation, satellite parameters, etc.). The question is also if such a feature is related to a specific class/UE design. At least for FR1 we need to see the benefit of integrating now such a feature, since it may not be essential. The scheduling is done in any case based on channel quality. If the channel is not good enough, the UE will not be scheduled with 64QAM. |
| Intelsat | We support the view that the NTN handheld UE has both the TN and NTN functionality and supports 64QAM. |
| Inmarsat | We agree with Qualcomm’s view and we would like to keep consistency of features, it seems that making 64QAM option would create an unnecessary deviation and require additional signalling. Moreover, we don’t see why we should prevent the use of 64QAM. |
| Hispasat | We support maintaining homogeneous specifications among terrestrial and not-terrestrials, under the agreed assumption that both networks will be available for the NTN handled UE, therefore 64QAM should be available on NTN as well. |
| MediaTek | We also support homogeneous specs for TN and NTN of course. But we just feel that this feature may not be needed for all scenarios as Thales mentioned, so would like to consider optionality. |

GTW Discussion (23/02/2022):

Further discuss below two options

- Supporting UL/DL 64QAM for NTN operation as optional with [per UE]

- Mandatory same as TN operation

Issue 1-1-2: UE feature for NTN – PUSCH

- Proposals

  - Option 1: [22-2] 64QAM for PUSCH

    - Features: [22. NR_NTN_solutions]

    - Index: [22-1]

    - Feature group: 64QAM for PUSCH

    - Components: 64QAM for PUSCH

    - Prerequisite feature groups: [N/A]

    - Need for the gNB to know if the feature is supported: [Yes]

    - Applicable to the capability signalling exchange between UEs (V2X WI only): [No]

    - Consequence if the feature is not supported by the UE: [UE does not support higher modulation 64QAM for PUSCH for NTN. The network cannot schedule 64QAM for PUSCH when DL propagation condition is good enough.]

    - Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC): [Per Band]

    - Need of FDD/TDD differentiation: [FDD only]

    - Need of FR1/FR2 differentiation: [FR1 only]

    - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: [N/A]

    - Note: [N/A]

    - Mandatory /Optional: [Optional with capability signalling]

- Recommended WF

  - TBD

  - Moderator Note: It seems that there is a typo. “DL” should be “UL” in the case of PUSCH.

Question: Which is the company preference for the above option?

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 |
| --- | --- |
| Qualcomm | We understand the agreement from last meeting was that UE will support 64QAM with UE capability signalling. However, on the other hand, NTN UE is supposed to support TN functionality, and TN UE is mandatory to support 64QAM for both UL and DL. In this case, we’d better to discuss the necessity of introducing a new signalling for 64QAM for NTN UE. It seems there is no need to introduce this additional UE feature signalling for NTN UE since UE with TN and NTN functionality should naturally support 64QAM for DL and UL. |
| CATT | Our understanding is that this capability is needed. To Qualcomm. If we don’t introduce UE capability, does it mean that SAN can scheduling PUSCH and PDSCH assuming all the satellite UE support 64QAM for both UL and DL? |
| Huawei | Based on the approved WF R4-2120774, we reached the following agreement. “RAN4 follow the assumption that NTN handheld User Equipment has both TN and NTN functionalities for FR1 in Rel-17 NTN WI from RAN4 requirements perspective.” For TN UE, the 64QAM is mandatory. That means the NTN handheld UE can also support 64QAM naturally since NTN handheld User Equipment has both TN and NTN functionalities for FR1. Maybe some logic issues can be observed when we specify 64QAM (DL and UL) capabilities for NTN satellite UE. Besides, it means some implementation restriction when capabilities are defined for UE. For 64QAM implementation, it’s better to check with UE vendors what the implemental restriction is for satellite UE supporting 64QAM. |
| ZTE | It’s fine for us to have UE capability, however it should be per UE instead per band from our understanding. |
| Hughes/EchoStar | We support the WF R4-2120774 “RAN4 follow the assumption that NTN handheld User Equipment has both TN and NTN functionalities for FR1 in Rel-17 NTN WI from RAN4 requirements perspective.”. We understand TN UE is mandatory to support 64QAM for both UL and DL. NTN UE is supposed to support TN functionality and vice-versa. |
| Omnispace | We agree Hughes/EchoStar views. |
| THALES | We sustain the approved WF R4-2120774 ([101-e][309] NTN_Solutions_Part1, THALES) (GTW 12/11/2021): RAN4 follow the assumption that NTN handheld User Equipment has both TN and NTN functionalities for FR1 in Rel-17 NTN WI from RAN4 requirements perspective. However, we also agree that 64QAM may not be supported for all NTN scenarios (depending on orbit, elevation, satellite parameters, etc.). The question is also if such a feature is related to a specific class/UE design. At least for FR1 we need to see the benefit of integrating now such a feature, since it may not be essential. The scheduling is done in any case based on channel quality. If the channel is not good enough, the UE will not be scheduled with 64QAM. |
| Inmarsat | We agree with Qualcomm’s view and we would like to keep consistency of features, it seems that making 64QAM option would create an unnecessary deviation and require additional signalling. Moreover, we don’t see why we should prevent the use of 64QAM. |
| Hispasat | We support maintaining homogeneous specifications among terrestrial and not-terrestrials, under the agreed assumption that both networks will be available for the NTN handled UE, therefore 64QAM should be available on NTN as well. |
| MediaTek | We also support homogeneous specs for TN and NTN of course. But we just feel that this feature may not be needed for all scenarios as Thales mentioned, so would like to consider optionality. |

GTW Discussion (23/02/2022):

Further discuss below two options

- Supporting UL/DL 64QAM for NTN operation as optional with [per UE]

- Mandatory same as TN operation

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Example 1

| Company | Comments |
| --- | --- |
| XXX | See above discussion from 1st round. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1-3 | Further discuss below two options - Supporting UL/DL 64QAM for NTN operation as optional with [per UE] - Mandatory same as TN operation |
| Issue 1-1-1 UE feature for NTN – PDSCH | Moderator note: Please note that for the time being the discussion is for FR1 only (handheld UE). The current options are described below. - Option 1: [22-1] 64QAM for PDSCH - Features: [22. NR_NTN_solutions] - Index: [22-1] - Feature group: 64QAM for PDSCH - Components: 64QAM for PDSCH - Prerequisite feature groups: [N/A] - Need for the gNB to know if the feature is supported: [Yes] - Applicable to the capability signalling exchange between UEs (V2X WI only): [No] - Consequence if the feature is not supported by the UE: [UE does not support higher modulation 64QAM for PDSCH for NTN. The network cannot schedule 64QAM for PDSCH when DL propagation condition is good enough.] - Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC): [Per Band] - Need of FDD/TDD differentiation: [FDD only] - Need of FR1/FR2 differentiation: [FR1 only] - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: [N/A] - Note: [N/A] - Mandatory /Optional: [Optional with capability signalling] - Option 2: Mandatory UE 64QAM for PDSCH (no UE feature) |
| Issue 1-1-2 UE feature for NTN – PUSCH | Moderator note: Please note that for the time being the discussion is for FR1 only (handheld UE). The current options are described below. - Option 1: [22-2] 64QAM for PUSCH - Features: [22. NR_NTN_solutions] - Index: [22-1] - Feature group: 64QAM for PUSCH - Components: 64QAM for PUSCH - Prerequisite feature groups: [N/A] - Need for the gNB to know if the feature is supported: [Yes] - Applicable to the capability signalling exchange between UEs (V2X WI only): [No] - Consequence if the feature is not supported by the UE: [UE does not support higher modulation 64QAM for PUSCH for NTN. The network cannot schedule 64QAM for PUSCH when UL propagation condition is good enough.] - Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC): [Per Band] - Need of FDD/TDD differentiation: [FDD only] - Need of FR1/FR2 differentiation: [FR1 only] - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: [N/A] - Note: [N/A] - Mandatory /Optional: [Optional with capability signalling] - Option 2: Mandatory UE 64QAM for PUSCH (no UE feature) |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Questions and Tables to be added.

# Topic #2: NTN SAN Class

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| TDoc Number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2205048 | Ericsson | Observation1: Considering the different noise figure value for the different type of satellites, the following SAN classes should be considered: LEO and GEO. Observation2: Considering the different IoT level for the different type of satellites, the following SAN classes should be considered: LEO600, LEO1200 and GEO. And, based on those observations, we made the following proposal: Proposal: RAN4 should introduce the following SAN classes: GEO, LEO600 and LEO1200 for SAN type 1-H and type 1-O. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description: Satellite Access Node (BS) Class

Moderator Note: Each of the constellation is scaled with respect to altitude, orbit, etc.., in order to have similar SINR/QoS, and therefore similar performances.

Agreement:

It’s FFS whether separate NTN gNB classes needed or not for Rel-17 which pending on further check on the RF requirements.

- If no difference observed from RAN4 RF requirements perspective, then only single NTN BS class will be introduced as wide area BS.

- All NTN BS classes can be potentially considered equivalent as to Wide Area BS (e.g. if all classes have the same requirements).

- At least introduce NTN BS class with wide coverage

The Classes intended to be used for differentiate the RF requirements.

Below candidate NTN gNB class can be considered as starting point:

- GEO, LEO@600, LEO@1200

- FFS whether need to LEO@600, LEO@1200 can be merged as single class

Moderator Note: Please see agreements from RAN4#101-e (approved R4-2120774)

Agreement R4-2120774:

Proposal 2-2-1-1: Continue discussion for NTN gNB class. Below candidate NTN gNB class can be considered as starting point:

- GEO, LEO@600, LEO@1200

- FFS whether need to LEO@600, LEO@1200 can be merged as single class

Moderator Note: Companies should indicate which requirements may be different, in order to define different Satellite Access Node classes.

Moderator Note: Companies should indicate which requirements may be different, in order to define different Satellite Access Node classes.

Moderator Note: The power limitation on “satellite access node” are manufacture declaration basis, no limitation in RAN4 specification.

Agreement GTW Discussion on 2nd Nov, RAN4#101-e:

Issue 1-2-1: Base station output power

- The power limitation on “satellite access node” are manufacture declaration basis, no limitation in RAN4 specification.

- Some background information from regulatory can be considered to be included in the TR for information.

Open issues and candidate options before e-meeting:

Issue 2-1-1: Satellite Access Node Class (BS Class)

- Proposals

  - Option 1 [Ericsson]: RAN4 should introduce the following SAN classes: GEO, LEO600 and LEO1200 for SAN type 1-H and type 1-O.

  - Option 2 [Moderator]: Propose to continue discussion and add classes later-on through CRs, depending on the ACLR values (if RAN4 identifies & defines different ACLR SAN values per GEO/LEO orbit type).

    - Note1: for the time being different NF values are not justifying different classes,

    - Note2: all the core requirements from the coexistence scenarios are currently the same for GEO and LEO. Please recall the following agreed values in RAN#101-bis-e: SAN ACLR is 24 dB (independently of the orbit), SAN ACS is [38] dB (independently of the orbit).

- Recommended WF

  - TBD

  - Moderator Note: we should focus to have a stable TS version at this meeting, because we need to submit TS with the core requirements by the next plenary meeting, and (as a general recommendation from the chairman) is preferably not to have an exception sheet.

Question: Which option (listed above) do you prefer? Please provide your answer(s) e.g. “Yes” or “No”.

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 | Comments Option 2 |  |
| --- | --- | --- | --- |
| Qualcomm |  | OK with Option 2. |  |
| CATT | From requirement point of view, there are indeed differences for SAN loaded on different satellite types. E.g. GEO LEO1200 and LEO 600. Maybe we could consider down select from 3 classes to 2. LEO 1200 and LEO600 have the same noise figure and similar power (maybe 3dB difference?). We are also ok to further discuss. |  |  |
| Ericsson | Yes | This would be acceptable but the we need to clarify we would need to have only 1 set of requirements (most stringent limits then) for ACLR, OBUE, REFSENS, dynamic range, ACS, ICS.. which is not the case right now looking at previous agreements and proposal for this meeting. |  |
| ESA | Two classes (GEO and LEO) following the option 3 in the next issue. |  |  |
| ZTE | We support the option 1 and we have similar observations as Ericsson that different RF requirement would be expected for LEO 600, LEO1200 and GEO, therefore we propose to have different SAN class |  |  |
| THALES | Agree only if: - 2 classes (GEO and LEO) - differentiation also in terms of ACLR (but this has to be decided in Part2 related to coexistence studies) and potentially other parameters - no definition based on SAN power limitation - [very important] companies are willing to update their respective TPs and re-submit them during the 2nd round of discussions, so we can integrate 2 classes in the current TS 38.108, at this meeting. The class differentiation should not be based only on NFs (since we can have a multitude of NFs, based on each satellite implementation). Moreover, ACS seems to be very similar. | Agree. This proposal is to simplify the working process for the submission of the TS. The modifications can be also done after the TS initial submission to RAN plenary. |  |
| Hughes/EchoStar | We support having more than one SAN classes, example 2 classes. Also support moderators’ view to focus on having a stable TS version at this meeting, because we need to submit TS with the core requirements by the next plenary meeting. |  |  |
| Ligado Networks | We support a minimum of 2 classes (GSO/GEO and LEO) and are open to supporting 2 separate LEO classes if required. |  |  |
| Huawei | Option 1. (see also TP in R4-2205976). For sake of spec drafting, the starting point shall be rather multiple classes (with related requirements) which can be further merged into one, if technically justified. |  | Disagree with the CR option as this would lead to modifications of core requirements (Cat B CR beyond the core spec deadline is NOT allowed) in parallel to the conformance requirements work. |
| Omnispace | We support having more than 1 SAN classes, if there is good justification. |  |  |
| Kepler | We support having more than 1 SAN classes. |  |  |
| Intelsat | We support having at least two classes (GEO/LEO) |  |  |
| Inmarsat | We share the view that we should at least define 2 separate classes, for GSO/GEO and for LEO. Whether it is necessary for now to establish multiple LEO classes can be discussed. We can support that option as well if other companies think it’s absolutely necessary. |  |  |
| Hispasat | We support having more than 1 SAN classes, at least considering GEO and LEO and open to more specific classes based on the nature of satellite orbits/systems. |  |  |

GTW Agreement (23/02/2022):

Introduce SAN classes in Rel-17: LEO and GEO

Further discuss the requirements associated with SAN classes in case by case manner including [IoT level/ICS, NF, ACLR/ACS, Emission]

Note: Introducing additional classes in future release not precluded.

The following issues 2-1-2 and 2-1-3 have been added after the initial pre-meeting discussion:

Issue 2-1-2: SAN number of classes

- Proposals

  - Option 1: Do not introduce any SAN class, but then clarify how to handle the different limits for Rx requirements

  - Option 2: RAN4 should introduce one single SAN class: GEO, LEO600 and LEO1200, for SAN type 1-H and type 1-O.

  - Option 3 (based on pre-meeting discussion): RAN4 should introduce two SAN classes: GEO and LEO (for both LEO600 and LEO1200), for SAN type 1-H and type 1-O.

- Recommended WF

  - TBD

  - Moderator Note: we should focus to have a stable TS version at this meeting, because we need to submit TS with the core requirements by the next plenary meeting, and (as a general recommendation from the chairman) is preferably not to have an exception sheet.

Question: Which option (listed above) do you prefer? Please provide your answer(s) e.g. “Yes” or “No”.

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 | Comments Option 2 | Comments Option 3 |
| --- | --- | --- | --- |
| CATT |  |  | This could be a good compromise. |
| Ericsson | This would be acceptable but then we need to clarify we would have only 1 set of requirements (most stringent limits then) for ACLR, OBUE, REFSENS, dynamic range, ACS, ICS.. which is not the case right now looking at previous agreements and various proposals for this meeting. | This would be acceptable but then we need to clarify we would have only 1 set of requirements (most stringent limits then) for ACLR, OBUE, REFSENS, dynamic range, ACS, ICS.. which is not the case right now looking at previous agreements and various proposals for this meeting. | This would be acceptable as well, but the LEO requirements should then be based on the most stringent values from LEO600 and LEO1200. |
| ESA |  |  | This is our preference |
| ZTE | We still believe there options are necessary, otherwise we just need to consider the worst requirement for SAN class |  |  |
| Hughes/EchoStar |  |  | This is our preference (preferably GEO and NGSO classes) |
| THALES | This is easier to integrate for the time being, seeing the current contributions. We need support from other companies if we go for Option 3. If for example we cannot reach an agreement in Part2 to separate ACLR and/or ACS per GEO and LEO, we would still prefer Option 1. | This is easier to integrate for the time being, seeing the current contributions. We need support from other companies if we go for Option 3. If for example we cannot reach an agreement in Part2 to separate ACLR and/or ACS per GEO and LEO, we would still prefer Option 2. | Agree with 2 classes (GEO and LEO) if: - differentiation also in terms of ACLR (but this has to be decided in Part2 related to coexistence studies) and potentially other parameters as mentioned by Ericsson. - no definition based on SAN power limitation - [very important] companies are willing to update their respective TPs and re-submit them during the 2nd round of discussions, so we can integrate 2 classes in the current TS 38.108. The class differentiation should not be based only on NFs (since we can have a multitude of NFs, based on each satellite implementation). Moreover, ACS values seem to be very similar to GEO and LEO. |
| Ligado Networks |  |  | We support option 3 of having separate SAN classes for GSO/GEO and LEO |
| Huawei | Topic 2-1-1 already covers this discussion. Based on the NF and IoT discussion, none of the options would fit as 3 classes seems to be needed. |  |  |
| Omnispace |  |  | We support option3. |
| Kepler |  |  | We support option 3 (GEO and LEO). |
| Intelsat |  |  | We support option 3 |
| Inmarsat |  |  | We support Option 3, but probably we could define just 2 separate SAN classes for GSO/GEO and NGSO/LEO. Further classes could be considered if deemed necessary. |
| Nokia |  |  | We are okay with this option but LEO should be based on the most stringent of LEO600 and LEO 1200 |
| Hispasat |  |  | We support option 3. |

GTW Agreement (23/02/2022):

Introduce SAN classes in Rel-17: LEO and GEO

Further discuss the requirements associated with SAN classes in case by case manner including [IoT level/ICS, NF, ACLR/ACS, Emission]

Note: Introducing additional classes in future release not precluded.

Issue 2-1-3: SAN class differentiation (if 2 classes to be introduced, based on pre-meeting discussion)

- Proposals

  - Option 1 : (At least) the following parameters should be considered for GEO and LEO SAN classes differentiation, for SAN type 1-H and type 1-O:

    - ACLR (i.e. an ACLR for GEO SAN, and different ACLR for LEO SAN)

    - ACS (i.e. an ACS for GEO SAN, and different ACS for LEO SAN)

    - NF (i.e. different NF for GEO and for LEO, to be considered together with previous parameters)

- Recommended WF

  - TBD

  - Moderator Note: we should focus to have a stable TS version at this meeting, because we need to submit TS with the core requirements by the next plenary meeting, and (as a general recommendation from the chairman) is preferably not to have an exception sheet.

Question: Which option (listed above) do you prefer? Please provide your answer(s) e.g. “Yes” or “No”.

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 |
| --- | --- |
| CATT | Despite ACLR/ACS/NR, there are more requirement differentiation, e.g. dynamic range, ICS, output power, protection of own receivers… |
| Ericsson | From the agreements on REFSENS, we need at least to consider the different NF values. From the proposals made in this meeting, we would also need to consider IoT level (dyn range) . To keep it simple, we would still prefer to have one unique ACLR and ACS value, but this could also be considered. |
| ZTE | ACLR and ACS requirement could be same for different SAN class, however there are still other requirement as mentioned by CATT, it would be different for different class; |
| Hughes/EchoStar | We believe only for the differentiation of ACLR and ACS and not other requirement. |
| Ligado Networks | We agree that different parameters should be specified for different SAN classes including ACLR, ACS based on the simulation results. |
| Huawei | Agree with CATT and ZTE concerns. |
| THALES | Let us see what other parameters we can take onboard, and if companies agree to separate/define different SAN ACLR and SAN ACS for GEO and LEO in Part2. In any case, the parameters have to be considered together, and in only 2 classes. We also count on the help from other companies to modify their respective TPs if we follow such approach. |
| Inmarsat | We believe that at least different ACLR and ACS should be specified for different SAN classes based on simulation results. Other parameters – open to discuss. |

GTW Agreement (23/02/2022):

Introduce SAN classes in Rel-17: LEO and GEO

Further discuss the requirements associated with SAN classes in case by case manner including [IoT level/ICS, NF, ACLR/ACS, Emission]

Note: Introducing additional classes in future release not precluded.

## Companies views’ collection for 1st round

### Open issues

Example 1

| Company | Comments |
| --- | --- |
| XXX | Please see above. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1-1: | There are currently no proposals to be considered for the 2nd round of discussions for issue 2-1-1. Companies will follow TP submission recommendation with 2 classes. GTW Agreement (23/02/2022): Introduce SAN classes in Rel-17: LEO and GEO Further discuss the requirements associated with SAN classes in case by case manner including [IoT level/ICS, NF, ACLR/ACS, Emission] Note: Introducing additional classes in future release not precluded. |
| Issue 2-1-2: SAN number of classes | There are currently no proposals to be considered for the 2nd round of discussions for issue 2-1-1. GTW Agreement (23/02/2022): Introduce SAN classes in Rel-17: LEO and GEO Further discuss the requirements associated with SAN classes in case by case manner including [IoT level/ICS, NF, ACLR/ACS, Emission] Note: Introducing additional classes in future release not precluded. |
| Issue 2-1-3: SAN class differentiation (if 2 classes to be introduced, based on pre-meeting discussion) | There are currently no proposals to be considered for the 2nd round of discussions for issue 2-1-1. GTW Agreement (23/02/2022): Introduce SAN classes in Rel-17: LEO and GEO Further discuss the requirements associated with SAN classes in case by case manner including [IoT level/ICS, NF, ACLR/ACS, Emission] Note: Introducing additional classes in future release not precluded. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Companies to discuss if any concerns with the approach, or any other parameters to be considered.

# Topic #3: TS (writing) assumptions

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| TDoc Number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2205051 | Ericsson | Observation1: There is no procedure to guarantee a NTN UE will not connect to a SAN when there is adjacent TN coverage. Observation2: For all cases, the coexistence simulations have always assumed that the NTN UEs will be either dropped at the TN clusters edge or with an isolation distance from this edge. Proposal1: Capture the following note in TS 38.108 and TS 38.101-5: Note: NTN operation is assumed to be outside a Terrestrial Network operating in an adjacent channel/band. Proposal2: Capture the above note in the scope sub-clause of TS 38.108 and TS 38.101-1. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Moderator Note1: Please see updated NR NTN WID from RAN#94-e in RP-213691 (agreed):

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 1) | Title | For info at TSG# | For approval at TSG# | Remarks |
| 38.863 | Internal TR | Non-terrestrial networks (NTN)related RF and co-existence aspects | 94-e | 95-e | Led by RAN4, rapporteur: Yiran Jin, yiran.jin@samsung.com Core part; |
| 38.101-5 | TS | NR; User Equipment (UE) radio transmission and reception, part 5: Satellite access Radio Frequency (RF) and performance requirements |  | 95-e | Led by RAN4, rapporteur: Yiran Jin, yiran.jin@samsung.com Core part |
| 38.108 | TS | NR; Satellite Access Node radio transmission and reception | 94-e | 95-e | Led by RAN4, rapporteur: Dorin Panaitopol, dorin.panaitopol@thalesgroup.com Core part; |
| 38.181 | TS | NR; Satellite Access Node conformance testing | 96 | 97 | Led by RAN4, rapporteur: Yuexia Song, songyuexia@catt.cn Performance part; |

Moderator Note2: Please also note that the draft skeletons of TS 38.108 and TS 38.101-5 have been approved in R4-2203080, while the (updated) draft skeleton of TR 38.863 has been approved in R4-2203130.

### Sub-topic 3-1

Sub-topic description: General approach to write NTN TS

Open issues and candidate options before e-meeting:

Issue 3-1-1: General approach to write NTN TS (i.e. TS 38.108 and TS 38.101-5)

- Proposals

  - Option 1: Capture the following note in TS 38.108 and TS 38.101-5:

    - Note: NTN operation is assumed to be outside a Terrestrial Network operating in an adjacent channel/band.

- Recommended WF

  - TBD

Question: Which is the company preference for the above option?

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 |
| --- | --- |
| Qualcomm | We prefer NOT to capture the Note in the TS. It can be captured in the TR that is the typical RAN4 working approach to capture the simulation assumptions. In addition, for case 1, even NTN UEs access to the SAN, it will not have impact on the TN performance. It only might lead to the DL performance loss for NTN UE. |
| CATT | What the note descripts is not always the case. It depends on the band allocation. |
| Ericsson | Option 1, this note is needed as it’s a key assumption for all coex simulations. If this is only mentioned in the TR, this would create some misunderstanding on the interpretation of the corresponding limits in the TS. To Qualcomm: The intention with this note is not really to warn about any TN performance impact but mainly to clarify those limits guarantee a minimum NTN performance based on this assumption. To CATT: Even if the TN is operating in an adjacent band, the NTN UEs should still not be able connect if they are in TN coverage, right? The assumptions taken in our simulations are also valid if this is an adjacent band (Dfobue and Dfoob are applicable in the adjacent band). |
| Huawei | How to coordinate the satellite operation and Terrestrial Network operating can follow the regulation. Not sure RAN4 should specify something to restrict the specific deployment. |
| ZTE | No strong opinions on that issue since this TN DL interfering NTN DL case, it’s true that NTN UE cannot operate in the adjacent channel of TN network. |
| Hughes/EchoStar | Prefer NOT to capture note in the TS. |
| Ligado Networks | Even for TN, simulation assumptions were always captured in TR and not in the TS. We should follow the same procedure. Deployment constraints are based on local regulations and/or bilateral agreements. |
| THALES | We can mention in the TR 38.863 only (and not in the TS). In any case, we already have a definition for the NTN operation. For the time being, the NTN operation in FR1 considers n256 and n255. |
| Kepler | Prefer NOT to capture the note. |
| Intelsat | We would prefer not to capture the note. |
| Inmarsat | There is no scope to add this note to the specification. If this is useful as part of the simulation assumptions, it can be added to the TR, but we should remember that deployment constraints are subject of regulation and inter-operator agreements, so even adding this note to the TR can only serve to clarify the simulation assumptions. |
| Nokia | We think the note is needed with the reasoning listed by Ericsson. We are however fine to add the note only in the TR with the understanding that this have been the basis for the simulation assumptions leading to the defined requirements. |
| Hispasat | Prefer NOT to capture this note in the TS. |
| Omnispace | We prefer not to capture the note in the TS. |

Issue 3-1-2: Scope of TS 38.108 and TS 38.101-5:

- Proposals

  - Option 1: Capture the following note in the scope sub-clause of TS 38.108 and TS 38.101-1.

    - Note: NTN operation is assumed to be outside a Terrestrial Network operating in an adjacent channel/band.

- Recommended WF

  - TBD

Question: Which is the company preference for the above option?

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 |
| --- | --- |
| Qualcomm | We prefer NOT to capture the Note in the TS. It can be captured in the TR that is the typical RAN4 working approach to capture the simulation assumptions. In addition, for case 1, even NTN UEs access to the SAN, it will not have impact on the TN performance. It only might lead to the DL performance loss for NTN UE. |
| CATT | What the note descripts is not always the case. It depends on the band allocation. |
| Ericsson | We proposed option 1 but we are open for any other proposal. As this assumption was mainly used to determine ACLR/ACS, we could also capture it in ACLR and ACS sub-clauses only. |
| Huawei | How to coordinate the satellite operation and Terrestrial Network operating can follow the regulation. Not sure RAN4 should specify something to restrict the specific deployment. |
| ZTE | No strong opinions on that issue since this TN DL interfering NTN DL case, it’s true that NTN UE cannot operate in the adjacent channel of TN network. |
| Hughes/EchoStar | Prefer NOT to capture note in the TS. |
| Ligado Networks | Even for TN, simulation assumptions were always captured in TR and not in the TS. We should follow the same procedure. Deployment constraints are based on local regulations and/or bilateral agreements. |
| THALES | We can mention in the TR 38.863 only (and not in the TS). In any case, we already have a definition for the NTN operation. For the time being, the NTN operation in FR1 considers n256 and n255. |
| Kepler | Prefer NOT to capture the note. |
| Intelsat | We would prefer not to capture the note. |
| Inmarsat | This note should NOT be captured in the TS. If this is useful as part of the simulation assumptions, it can be added to the TR, but we should remember that deployment constraints are subject of regulation and inter-operator agreements, so even adding this note to the TR can only serve to clarify the simulation assumptions. |
| Hispasat | Prefer NOT to capture this note in the TS. |
| Omnispace | We prefer not to capture the note in the TS. |
| MediaTek | Prefer not to capture any note in the TS. Agree with Qualcomm that the issue observed that drove the isolation consideration was possible NTN DL receive performance degradation, not TN degradation. |

## Companies views’ collection for 1st round

### Open issues

Example 1

| Company | Comments |
| --- | --- |
| XXX | Please see above |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 3-1-1: | It seems that the majority of the companies would prefer the following proposal: Proposal 3-1-1: Capture the following note in TR 38.863 (and not in TS 38.108 and TS 38.101-5): - Note: NTN operation is assumed to be outside a Terrestrial Network operating in an adjacent channel/band. |
| Issue 3-1-2: | It seems that the majority of the companies would prefer the following proposal: Proposal 3-1-2: Capture the following note in the scope sub-clause of TR 38.863 (and not in TS 38.108 and TS 38.101-1): - Note: NTN operation is assumed to be outside a Terrestrial Network operating in an adjacent channel/band. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Questions and tables to be added for the second round.

# Topic #4: pCRs/CRs

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| TDoc Number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2205232 | HUGHES Network Systems Ltd | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. TP TR 38.863 7.4.1 NTN UE Requirement (General) |
| R4-2203953 | CATT | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. TP for 38.108: clause 5.3&5.4 on system parameters |
| R4-2204507 | Qualcomm Incorporated | TP to TS 38.101-5 to be discussed in the dedicated 1st round & 2nd round folders. TP on TS 38.101-5 for UE channel bandwidth and channel arrangement |
| R4-2203956 | CATT | pCR moved to [102-e][310] |
| R4-2205673 | THALES | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clause 4.3 Requirement reference points - TS 38.108 |
| R4-2205730 | THALES | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clause 4.4 Satellite Access Node classes - TS 38.108 |
| R4-2205111 | Ligado Networks | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. TP for TR 38.863: Regulatory aspects for NTN satellite access nodes and UEs operating in UL1626.5-1660.5 MHz and DL 1525-1559 MHz frequencies ranges |
| R4-2205314 | HUGHES Network Systems Ltd | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. TP to TR 38.863 on Section 5.2 NTN Satellite band |
| R4-2205733 | THALES | Withdrawn |
| R4-2205555 | Nokia, Nokia Shanghai Bell | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. TP to TR 38.863 Regulatory aspects for HAPS |
| R4-2205671 | THALES | TP to TS 38.101-5 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clause 3 - TS 38.101-5 |
| R4-2205672 | THALES | TP to TS 38.101-5 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clause 4 - TS 38.101-5 |
| R4-2205667 | THALES | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Annex B - TS 38.108 |
| R4-2205921 | THALES | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clauses 7, 7.1, 7.2, 7.3 in TR 38.863 |
| R4-2204195 | Softbank, Deutsche Telekom, Ericsson, NTT Docomo, KDDI, Nokia, Intelsat | CR to TS 38.104 to be discussed in the dedicated 1st round & 2nd round folders. CR for TS 38.104: capturing HAPS requirements |
| R4-2203952 | CATT | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. TP for 38.108: clause 4.3 requirement reference point |
| R4-2205437 | HUGHES Network Systems Ltd | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. TP to TR 38.108 on 4.5 Regional Requirement |
| R4-2205476 | ZTE Corporation | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. TP for TS 38.108: Genera(5.1) and Operating Band(5.2) |
| R4-2205472 | ZTE Corporation | TP to TS 38.101-5 to be discussed in the dedicated 1st round & 2nd round folders. TP for TS 38.101-5: Genera(5.1) and Operating Band(5.2) |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1

Sub-topic description: pCRs to TR 38.863

Open issues and candidate options before e-meeting:

Issue 4-1-1: pCR to TR 38.863 – see R4-2205232 (HUGHES Network Systems Ltd), TP TR 38.863 7.4.1 NTN UE Requirement (General)

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | To be revised | See comments in the dedicated folder. |
| Huawei | Disagree | These are not UE RF requirements, They are just some high level principle about how to develop the TS 38.101-5. I’m not it have to be included in this technical report as the principles have been agreed in RAN plenary. |
| ZTE | Agree |  |
| Hughes/EchoStar | Agree |  |
| THALES | Agree |  |
| Intelsat | Agree pCR |  |
| Omnispace | Agree |  |

Moderator recommendation: to be revised. Discussion to continue during 2nd round.

Issue 4-1-2: pCR to TR 38.863 – see R4-2205111 (Ligado Networks), TP for TR 38.863: Regulatory aspects for NTN satellite access nodes and UEs operating in UL1626.5-1660.5 MHz and DL 1525-1559 MHz frequencies ranges

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | To be revised | See comments in the dedicated folder. |
| ZTE |  | The band n255 and n24 should be the same aligned, however the figure is not aligned with that. In addition, more clarifications on US380, is that co-channel or non co-channel? The reference of FCC Orders 03-15 and 20-48 are also needed. |
| Hughes/EchoStar | Agree | Simplify the ITU footnotes per EAB’s suggestion |
| Ligado Networks |  | [Ericsson]: we can trim, but we noticed that all the notes for n256 were captured; question – also, is it ok to have the note in the main table but not the detailed description in the footnote? It will just be hanging. [ZTE]: Figure will be revised to align n255 and n24. US380 does not restrict co-cochannel spectrum use between satellite and terrestrial within US; it is possible for the authorized operators to deploy co-channel with the necessary geographic separation in the same way CEPT allows CGC between n256 and n65. References for FCC Orders 03-15 and 20-48 will be added in the revised |
| THALES | Agree | Some clarification required as asked by other companies. |

Moderator recommendation: to be revised. Discussion to continue during 2nd round.

Issue 4-1-3: pCR to TR 38.863 – see R4-2205314 (HUGHES Network Systems Ltd), TP to TR 38.863 on Section 5.2 NTN Satellite band

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Qualcomm |  | The sentence “Also, the NTN band n256 will fully be fully overlapped by TN NR bands n65” depends on the duplexer assumption for n256. |
| ZTE |  | n256 and Complementary Ground Component (CGC) could operate simultaneously operating in the upper 30 MHz portion of n65 in CEPT countries. Simultaneously operation, is that co-channel or adjacent channel for TN and NTN? |
| Hughes/EchoStar | Agree |  |
| THALES | Agree |  |
| Intelsat | Agree |  |
| Omnispace | Agree |  |
| Hughes/EchoStar | Agree | To Qualcomm - this regulatory matter/topic and not related to duplexer assumptions for n256 |

Moderator recommendation: Document seems to be agreeable, however discussion to continue during 2nd round.

Issue 4-1-4: pCR to TR 38.863 – see R4-2205555 (Nokia, Nokia Shanghai Bell), TP to TR 38.863 Regulatory aspects for HAPS

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Nokia | Agree | This corrects and ads the correct reference. |

Moderator recommendation: Document seems to be agreeable, however discussion to continue during 2nd round. It seems that not all companies had the time to review the document.

Issue 4-1-5: pCR to TS 38.863 – see R4-2205921 (THALES), Draft text proposal for Clauses 7, 7.1, 7.2, 7.3 in TR 38.863

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | To be revised | See comments in the dedicated folder. |
| ZTE |  | SAN class need more discussions on that, TP could be updated based on SAN class agreement. |
| Hughes/EchoStar | pending | TP will need update based on agreement on different SAN classes |
| Ligado Networks | Pending agreement on topic 2 | TP will require an update based on agreement on different SAN classes |
| THALES |  | We will update accordingly based on SAN class agreements in this meeting and resubmit the TP. |

Moderator recommendation: Revision required, document to be updated based on SAN classes agreement.

### Sub-topic 4-2

Sub-topic description: pCRs to TS 38.108

Open issues and candidate options before e-meeting:

Issue 4-2-1: pCR to TS 38.108 – see R4-2203953 (CATT), TP for 38.108: clause 5.3&5.4 on system parameters

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Qualcomm | agree partially | It was agreed in last meeting that the NTN spec should reuse the from TN TS as much as possible to avoid the duplication. The channel bandwidth and arrangement are same as TS38.104. |
| CATT | Agree | To Qualcomm, It has different agreement for 38.108 and 38.101-5. For 38.108, it is agreed that the requirement will be copy and pasted from 38.104 even though it is the same. Because, for SAN, different terminologies are using. Base station is not applicable any more. |
| Ericsson | To be revised | See comments in the dedicated folder. |
| ZTE |  | he number of RSAN configured in any BS channel bandwidth shall ensure that the minimum guardband specified in this clause is met. It should be RBs, right? |
| THALES |  | Comments will be added in the folder |

Moderator recommendation: Revision required, continue discussion for the 2nd round.

Issue 4-2-2: pCR to TS 38.108 – see R4-2205673 (THALES), Draft text proposal for Clause 4.3 Requirement reference points - TS 38.108

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | To be merged with 3952 | See comments in the dedicated folder. |
| ZTE | Agree |  |
| THALES |  | Yes, it can be merged with leading company contribution. We will provide the inputs in 3952. |

Moderator recommendation: R4-2205673 to be merged with R4-2203952. Comments to be provided directly in 3952.

Issue 4-2-3: pCR to TS 38.108 – see R4-2205730 (THALES), Draft text proposal for Clause 4.4 Satellite Access Node classes - TS 38.108

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Pending on #class decision | See comments in the dedicated folder. |
| ZTE |  | we think that different SAN class is still needed. |
| Ligado Networks | Pending agreement on topic 2 | pCR will require an update based on agreement on different SAN classes |
| THALES |  | We will update accordingly based on SAN class agreements in this meeting and resubmit the TP. |

Moderator recommendation: Revision required, document to be updated based on SAN classes agreement.

Issue 4-2-4: pCR to TS 38.108 – see R4-2205667 (THALES), Draft text proposal for Annex B - TS 38.108

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | To be revised | See comments in the dedicated folder. |
| ZTE |  | Based on the work split, ZTE’s also drafting this section based on the reference approach. |
| THALES |  | To ZTE: This is true, but it seems that there was no contribution from ZTE in this meeting on this topic, so we suggest keeping the contribution. To all companies: we tried to update as much as possible in order to have a stable TS for submission in the plenary. It was a very short time between the 2 RAN4 meetings, and also some holidays. For this reason, some extra contributions were made as support/as reference for this meeting on some topics considered as important. Sorry for any inconvenience caused. Our understanding is that the leading company has priority, but some other companies can still submit contributions on the same topic. The general assumptions are: - if there is a contribution from the leading company for a given meeting, it will be some merge with contribution from leading company, of course. - if there is not, then we should consider the contribution from the secondary company (please see the discussion on the reflector), in order to update as much as possible at this meeting. We hope this approach is fine for everyone (at least for this meeting) in order to advance faster. |

Moderator recommendation: To be revised, discussion to continue during 2nd round.

Issue 4-2-5: pCR to TS 38.108 – see R4-2203952 (CATT), TP for 38.108: clause 4.3 requirement reference point

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | To be revised | See comments in the dedicated folder. |
| ZTE |  | For a SAN type 1-O the transceiver unit array must contain at least 8 transmitter units and at least 8 receiver units. Transmitter units and receiver units may be combined into transceiver units. The transmitter/receiver units have the ability to transmit/receive parallel independent modulated symbol streams. The above assumptions for 8 transmitter units and 8 receivers are not needed since this assumption is for BS type 1-O with MIMO assumption. |
| THALES | To be updated with R4-2205673 | Agree with ZTE. In R4-2205673 we removed some unnecessary parts. You can update directly the text. |

Moderator recommendation: To be revised based on R4-2205673, discussion to continue during the 2nd round.

Issue 4-2-6: pCR to TS 38.108 – see R4-2205437 (HUGHES Network Systems Ltd), TP to TR 38.108 on 4.5 Regional Requirement

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | To be revised | See comments in the dedicated folder. |
| Hughes/EchoStar |  | EAB’s comments referred to Ligado |

Moderator recommendation: Potential revision needed, to be discuss during the 2nd round.

Issue 4-2-7: pCR to TS 38.108 – see R4-2205476 (ZTE Corporation), TP for TS 38.108: General (5.1) and Operating Band (5.2)

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | To be revised | See comments in the dedicated folder. |

Moderator recommendation: Potential revision needed, to be discuss during the 2nd round.

### Sub-topic 4-3

Sub-topic description: pCRs to TS 38.101-5

Open issues and candidate options before e-meeting:

Issue 4-3-1: pCR to TS 38.101-5 – see R4-2204507 (Qualcomm Incorporated), TP on TS 38.101-5 for UE channel bandwidth and channel arrangement

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | To be merged with 4808 |  |
| ZTE |  | NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: This UE channel bandwidth is optional in this release of the specification. In Table 5.3.5-1, the following notes are not necessary. |
| Hughes/EchoStar |  | OK with EAB’s comment |

Moderator recommendation: Document to be revised and continue discussion in the 2nd round. It was a request to merge with R4-2204808 (which is in Part4). Not clear why this request, since different Clauses from 4808.

Issue 4-3-2: pCR to TS 38.101-5 – see R4-2205671 (THALES), Draft text proposal for Clause 3 - TS 38.101-5

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | To be revised | See comments in the dedicated folder. |
| ZTE |  | PSCCH Physical Sidelink Control CHannel PSSCH Physical Sidelink Shared CHannel The above abbreviations are for sidelink and it could be removed. And some terms might be also not used in the UE spec, |
| THALES |  | Sure, of course. |

Moderator recommendation: Document to be revised and continue discussion in the 2nd round.

Issue 4-3-3: pCR to TS 38.101-5 – see R4-2205672 (THALES), Draft text proposal for Clause 4 - TS 38.101-5

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Qualcomm |  | See the comments in the pCR |
| Ericsson | To be revised | See comments in the dedicated folder. |
| Huawei |  | It should be further discussed whether conformance specification for satellite UE can be included in 3GPP TS 38.521-1 by RAN5. |
| THALES |  | To Huawei: we followed the recommendation from TS 38.101-5 rapporteur for this issue. |

Moderator recommendation: Document to be revised and continue discussion in the 2nd round. On the other hand, we can keep the reference and update it accordingly with a CR in later meetings.

Issue 4-3-4: pCR to TS 38.101-5 – see R4-2205472 (ZTE Corporation), TP for TS 38.101-5: General (5.1) and Operating Band (5.2)

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Qualcomm |  | Suggest to removing the FR2 frequency range at this stage. It is still not clear how to introduce the FR2 frequency for NTN, e.g., Ka band is not all in FR2-1. |
| Ericsson | To be revised | See comments in the dedicated folder. |
| ZTE |  | Fine to remove the FR2 |
| Hughes/EchoStar | To be revised | Suggest to removing the FR2 section, OK with EAB’s edits |
| Omnispace |  | Fine if we remove FR2 |

Moderator recommendation: Document to be revised and continue discussion in the 2nd round.

### Sub-topic 4-4

Sub-topic description: CRs to TS 38.104

Open issues and candidate options before e-meeting:

Issue 4-4-1: CR to TS 38.104 – see R4-2204195 (Softbank, Deutsche Telekom, Ericsson, NTT Docomo, KDDI, Nokia, Intelsat), CR for TS 38.104: capturing HAPS requirements

- Proposals

  - Option 1: Agree CR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [102-e][308] folders in both Round 1 and Round 2.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Agree with the CR |  |
| Huawei | Agree |  |
| ZTE | Partially disagree | Why BS type 2-o is also inclued in HAPS? |
| Intelsat | Agree |  |
| Nokia | Agree | We are okay to remove BS type 2-O |
| SoftBank | Agree | Okay for remove BS type 2-O |

Moderator recommendation: Document to be revised and continue discussion in the 2nd round.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | See above. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

TP revisions in the 2nd round.

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2205232 (HUGHES Network Systems Ltd) | to be revised |
| R4-2205111 (Ligado Networks) | to be revised |
| R4-2205314 (HUGHES Network Systems Ltd) | to be revised |
| R4-2205555 (Nokia, Nokia Shanghai Bell) | to be revised in case of any further comment |
| R4-2205921 (THALES) | to be revised |
| R4-2203953 (CATT) | to be revised |
| R4-2205673 (THALES) | to be merged with R4-2203952 |
| R4-2205730 (THALES) | to be revised |
| R4-2205667 (THALES) | to be revised |
| R4-2203952 (CATT) | to be revised based on R4-2205673 |
| R4-2205437 (HUGHES Network Systems Ltd) | to be revised |
| R4-2205476 (ZTE Corporation) | to be revised |
| R4-2204507 (Qualcomm Incorporated) | to be revised |
| R4-2205671 (THALES) | to be revised |
| R4-2205672 (THALES) | to be revised |
| R4-2205472 (ZTE Corporation) | to be revised |
| R4-2204195 (Softbank et al.) | to be revised |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Discussion to continue in dedicated tables and folders, in the 2nd round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2205232 (HUGHES Network Systems Ltd) | Company A |
|  | Company B |
| R4-2205111 (Ligado Networks) | Company A |
|  | Company B |
| R4-2205314 (HUGHES Network Systems Ltd) |  |
| R4-2205555 (Nokia, Nokia Shanghai Bell) |  |
| R4-2205921 (THALES) |  |

Discussion to continue in dedicated tables and folders, in the 2nd round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2203953 (CATT) |  |
| R4-2205730 (THALES) | Company A |
|  | Company B |
| R4-2205667 (THALES) | Company A |
|  | Company B |
| R4-2203952 (CATT) |  |
| R4-2205437 (HUGHES Network Systems Ltd) |  |
| R4-2205476 (ZTE Corporation) |  |

Discussion to continue in dedicated tables and folders, in the 2nd round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2204507 (Qualcomm Incorporated) | Company A |
|  | Company B |
| R4-2205671 (THALES) | Company A |
|  | Company B |
| R4-2205672 (THALES) |  |
| R4-2205472 (ZTE Corporation) |  |
| R4-2204195 (Softbank et al.) |  |

# Topic #5: HAPS Generalities

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| TDoc Number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2205554 | Nokia, Nokia Shanghai Bell | Observation 1: A new TS is to be introduced for NTN satellite access by RAN4 for both the BS (38.108) and UE (38.101-5) RF and performance requremens. Observation 2: HAPS specific technical specifications, if any, is agreed to be introduced in 38.104. Observation 3: There is no need to define UE specific requirements related to HAPS. Observation 4: Co-existence considarations when HAPS is deployed is no different thatn those for TN. Observation 5: HAPS have already been deployed utilizing LTE; it should be natural also to support these deployments in NR spectrum. Proposal 1: NR UEs as defined by current TS 38.101-1 can support HAPS deplyments with no additional changes needed in TS 38.101-1. Observation 6: RAN4 shall identify a existing NR band(s) which can be considered for HAPS operation or define a new band(s). Observation 7: NR band n1 fall in the ITU designated spectrum for HAPS. Proposal 2: Specify that HAPS can be deployed in band n1. |
| R4-2203536 | SoftBank, Deutsche Telekom, Ericsson, NTT Docomo, KDDI, Nokia, Intelsat | Proposal 1: Add the description of HAPS operating band to TS 38.104 clause 5.2 as below. ## 5.2 Operating bands NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2. NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation. NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n41, n65, n66, n70, n71, n74, n90 which are defined in Table 5.2-1. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 5-1

Sub-topic description: Spectrum/NR bands for HAPS

Open issues and candidate options before e-meeting:

Issue 5-1-1: NR operating band for HAPS

- Proposals

  - Option 1: Specify that HAPS can be deployed in band n1.

- Recommended WF

  - Option 1 (if no other options).

Question: Which is the company preference for the above option?

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 |
| --- | --- |
| Ericsson | Option1 |
| Huawei | Option 1 |
| Nokia | Option 1 |
| SoftBank | Option 1 |

Issue 5-1-2: Description of HAPS operating band to TS 38.104

- Proposals

## 5.2	Operating bands

NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n41, n65, n66, n70, n71, n74, n90 which are defined in Table 5.2-1.

Option 1: Add the description of HAPS operating band to TS 38.104 clause 5.2 as below.

- Recommended WF

  - Option 1 (if no other options).

Question: Which is the company preference for the above option?

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 |
| --- | --- |
| Ericsson | Option 1 |
| Huawei | Option 1 |
| Nokia | Option 1 |
| SoftBank | Option 1 |

### Sub-topic 5-2

Sub-topic description: HAPS technical specifications related to UE side

Open issues and candidate options before e-meeting:

Issue 5-2-1: HAPS UE

- Proposals

  - Option 1: NR UEs as defined by current TS 38.101-1 can support HAPS deployments with no additional changes needed in TS 38.101-1

- Recommended WF

  - Option 1 (if no other options).

Question: Which is the company preference for the above option?

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 |
| --- | --- |
| Qualcomm | OK with option 1. |
| Ericsson | Option 1 |
| Huawei | OK with option 1. |
| Nokia | Option 1 - HAPS have already been deployed utilizing LTE and LTE UE. Extension to NR is straightforward. |
| SoftBank | Option 1 |

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | See above |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 5-1-1: NR operating band for HAPS | Proposal 5-1-1: Specify that HAPS can be deployed in band n1. |
| Issue 5-1-2: Description of HAPS operating band to TS 38.104 | Proposal 5-1-2: Add the description of HAPS operating band to TS 38.104 clause 5.2 as below. ## 5.2 Operating bands NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2. NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation. NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n41, n65, n66, n70, n71, n74, n90 which are defined in Table 5.2-1. |
| Issue 5-2-1: HAPS UE | Proposal 5-2-1: NR UEs as defined by current TS 38.101-1 can support HAPS deployments with no additional changes needed in TS 38.101-1 |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Tables and questions to be added for the 2nd round.

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| Way Forward on NTN_solutions_Part1 | THALES | Document # R4-220xxxx WF [102-e][308] NTN_Solutions_Part1 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2207165 | Email discussion summary for [102-e][308] NTN_Solutions_Part1 | THALES | To be revised | R4-2207165 revised to R4-220xxxx |
| R4-2205232 | TP TR 38.863 7.4.1 NTN UE Requirement (General) | HUGHES Network Systems Ltd | To be revised |  |
| R4-2205111 | TP for TR 38.863: Regulatory aspects for NTN satellite access nodes and UEs operating in UL1626.5-1660.5 MHz and DL 1525-1559 MHz frequencies ranges | Ligado Networks | To be revised |  |
| R4-2205314 | TP to TR 38.863 on Section 5.2 NTN Satellite band | HUGHES Network Systems Ltd | to be revised |  |
| R4-2205555 | TP to TR 38.863 Regulatory aspects for HAPS | Nokia, Nokia Shanghai Bell | to be revised | to be revised in case of any further comment |
| R4-2205921 | Draft text proposal for Clauses 7, 7.1, 7.2, 7.3 in TR 38.863 | THALES | to be revised |  |
| R4-2203953 | TP for 38.108: clause 5.3&5.4 on system parameters | CATT | to be revised |  |
| R4-2205673 | Draft text proposal for Clause 4.3 Requirement reference points - TS 38.108 | THALES | to be merged | to be merged with R4-2203952 |
| R4-2205730 | Draft text proposal for Clause 4.4 Satellite Access Node classes - TS 38.108 | THALES | to be revised |  |
| R4-2205667 | Draft text proposal for Annex B - TS 38.108 | THALES | to be revised |  |
| R4-2203952 | TP for 38.108: clause 4.3 requirement reference point | CATT | to be revised | to be revised based on R4-2205673 |
| R4-2205437 | TP to TR 38.108 on 4.5 Regional Requirement | HUGHES Network Systems Ltd | to be revised |  |
| R4-2205476 | TP for TS 38.108: General (5.1) and Operating Band (5.2) | ZTE Corporation | to be revised |  |
| R4-2204507 | TP on TS 38.101-5 for UE channel bandwidth and channel arrangement | Qualcomm Incorporated | to be revised |  |
| R4-2205671 | Draft text proposal for Clause 3 - TS 38.101-5 | THALES | to be revised |  |
| R4-2205672 | Draft text proposal for Clause 4 - TS 38.101-5 | THALES | to be revised |  |
| R4-2205472 | TP for TS 38.101-5: General (5.1) and Operating Band (5.2) | ZTE Corporation | to be revised |  |
| R4-2204195 | CR for TS 38.104: capturing HAPS requirements | Softbank, Deutsche Telekom, Ericsson, NTT Docomo, KDDI, Nokia, Intelsat | to be revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm | Bin Han | binhan@qti.qualcomm.com |
| Nokia | Johannes Hejselbaek | Johannes.hejselbaek@nokia.com |
| THALES | Dorin Panaitopol |  |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Appendix: Companies contribution summary

Contribution summaries for [102-e][308] NTN_Solutions_Part1 thread are as follows:

| TDoc Number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2203964 | CATT | It is proposed the following is included in the Rel-17 UE feature list for NTN 64QAM: see also Annex 8. |
| R4-2205232 | HUGHES Network Systems Ltd | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. TP TR 38.863 7.4.1 NTN UE Requirement (General) |
| R4-2205554 | Nokia, Nokia Shanghai Bell | Observation 1: A new TS is to be introduced for NTN satellite access by RAN4 for both the BS (38.108) and UE (38.101-5) RF and performance requremens. Observation 2: HAPS specific technical specifications, if any, is agreed to be introduced in 38.104. Observation 3: There is no need to define UE specific requirements related to HAPS. Observation 4: Co-existence considarations when HAPS is deployed is no different thatn those for TN. Observation 5: HAPS have already been deployed utilizing LTE; it should be natural also to support these deployments in NR spectrum. Proposal 1: NR UEs as defined by current TS 38.101-1 can support HAPS deplyments with no additional changes needed in TS 38.101-1. Observation 6: RAN4 shall identify a existing NR band(s) which can be considered for HAPS operation or define a new band(s). Observation 7: NR band n1 fall in the ITU designated spectrum for HAPS. Proposal 2: Specify that HAPS can be deployed in band n1. |
| R4-2203953 | CATT | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. TP for 38.108: clause 5.3&5.4 on system parameters |
| R4-2204507 | Qualcomm Incorporated | TP to TS 38.101-5 to be discussed in the dedicated 1st round & 2nd round folders. TP on TS 38.101-5 for UE channel bandwidth and channel arrangement |
| R4-2203956 | CATT | pCR moved to [102-e][310] |
| R4-2205673 | THALES | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clause 4.3 Requirement reference points - TS 38.108 |
| R4-2205730 | THALES | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clause 4.4 Satellite Access Node classes - TS 38.108 |
| R4-2205048 | Ericsson | Observation1: Considering the different noise figure value for the different type of satellites, the following SAN classes should be considered: LEO and GEO. Observation2: Considering the different IoT level for the different type of satellites, the following SAN classes should be considered: LEO600, LEO1200 and GEO. And, based on those observations, we made the following proposal: Proposal: RAN4 should introduce the following SAN classes: GEO, LEO600 and LEO1200 for SAN type 1-H and type 1-O. |
| R4-2205111 | Ligado Networks | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. TP for TR 38.863: Regulatory aspects for NTN satellite access nodes and UEs operating in UL1626.5-1660.5 MHz and DL 1525-1559 MHz frequencies ranges |
| R4-2205314 | HUGHES Network Systems Ltd | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. TP to TR 38.863 on Section 5.2 NTN Satellite band |
| R4-2205733 | THALES | Withdrawn |
| R4-2205555 | Nokia, Nokia Shanghai Bell | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. TP to TR 38.863 Regulatory aspects for HAPS |
| R4-2205671 | THALES | TP to TS 38.101-5 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clause 3 - TS 38.101-5 |
| R4-2205672 | THALES | TP to TS 38.101-5 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clause 4 - TS 38.101-5 |
| R4-2205667 | THALES | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Annex B - TS 38.108 |
| R4-2205921 | THALES | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clauses 7, 7.1, 7.2, 7.3 in TR 38.863 |
| R4-2205051 | Ericsson | Observation1: There is no procedure to guarantee a NTN UE will not connect to a SAN when there is adjacent TN coverage. Observation2: For all cases, the coexistence simulations have always assumed that the NTN UEs will be either dropped at the TN clusters edge or with an isolation distance from this edge. Proposal1: Capture the following note in TS 38.108 and TS 38.101-5: Note: NTN operation is assumed to be outside a Terrestrial Network operating in an adjacent channel/band. Proposal2: Capture the above note in the scope sub-clause of TS 38.108 and TS 38.101-1. |
| R4-2204195 | Softbank, Deutsche Telekom, Ericsson, NTT Docomo, KDDI, Nokia, Intelsat | CR to TS 38.104 to be discussed in the dedicated 1st round & 2nd round folders. CR for TS 38.104: capturing HAPS requirements |
| R4-2203952 | CATT | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. TP for 38.108: clause 4.3 requirement reference point |
| R4-2203536 | SoftBank, Deutsche Telekom, Ericsson, NTT Docomo, KDDI, Nokia, Intelsat | Proposal 1: Add the description of HAPS operating band to TS 38.104 clause 5.2 as below. ## 5.2 Operating bands NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2. NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation. NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n41, n65, n66, n70, n71, n74, n90 which are defined in Table 5.2-1. |
| R4-2205437 | HUGHES Network Systems Ltd | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. TP to TR 38.108 on 4.5 Regional Requirement |
| R4-2205476 | ZTE Corporation | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. TP for TS 38.108: Genera(5.1) and Operating Band(5.2) |
| R4-2205472 | ZTE Corporation | TP to TS 38.101-5 to be discussed in the dedicated 1st round & 2nd round folders. TP for TS 38.101-5: Genera(5.1) and Operating Band(5.2) |

# Appendix: Proposed Rel-17 UE feature list for NTN 64QAM

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [22. NR_NTN_solutions | [22-1] | 64QAM for PDSCH | 64QAM for PDSCH | [N/A] | [Yes] | [No] | [UE does not support higher modulation 64QAM for PDSCH for NTN. The network cannot schedule 64QAM for PDSCH when DL propagation condition is good enough.] | [Per Band] | [FDD only] | [FR1 only] | [N/A] | [N/A] | [Optional with capability signalling] |
| [22. NR_NTN_solutions | [22-2] | 64QAM for PUSCH | 64QAM for PUSCH | [N/A] | [Yes] | [No] | [UE does not support higher modulation 64QAM for PUSCH for NTN. The network cannot schedule 64QAM for PUSCH when DL propagation condition is good enough.] | [Per Band] | [FDD only] | [FR1 only] | [N/A] | [N/A] | [Optional with capability signalling] |