3GPP TSG-RAN WG4 Meeting # 102-e												         R4-2207201

Electronic Meeting, February 21 – March 3, 2022

Agenda item:			11.1

Source:	Intel Corporation

Title:	WF on FR2-2 OTA test methods

Document for:	Approval

# Introduction

This Way Forward covers discussion agreements for the Study on Enhanced Test Methods in FR2

# Topics

## General

### Remaining work of SI

Candidate options:

- Proposal 1:	 RAN4 should conclude the preliminary MU assessments for UE RF, RRM, and demodulation based on the agreed assumption on the number of UE antenna elements.

- Proposal 2:	 RAN4 should conclude the max achievable SNR for demodulation and for RRM in the beam peak direction.

- Proposal 3:	 RAN4 should conclude the applicability of Objectives 1-5 to FR2-2 based on the agreements below and capture the related agreements in TR38.884.

  - As a starting point, the same High DL power and low UL power test cases for which NF based solutions (i.e. CFFNF, CFFDNF, and CFFdeltaNF) are applicable in FR2-1, can be considered for NF based solutions applicability in FR2-2. In case relaxations are needed for IFF/DFF methods for a given test case, it is up to RAN5 to confirm applicability of NF based solutions

  - At least, RSRPB based Rx beam peak search, Single link polarization measurement and Fast Spherical Coverage Method can be applied to 52.6-71GHz directly

For Proposal 1 and Proposal 2, consider the content of previous agreements captured below.

- Agreement: MU assessment for FR2-2 will focus on PC3 in Rel-17 timeline.  This, however, does not deprioritize the general work on other UE types in the WI (i.e., FWA and vehicular).

- Agreement: Approve the proposal (below) and extend applicability of Objective 2 and Objective 5 solutions to FR2-2.

  - Proposal: Applicability of methodology enhancements of three methods in Objective 5 can be extended to FR2-2. Objective 3 discussions should be postponed until core requirements are discussed. Lastly, we should further discuss the remaining objectives

Agreement: Proposal 1, Proposal 2 and Proposal 3 are agreed.

## OTA test methods for UE RF, RRM and demodulation for 52.6~71GHz

### General aspects

#### Testing and calibration

Candidate option:

- Proposal 1: RAN4 should confirm if the testing and calibration aspects detailed in Clause 5.2.1.3 of TR 38.810 can be extended to FR2-2.

Agreement: Approve Proposal 1; testing and calibration aspects can be extended to FR2-2. Relevant text is already included in R4-2207202.

#### Radiating aperture

Candidate options:

- Option 1: Yes, 5cm can be reused for D

  - Proposal: If D = 5cm is reused for FR2-2, a column for 71 GHz will be added to the minimum range length of DFF table in TR 38.810 (Table 5.2.1.2-1).

- Option 2: No, a different value is needed

Agreement:

- Option 1: reusing D = 5cm for PC3

- Additional information can be included into TR for the background if needed

### UE types

#### Single-element antenna assumption

Candidate options:

- Proposal 1: Feedback is requested from chipset vendors/device manufacturers which single-element antenna assumptions should be considered for PC1, PC2, and PC3 in FR2-2.

- Proposal 2: Single UE antenna element pattern parameters can be reused as Table G.1.1-1 in TR38.810, and half-power beamwidth and gain need to be further confirmed.

Agreement:

- Single UE antenna element pattern parameters can be reused as Table G.1.1-1 in TR38.810, with below exceptions:

  - half-power beamwidth: [80º/60º] as starting point

  - Gain: [5dBi]

#### Worst-case antenna array configuration (MxN) for PC1 and PC2

Candidate option:

- Proposal: Feedback is requested to clarify the worst-case antenna array configuration (MxN) for PC1 and PC2 UEs in FR2-2.

Agreement: Worst-case antenna array configuration (MxN) for PC1 and PC2 on MU assessment

- PC1: [144(12*12)]

- PC2: [40 (10*4)]

#### Beam steering assumptions of PC3, PC1 and PC2

Candidate option:

- Proposal: Feedback is requested to clarify the beam steering assumptions for PC1, PC2, and PC3 UEs in FR2-2.

Agreement: Reuse the beam steering assumptions in TR38.810 for PC3

### Test methodology for UE RF

#### MIMO EVM measurement

Candidate options:

- Proposal 1:  For two-layer uplink MIMO in FR2, define the zero-forcing receiver as the inverse of the effective channel matrix so that

GZF=H-1

- Proposal 2:	Agree to the text proposal for Section 5.2.3.1.1.2 of TR38.884-130 in the Appendix.

Agreement: Approve Proposal 1 and Proposal 2. Include relevant text proposal content in R4-2207202.

### Test methodology for RRM

#### Informative assessment of testable RRM DL SNR range

Agreement:

- Perform informative assessment of testable RRM DL SNR range for FR2-2 for the first and second scenario of RRM requirements and for both types of RRM requirements

- Assume [7] dB gain between fine and rough beams for FR2-2

- Above assumption has no impact on RRM core requirements discussion.

### Test methodology for Demodulation and CSI

#### FR2-2 max achievable DL SNR adjustment

Further work on the TP based on the following ways to change test methodology

- Acceptable ∆thermal decrease

  - Option 1: 1 dB (similar to FR2-1)

  - Option 2: 2 dB

  - Other options are not precluded

- Adjustment of TE

- Acceptable restriction of allocation size within CBW

- Note: ∆thermal decrease, and allocation size within CBW are included in the side condition for demodulation and CSI requirements.

#### Path delay grid

Candidate options:

- Option 1: Limit max applicable CBW and sampling frequency by 400MHz

- Option 2: Define accurate delay grid

  - Specify validation procedure and acceptance criteria for channel model implementation tolerances.

Agreement: Option 1: Limit maximum applicable CBW and minimum sampling frequency by [400MHz]

# References

- R4-2207452, “Email discussion summary for [102-e][337] FR2_enhTestMethods,” Moderator (Intel), RAN4 #102-e, March 2022