3GPP RAN WG4 Meeting #102-e		R4-2207203

Electronic Meeting, February 21 - March 3, 2022

Agenda item:	11.1.2.3

Source:	Apple

Title:	TP to TR38.884 on minimum SNR for RRM test cases for band n263

Work Item:	FS_FR2_enhTestMethods

Document for:	Approval

# 1	Introduction

During the RAN4 #101-bis meeting, RAN4 agreed the following tentative requirement for band n263 REFSENS:

Agreement: Min SENS for n263 400 MHz, based on averaging the proposals in the table is [-73.0 dBm]

Based on our contribution on minimum SNR for demodulation from the RAN4 #101-bis meeting [R4-2200907], we provide a text proposal to TR38.884.

This contribution is updated with revised SNR calculations [R4-2207204] and is based on the following agreement reached during the course of the RRM related discussions during the meeting:

Agreement: Perform informative assessment of testable RRM DL SNR range for FR2-2 for the first and second scenario of RRM requirements and for both types of RRM requirements Assume [7] dB gain between fine and rough beams for FR2-2 Above assumption has no impact on RRM core requirements discussion.

The following spherical coverage requirement agreements were used as well:

Agreement: Calculate the spherical coverage 50%ile drop based on the averaged value between 2 panel and 1 panel values in the table below. average between 2 panel and 1 panel = (8.5 + 13.9)/2 = 11.2 dB drop HW: Suggest to average across all the values, which results 12.2dB. In addition, my calculation of the linear averaging is 9.8dB for 2 panels and 14.1dB for 1 panel. Maybe some alignment is needed. Agreement: PC3 REFSENS is -72 dBm for n263 400 MHz.

# 2	Text Proposal

<begin text proposal>

# 6	UE RRM testing methodology enhancements

## 6.1	Extension of frequency applicability for band n262

Following the derivation of band-dependent parameters provided in Clause 7.1 and reusing the assumptions from Clause 6.2.1.4 of TR38.810 related to AoA scenarios and requirement types, the maximum achievable SNR for the RRM test setups with a PC3 DUT can be summarized as shown in Table 6.1-1 below.

Table 6.1-1: Comparison of maximum SNR between TR38.810 and preliminary extension to band n262

| RRM test setup | UE | CBW (MHz) | Max SNR (dB) [TR38.810] | Max SNR (dB) [n262] |
| --- | --- | --- | --- | --- |
| Scenario 1 (1AoA RX beam peak) for Type 1 Requirements (“Fine” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | [19.7] | [15.2] |
|  |  | 200 | [16.7] | [12.2] |
|  | Multi-band UE (NOTEs 1,2) | 100 | [17.7] | [14.2] |
|  |  | 200 | [14.6] | [11.2] |
| Scenario 1 (1AoA RX beam peak) for Type 2 Requirements (“Rough” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | [12.5] | [7.6] |
|  |  | 200 | [9.5] | [4.6] |
|  | Multi-band UE (NOTEs 1,2) | 100 | [10.5] | [6.6] |
|  |  | 200 | [7.5] | [3.6] |
| Scenario 2 (1AoA RX non-beam peak) for Type 1 Requirements (“Fine” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | [6.7] | [-2.7] |
|  |  | 200 | [3.7] | [-5.7] |
|  | Multi-band UE (NOTEs 1,2) | 100 | [4.7] | [-3.7] |
|  |  | 200 | [1.7] | [-6.7] |
| Scenario 2 (1AoA RX non-beam peak) for Type 2 Requirements (“Rough” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | Not usable | Not usable |
|  |  | 200 | Not usable | Not usable |
|  | Multi-band UE (NOTEs 1,2) | 100 | Not usable | Not usable |
|  |  | 200 | Not usable | Not usable |
| NOTE 1: For ∑MBp from TS 38.101-2 [2] Table 6.2.1.3-4 allows up to 2 dB in Rel-15 NOTE 2: From Rel-16 and later ∑MBp can be rounded up to 1.0 dB for all bands NOTE 3: The parameters and values in this table are preliminary and subject to further refinement by RAN5 as part of their conformance test development work. |  |  |  |  |

## 6.2	Extension of frequency applicability for FR2-2

### 6.2.1	General

Unless otherwise stated, all test methods and measurement setup for FR2 RRM methodology defined in TR 38.810 [3] Clause 6 and in TS 38.508-1 [10] Clause 7 are applicable for FR2-2.

### 6.2.2	Noc methodology

The Noc level for Scenario 1 (1AoA RX beam peak) for Type 1 Requirements ("Fine" RX beams) and Mode 1 Configuration (S+N) described in TR 38.810 Clause 6.2.1.4.3 need to be adjusted and can be derived similar to the Noc level for UE demodulation test methods described in Clause 7.2.2.

### 6.2.3	Maximum SNR derivation

Following the derivation of band-dependent parameters provided in Clause 7.2 and reusing the assumptions from Clause 6.2.1.4 of TR 38.810 related to AoA scenarios and requirement types, the maximum achievable SNR for the RRM test setups can be summarized as shown in Table 6.2.3-1.

Table 6.2.3-1: Maximum SNR preliminary extension to band n263

| RRM test setup | UE | CBW (MHz) | Max SNR (dB) [n263] |
| --- | --- | --- | --- |
| Scenario 1 (1AoA RX beam peak) for Type 1 Requirements (“Fine” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | [7.7] |
|  |  | 400 | [-0.6] |
|  | Multi-band UE | 100 | TBD |
|  |  | 400 | TBD |
| Scenario 1 (1AoA RX beam peak) for Type 2 Requirements (“Rough” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | < -20 (NOTE 1) |
|  |  | 400 | < -20 (NOTE 1) |
|  | Multi-band UE | 100 | TBD |
|  |  | 400 | TBD |
| Scenario 2 (1AoA RX non-beam peak) for Type 1 Requirements (“Fine” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | < -20 (NOTE 1) |
|  |  | 400 | < -20 (NOTE 1) |
|  | Multi-band UE | 100 | TBD |
|  |  | 400 | TBD |
| Scenario 2 (1AoA RX non-beam peak) for Type 2 Requirements (“Rough” RX beams) and Mode 1 Configuration (S+N) | Single band UE | 100 | < -20 (NOTE 1) |
|  |  | 400 | < -20 (NOTE 1) |
|  | Multi-band UE | 100 | TBD |
|  |  | 400 | TBD |
| NOTE 1: Result does not converge |  |  |  |

<end text proposal>