3GPP TSG-RAN WG4 Meeting # 102-e	R4-2207216

Electronic Meeting, 21 February – 3 March 2022

Agenda Item:	10.16.4

Source: 	Nokia, Nokia Shanghai Bell

Title: 	WF on >52.6 GHz BS RF Tx Requirements

Document for:	Approval

# 1.	Background

Contributions on BS RF TX requirements for 52.6 – 71 GHz in RAN4#102 have a variety of proposals:

[1]	R4-2203577	On BS RF transmitter requirements for the frequency range 52 to 71 GHz	Ericsson

[2]	R4-2203649	Proposals on BS transmitter requirements for extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

[3]	R4-2203975	Discussion on BS TX RF requirements for 52 6-71GHz	CATT

[4]	R4-2204436	Discussion on BS RF Tx requirements for 52.6 – 71 GHz		NEC

[4]	R4-2205460	Further discussion on BS Tx requirements for 52.6-71GHz	ZTE Corporation

[5]	R4-2206119	Discussion on the remaining BS RF requirements for FR2-2: Tx requirements		Huawei, HiSilicon

The way forward from the discussion of the contributions are summarized as follows.

# 2.	Way forward

2.1	TAE

Agreements:

-	MIMO TAE: Introducing 32.5 ns for both 480kHz and 960kHz SCS.

-	Intraband contiguous CA TAE: Introducing 32.5 ns for both 480kHz and 960kHz SCS.

-	Intraband non-contiguous CA TAE:

-	Agree that 260ns requirement is applicable for intraband non-contiguous CA TAE.

-	FFS whether to specify such requirements to BS specification pending on the conclusion in main and RRM sessions for the supporting on non-contiguous CA in Rel-17.

2.2	EVM

Agreements:

-	Add new tables with information only agreed values and remaining FFS 480 kHz and 960 kHz as described in Table 2.4-3 and Table 2.4-4.

-	Put 50% EVM window length within [] until conformance test feasibility issues are resolved.

-	Note: The value on EVM window length can be further discussed and updated if needed.

-	FFT sizes to be also double checked by companies.

2.3	Emissions: OBUE

Agreement:

-	For fOBUE set the upper band size boundary to 14000 MHz.

2.4	Emissions: Spurious emissions

Agreements:

-	The frequency step sizes for n264:

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| [n264] | [46] | [61] | [62.5] | [74.5] | [76] | [91] |

-	Introduce requirements for n264 in draftCR only if introduction of licensed band is agreed in main session.

2.5	ACLR

Agreement:

-	FFS whether to specify such requirements to BS specification pending on the conclusion in main session for the supporting on non-contiguous CA in Rel-17.