3GPP TSG-WG RAN4 Meeting #102-e	R4-2207240

Electronic Meeting, February 21 – March 3, 2022

WF on CRS-IM in scenarios with overlapping spectrum for LTE and NR with 30kHz SCS

CMCC

- Background

WFs approved in the previous meetings:

[1]	R4-2108662, WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR, RAN4 #99e.

[2]	R4-2115740, WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR, RAN4 #100e.

[3]	R4-2120705, WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR, RAN4 #101e.

[4]	R4-2203029, WF on CRS-IM in scenarios with overlapping spectrum for LTE and NR with 30kHz SCS, RAN4 #101bis-e.

- WF on Feasibility evaluation for 30 kHz SCS scenario

Whether to define CRS-IM requirements for 30 kHz SCS scenario

- Only define CRS-IM requirements for scenario with 30 kHz SCS assuming [10%] interference loading, 4 CRS ports and 1+1 DMRS configuration under the condition with enough performance discrimination between CRS-IM on and CRS-IM off i.e. at least 1dB performance difference observed

  - Channel BW: 20MHz

  - FFS on special slot configuration

  - Interested companies can bring results with Rel-15 rate matching (symbol level)

Implementation of LLR weighting in 30 kHz SCS scenario

- Up to UE implementation if no simulation result mis-alignment due to this issue.

- WF on CRS-IM Test Setup for 30kHz SCS Scenario

CBW and SCS for target and interference cells

- TDD 30kHz/20MHz for target cell and TDD 15kHz/20MHz for interference cells

  - the center frequency among target cell and interference cells are the same

TDD configuration for target cell with 30kHz SCS

- Use 7DS2U for the target cell with TDD 30kHz SCS

- FFS the configuration in the special slot

  - Option 1: S=6D+4G+4U

  - Other Options are not precluded

Time offset among cells

- Reuse the agreement for inter-cell IRC in 30 kHz SCS scenario, 1.5 us for cell 1, -0.5 us for cell 2

Simulation assumptions (Informative)

| Parameters | Value |
| --- | --- |
| TDD pattern for target cell | 7DS2U with S=6D+4G+4U Other special slot configurations are not precluded, interested companies can bring results. |
| CRS ports for interference cells | 4 CRS ports |
| SCS/CBW for target cell | 30kHz/20MHz |
| SCS/CBW for interference cells | 15kHz/20MHz |
| Center frequency | Same center frequency among target cell and interference cells |
| Time offset for interference cells | 1.5 us, -0.5 us |
| Inteference loading | 10% |
| DMRS configuration | 1 + 1 |
| Other parameters | Reuse the parameters from 15kHz Scenario |

Note: Companies are encouraged to provide simulation results with and without CRS-IM in next meeting

- WF on CR work split

| Section | Responsibility |
| --- | --- |
| General and applicability sections | CMCC |
| PDSCH requirements TDD | Huawei |
| Annex A: FRC | Intel |

Note: Under the premise of 1dB CRS-IM gain, the CRs can be submitted