3GPP TSG-RAN WG4 Meeting # 102-e	R4-2xxxxxx

Electronic Meeting, Feb.21st – Mar.3rd 2022

Title: 	Way Forward on MMSE-IRC for intra-cell inter-user interference

Source: 	Huawei, HiSilicon

Agenda item:	10.2.2.2

Document for:	Discussion

Inter-user interference modelling

DMRS scrambling ID for target UE and co-scheduled UE

- Background

- Option 1: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups. - Option 2: Same scrambling ID for all cases - Option 3: Configure variable scrambling ID during the test.

- Agreement

- Same scrambling ID for all cases

Simulation assumptions

Companies can provide the simulation results in next meeting based on the assumptions in Table 2-1:

Table 2-1: Simulation assumptions for MU-MIMO

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 for FDD and 40/30 for TDD |  |
| Duplex mode |  |  | FDD and TDD |  |
| TDD pattern |  |  | 7D1S2U S=6D+4G+4U |  |
| MCS |  |  | 13 | 16 QAM random symbols |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | DMRS port allocation |  | For rank 1+1: Port 1000 For rank 2+2: Port 1000 and 1001 | For rank 1+1: Port 1001 For rank 2+2: Port 1002 and 1003 |
| MIMO configuration |  |  | 2T2R ULA Low and 2T4R ULA Low for case with rank1+1 and 4T4R ULA Low (Note 1) for case with rank 2+2 |  |
| Number of CDM groups without data |  |  | 1 for case with rank 1+1 and 2 for case with rank 2+2 |  |
| HARQ process number |  |  | 4 for FDD and 8 for TDD |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - For case with rank 1+1: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of Co-scheduled UE is not equal to any column of precoding matrix of Target UE - For case with rank 2+2: Select the precoding matrix to ensure orthogonality with Target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | For rank 1+1: TDLC300-100 For rank 2+2: TDLA30-10 |  |
| Receiver type |  |  | MMSE-IRC | N/A |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |

UE feature list

- Backgroud

- Option 1: No need to introduce new UE feature, requirements release independent from Rel-15 - Option 2: Mandatory with or without UE capability signaling - Option 3: Optional with or without UE capability and applicable from Rel-17 - Option 4: Optional without UE capability and applicable from Rel-15

- Agreement

  - Optional without UE capability signalling for Rel-15/16 UE and mandatory from Rel-17

    - Define MMSE-IRC requirements in Rel-17 TS 38.101-4 without applicability rule.

    - Requirements are release independent from Rel-15. Add clarification in 38.307 that MMSE-IRC requirements are optional for Rel-15 and 16 UEs and can only be executed based on UE declaration.

CR split

| Section | Company |
| --- | --- |
| General and applicability section | ZTE |
| PDSCH requirements – 2 Rx | Intel |
| PDSCH requirements – 4 Rx | China Telecom |
| Annex A: FRC | Intel |
| Annex B: MU-MIMO Beamforming Model | Huawei |