3GPP TSG-RAN WG4 Meeting # 102-e                                                        R4-2207335

# Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	TP for TS 38.108: Genera(5.1) and Operating Band(5.2)

Agenda Item:	10.13.3

Document for:	Approval

- Introduction

In terms of the work split in the approved WF[1]. This contribution provides a text proposal to TS38.108 [2] on sub-clause 5.1(General) and sub-clause 5.2 (operating band)

- Reference

[1] R4-2203080, Way Forward on NTN_solutions_Part1, THALES

[2] R4-2203086,  Draft skeleton for TS 38.101-8, THALES

- Text Proposal

----- Start of TP -----

- 5				Operating bands and channel arrangement

  - 5.1				General

The channel arrangements presented in this clause are based on the operating bands and SAN channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and SAN channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NTN satellite can operate according to the present version of the specification are identified as described in table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |

  - 5.2				Operating bands

NTN satellite is designed to operate in the operating bands defined in table 5.2-1.

Table 5.2-1: NTN satellite operating bands in FR1

| NTN satelliteoperating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| NOTE 1: NTN satellite bands are numbered in descending order from n256.. |  |  |  |

----- End of TP -----