3GPP TSG-RAN WG4 Meeting # 102-e 		                       R4-2207370

Electronic Meeting, February 21st – March 3rd, 2022

Title: 	TP to TS 38.108 on 6.0 Conducted transmitter characteristics

Source: 	Hughes/EchoStar

Agenda item:	10.13.3.3

Document for:	Approval

# Introduction

This contribution provides a text proposal on 6.0 Conducted transmitter characteristics for TS 38.108 [1] based on the approved WF in [2].

# Reference

[1] R4-2203087 (3GPP TS 38.108 V0.0.1))

[2] WF R4-2203080

# Text proposal

---------------------------------------------------Start of Text proposal---------------------------------------------------------

# 6	Conducted transmitter characteristics

## 6.1	General

Unless otherwise stated, the conducted transmitter characteristics are specified at the TAB connector for SAN type 1-H, with a full complement of transceiver units for the configuration in normal operating conditions.

For SAN type 1-H the manufacturer shall declare the minimum number of supported geographical cells (i.e., geographical areas covered by beams). The minimum number of supported geographical cells (Ncells) relates to the BS setting with the minimum amount of cell splitting supported with transmission on all TAB connectors supporting the operating band, or with minimum amount of transmitted beams.

For SAN type 1-H manufacturer shall also declare TAB connector TX min cell groups. Every TAB connector of the SAN type 1-H supporting transmission in an operating band shall map to one TAB connector TX min cell group supporting the same operating band, where mapping of TAB connectors to cells/beams is implementation dependent.

---------------------------------------------------End of Text proposal---------------------------------------------------------