3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2207387

# Electronic meeting, January 17th – 25th, 2022

Source:			CATT

Title:			WF on open issue for Satellite Access Node

Agenda item:		6.13.3

Document for:		Approval

RF requirements for satellite access node

SAN type 1-H

64QAM EVM

Reuse the EVM requirement value for 64QAM from NR BS specification, i.e. 8 % as optional requirement subject to manufacturer declaration.

Operating band unwanted emissions

Will be addressed in a separate WF

Spurious emissions

Will be addressed in a separate WF

Intra-system intermodulation requirements

Not to define intra-system intermodulation requirement in 38.108.

Capture some background/techincial justification  information into TR if needed in the next meeting.

Dynamic range

Define single IOT requirement for LEO SAN Class: with [15] dB

ICS

Define ICS level as 12 dB for GEO, 24dB for LEO.

In-band blocking

Not to introduce in-band blocking requirements in Rel-17

SAN type 1-O

Reference point

Use the following figure as reference point for SAN type 1-O. move RIB outside SAN

EVM requirement

Mirror the OTA EVM requirement value for 64QAM from the conducted 1-H requirement, i.e. 8 % as optional requirement subject to manufacturer declaration.

“basic limit” concept and requirement scaling

Reuse the basic limit terminology for the NTN SAN unwanted emissions requirements/spurious emission without scaling factor

Include some backgroud/techinical justification into TR if needed

Do not define limitation on number of TRXU units

OTA Intra-system intermodulation

Not to define intra-system OTA IMD requirement

OTA reference sensitivity level

Specify OTA REFSENS requirement using the same limits than for SAN 1-H, adjusted with SAN ΔOTAREFSENS.

OTA dynamic range

Specify OTA dynamic using the same limits as 1-H ones but adjusted with the SAN ΔminSENS for both wanted signal and interferer values.

OTA ICS

It is proposed that OTA In-channel selectivity is specified using the same limits as SAN type 1-H ones but adjusted with the SAN ΔminSENS for both wanted signal and interfere values.

OTA in-band blocking

Following decision of SAN type 1-H, do not define OTA in-band blocking requirement.

References

[1] R4-2203948,

[2] R4-2205046

[3] R4-2205047

[4] R4-2205049

[5] R4-2205468

[6] R4-2205469

[7] R4-2205977

[8] R4-2205978

[9] R4-2203949

[10] R4-2205980

[11] R4-2203950

[12] R4-2205982

[13] R4-2203951

[14] R4-2205285

[15] R4-2205985