3GPP TSG-RAN4 Meeting #102-e                                                         R4-2207406

# Online, , 21st Feb 2022 - 3rd Mar 2022						 (Revision of R4-2205110)

Source:	Ligado Networks, Inmarsat

Title:	TP for TR 38.863: Updates to UE Maximum Output Power for n255

Agenda item:			10.13.4.1

Document for:	Approval

# 1. Introduction

This TP proposes updates to the tolerances for the maximum output power for n255 that were copied over from that specified for terrestrial operations in n24 [1] for the US. Additional tolerances were specified at the request of one company to meet the out of band emission requirements for FCC for terrestrial operations in n24. The stricter out of band emission requirements of FCC are not applicable for NTN satellite operations in n255.

# 2. Reference

- TR 38.863 v0.2.0, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence (Release 17)

# 3. Text Proposal

<Start of Text Proposal>

#### 7.4.2.2	Conducted transmitter characteristics

##### 7.4.2.2.1	Maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 7.4.2.2.1-1: UE Power Class

| NTN satellite band # | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23 | ±2 |
| n255 | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

<End of Text Proposal>