3GPP TSG-RAN WG4 Meeting # 102-e 	R4-2206820

Electronic Meeting, Feb 21st - Mar 3rd, 2022

Agenda item:			5.1.2.1

Source:			Samsung

Title:	WF on NR eMIMO RRM Maintenance

Document for:		Approval

Introduction

This contribution contains the WF for Rel-16 [102-e][204] Maintenance_NR_eMIMO_NWM issues discussed in RAN4#102-e w.r.t. core part and performance requirement maintenance.

Way Forward on eMIMO WI

RRM Core Requirement Maintenance

SSB-based PL-RS switching in FR2

- If a SSB resource is configured as Pathloss reference signal and it has already been configured as L1 measurement reference signal in FR2, longer delay for PL-RS switching is expected than the current delay requirement defined in TS38.133 clause 8.14.

  - RAN4 will further discuss how to resolve this technical issue in the next meeting:

    - Option 1: Add a Note for Pathloss RS switching delay requirement in the spec to clarify longer delay is expected for this case.

    - Other options are not precluded.

Performance Requirement Maintenance

Test Case for Pathloss Reference Signal Switching Delay

- Define the test case for Pathloss Reference Signal Switching Delay core requirement in Rel-16

- RAN4 may further improve the test procedures and technical details for the test case

  - FFS how to trigger 2 PHR reports before and after the switching in test case for comparing pathloss;

  - FFS whether configuring UL grant need to be explicitly specified in the test procedure;

  - Other issues are not precluded.