Background from R4-2205468, LEO OBUE requirement in 1st and 2nd breaking point seems agreeable except then measurement bandwidth. For GEO OBUE requirement, ACLR requirement would be updated in the thread 309.

Table 1.  GEO UEM limit values

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | 51dBm-24dBc-10*log10(5*10)+1dB margin=11dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | 10dBm | 100 kHz |

For other types of SAN class, similar as what has been adopted where power difference should be taken into account.

Table 2. LEO-1200 OBUE requirements

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | 53dBm-24dBc-10*log10(5*10)+1dB margin=13dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | 12 | 100 kHz |

Table 3. LEO-600 OBUE requirements

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | 47dBm-24dBc-10*log10(5*10)+1dB margin=7dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | 6 | 100 kHz |

GTW agreement:

Measurement bandwidth: 4kHz

General principle: Follow ITU recommendation 15-41 (SM.1541) at least for the frequency offset range within first two break points

It is proposed to further discuss in the 2nd round based on the TP from leading company.

Further action in 2nd round:

| Issue 3-3: Consideration of ACLR for GEO and LEO SAN | Agreement in GTW (2/23): Option 1 Use separate ACLR values for GEO and LEO SANs. Candidate options: N/A Recommendations for 2nd round: Further discuss the corresponding modification in the revised R4-2204333. |
| --- | --- |
| Issue 3-4: GEO SAN ACLR value | Agreement in GTW (2/23): LEO SAN ACLR: 24dBc Candidate options: Option 1: 14 dBc and capture relative contents in draft TR 38.863, noting that a further check on non-AAS results may be needed. Recommendations for 2nd round: Further discuss Option 1 and the corresponding modification in the revised R4-2204333. |

Table 1.  GEO UEM limit values

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | -10*log10(100/4) | 4 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | 51dBm-14dBc-10*log10(5*10)+1dB margin-10*log10(100/4)=21dBm-14=7dBm | 4 kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | -13 | 4 kHz |

For GEO SAN OBUE requirement, to be simplified as following:

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz |  | 4 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | 7dBm | 4 kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | -13 | 4 kHz |

For LEO600KM and LEO1200KM, since only one SAN class would be defined, then based the comparison of SAN OBUE requirement in Table 2 and Table 3 as illustrated in the following figure, it is proposed to use the LEO 600KM SAN OBUE requirement as baseline. In addition, measurement bandwidth should be 4KHz instead of 100KHz, therefore the emission mask would be refined as following:

Table 3. LEOOBUE requirements

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | -10*log10(100/4) | 4 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | 47dBm-24dBc-10*log10(5*10)+1dB-10*log10(100/4)margin=7dBm-14.0 | 4 kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | -13 | 4 kHz |

For LEO SAN OBUE requirement, to be simplified as following:

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz |  | 4 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | -7 | 4 kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | -13 | 4 kHz |

Note: please satellite vendor confirm that which section in ITU-R SM1541 could be referred for measurement bandwidth definition and spurious emission requirement proposed here.