3GPP TSG-RAN WG4 Meeting #102- e 												R4-22XXXXX

Electronic Meeting, 21 February – 03 March 2022

Agenda item:			10.1.1

Source:	vivo, CAICT

Title:	WF on NR MIMO OTA

Document for:	Approval

Introduction

This is the Way Forward on NR MIMO OTA.

Way Forward on Topic #1: General and Testing

Sub-topics summary

Sub-topic 1-1 FR1 channel model validation

Issue 1-1-1: PDP reference for FR1 channel model validation

Agreement:

- Option 2: Adopt the FR1 CDL-C UMi reference values in the following Tables:

- CDL-C UMi PDP Reference Values at ≤ 2.5 GHz

| Cluster | Delay [ns] | Power [dB] |
| --- | --- | --- |
| 1 | 0 | -30.7 |
| 2-5 | 20 | -19.2 |
| 6-10 | 65 | 0 |
| 11-12 | 130 | -31.4 |

- CDL-C UMi PDP Reference Values at > 2.5 GHz

| Cluster | Delay [ns] | Power [dB] |
| --- | --- | --- |
| 1 | 0 | -30.7 |
| 2-5 | 20 | -19.2 |
| 6-10 | 65 | 0 |
| 11-12 | 130 | -31.4 |

Issue 1-1-2: PDP pass/fail limits for FR1 channel model validation

Sub-issue 1-1-2-1: PDP pass/fail limits for FR1 CDL-C UMi channel model validation

- For FR1 CDL-C UMi channel model validation, which option is agreeable?

  - Option 1: Adopt the PDP pass/fail limits in R4-2119093 for FR1 CDL-C UMi channel model validation as below

|  | Power Tolerance | Delay Tolerance |
| --- | --- | --- |
| Paths from 0dB to 10dB | [±1dB] | [±6ns] |
| Paths from 10dB to 20dB | [±2.5dB] | [±6ns] |
| Paths from 20dB to 30dB | [±5dB] | [±6ns] |
| Paths from 30dB to 40dB | [±10dB] | [±6ns] |

  - Option 2: Adopt the PDP pass/fail limits in R4-2119093 for FR1 CDL-C UMi channel model validation as below (CAICT’s proposal from the 1st round)

|  | Power Tolerance | Delay Tolerance |
| --- | --- | --- |
| Paths from 0dB to 20dB | [±2.5dB] | [±6ns] |
| Paths from 20dB to 30dB | [±5dB] | [±6ns] |
| Paths beyond 30dB | [±10dB] | [±6ns] |

  - Option 3: others

Tentative Agreement:

  - Option 1

Sub-issue 1-1-2-2: PDP pass/fail limits for FR1 CDL-C UMa channel model validation

options:

- Option 1: For FR1 CDL-C UMa channel model validation, adopt the following PDP pass/fail limits

|  | Power Tolerance | Delay Tolerance |
| --- | --- | --- |
| Paths from 0dB to 10dB | ±1dB | ±6ns |
| Paths from 10dB to 20dB | ±2.5dB | ±6ns |
| Paths from 20dB to 30dB | ±5dB | ±6ns |
| Paths from 30dB to 40dB | - +/-10 dB at 290 ns for Beam 1 - +/-7.5 dB for others | [±6ns] |

- Option 2: Using the same PDP pass/fail limits for both CDL-C UMi and CDL-C UMa;

Agreement:

  - Option 2

Issue 1-1-3: Spatial Correlation pass/fail limits for FR1 CDL-C UMa channel model validation

Agreement:

  - For CDL-C UMa 3600MHz, the pass/fail limits are formed as bands of ±10% of correlation capped at 100% for the upper limit for target correlation of 65% and above. For target correlations below 65%, the band is widened to ±20% capped at 0%.

  - For others, the pass/fail limits are formed as bands of ±10% of correlation capped at 100% for the upper limit for target correlation of 35% and above. For target correlations below 35%, the band is widened to ±20% capped at 0%.

Way Forward on Topic #2: Performance requirements

Sub-topics summary

Sub-topic 2-1 FR1 MIMO OTA lab alignment activity

Issue 2-1-1: Reference value for FR1 MIMO OTA lab alignment

Agreements:

  - Removal of apparent outlier (if identified) should be considered in the average processing to derive reference value.

  - Define the reference and pass/fail limit of lab alignment together.

  - FFS the average approach of the measurement results submitted by test labs to derive reference value.

    - Inverse average is the 1st priority

    - linear average

  - FFS how to identify/treat the “apparent outlier” in the average processing to derive reference value.

Issue 2-1-2: Pass/fail limit for FR1 MIMO OTA lab alignment

Agreements:

The pass/fail limit for lab PAD alignment: the maximum deviation of TRMS between each performance alignment lab and Reference Value

  - Starting point: [+/- 0.75MU], i.e., +/- 2.25 dB for band <3GHz and +/- 2.55 dB for band >3GHz.

  - Further check is allowed after having some PAD test results next RAN4 meeting.

[It should be noted that in the framework for NR MIMO OTA requirement, the re-test of a test lab is allowed, if large offset is found from the submitted results.]

Issue 2-1-3: Arrangement of PAD test

Agreement:

  - After submitting valid channel model validation results to 3GPP RAN4 #103-e meeting, SGS TW can be arranged to test the Performance Alignment Devices (PADs) in the last labs. (The shipping order for these PADs will be Beijing  Shanghai  Cupertino  New Taipei City)

Sub-topic 2-2 FR1 MIMO OTA performance test campaign

Issue 2-2-1: Framework for FR1 MIMO OTA performance test campaign

Agreement:

  - Approve the Framework for FR1 MIMO OTA performance test campaign in R4-2207302.

Issue 2-2-2: How to avoid the same UE model measured in several labs

Option 1: share all the supported band information of each device as a rough approach to avoid same model being measured by many times.

Option 2: disclosure all the UE models in 3GPP without any repeated UE models as a precise way

- Option 2a: among a small group, i.e. just test labs those join the lab alignment activity and will provide measurement results to define MIMO OTA requirements

- Option 2b: among all RAN4 interested companies, i.e. in a public contribution or via RAN4 reflector

Option 3: do not consider this aspect when specify FR1 MIMO OTA requirement

Pros&Cons:

option 1 is simple and anonymous but can not directly avoid same UE model;

option 2 can potentially address the issue but conflicts with anonymous approach, then mapping results to UE model can be roughly estimated if very few results (e.g. 1~2 devices) submitted from a lab. Besides, option 2 still can not fundamentally force lab to avoid the same UE model testing, if cheating exists.

option 3: CDF curve of data pool will be impacted, but FFS the impacts on final requirement, given additional margin will be discussed anyway based on CDF analysis

Agreement:

  - TBA

Issue 2-2-3: FR1 MIMO OTA TRMS requirements development

Agreement:

  - TBA.

Sub-topic 2-3 Mechanical mode of FR1 MIMO OTA performance

Issue 2-3: How to treat the mechanical mode in FR1 MIMO OTA performance requirement

Agreement:

  - The TRMS requirements only apply to the primary mechanical mode for devices having multiple mechanical modes.

  - The primary mechanical mode for devices having multiple mechanical modes shall be declared by the manufacturers.

Sub-topic 2-4 FR2 MIMO OTA performance requirements

Issue 2-4-1: How to calculate the sensitivity values by simulation SNR

Agreements:

  - Simulation results should be aligned firstly before specifying FR2 MIMO OTA requirements.

  - FFS whether both simulation results and measurement results should be taken into account when defining FR2 MIMO OTA requirements.

  - FFS the gap between simulation results and measurement results of FR2 UEs.

Issue 2-4-2: How to evaluate the offset of equivalent SNR due to the non-ideal factors

Agreement:

  - Use “Rel-15 EIS requirements” as the baseline UE assumption for MIMO OTA simulation analysis.

Issue 2-4-3: FR2 MIMO OTA simulation results for 36 test directions

Agreement:

  - RAN4 to use the limits of FR2 channel model validation for power and delay tolerance, and AoA/ZoA offsets to evaluate the maximum impact on the FR2 MIMO OTA simulation results as a starting point.

Sub-topic 2-5 MU budget for FR2 MIMO OTA

Issue 2-5-1: MU budget for FR2 MIMO OTA 3D-MPAC

Agreement:

  - Review and check whether R4-2207301 is agreeable as a starting point for further discussion.

Reference

[1] R4-2207424, Email discussion summary for [102-e][334] NR_MIMO_OTA, Moderator (CAICT), RAN4#102-e, Feb 2022.