3GPP TSG-RAN WG4 Meeting #102-e	R4-2206147

Online Meeting, 21 February – 03 March 2022

3GPP TSG RAN WG1 #108-e		R1-2202599

e-Meeting, February 21st – March 3rd, 2022

Title:	Reply LS on interruption for PUCCH SCell activation in invalid TA case

Response to:	R1-2200871 (R4-2120420)

Release:	Rel-17

Work Item:	NR_RRM_enh2-Core

Source:	RAN WG1

To:	RAN WG4

Cc:	RAN WG2

Contact Person:

#### Name: Chi-Hsuan Hsieh

#### E-mail Address:	CH.Hsieh@mediatek.com

Attachments:	None

1. Overall Description:

RAN1 thanks RAN4 for the LS regarding interruption for PUCCH SCell activation in invalid TA case. Regarding RAN4’s questions Q1/Q2:

- Q1: Can UE capability diffNumerologyAcrossPUCCH-Group defined in TS 38.306 be used to allow indication of UE support of parallel transmission of PRACH on PUCCH SCell and physical channels/signals with different SCS on other activated serving cells?

  - If YES, RAN4 would like to respectfully ask RAN2 to clarify that diffNumerologyAcrossPUCCH-Group applies to PRACH in the corresponding “Definitions for parameters” in TS 38.306.

  - If NO, are there any other existing capabilities that can serve this purpose? If so, RAN4 would like RAN1 to indicate the capabilities.

- Q2: If the answer in Q1 is NO, and there is no capabilities that can serve the purpose, what would be the expected UE behavior when a target PUCCH SCell PRACH has a different SCS from SpCell/SCell physical channels/signals according to RAN1/2 spec? e.g. Shall UE be able to simultaneously transmit PRACH on PUCCH SCell and physical channels/signals on other activated serving cells irrespective of SCS configurations?

RAN1’s reply is included below in paragraph A1/A2:

- A1:

- The answer is “NO”, since according to the capability description in 38.306, diffNumerologyAcrossPUCCH-Group is only related to data/control (data refers to PUSCH and control refers to PUCCH), but not to PRACH transmission

  - The relevant capability in 38.306 which serves RAN4’s designated purpose is parallelTxPRACH-SRS-PUCCH-PUSCH. Parallel PRACH transmissions are not supported for CA or for DC within the same cell-group.

- The following spec texts quoted from 38.306 are for RAN4’s reference

diffNumerologyAcrossPUCCH-Group Indicates whether different numerology across two NR PUCCH groups for data and control channel at a given time in NR CA and (NG)EN-DC/NE-DC is supported by the UE.

parallelTxPRACH-SRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band CA band combination.

- A2:

- For inter-band CA capability of simultaneous PRACH transmission on PUCCH SCell and SRS/PUCCH/PUSCH transmission on other activated serving cells, it is provided by parallelTxPRACH-SRS-PUCCH-PUSCH as from 38.306, regardless of the SCS combination. For intra-band CA, according to 38.213 8.1, UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when the gap between PRACH transmission and PUSCH/PUCCH/SRS transmission is less than N symbols; hence, UE does not transmit PRACH and PUSCH/PUCCH/SRS simultaneously for intra-band CA. Besides, it seems RAN4’s question is the case where the UE transmits PRACH on PUCCH SCell in the secondary PUCCH group and PUCCH/PUSCH/SRS on a UL cell in the primary PUCCH group; then to RAN1’s understanding this must be inter-band simultaneous transmissions.

- The following spec texts quoted from 38.213 8.1 are for RAN4’s reference

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration for the active UL BWP. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214].

2. Actions:

To RAN4:

RAN1 kindly asks RAN4 to take the above response into account for the corresponding future works.

3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting 109e 	16 May – 27 May 2022		            e-Meeting

TSG RAN WG1 Meeting #110                          22 Aug. – 26 Aug. 2022                              Toulouse, FR