3GPP TSG-RAN WG4 Meeting #102-e	R4-2206152

Online Meeting, 21 February – 03 March 2022

3GPP TSG RAN WG1 Meeting #108-e	 R1-2202778

e-Meeting, February 21 – March 3, 2022

Title:	Reply LS on beam information of PUCCH SCell in PUCCH SCell activation procedure

Response to:	R1-2108704/R4-2115339

Release:	Rel-17

Work Item:	NR_RRM_enh2-Core

Source:	RAN WG1

To:	RAN WG2, RAN WG4

Cc:

Contact Person:

#### Name:	Frank Long

E-mail Address:	frank.longyi {@ }huawei.com

Attachments:

1. Overall Description:

RAN1 thanks RAN4 for the LS on beam information of PUCCH SCell in PUCCH SCell activation procedure. RAN1 has follow-up discussion on the UE capability and has achieved the following agreements,

Proposal 1-1a: For the new UE capability xx-1 for cross-PUCCH group CSI reporting,

- In column “Feature group”, it is “CSI reporting cross PUCCH group”

Proposal 1-2: For the new UE capability xx-1 for cross-PUCCH group CSI reporting, the “Type” column is per BC if the capability is introduced from Rel-16, otherwise per UE.

Proposal 1-1b-rev3: For the new UE capability xx-1 for cross-PUCCH group CSI reporting,

- In column “Components”, it is as follows,

  - Support reporting CSI of an active/being activated SCell belonging to secondary PUCCH group by PUSCH or PUCCH of active serving cells belonging to primary PUCCH group, for both during and after SCell activation procedure.

  - Support reporting CSI of an active/being activated SCell belonging to primary PUCCH group by PUSCH or PUCCH of active serving cells belonging to secondary PUCCH group, for both during and after SCell activation procedure.

  - Support for P-CSI and A-CSI for cross-PUCCH group CSI reporting

    - Indication for UE CSI computation time for A-CSI report = {same as no-cross-PUCCH-group, relaxed}

  - Additional indication for support/not of SP-CSI on PUCCH for cross-PUCCH group CSI reporting

  - Additional indication for support/not of SP-CSI on PUSCH for cross-PUCCH group CSI reporting

  - UE indicates one or multiple supported carrier type pairs(s), each carrier type pair is {carrier type in a PUCCH-group in which CSI measurement is performed, carrier type in the other PUCCH-group in which CSI report is performed}, where a carrier type is one of {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2}

- Add the following in Note column:

  - Component 3: if “relaxed” is reported, then indicate additional number of symbols required in addition to existing Z and Z’ for aperiodic CSI report for cross-PUCCH group CSI reporting, which is per SCS (the same SCS set definition as in S5.4 of TS 38.214) reported and has candidate values {val#1, val#2, val#3}.

  - Note: the candidate value {val#1, val#2, val#3} is with range from 14 to 56 symbols only, their exact values are FFS.

- Add in Note column: RAN1 didn’t discuss the potential conflicts with the definition of PUCCH group that was discussed in RAN2

- Note: The UE capability is introduced from Rel-16.

Proposal 1-3: For the new UE capability xx-1 for cross-PUCCH group CSI reporting, the column “Prerequisite feature groups” is that FG 2-35 and either FG 6-7 or FG 22-7.

Proposal 1-4: For the new UE capability xx-1 for cross-PUCCH group CSI reporting, it is Optional with capability signalling.

Proposal 1-5-rev: For the new UE capability xx-1 for cross-PUCCH group CSI reporting,

- In column “Need for the gNB to know if the feature is supported”, it is “Yes”.

- In column “Applicable to the capability ignaling exchange between Ues (Sidelink WI only)”, it is “N/A”.

- In column “Consequence if the feature is not supported by the UE”, it is “Cross-PUCCH group CSI reporting may not be supported”.

A table of UE capability that reflects all the agreements above can be found in Appendix.

2. Actions:

To: RAN2, RAN4

ACTION: RAN1 respectfully asks RAN2 and RAN4 to take the above RAN1 agreements into account in their future work.

3. Date of Next TSG-RAN1 Meetings:

3GPP RAN1#109-e				16 - 27, May 2022	                Online

3GPP RAN1#110				22 - 26, August 2022	        Toulouse, FR

Appendix

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Further RRM enhancement for NR and MR-DC | 22-13 | CSI reporting cross PUCCH group | - Support reporting CSI of an SCell belonging to secondary PUCCH group by PUSCH or PUCCH of active serving cells belonging to primary PUCCH group, for both during and after SCell activation procedure. - Support reporting CSI of an SCell belonging to primary PUCCH group by PUSCH or PUCCH of active serving cells belonging to secondary PUCCH group, for both during and after SCell activation procedure. - Support for P-CSI and A-CSI for cross-PUCCH group CSI reporting - Indication for UE CSI computation time for A-CSI report = {same as no-cross-PUCCH-group, relaxed} - Additional indication for support/not of SP-CSI on PUCCH for cross-PUCCH group CSI reporting - Additional indication for support/not of SP-CSI on PUSCH for cross-PUCCH group CSI reporting - UE indicates one or multiple supported carrier type pairs(s), each carrier type pair is {carrier type in a PUCCH-group in which CSI measurement is performed, carrier type in the other PUCCH-group in which CSI report is performed}, where a carrier type is one of {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} - Note: The UE capability is introduced from Rel-16. | FG 2-35 and either FG 6-7 or FG 22-7 | Yes | N/A | Cross-PUCCH group CSI report may not be supported | per BC if the capability is introduced from Rel-16, otherwise per UE. | [No] | [No] | N/A | Note: RAN1 didn’t discuss the potential conflicts with the definition of PUCCH group that was discussed in RAN2 Component 3: if “relaxed” is reported, then indicate additional number of symbols required in addition to existing Z and Z’ for aperiodic CSI report for cross-PUCCH group CSI reporting, which is per SCS (the same SCS set definition as in S5.4 of TS 38.214) reported and has candidate values {val#1, val#2, val#3}. Note: the candidate value {val#1, val#2, val#3} is with range from 14 to 56 symbols only, their exact values are FFS. | Optional with capability signaling |