3GPP TSG-RAN WG4 Meeting # 103-e	                                       R4-2207707

Electronic Meeting, 09 – 20 May, 2022

Agenda Item:	11.2.2.2

Source: 	China Telecom

Title: 	Considerations for wider CBW solutions on Signaling and configuration aspects

Document for:	Discussion

- Introduction

During the last meeting, the TP [1] on channel bandwidth configuration aspects and the issue about PRB grid misalignment between SIB1channel bandwidth and reconfigured UE dedicated channel bandwidth based on the 100 kHz channel raster [2] are discussed, this document gives further considerations on above two topics.

- Discussion

2.1 Cell-specific carrier bandwidth

In our understanding, the carrierBandwidth IE carried in SIB1 is the cell-specific transmission bandwidth configuration, which is used by all UEs in this cell to map it unambiguously to a regular UE channel bandwidth defined in TS 38.101. So, the carrier bandwidth in SIB1 is the cell-specific UE channel bandwidth rather than BS channel bandwidth, which is consistent with the statement in TS 38.101-2 that UE does not need to be aware of the BS channel bandwidth. Based on the analysis above, the following observations can be made:

Observation 1: the carrier bandwidth indicated by SIB1 is cell-specific UE channel bandwidth instead of BS channel bandwidth. It is ok to re-configure a UE-specific channel bandwidth wider than the one indicated in SIB1.

Observation 2: The carrierBandwidth in SIB1 must correspond to the maximum transmission bandwidth configuration NRB defined in TS 38.101 so that UE can map it unambiguously to a regular UE channel bandwidth.

2.2 UE-specific channel bandwidth

Based on the CR RP-182896 and CR RP-1902778, UE-specific channel bandwidth is only used for the purpose of channel bandwidth and location determination, and only the channel bandwidth defined in TS 38.101-1 and TS 38.101-2 will be indicated in downlinkChannelBW-PerSCS-List and uplinkChannelBW-PerSCS-List. The location and bandwidth of a BWP is always determined by offsetToCarrier contained in ServingCellConfigConmmon / ServingCellConfigConmmonSIB, instead of the one indicated in downlinkChannelBW-PerSCS-List and uplinkChannelBW-PerSCS-List. When UE-specific BWP are configured, UE-specific channel bandwidth is provided with dedicated signaling so that UE can operate in a regulatory compliant way. Based on the analysis above, the following observations can be made:

Observation 3: When UE-specific BWP is configured, UE-specific channel bandwidth provided by dedicated signaling is used by UE to set channel bandwidth in order to be able to operate in a regulatory compliant way.

Observation 4: The channel bandwidth value indicated in dedicated signaling must correspond to the maximum transmission bandwidth configuration NRB defined in TS 38.101 so that UE can map it unambiguously to a regular UE channel bandwidth.

- Conclusion

In this contribution, we discuss the approach of wider CBW and provide our considerations:

Observation 1: the carrier bandwidth indicated by SIB1 is cell-specific UE channel bandwidth instead of BS channel bandwidth. It is ok to re-configure a UE-specific channel bandwidth wider than the one indicated in SIB1.

Observation 2: The carrierBandwidth in SIB1 must correspond to the maximum transmission bandwidth configuration NRB defined in TS 38.101 so that UE can map it unambiguously to a regular UE channel bandwidth.

Observation 3: When UE-specific BWP is configured, UE-specific channel bandwidth provided by dedicated signaling is used by UE to set channel bandwidth in order to be able to operate in a regulatory compliant way.

Observation 4: The channel bandwidth value indicated in dedicated signaling must correspond to the maximum transmission bandwidth configuration NRB defined in TS 38.101 so that UE can map it unambiguously to a regular UE channel bandwidth.

- Reference

R4-2204622, “TP for 38.844: configuration for the case of larger channel bandwidths than licensed bandwidth”, Ericsson.

R4-2204514, “Considerations for wider CBW solutions”, China Telecom.