3GPP TSG-RAN4 Meeting #103-e	R4-2207762

# Online, 9 – 20 May, 2022

Agenda item:	9.7.1

Source:	Apple

Title:	On remaining issues for R17 FR1 HST

Document for:	Discussion

# Introduction

In RAN4#102e RAN4 requirements for R17 FR1 HST was widely discussed. Corresponding agreement and open items are captured in the approved WF [1]. In this contribution, we continue discussing the open issues listed in [1].

# Discussion

Issue 3-1-1: how to specify the L1-SINR requirements for high speed scenario

Agreements in 1st round

- For L1-SINR for FR1 HST

  - The existing measurement delay requirements for L1-SINR is reused

  - The existing accuracy requirements for L1-SINR is reused

    - Whether and how to define applicability of side condition for the L1-SINR accuracy requirements can be further discussed and concluded in the perf. phase

      - Option 1: the upper bound of the side condition for L1-SINR accuracy is 5dB

      - Option 2: no upper bound of side condition for L1-SINR accuracy

Option 1 is supported. First, 5dB cap comes from high Doppler shift which has been widely discussed since LTE HST speed train WI. Second, even though it is up to NW whether to configure L1-SINR, we don’t think there is much room for NW Tx beam determination based on L1-SINR on top of L1-RSRP in high-speed train scenario, due to the specific UE trajectory and linear deployment of RRH.

Proposal 1: Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB.

# Conclusion

In this contribution, we further discuss the open issues according to [1]. After discussion, the following conclusions are provided:

Proposal 1: Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB.

# References

- R4-2206845, WF on RRM for FR1 HST, CMCC

- R4-2202590, WF on RRM for FR1 HST, CMCC

- R4-2120285, WF on RRM for FR1 HST, CMCC