3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207797

# Electronic Meeting, May 09 - May 20, 2022

Agenda item:	9.7.3

Source:	Apple Inc.

Title:	Simulation results for PDSCH CA Requirements in HST

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#102-e UE demodulation for FR1 HST was discussed and way forward [1] was agreed.

Issue 1-1: UE capability for HST-SFN CA - Agreements in GTW: - per band combination granularity Issue 1-2: Release independent - Agreements in GTW: - From demodulation perspective, Rel-17 FR1 HST-SFN CA demodulation is release independent from Rel-16. Issue 1-5: FDD BW - Agreements in GTW: - For FDD 15KHz SCS, specify PDSCH requirements on single carrier of BW of {35, 45} MHz for HST CA. - Companies are encouraged to bring simulation results in next meeting.

In this contribution we present simulation results for PDSCH in CA for additional CBW in FDD.

# 2. Simulation Results

## Simulation results for HST-SFN JT

For the simulation parameters agreed in [1], we present for different CBW for FDD and TDD duplex modes.

Table 1: Simulation Results for FDD 2x2 for HST-SFN JT

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 1-1 | 5 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 |  |  |
| 1-2 | 10 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 |  |  |
| 1-3 | 15 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 |  |  |
| 1-4 | 20 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 |  |  |
| 1-5 | 25 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 |  |  |
| 1-6 | 30 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 |  |  |
| 1-7 | 35 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 |  |  |
| 1-8 | 40 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 |  |  |
| 1-9 | 45 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 |  |  |
| 1-10 | 50 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 |  |  |

Table 3: Simulation Results for FDD 2x4 for HST-SFN JT

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 1-1 | 5 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 |  |  |
| 1-2 | 10 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 |  |  |
| 1-3 | 15 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 |  |  |
| 1-4 | 20 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 |  |  |
| 1-5 | 25 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 |  |  |
| 1-6 | 30 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 |  |  |
| 1-7 | 35 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 |  |  |
| 1-8 | 40 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 |  |  |
| 1-9 | 45 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 |  |  |
| 1-10 | 50 / 15 | 16QAM, 0.48 | HST-SFN | 2x4 |  |  |

## Simulation results for HST-DPS

For the simulation parameters agreed in [1], we present for different CBW for FDD and TDD duplex modes.

Table 5: Simulation Results for FDD 2x2 for HST-DPS

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 1-1 | 5 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-2 | 10 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-3 | 15 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-4 | 20 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-5 | 25 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-6 | 30 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-7 | 35 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-8 | 40 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-9 | 45 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |
| 1-10 | 50 / 15 | 64QAM, 0.43 | HST-DPS | 2x2 | 11.0 | 13.5 |

Table 7: Simulation Results for FDD 2x4 for HST-DPS

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 1-1 | 5 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 7.8 | 10.3 |
| 1-2 | 10 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.3 | 10.8 |
| 1-3 | 15 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.1 | 10.6 |
| 1-4 | 20 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 7.7 | 10.2 |
| 1-5 | 25 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.1 | 10.6 |
| 1-6 | 30 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.4 | 10.9 |
| 1-7 | 35 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.3 | 10.8 |
| 1-8 | 40 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.2 | 10.7 |
| 1-9 | 45 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.3 | 10.8 |
| 1-10 | 50 / 15 | 64QAM, 0.43 | HST-DPS | 2x4 | 8.4 | 10.9 |

# 3. Conclusion

In this paper, we provide simulation results for PDSCH requirements in CA for HST scenarios.

# Reference

- R4-2207194, “Way Forward on UE demodulation for FR1 HST”, CMCC.