3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207811

# Electronic Meeting, May 09 - May 20, 2022

Agenda item:	9.19.5.2.2

Source:	Apple Inc.

Title:	On PDCCH PBCH Demod Requirements for Reduced Capability Devices in NR

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#102-e RedCap UE demodulation and CSI reporting requirements was discussed and way forward [1] was agreed. In this contribution we present our views on open issues and initial simulation results for PDCCH and PBCH demodulation requirements for RedCap UE.

# 2. Discussion

PDCCH Demod Requirements

In [1] the simulation assumptions for PDCCH demod requirements for RedCap UE were agreed. The scope of PDCCH demod requirements is still open.

Aggregation level(s)

- Option 1: Specify PDCCH demodulation requirements with AL4 and AL8.

- Option 2: Specify PDCCH demodulation requirements with AL4 or AL8.

Firstly, we don’t see the need to introduce requirements for both AL 4 and AL8 for each set – FR1 FDD, FR1 TDD and FR2. PDCCH demodulation is tested along with PDSCH demod requirements. For PDSCH demod we typically use AL 8 for requirements to ensure error free PDCCH transmission. Since Al 8 is covered in PDSCh demod requirements, we can use AL 4 for PDCCH demod requirements for RedCap UE.

Proposal #1: Use AL 4 for PDCCH requirements for RedCap UE.

Initial Simulation Results

For the agreed simulation assumptions [1], we provide initial simulation results for PDCCH demod requirements. Since FR2 requirements for RedCap UE will be reused from existing Rel-15 requirements, we only provide results for FR1.

FR1 FDD

Table 1: Simulation Results for FR1 FDD

| RB size | Duration | AL | Propagation condition | Antenna configuration | SNR @ 1% Pm-dsg(dB) |
| --- | --- | --- | --- | --- | --- |
| 48 | 2 | 4 | TDLA30-10 | 1x1 | 13.0 |
| 48 | 1 | 8 | TDLA30-10 | 2x1 low | 2.7 |
| 48 | 2 | 4 | TDLA30-10 | 1x2 low | 4.0 |
| 48 | 1 | 8 | TDLA30-10 | 2x2 low | -2.4 |

FR1 TDD

Table 2: Simulation Results for FR1 TDD

| RB size | Duration | AL | Propagation condition | Antenna configuration | SNR @ 1% Pm-dsg(dB) |
| --- | --- | --- | --- | --- | --- |
| 48 | 2 | 4 | TDLC300-100 | 1x1 | 6.0 |
| 48 | 2 | 8 | TDLC300-100 | 2x1 low | 0.4 |
| 48 | 2 | 4 | TDLC300-100 | 1x2 low | 1.0 |
| 48 | 2 | 8 | TDLC300-100 | 2x2 low | -3.0 |

PBCH Demod Requirements

The scope of PBCH demod requirements and simulation assumptions was agreed in [1].

Whether to define 1Rx PBCH demodulation requirements in the case SS/PBCH block index is known - Not to define 1Rx PBCH demodulation requirements in the case SS/PBCH block index is known - Define only 2Rx requirements for FR2

For the agreed simulation assumptions [1], we provide initial simulation results for PBCH demod requirements. Since FR2 requirements for RedCap UE will be reused from existing Rel-15 requirements, we only provide results for FR1.

Table 3: Simulation Results for PBCH with unknown SSB index

| BW / SSB SCS | Propagation condition | Antenna configuration | SSB Pattern | SNR @ 1% Pm-bch(dB) |
| --- | --- | --- | --- | --- |
| 10MHz / 15kHz | TDLC300-100 | 1x1 | Case A | -5.2 |
| 20MHz / 30kHz | TDLA30-10 | 1x1 | Case B | -2.5 |
|  |  |  | Case C | -2.8 |
| 10MHz / 15kHz | TDLC300-100 | 1x2 low | Case A | -8.2 |
| 20MHz / 30kHz | TDLA30-10 | 1x2 low | Case B | -6.9 |
|  |  |  | Case C | -7.0 |

# 3. Conclusion

In this paper, we provide our views on open issues and initial simulation results for PDCCH and PBCH demodulation requirements for RedCap UE. Our observations and proposals are captured below:

Proposal #1: Use AL 4 for PDCCH requirements for RedCap UE.

# Reference

- R4-2207206, “WF on RedCap UE demodulation and CQI reporting requirements”, Ericsson.