3GPP TSG RAN WG4 Meeting #103-e	R4-2207856

Online, 9th – 20th May 2022

Agenda item:	8.17.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-31: EN-DC_1-11_n3-n77-n79

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 3B NR of DC_1-11_n3-n77-n79 was approved in RAN#94 [1]. This TP is to capture the basic aspects for the EN-DC.

2	Text Proposal

[Unchanged Parts Skipped]

7.X	DC_1-11_n3-n77-n79

7.X.1	Operating bands for DC

Table 7.x.1-1: EN-DC band combination (five bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-11_n3-n77-n79 | CA_1-11 | CA_n3-n77-n79 |

7.X.2	Inter-band DC Configurations

Table 7.x.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-11A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_1A-11A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_11A_n3A DC_11A_n77A DC_11A_n79A |

7.X.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR3DL/1UL are already covered by the fallback configurations, DC_1-11_n3-n77, DC_1-11_n3-n79, DC_1-11_n77-n79, DC_1_n3_n77-n79 and DC_11_n3-n77-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.X.4	∆TIB and ∆RIB values

For DC_1-11_n3-n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n3-n77-n79 | 1 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n77 | 0.8 |
|  | n79 | 0.8 |

Table 8.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n3-n77-n79 | 1 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n77 | 0.5 |
|  | n79 | 0.5 |

7.X.5	MSD

As mentioned in 7.X.3, there is no need to specify additional MSD requirement for this UL DC configuration.

[Unchanged Parts Skipped]

7.  Reference

[1]  RP-212885  Revised WID on Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL), ZTE